3GPP TSG-RAN WG4 Meeting #108	R4- 2313488

Toulouse, France, 21 – 25 August 2023

Source: 	Huawei, HiSilicon

Title:	A-MPR for NS_50 with 35MHz channel BW

Agenda Item:	7.26.2.1

Document for:	Agreement

# 1	Introduction

It has been requested to add support of 35MHz channel BW for band n39 and n98. A draft CR was provided in [1], where the major unresolved issue is the A-MPR requirements for NS_50 for both PC3 and PC2. In the sequel, we share our simulation results and proposals for the A-MPR.

# 2	Simulation Results

The simulation assumptions are as follows:

- Power class 2

- IQ image = -28 dB and LO leakage = -28 dBc

- CIM3 = -60 dB

- PA calibration point is 20 MHz BW, 15 kHz SCS, QPSK, DFT-s-OFDM, 100 RB#0 with 1 dB MPR

- No IBE or EVM is evaluated.

## 2.1 Existing spurious emission requirements for NS_50

##### 6.5.3.3.16	Requirement for network signalling value "NS_50"

When "NS_50" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.16-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.16-1: Additional requirements for "NS_50"

| Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency range | 1805 | - | 1855 | -40 | 1 | 1 |
| Frequency range | 1855 | - | 1880 | -15.5 | 5 | 1, 2, 3 |
| NOTE 1: This requirement is applicable for carriers with aggregated channel bandwidths confined in 1885-1920 MHz for ≤ 30MHz channel BWs and confined in 1880-1920 MHz for 40MHz channel BW. NOTE 2: The requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 3: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |

The additional emission requirements as signalled by NS_50 [2] are duplicated below. It can be seen that an extra guard band of 5MHz is specified for BW≤ 30MHz.

Proposal 1: Update the existing spurious emission requirements for NS_50 in support of 35MHz channel BW, and specify 5MHz guard band similar to the requirements for BW≤ 30MHz.

## 2.2 Simulation results for PC3 and PC2

The simulation results are presented below by considering the MPR and showing the A-MPR only when it exceeds the MPR, i.e. Figures in Table 1 and 2 show the A-MPR if A-MPR>MPR for NS_50.

Table 1: PC3 A-MPR if A-MPR>MPR for BW=35MHz

| Modulation | DFT-s-OFDM | CP-OFDM |
| --- | --- | --- |
| QPSK |  |  |
| 16QAM |  |  |
| 64QAM |  |  |
| 256QAM |  |  |

Table 2: PC2 A-MPR if A-MPR>MPR for BW=35MHz

| Modulation | DFT-s-OFDM | CP-OFDM |
| --- | --- | --- |
| QPSK |  |  |
| 16QAM |  |  |
| 64QAM |  |  |
| 256QAM |  |  |

# 3	A-MPR requirements

The A-MPR requirements for 35MHz channel BW as proposed in [1] are duplicated below.

Table 6.2.3.19-1: A-MPR regions for NS_50 (Power Class 3)

| Channel Bandwidth (MHz) | RBstart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- |
| 25 MHz | ≤ LCRB*12*SCS - 5 | > 5 | A7 |
|  | ≤ 6.48 | ≤ 1.44 | A8 |
|  |  | ≤ 3.6 | A9 |
| 30 MHz | ≤ LCRB*12*SCS - 5 | > 5 | A7 |
|  | ≤ 8.64 | ≤ 1.44 | A8 |
|  |  | ≤ 3.6 | A9 |
| 35MHz | [≤ 1.62] | [> 0] | A1 |
|  | [> 1.62, ≤ 7.92] | [≤ 9.0] | A3 |
|  | [> 1.62, ≤ 13.68] | [> 9.0] | A2 |
|  | [> 13.68, ≤ 30.6] | [> max (29.7 –RBstart *12*SCS, 0)] | A6 |
|  | [> 30.6] | [> 0] | A5 |
| 40 MHz | ≤ 4.32 | > 0 | A1 |
|  | > 4.32, ≤ 10.44 | ≤ 10.8 | A3 |
|  | > 4.32, ≤ 18 | > 10.8 | A2 |
|  | > 18, ≤ 31.68 | > max (31.68 – RBstart*12*SCS, 0) | A6 |
|  | > 31.68 | > 0 | A5 |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.19-2. |  |  |  |

Table 6.2.3.19-3: A-MPR regions for NS_50 (Power Class 2)

| Channel Bandwidth (MHz) | RBstart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- |
| 35MHz | [≤ 1.62] | [> 0] | A1 |
|  | [> 1.62, ≤ 7.92] | [≤ 9.0] | A3 |
|  | [> 1.62, ≤ 13.68] | [> 9.0] | A8 |
|  | [> 13.68, ≤ 30.6] | [> max (29.7 –RBstart *12*SCS, 0)] | A4 |
|  | [> 30.6] | [> 0] | A9 |
| 40 MHz | ≤ 4.32 | > 0 | A1 |
|  | > 4.32 | > RBstart*12*SCS + 11.88 | A8 |
|  | > 4.32, ≤ 12.96 | ≤ 10.8 | A3 |
|  | > 4.32, ≤ 18 | > 10.8, <= RBstart*12*SCS + 11.88 | A7 |
|  | > 18, ≤ 31.68 | > max (31.68 – RBstart*12*SCS, 0) | A4 |
|  | > 31.68 | > 0 | A9 |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.19-4. |  |  |  |

Based on the simulation results presented in Section 2, the proposed A-MPR requirements in [1] are acceptable, except that the region with A-MPR value of A3 needs to be slightly enlarged.

Proposal 2: Based on our simulation results, it’s proposed to slightly enlarge the A-MPR region for A3 for both PC3 and PC2. The modification based on the draft CR in [1] is shown as below.

| Channel Bandwidth (MHz) | RBstart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- |
| 35MHz | [≤ 1.62] | [> 0] | A1 |
|  | [> 1.62, ≤ 9.36] | [≤ 9.0] | A3 |

# 4 Conclusions

The PC3 and PC2 A-MPR for NS_50 with 35MHz channel BW have been discussed. Based on our simulation results, the following proposals are presented.

Proposal 1: Update the existing spurious emission requirements for NS_50 in support of 35MHz channel BW, and specify 5MHz guard band similar to the requirements for BW≤ 30MHz.

Proposal 2: Based on our simulation results, it’s proposed to slightly enlarge the A-MPR region for A3 for both PC3 and PC2. The modification based on the draft CR in [1] is highlighted as below.

| Channel Bandwidth (MHz) | RBstart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- |
| 35MHz | [≤ 1.62] | [> 0] | A1 |
|  | [> 1.62, ≤ 9.36] | [≤ 9.0] | A3 |

# References

[1] R4-2310248, Draft CR for 38.101-1: Addition of 35MHz for n39 and n98, CMCC, RAN4#107

[2] TS 38.101-1 v18.2.0