3GPP TSG-RAN WG4 Meeting #	108                                         R4-23xxxxx

Toulouse, France, August 21 – August 25, 2023

Agenda item:		8.33.3

Source:	Dell Technologies

Title:	 Further Discussion on mobile IAB RF Requirements

Document for:	Discussion

# Introduction

## RF Requirements

## 2.1 IAB-MT RF requirements

Agreement:

- Based on the outcome of the coexistence study:

  - Further study whether existing wide area IAB-MT RF or local area IAB-MT RF requirements can be reused for mobile IAB, or a new set of requirements need to be specified.

  - Limit output power in UL or introduce output power control dynamics following UE.

## IAB-MT antenna modelling

IAB-MT antenna modelling:

Agreement:

- Consider the 3 panel with 120-degree shift IAB-MT antenna model in coexisting simulation.

IAB-DU antenna modelling:

Agreement:

- Use same assumption for mobile IAB-DU and IAB-MT as starting point for initial co-existence simulations.

- Other options not precluded for mobile IAB-DU antenna modelling and deployment scenarios.

In RAN4-#107 meeting, the WF on mobile IAB RF requirements and co-existence, R4-2309863 [1], is approved. In this WF, the agreement for mobile IAB-MT RF requirements is described as followings:

In this contribution, the RF requirements for mobile IAB-MT are further discussed with our views based on above WF.

- Discussions

## 2.1 IAB-MT RF requirements

For static IAB-MT output power, the limited output power and dynamic range can be specified based on the IAB -MT classes, such as based on Table 6.2.1-2 in 38.174 for IAB-MT type 1-H rated output power limits for IAB-MT.

For mobile IAB-MT output power and power control dynamics, since we have to consider the mobility of IAB-MT, the static IAB-MT requirements are no more appliable.  Considering mobile IAB-MT has similar functionality as UE, the requirement of power control for mobile IAB-MT can be treated as the same manner as UE and the similar approach for UE output power control and dynamics can be therefore specified for mobile IAB-MT.

Proposal 1: It is suggested output power control dynamics for mobile IAB-MT may be appliable with the same manner as UE.

2.2 IAB-DU antenna modelling

Based on the previous meeting agreements, it was defined that the antenna installation for mobile IAB-MT is outside the vehicle (e.g rooftop) and for mobile IAB-DU is inside the vehicle.

For mobile IAB-MT antenna modelling, in last meeting, it was agreed to consider the 3 panel with 120-degree shift IAB-MT antenna model in coexisting simulation.

For mobile IAB-DU antenna modelling, since the installation of IAB-DU antenna is inside of vehicle there may be different deployment scenarios. Some of factors such as antenna configuration and placement in vehicle will be quite different from mobile IAB-MT.   We think it may need to modify the IAB-DU antenna modelling from IAB-MT though it is agreed that using the same assumption for mobile IAB-DU and IAB-MT antenna modelling as starting point.

Proposal 2: It is fine to reuse IAB-MT antenna modelling for mobile IAB as starting point and suggested to further study and specify the antenna modelling for mobile IAB-DU.

- Conclusion

In this contribution, we have further discussed RF requirement issues for mobile IAB and make the following proposals:

Proposal 1: It is suggested output power control dynamics for mobile IAB-MT may be appliable with the same manner as UE.

Proposal 2: It is fine to reuse IAB-MT antenna modelling for mobile IAB as starting point and suggested to further study and specify the antenna modelling for mobile IAB-DU.

- References

[1] R4-2309863 WF on mIAB RF requirement and coexistence

[2] R4-2310452 Summary for [107][314] NR_mobile_IAB_RF

[3] R4-2308619 Considerations on mobile IAB RF requirements

[4] RP-222671 Mobile IAB (Integrated Access and Backhaul) for NR