3GPP TSG-RAN WG4 Meeting #108	R4-2313499

Toulouse, France, August 21 – August 25, 2023

Agenda item:	5.1.3

Source:            	Google Inc.

Title:                	Discussion on IE supportedBandwidthCombinationSetIntraENDC

Document for:	Approval

# Introduction

In RAN4#98-e, BCS reporting for inter-band EN-DC band combination with one additional intra-band EN-DC component has been discussed. The LS to RAN2, some RAN4/RAN2 CRs are agreed to clarify the definition of IE supportedBandwidthCombinationSetIntraENDC. In Rel-17, it is observed in TS 38.101-3 that DC_3A-41A_n3A-n41A is introduced in sub-clause 5.5B.4.3 and DC_1A-3A-41A_n3A-n41A is introduced in sub-clause 5.5B.4.4, which are inter-band EN-DC band combination with two additional intra-band EN-DC components. In our understanding, the current definition of IE supportedBandwidthCombinationSetIntraENDC is only applicable to inter-band EN-DC band combination with only one intra-band EN-DC component. Hence, it would be ambiguous about how to indicate supportedBandwidthCombinationSetIntraENDC for inter-band EN-DC band combination such as DC_3A-41A_n3A-n41A and DC_1A-3A-41A_n3A-n41A with two additional intra-band EN-DC components, e.g., DC_3A_n3A and DC_41A_n41A. In this paper, we would like to share our view in the following.

| EN-DC configuration | Uplink EN-DC configuration (NOTE1) |
| --- | --- |
| DC_3A-41A_n3A-n41A | DC_3A_n3A4 DC_3A_n41A DC_41A_n3A |
| DC_1A-3A-41A_n3A-n41A | DC_1A_n3A DC_1A_n41A DC_3A_n3A4 DC_3A_n41A DC_41A_n3A |

# Discussion

In current RRC signaling, if the UE would like to support an inter-band EN-DC band combination with additional intra-band EN-DC component, the UE can inform the network about the supported channel bandwidth for each LTE and NR band by reporting E-UTRA bandwidth combination sets (LTE CA BCS), NR bandwidth combination sets (NR CA BCS), and EN-DC bandwidth combination sets (EN-DC BCS). According to TS38.101-3 sub-clause 4.2, the UE should support a product set or a common set among these three bandwidth combination sets which it signals to support.

[38.101-3][Sub-clause 4.2][V17.10.0]

A terminal which supports an EN-DC or NE-DC configuration shall support:

If any subsets of the EN-DC or NE-DC configuration do not specify its own bandwidth combination sets in 5.3B, then the terminal shall support the same E-UTRA bandwidth combination sets it signals the support for in E-UTRA CA configuration part of E-UTRA – NR DC and shall support the same NR bandwidth combination sets it signals the support for in NR CA configuration part of E-UTRA – NR DC.

Else if one of the subsets of the EN-DC or NE-DC configuration specify its own bandwidth combination sets in 5.3B, then the terminal shall support a product set of channel bandwidth for each band specified by E-UTRA bandwidth combination sets, NR bandwidth combination sets, and EN-DC or NE-DC bandwidth combination sets it singnals the support. A terminal which supports an inter-band EN-DC or NE-DC configuration with a certain UL configuration shall support the all lower order DL configurations of the lower order EN-DC or NE-DC combinations, which have this certain UL configuration and the fallbacks of this UL configuration.

For example, if the UE would like to indicate EN-DC BCS in inter-band EN-DC band combination with additional intra-band EN-DC component, the definition of IE supportedBandwidthCombinationSetIntraENDC in TS38.306 should be followed by the UE for EN-DC BCS reporting. In the IE definition, it is observed that the EN-DC BCS reporting criteria can be divided into two categories. The first category is that the UE should be mandatory to report EN-DC BCS if both UL and DL configuration for the intra-band EN-DC component is supported within inter-band EN-DC band combination, e.g., DL DC_2A-7A_n7A with UL DC_7A_n7A. The other category is that the UE should be optional to report EN-DC BCS if both UL and DL configuration for the intra-band EN-DC component is not supported within inter-band EN-DC band combination, e.g., DL DC_2A-7A_n7A with UL DC_2A_n7A, where the network would assume the UE supports BCS0 if the IE is not indicated.

[38.306][V17.5.0]

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| supportedBandwidthCombinationSetIntraENDC Defines the supported bandwidth combination set for a band combination that allows configuration of at least one EUTRA serving cell and at least one NR serving cell in the same band, as defined in the TS 38.101-3 [4], table 5.3B.1.2-1 and table 5.3B.1.3-1. - For intra-band (NG)EN-DC with additional inter-band CA component(s) of LTE and/or NR, the field defines the bandwidth combinations for the intra-band (NG)EN-DC component. - For intra-band NE-DC with additional inter-band CA component(s) of LTE and/or NR, the field defines the bandwidth combinations for the intra-band NE-DC component. Field encoded as a bit map, where bit N is set to "1" if UE support Bandwidth Combination Set N for this band combination as defined in the TS 38.101-3 [4]. The leading / leftmost bit (bit 0) corresponds to the Bandwidth Combination Set 0, the next bit corresponds to the Bandwidth Combination Set 1 and so on. - It is mandatory if the band combination is an intra-band (NG)EN-DC/NE-DC combination supporting both UL and DL intra-band (NG)EN-DC/NE-DC parts with additional inter-band NR/LTE CA component. - It is optional if the band combination is an intra-band (NG)EN-DC/NE- DC combination without supporting UL in both the bands of the intra- band (NG)EN-DC/NE-DC UL part. If not included, the network assumes the UE supports BCS0 as defined in TS 38.101-3 [4], table 5.3B.1.2-1 and table 5.3B.1.3-1 for the intra-band (NG)EN-DC/NE-DC. | BC | CY | N/A | N/A |

However, in the current definition for EN-DC BCS reporting, it works well only for inter-band EN-DC band combination with one additional intra-band EN-DC component. The rule for EN-DC BCS reporting becomes ambiguous if there are two additional intra-band EN-DC components within inter-band EN-DC band combination. If the UE decides to report EN-DC BCS for these kind of band combinations, the network cannot differentiate which one of intra-band EN-DC component is associated with the reported EN-DC BCS.

Take DC_1A-3A-41A_n3A-n41A as an example, which is the inter-band EN-DC band combination with two additional intra-band EN-DC components, e.g., DC_3A_n3A and DC_41A_n41A. According to BCS definition in RAN4 specification, the UE can indicate “BCS0” for LTE CA component CA_1A-3A-41A and indicate “BCS0/BCS1/BCS2” for NR CA component CA_n3A-n41A. For EN-DC BCS, the UE can indicate “BCS0/BCS1” for intra-band EN-DC components DC_3A_n3A and indicate “BCS0/BCS1/BCS2” for intra-band EN-DC component DC_41A_n41A. However, there is only one BCS IE can be used to indicate for these two intra-band EN-DC components. Depending on different implementations, the network may have different interpretations for this IE. In one implementation, if the UE decides not to reporting BCS for EN-DC, it would be possible for the network to assume the UE supports BCS0 for DC_3A_n3A and BCS0 for DC_41A_n41A. However, in other implementations, if the UE decides to report BCS1 in EN-DC BCS, the network cannot know whether BCS1 is associated with DC_3A_n3A or DC_41A_n41A.

[36.101][V17.10.0]

| E-UTRA CA Configuration | Uplink CA configurations (NOTE 5) | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-3A-41A9 | CA_1A-3A | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 41 |  |  | Yes | Yes | Yes | Yes |  |  |

[38.101-1][V17.10.0]

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n41A | n418 CA_n3A-n41A8 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30 | 1 |
|  |  | n41 | 10, 15, 20, 40, 50, 60 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |

[38.101-3][V17.10.0]

| Downlink EN-DC configuration | Uplink EN-DC configurations | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Channel bandwidths for E-UTRA carrier (MHz) | Channel bandwidths for NR carrier (MHz) | Channel bandwidths for E-UTRA carrier (MHz) |  |  |
| DC_3A_n3A | DC_3A_n3A |  | 5, 10, 15, 20, 25, 30 | 5, 10, 15, 20 | 50 | 0 |
|  |  |  | 5, 10, 15, 20, 25, 30 | 5, 10, 15, 20 | 50 | 1 |
|  |  | 5, 10, 15, 20 | 5, 10, 15, 20, 25, 30 |  |  |  |
| DC_41A_n41A | DC_41A_n41A | 20 | 40, 60, 80,100 |  | 120 | 0 |
|  |  |  | 40, 60, 80,100 | 20 |  |  |
|  |  | 20 | 40, 50, 60, 80,100 |  | 120 | 1 |
|  |  |  | 40, 50, 60, 80,100 | 20 |  |  |
|  |  | 20 | 10, 20, 30, 40, 50, 60, 80,100 |  | 120 | 2 |
|  |  |  | 10, 20, 30, 40, 50, 60, 80,100 | 20 |  |  |
|  |  | 10 | 20, 30, 40, 50, 60, 80,100 |  |  |  |
|  |  |  | 20, 30, 40, 50, 60, 80,100 | 10 |  |  |

In the following table, we summarize the possible DL/UL configurations and how the IE would be indicated by using the example band combination DC_1A-3A-41A_n3A-n41A with the assumption of reporting BCS0 for LTE CA, BCS0 for NR CA, BCS0 for intra-band EN-DC DC_3A_n3A and BCS2 for intra-band DC_41A_n41A.

In Case#1/Case#2/Case#4/Case#5, the UL configurations meet the optional reporting condition for EN-DC BCS reporting rule, which means that the UE does not need to report BCS0 for EN-DC and the network would assume the UE support BCS0. In one implementation, if the UE decides not to report BCS for EN-DC, the network may interpret BCS0 for both intra-band EN-DC DC_3A_n3A and DC_41A_n41A, where the network may use BCS0 for DC_41A_n41A as the reference BW information to configure the UE instead of using BCS2. In another implementation, if the UE decides to report BCS2 in EN-DC BCS, the network may interpret BCS2 for both intra-band EN-DC DC_3A_n3A and DC_41A_n41A, where the network cannot acquire BCS2 for DC_3A_n3A due to no BCS2 definition for DC_3A_n3A in the specification. Hence, for both implementations, the network cannot know the exact supported BCS for these two additional intra-band EN-DC components by only one IE supportedBandwidthCombinationSetIntraENDC, where the UE can actually support EN-DC BCS0 for DC_3A_n3A and EN-DC BCS2 for DC_41A_n41A.

In Case#3, the UL configuration meets the mandatory reporting condition for EN-DC BCS reporting rule, and the UE should indicate EN-DC BCS to the network. In one implementation, if the UE decides to report BCS0 in EN-DC BCS, the network may interpret BCS0 for both intra-band EN-DC DC_3A_n3A and DC_41A_n41A, where the network may use BCS0 for DC_41A_n41A as the reference BW information to configure the UE instead of using BCS2. In another implementation, if the UE decides to report BCS2 in EN-DC BCS, the network may interpret BCS2 for both intra-band EN-DC DC_3A_n3A and DC_41A_n41A, where the network cannot acquire BCS2 for DC_3A_n3A due to no BCS2 definition for DC_3A_n3A. Hence, for both implementations, the network still cannot know the exact supported BCS for these two additional intra-band EN-DC components by only one IE supportedBandwidthCombinationSetIntraENDC, where the UE can actually support EN-DC BCS0 for DC_3A_n3A and EN-DC BCS2 for DC_41A_n41A.

Moreover, from n41 BW configuration perspective, n41 BW configuration are {40,60,80,100} (MHz) in DC_41A_n41A_BCS0, {10,20,30,40,50,60,80,100} (MHz) in DC_41A_n41A_BCS2, and {10,15,20,40,50,60,80,90, 100} (MHz) in CA_n3A-n41_BCS0. In one implementation for EN-DC BCS reporting, if the network interprets DC_41A_n41A_BCS0 as the reference BW information, the UE should support n41 BW configuration {40, 60, 80,100} (MHz) due to the product set between DC_41A_n41A_BCS0 and CA_n3A-n41_BCS0. In another implementation for EN-DC BCS reporting, if the network interprets DC_41A_n41A_BCS2 as the reference BW information, the UE should support n41 BW configuration {10, 20, 40, 50, 60, 80, 100} (MHz) due to the product set between DC_41A_n41A_BCS2 and CA_n3A-n41_BCS0. Therefore, if the network cannot exactly understand that whether DC_41A_n41A_BCS2 or  DC_41A_n41A_BCS0 is supported by the UE, the network may use DC_41A_n41A_BCS0 to configure n41 BW={40, 60, 80,100} instead of using DC_41A_n41A_BCS0 to configure n41 BW={10,20,40,50,60,80,100}, where n41 BW={10, 20, 50} cannot be configured if the network interprets the UE supports DC_41A_n41A_BCS0.

Furthermore, from n3 BW configuration perspective, n3 BW configuration are all {5, 10, 15, 20, 25, 30} in DC_3A_n3A_BCS0, DC_3A_n3A_BCS1 and CA_n3A-n41_BCS0. However, although the n3 BW configuration is the same, the network still cannot configure n3 BW configuration with LTE B3 carrier frequency lower than NR n3 carrier frequency if the network interprets the UE supports DC_3A_n3A_BCS0 instead of DC_3A_n3A_BCS1.

| Example Band Combination: DC_1A-3A-41A_n3A-n41A |  |  |  |
| --- | --- | --- | --- |
| BCS categories |  | Supported BCS |  |
| LTE CA part BCS |  | CA_1A-3A-41A_BCS0 |  |
| NR CA part BCS |  | CA_n3A-n41A_BCS0 |  |
| Intra-band EN-DC part BCS |  | DC_3A_n3A_BCS0 |  |
| Intra-band EN-DC part BCS |  | DC_41A_n41A_BCS2 |  |
| Case | DL Configuration | UL Configuration | supportedBandwidthCombinationSetIntraENDC |
| Case#1 | DC_1A-3A-41A_n3A-n41A | DC_1A_n3A | Not report BCS0 for DC_3A_n3A? or Report BCS2 for DC_41A_n41A? |
| Case#2 | DC_1A-3A-41A_n3A-n41A | DC_1A_n41A | Not report BCS0 for DC_3A_n3A? or Report BCS2 for DC_41A_n41A? |
| Case#3 | DC_1A-3A-41A_n3A-n41A | DC_3A_n3A | Report BCS0 for DC_3A_n3A? or Report BCS2 for DC_41A_n41A? |
| Case#4 | DC_1A-3A-41A_n3A-n41A | DC_3A_n41A | Not report BCS0 for DC_3A_n3A? or Report BCS2 for DC_41A_n41A? |
| Case#5 | DC_1A-3A-41A_n3A-n41A | DC_41A_n3A | Not report BCS0 for DC_3A_n3A? or Report BCS2 for DC_41A_n41A? |

Hence, in order to solve the ambiguous issue by indicating only one IE for intra-band EN-DC BCS reporting in inter-band EN-DC band combination with two additional intra-band EN-DC components, we propose to introduce a new capability signalling to indicate intra-band EN-DC BCS for the second intra-band EN-DC component.

Proposal 1: Introduce a new UE capability signaling to indicate intra-band EN-DC BCS for the second intra-band EN-DC component in inter-band EN-DC band combination with two additional intra-band EN-DC components.

Proposal 2: Send LS in the Annex to RAN2.

# Conclusion

The proposals in this contribution are summarized in the following.

Proposal 1: Introduce a new UE capability signaling to indicate intra-band EN-DC BCS for the second intra-band EN-DC component in inter-band EN-DC band combination with two additional intra-band EN-DC components.

Proposal 2: Send LS in the Annex to RAN2.

# Reference

- R4-2102149, “Reply LS to RP-202935 on BCS reporting and support for intra-band EN-DC band combinations”, RAN4#98-e, T-Mobile USA

- R4-2103401, “Second Reply LS to RP-202935 = R4-2100025 on BCS reporting and support for intra-band EN-DC band combinations”, RAN4#98-e, T-Mobile USA

- R4-2103402, “CR for 38.101-3: clarification of intra-band EN-DC BCS applicability”, RAN4#98-e, T-Mobile USA

- R4-2103403, “CR for 38.101-3: clarification of intra-band EN-DC BCS applicability”, RAN4#98-e, T-Mobile USA

- R4-2103404, “CR for 38.101-3: clarification of intra-band EN-DC BCS applicability”, RAN4#98-e, T-Mobile USA

- R2-2102216, “Clarification on the supportedBandwidthCombinationSetIntraENDC capability”, RAN2#113-e, Nokia, Nokia Shanghai Bell

- R2-2102217, “Clarification on the supportedBandwidthCombinationSetIntraENDC capability”, RAN2#113-e, Nokia, Nokia Shanghai Bell

# Annex

3GPP TSG-RAN WG4 Meeting #108	R4-23XXXXX

Toulouse, France, August 21 – August 25, 2023

Title:	[Draft] LS on IE supportedBandwidthCombinationSetIntraENDC

Response to:

Release:	Rel-17

Work Item:	TEI17

Source:	RAN4

To:	RAN2

Cc:

Contact Person:

Name: Clement Huang

E-mail Address: clementhuang@google.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN4 confirms that the current definition of IE supportedBandwidthCombinationSetIntraENDC cannot handle intra-band EN-DC BCS reporting for inter-band EN-DC band combination with two additional intra-band EN-DC components, e.g., DC_3A_n3A and DC_41A_n41A shown as the following table, where these band combinations are introduced from Rel-17.

In order to solve the issue, RAN4 would like to request RAN2 to check whether it is feasible to introduce a new capability signaling to indicate intra-band EN-DC BCS for the second intra-band EN-DC component in inter-band EN-DC band combination with two additional intra-band EN-DC components.

| EN-DC configuration | Uplink EN-DC configuration (NOTE1) |
| --- | --- |
| DC_3A-41A_n3A-n41A | DC_3A_n3A4 DC_3A_n41A DC_41A_n3A |
| DC_1A-3A-41A_n3A-n41A | DC_1A_n3A DC_1A_n41A DC_3A_n3A4 DC_3A_n41A DC_41A_n3A |

2. Actions:

To RAN2:

ACTION: RAN4 respectfully asks RAN2 to take the above decisions into account to update RAN2 specifications.

3. Date of Next TSG WG RAN4 Meetings:

TSG-RAN WG4 Meeting #108bis               9st October – 13th October 2023    Xiamen, CN

TSG-RAN WG4 Meeting #109                  13st November – 17th November 2023    Chicago, US