3GPP TSG-RAN WG4 Meeting # 108	R4-2313502

# Toulouse, France,21th – 25th Aug, 2023

Source: 	Apple

Title: 	TP for TR 38.876: ATG UE Tx requirement

Agenda Item:	8.13.2.2

Document for:	Approval

# Introduction

This contribution presents a text proposal on ATG UE Tx requirement for TR 38.876.

# Reference

[1] R4-2309106, TR38.876 v0.4.0

# Text proposal

----------------------------------------------------------Start of Text Proposal------------------------------------------------------------

#### 7.1.2.9 Transmit signal quality

Transmit signal quality include frequency error and transmit modulation quality.

For frequency error, frequency pre-compensation is assumed for ATG UE. The existing requirement defined for NTN UE in TS 38.101-5 will be reused as baseline.

Regarding modulation scheme, QPSK/16QAM/64QAM will be supported as mandatory. For 256QAM, there are some challenges for ATG due to large propagation distance. While according to the evaluation for 2GHz and 4GHz in [x], the SNR above 25dB can still be seen in the considered scenarios for both UL and DL. UL 256QAM is supported for ATG UE as optional feature.

Figure 7.1.2.9-1

Figure 7.1.2.9-2

For transmit modulation quality, the following requirement for FR1 UE in 38.101-1 will be reused for ATG UE.

| Parameter | Unit | Average EVM Level |
| --- | --- | --- |
| QPSK | % | 17.5 |
| 16 QAM | % | 12.5 |
| 64 QAM | % | 8 |
| 256 QAM | % | 3.5 |

#### 7.1.2.10 Occupied bandwidth

The Occupied bandwidth requirement in subclause 6.5.1 of 38.101-1 [4] is applicable for ATG UE operating in FR1.

#### 7.1.2.11 SEM requirements

TBA

#### 7.1.2.12 ACLR requirements

FR1 PC3 ACLR requirement in 38.101-1 is reused for ATG CPE.

#### 7.1.2.13 Spurious emission

The general spurious emission requirement in subclause 6.5.3.1 of 38.101-1 [4] is applicable for ATG UE operating in FR1.

#### 7.1.2.14 Spurious emissions for UE co-existence

Given the separation distance between ATG UE and TN UE is quite large, it is expected that the achievable MCL can be large enough to overcome the UE-UE interference. This requirement will not be specified for ATG UE.

#### 7.1.2.15 Transmit intermodulation

---------------------------------------------------------End of Text Proposal--------------------------------------------------------------