3GPP TSG-RAN WG4 Meeting #108	R4-2313503

Toulouse, France, August 21 – August 25, 2023

Agenda item:	8.5.2

Source:            	Google Inc.

Title:                	Discussion on NR channel raster enhancement

Document for:	Approval

# Introduction

In RAN4#107 meeting, NR channel raster enhancements has been discussed and some agreements and open issues are captured in the WF [1]. It is agreed that the cell-specific channel BW advertised in SIB1 does not have to be placed on the 100KHz. In addition, two approaches with some alternatives are listed in the WF for further study.

- SIB1 positioning off the 100 kHz grid for legacy UEs

Agreement:

- There is no backwards compatibility issue, the carrierBandwidth advertised in SIB1 does not have to be placed on the 100kHz raster.

- SIB1 positioning off the 100 kHz grid for legacy UEs

Agreement:

- There is no backwards compatibility issue, the carrierBandwidth advertised in SIB1 does not have to be placed on the 100kHz raster.

The Approach 1 is to specify a new channel raster for both UE and gNB in all FR1 bands below 3GHz that currently have 100KHz channel raster. However, three options (5KHz,10KHz,50KHz) for step size in the new channel raster is not determined.

### Proposed alternatives for further study

  - Approach 1: Specify a new channel raster

- New channel raster step size:

Option 1: 5 kHz

Option 2: 10 kHz

Option 3: 50kHz

- The new channel raster should be specified for:

both UE and gNB.

- For which bands this new channel raster should be specified:

All FR1 bands below 3GHz that that currently have 100 kHz channel raster

Way forward:

For the next meeting, companies are encouraged to detail the expected specification’ updates of their preferred approach.

### Proposed alternatives for further study

  - Approach 1: Specify a new channel raster

- New channel raster step size:

Option 1: 5 kHz

Option 2: 10 kHz

Option 3: 50kHz

- The new channel raster should be specified for:

both UE and gNB.

- For which bands this new channel raster should be specified:

All FR1 bands below 3GHz that that currently have 100 kHz channel raster

Way forward:

For the next meeting, companies are encouraged to detail the expected specification’ updates of their preferred approach.

Approach 2 is not to specify new channel raster and two alternatives are listed for further study. The Alternative 1 is to clarify that the mapping rule for the channel raster to resource element in UE and BS specification clause 5.4.2.2 needs to include the centre of a carrier grid of a serving cell for at least one numerology as advertised in SIB1. Then, the network can use RRC signaling to reconfigure UE-specific channel BW within a wider cell-specific channel BW. The alternative 3 is that the UE can signal a capability to support a UE-specific channel BW off the 100 kHz channel raster with some BW configuration conditions, and the network can configure the UE-specific channel BW by RB grid of the carrier bandwidth in SIB1, where only some sparser RB grids need to be specified as the new raster.

### Proposed alternatives for further study

  - Approach 2: Do not specify new channel raster entries

    - Alternative 1

    - Clarify in clause 5.4.2.2 of both the BS and UE specifications that the “RF channel” is mapped to the channel raster at the centre of a carrier grid of a serving cell for at least one numerology as advertised in SIB1.

    - The network should be able to use the RRC specification for configuring the UE with locations of the UE-specific channel BW within a wider cell-specific bandwidth;

    - Alternative 3:

- For operating bands with a 100 kHz channel raster, the UE can signal a capability to support a UE specific channel BW that

  - consists of a contiguous subset of RBs from SCS-SpecificCarrier in SIB1 and

  - is a maximum transmission BW configuration

  - but need not be centered on the channel raster.

- For UEs with the capability to support a	 UE specific channel BW off the 100 kHz raster in corresponding operating bands, the natural raster for the UE specific channel BW is the RB grid of the carrier bandwidth in SIB1. (For a given numerology and location of the SIB1 carrier bandwidth, its RB grid is considerably sparser than the proposed channel rasters and it includes only valid frequency locations, hence rather the RB grid of the carrier bandwidth in SIB1 should be specified as raster for the UE specific channel BW than a new channel raster.)

Way forward:

For the next meeting, companies are encouraged to detail the expected specification’ updates of their preferred approach.

### Proposed alternatives for further study

  - Approach 2: Do not specify new channel raster entries

    - Alternative 1

    - Clarify in clause 5.4.2.2 of both the BS and UE specifications that the “RF channel” is mapped to the channel raster at the centre of a carrier grid of a serving cell for at least one numerology as advertised in SIB1.

    - The network should be able to use the RRC specification for configuring the UE with locations of the UE-specific channel BW within a wider cell-specific bandwidth;

    - Alternative 3:

- For operating bands with a 100 kHz channel raster, the UE can signal a capability to support a UE specific channel BW that

  - consists of a contiguous subset of RBs from SCS-SpecificCarrier in SIB1 and

  - is a maximum transmission BW configuration

  - but need not be centered on the channel raster.

- For UEs with the capability to support a	 UE specific channel BW off the 100 kHz raster in corresponding operating bands, the natural raster for the UE specific channel BW is the RB grid of the carrier bandwidth in SIB1. (For a given numerology and location of the SIB1 carrier bandwidth, its RB grid is considerably sparser than the proposed channel rasters and it includes only valid frequency locations, hence rather the RB grid of the carrier bandwidth in SIB1 should be specified as raster for the UE specific channel BW than a new channel raster.)

Way forward:

For the next meeting, companies are encouraged to detail the expected specification’ updates of their preferred approach.

In addition, another open issue needs to be discussed is about how to introduce the UE capability in accordance with the solution for Approach 1 or Approach 2. Whether the UE capability should be per-band or per-UE and introduced from which release are needed to be determined. In this paper, we would like to share our view in the following.

### 3- UE capability

Open issues (to be further discussed once RAN4 has agreed on one of the alternatives):

FFS whether/how a new UE capability would be specified to support the WI objectives

FFS if UE capability should be per band or per UE.

FFS from which release should the UE capability be applicable.

### 3- UE capability

Open issues (to be further discussed once RAN4 has agreed on one of the alternatives):

FFS whether/how a new UE capability would be specified to support the WI objectives

FFS if UE capability should be per band or per UE.

FFS from which release should the UE capability be applicable.

# Discussion

In RAN#99, a new WI [2] on channel raster enhancement was approved to discuss on how to remove the restriction on the 100KHz channel raster for some RF bands below 3GHz. One objective of the WI is to specify necessary changes to the UE channel raster such that configuring a narrower UE channel BW inside a wider gNB channel BW is always possible. For this case, the PRB alignment needs to be considered between UE-specific channel BW and cell-specific channel BW. Hence, even/odd PRB issue would be happened when locating the carrier frequency for UE-specific channel BW by following the rule of 100KHz channel raster point.

In one example, with SCS=15KHz as the assumption, if the UE-specific CHBW is configured with even/odd PRBs within the cell-specific CHBW with even/odd PRBs, the offset between UE channel raster and BS channel raster can be determined by the equation m*100KHz = n*180KHz, where {m=9,n=5} could be a possible solution to obtain 900KHz raster shift to make PRB alignment. However, in another example, with SCS=15KHz as the assumption, if the UE-specific CHBW is configured with even/odd PRBs within the cell-specific CHBW with odd/even PRBs, the offset between UE channel raster and BS channel raster can be determined by the equation m*100KHz = n*180KHz + 90KHz, where no {m,n} can be found to make PRB alignment between the UE-specific CHBW and the cell-specific CHBW. In order to solve the PRB alignment issue based on the configuration of the narrower UE-specific CHBW within the wider cell-specific CHBW, two approaches are proposed in the WF for further discussion.

For approach 1, there are three options for the new channel raster step size needs to be determined. In our view, Option 1 (5 kHz) and Option 2 (10 kHz) can be always the solution to solve even/odd n*180KHz + 90KHz and even-even/odd-odd n*180KHz regardless of the value n=1,2,… . But Option 3 (50 kHz) can only be the solution to even/odd n*180KHz + 90KHz when the value n=2,7,12,… and to even-even/odd-odd n*180KHz when the value n=5,10,15,… . Although both Option 1 (5 kHz) and Option 2 (10 kHz) could be the solution to even/odd issue, Option 2 (10 kHz) is preferred because the relative smaller new raster points would be introduced compared to  Option 1 (5 kHz), and the flexible channel raster with forward capability can also be met.

For alternative 1 of approach 2, since the cell-specific channel CHBW advertised in SIB1 does not have to be placed on the 100kHz raster according to last meeting agreement, the clarification on clause 5.4.2.2 may force the UE to support any channel raster configured by the network, which may increase more UE design complexity than approach 1.

For alternative 3 of approach 2, the UE can signal a capability to support a UE-specific channel BW off the 100 kHz channel raster by RB grid of the carrier bandwidth in SIB1. Although the new raster points would be much less than approach 1, it would be less flexible in BW configuration and the issue cannot be solved for 40KHz channel raster shift due to guard band checking in band n28 with the configuration of 30MHz UE-specific channel CHBW within 40MHz cell-specific channel CHBW.

Hence, in our view, we prefer approach 1 and we would like to propose to introduce the new channel raster with 10KHz as the step size for both UE and gNB in all FR1 bands below 3GHz that currently have 100KHz channel raster.

Proposal 1: Introduce the new channel raster with 10KHz as the step size for both UE and gNB in all FR1 bands below 3GHz that currently have 100KHz channel raster.

Regarding the UE capability issue, if approach 1 with 10KHz channel step size is approved as the solution, we propose to introduce a UE capability to support the new channel raster and it should be optional per-UE capability from Rel-18.

Proposal 2: Introduce an optional per-UE capability from Rel-18 to indicate the support of new channel raster with 10KHz as the step size for all FR1 bands below 3GHz that currently have 100 kHz channel raster.

# Conclusion

The proposals in this contribution are summarized in the following.

Proposal 1: Introduce the new channel raster with 10KHz as the step size for both UE and gNB in all FR1 bands below 3GHz that currently have 100KHz channel raster.

Proposal 2: Introduce an optional per-UE capability from Rel-18 to indicate the support of new channel raster with 10KHz as the step size for all FR1 bands below 3GHz that currently have 100 kHz channel raster.

# Reference

- R4-2310269, “WF on NR channel raster enhancement”, RAN4#107, Ericsson

- RP-230813, “New WID on Channel raster enhancement”, RAN#99, Ericsson