3GPP TSG-RAN WG4 Meeting #108 	R4-2313508

Toulouse, France, August 21 – August 25, 2023

Title:

LS on clarification on UE behavior during RACH-less SCG activation

Response to:

Release:	Rel-18

Work Item:	Even Further RRM enhancement for NR and MR-DC (NR_RRM_enh3)

Source:	RAN4 #108

To:	RAN2

Cc:

Contact person:	Griselda Zhanxian Wang

griselda.wang@ericsson.com,

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	N/A

# 1	Overall description

RAN4 has been working on WID objective 2 on Even Further RRM enhancement for NR and MR-DC, which (as per RP-220977) is defined as per the following.

- FR1-FR1 NR-DC scenarios FR1 + FR1 NR-DC band combinations were introduced in Rel-16 and the relevant deployment scenarios are expected to be used globally. However, the RRM requirements for FR1 + FR1 NR-DC are missing, which may negatively affect the overall performance in NR-DC scenarios and shall be specified.

For RACH-less SCG activation, RAN4 has discussed the side condition for RACH-less activation for setting delay requirements.

According to RAN2 procedure, the RACH-less activation requires no beam failure declared and TAT time alignment timer running based on specification TS 38.321 clause 5.29.

However, one of the known conditions defined in RAN4 TS 38.133 clause 8.17 as during the last 5 seconds, UE has sent a valid L3 measurement report.

The UE behavior is unclear upon receive the SCG activation command, when the TAT timer is running while UE has not measured the target cell or not being able to send a valid L3 measurement report.

From UE perspective, the RACH-less activation cannot be guaranteed as the target cell can not be regarded as known cell.

From network perspective, the RACH-less activation procedure is expected.

# 2	Actions

To RAN2:

ACTION:

RAN4 respectfully asks RAN2 to clarify the UE behaviour for the following scenarios and define the related procedures in their specifications:

What UE behaviour shall be expected upon receiving the activation command, while TAT timer is still running,

- When the target cell has been measured while L3 report has not been triggered.

- When the target cell has not been measured.

# 3	Dates of the next TSG RAN4 meetings

RAN WG4 Meeting #108bis 		Oct 9th – Oct 13th , 2023,			Xiamen, China

RAN WG4 Meeting #109		Nov 13th – Nov 17th, 2023,			Chicago, US