3GPP TSG RAN WG4 Meeting #108	R4-2313519

Toulouse, France, August 21-25, 2023

Agenda Item:	8.33.4

Source: 	Ericsson

Title:	Analysis of mIAB intra-frequency HO and RRC re-establishment requirements

Document for:	Discussion

# Introduction

The work split on draft CRs to define the RRM core requirements for mobile IAB-MT (mIAB-MT) was agreed in the approved WF [1].

In this contribution we analyze the details of the handover and RRC connection release with redirection requirements for the mIAB-MT based on all the previous agreements.

# Analysis of mIAB-MT HO requirements

The following was agreed regarding the handover requirements for the mIAB-MT [1]:

Intra-frequency HO requirements

- Reuse the UE intra-frequency HO requirements from TS 38.133 clauses 6.1.1.2 and 6.1.1.4 as baseline.

According to the above agreement, the HO requirements for the mIAB-MT are based on the corresponding requirements for the UE defined in TS 38.133. However, some aspects used in the HO requirements for the UE should not be applicable to the mIAB-MT. The handover requirements should be introduced only for the non-DRX case as agreed in the following WF [2]:

Issue 1-10: DRX requirements handling

- Only introduce requirements for non-DRX

One difference is that the mIAB-MT supports 4 SMTCs instead of 2 SMTCs supported by the UE. Furthermore, the IAB requirements are applicable for FR2-1 as there has not been any agreement for definng IAB requirements FR2-2 bands. Therefore, the HO HO requirements for the mIAB-MT should be limited to FR2-1.

# Analysis of mIAB-MT RRC connection re-establishment requirements

The following was agreed regarding the RRC connection re-establishment requirements requirements for the mIAB-MT [1]:

RRC Re-establishment

- Re-use the UE RRC re-establishment requirements in TS 38.133 clause 6.2.1 for the mobile IAB as baseline.

Inter-frequency RRC re-establishment/RRC release with re-direction

- Do not introduce requirements for inter-frequency RRC re-establishment and for RRC release with re-direction.

Based on the above agreements, the RRC connection re-establishment requirements requirements for the mIAB-MT should be defined for only NR intra-frequency target cell and are based on the corresponding UE requirements in TS 38.133. There are few differences wrt the UE requirements which should be considered for the mIAB-MT. As stated in the previous section that the mIAB-MT supports 4 SMTCs instead of 2 SMTCs supported by the UE. Another difference is that as also described in the previous section that the IAB requirements are applicable to FR2-1 bands only.

# Summary

The following are the proposals related to the RRM core requirements for mIAB-MT:

- Proposal #1: The intra-frequency HO delay requirement for mIAB-MT is defined for non-DRX.

- Proposal #2: The intra-frequency HO and RRC connection re-establishment requirements for mIAB-MT are defined for up to 4 SMTCs.

- Proposal 3: Both intra-frequency HO and RRC connection re-establishment requirements for mIAB-MT are applicable to only FR2-1 bands.

A draft CR introducing intra-frequency HO and RRC re-establishment requirements for mIAB-MT is provided in [3].

# References

- R4-2310085, WF on NR mobile IAB RRM requirements, Qualcomm.

- R4-2306368, WF on Mobile IAB RRM requirements, Qualcomm.

- R4-2313520, Draft CR on intra-frequency HO and RRC re-establishment requirements for mobile IAB, Ericsson.