3GPP TSG-RAN WG4 Meeting #108	R4-2313533

Toulouse, France, Aug 21 – 25, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM Requirements for NR Sidelink CA

Agenda item:	8.30.3.1

Document for:	Discussion

# 1	Introduction

The WF on RRM requirements related to NR SL CA as agreed in RAN4#107 is found in [1]. Short list of agreements and WF is provided below.

#### Issue 1-1: Interruption requirements when NR SL UE performs SL carrier addition/release - Agreements - Define interruptions to WAN carriers when NR SL UE performs SL carrier addition/release - Do not define interruptions to SL carriers when NR SL UE performs SL carrier addition/release - Interruptions to WAN - Interruption are defined with granularity of the NR slot length - FFS on interruption ratio and length Issue 1-2: Delay requirements of carrier addition/release for NR SL CA - Agreements - The delay requirements for SL component carrier addition/release are not needed for Mode 2 operation and up to UE implementation. Issue 1-3: Requirements of Selection / Reselection of Synchronization Reference Source under NR SL CA operation FFS: - Option 1 (Huawei, LGE, MTK): - Wait for RAN1/RAN2 progress. - Option 2 (OPPO): - If N synchronization carriers can be configured based on RAN2 conclusion, the corresponding detection and measurement delay requirements should be scaled by N compared to that for the scenario when a single synchronization carrier is configured. - Option 3 (Ericsson): - The LTE V2X requirements defined in section 13.10 in TS 36.133 for selection/reselection of V2X SyncRefUE are reused for NR SL selection/reselection of SyncRefUE when supporting NR SL CA

In this discussion paper, we discuss the highlighted FFS issues left for further discussion.

# 2	Discussion

In this section, we discuss the open issues on RRM requirements for NR SL CA, as summarized in the WF from RAN4#107.

On Requirements of Selection / Reselection of Synchronization Reference Source under NR SL CA operation

## Regarding RRM requirements on selection/reselection of synchronization reference source captured under Issue 1-3, RAN2 has not started to discuss among three options mentioned in WF [1].

## The updated WID on NR sidelink evolution, RP-230077 [2], as agreed in 3GPP TSG RAN Meeting #99 states “specify mechanism to support NR sidelink CA operation based on LTE sidelink CA operation.” Therefore, the NR SL CA operation should be based on LTE SL CA operation. In particular, TS 36.133 Clause 13.10 defines the RRM requirements on Selection / Reselection of V2X Synchronization Reference Source for LTE V2X Carrier Aggregation. Thus, the legacy requirements of V2X synchronization reference source selection / re-selection under LTE SL CA is also applicable to NR CA and can be reused for NR SL CA.

## In future, if RAN1/2 modifies or enhances the legacy procedure of synchronization reference source selection / reselection in a way that requires changes in the corresponding RRM requirements, RAN4 can analyze the impact on RRM requirements and suggest the relevant enhancement.

TS 36.133 Clause 13.10 RRM requirements of (re)selection of V2X (re)selection of Synchronization reference source can be reused for NR SL CA operation.

# 3	Conclusions

This document has made the following Proposal:

- TS 36.133 clause 13.10 RRM requirements of (re)selection of V2X (re)selection of Synchronization reference source can be reused for NR SL CA operation.

# References

- R4-2310061, WF on SL evolution RRM requirements – SL CA and co-channel co-existence

- RP-230077, WID revision: NR sidelink evolution