3GPP TSG-RAN WG4 Meeting #108 	R4-2313534

Toulouse, France, Aug 21 – 25, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM Requirements for U2U Relay of NR Sidelink

Agenda item:	8.32.2

Document for:	Discussion

Introduction

From RAN4#107 agreements on Topic: “ [NR_SL_relay_enh] WF on RRM requirements for NR SL relay requirements R4-2310059[1]”, Below are the WF agreements.

#### Issue 1-1: The requirements for the selection/reselection of the relay UE <Agreement> - The legacy requirement on selection/reselection of relay UE is applicable to R18 scenarios. #### Issue 1-2: Interruption time in multipath scenario <Agreement> - The existing interruptions due to RRC reconfiguration and SL DRX are applicable to multipath scenario (Uu and U2N path). FFS whether and how to capture this in the specification. #### Issue 1-3: Measurement accuracy <Agreement> - Discovery signal measurements accuracy (clause 10.4.5 in TS38.133) can be applied. No need to define new requirement.

Discussion

There are no open points in WF from RAN4#107[1]. We would like to discuss the below agreement of WF[1]-

<Agreement> - The existing interruptions due to RRC reconfiguration and SL DRX are applicable to multipath scenario (Uu and U2N path). FFS whether and how to capture this in the specification.

According to the work item description on ‘NR sidelink relay enhancements, RP-223501 [2], objective 5, RAN4 should (only) specify RRM core requirements for relay discovery and (re)selection in UE-to-UE relay.

Any other requirements related to other Rel. 18 features are, hence, considered out of the scope of RAN4 work for release 18.

The objective of WID is to only specify RRM core requirements for relay discovery and (re)selection in UE-to-UE relay.

Any requirements related to Rel-18 features, including multipath scenarios, are out of the scope of WID.

Hence, any discussion outside the WID scope as define under objective 5 is waste of RAN4 time and should not be pursued without a corresponding change in the scope.

- Proposal 1: RAN4 not to discuss RRM requirements for multipath scenarios in SL relay operation.

Conclusions

Observations and proposals made in this paper are as follows:

- The objective of WID is to only specify RRM core requirements for relay discovery and (re)selection in UE-to-UE relay.

- Any requirements related to Rel-18 features, including multipath scenarios, are out of the scope of WID.

RAN4 not to discuss RRM requirements for multipath scenarios in SL relay operation.

- References

- R4-2310059, WF on SL Relay RRM Requirements.

- RP-223501, Revised WID on NR sidelink relay enhancements, RAN #98e