3GPP TSG-RAN WG4 Meeting #108                                                               R4-2313538

Toulouse, France, August 21st – 25th, 2023

Source:	Samsung

Title:	TP to TR 38.858 on SBFD Implementation feasibility for FR2 BS

Agenda item:			8.19.2.2.2

Document for:	Approval

# 1. Introduction

Rel-18 Study Item is approved on Study on evolution of NR duplex operation with the target to provide enhanced UL coverage, reduced latency, improved system capacity, and improved configuration flexibility for NR TDD operation. According to SID [RP-222110], in this RAN1 led SI tasks for RAN4 scope are explicitly stated as below:

- Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4).

In RAN4#106, the group has agreed on the following work-split:

| No. | Section for TR 38.858 | Responsible company |
| --- | --- | --- |
| 1 | 10.1 Background for analysis | Ericsson |
| 2 | 10.2 Feasibility of FR1 Wide Area BS aspects | Samsung |
| 3 | 10.3 Feasibility of FR1 Medium Range BS aspects | Nokia |
| 4 | 10.4 Feasibility of FR1 Local Area BS aspects | CATT |
| 5 | 10.5 Feasibility of FR2 BS aspects | Huawei |
| 6 | 10.6. FR1 Feasibility of UE aspects | MediaTek |
| 7 | 10.7 FR2 Feasibility of UE aspects | Qualcomm |
| 8 | 10.8 Summary | CMCC |
| 9 | 11.1 Impact on BS RF requirements | ZTE |
| 10 | 11.2 Impact on UE RF requirements | Qualcomm |
| 11 | 12 Adjacent channel co-existence evaluation results | Samsung |
| 12 | 13 Regulatory aspects for deploying the duplex enhancements in TDD unpaired spectrum | CableLabs |

In RAN4#106-bis-e, RAN4 has agreed that the TR section “Feasibility of FR1 Wide Area BS aspects” shall be further broken-down to harmonize the common understating from RAN4 if possible and summarize the input from companies [R4-2306006], and the TR skeleton breakdown for this section. By using the similar skeleton, we would like to provide our technical input on the section for FR2 BS feasibility.

# 2. Text Proposal

## < START OF Text Proposal >

## 9.5	Feasibility of FR2 BS aspects

### 9.5.1	Self-interference analysis

Editor's note: This section captures the typical assumption based on which the RSIC capability and analysis results.

### 9.5.1.1	Summary table for self-interference analysis

Table 9.5.1.1-1: FR2-1 BS Self-Interference Analysis Summary

| FR2-1 |  |  |  | Samsung |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BS class |  |  |  | FR2-1 BS |  |  |  |
| BS TX Power dBPTX = ① dBm |  |  |  | 30 dBm |  |  |  |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | 28 dBc |  |  |  |
|  |  | Frequency isolation techniques used |  | Without DPD |  |  |  |
|  | Spatial isolation | Spatial isolation capability dBαSI-spatial = ③ dBc |  | 87 dBc |  |  |  |
|  |  | Spatial isolation techniques used |  | TX/RX panel separation and RF barrier structure |  |  |  |
|  | TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  | 10 dBc |  |  |  |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  | Limited, ~0dB |  |  |  |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  | -95 dBm Note: provided by ①-②-③-⑨ dBm |  |  |  |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  | 0 dBc |  |  |  |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  | 0 dBc |  |  |  |
|  |  | RF IC techniques and other tech. (before LNA) |  | Not applicable |  |  |  |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | No impact |  |  |  |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  | -67 dBm Note: provided by ①-③-④-⑤dBm |  |  |  |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | 24 dBc |  |  |  |
|  |  | Frequency isolation techniques |  | Filtering |  |  |  |
|  |  | RX IMD | Rx IIP3 capability (dBm) | IM3 contribution is Neglectable |  |  |  |
|  |  |  | Rx IM3 contribution (dBm) |  |  |  |  |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | N/A |  |  |  |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | -91 dBm Note: provided by ①-③-④-⑤-⑥dBm |  |  |  |
|  | RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  | 10 dBc |  |  |  |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | Limited, ~0dB |  |  |  |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | 10 dBc |  |  |  |
| Overall RSIC capability dBRSIC (Note 1) |  |  |  | 129.5 dBc |  |  |  |
| Noise floor ⑩dBm |  |  |  | -83 dBm/100MHz |  |  |  |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | -89 dBm |  |  |  |
| Required RSIC budget (①-⑪dBc) |  |  |  | 119 dBc |  |  |  |
| SBFD configuration |  |  |  | DU (100MHz-100MHz) |  |  |  |
| Guardband assumption (if exist) |  |  |  | 5PRB |  |  |  |
| bandwidth over which suppression is achieved |  |  |  | 100MHz |  |  |  |
| Others |  |  |  |  |  |  |  |

### 9.5.1.2	Feasibility study on self-interference

### 9.5.1.2.1	Samsung

Different FR1 counterpart, the difference of feasibility analysis on self-interference for FR2 BS will be provided here for different factors.

Spatial Isolation by Antenna Design

We observe similar and even better antenna isolation performance with the FR2-1 26 GHz testbed where panel separation can be exploited.

Figure 9.5.1.2.1-1 shows the FR2-1 testbed using 2 Tx panels and 2 Rx panels. Like described in the case of the FR1 3.5 GHz testbed, the Tx panel and the Rx panel in the FR2-1 26 GHz testbed are separated by a separation distance. Additional Tx/Rx isolation performance is then enabled by using an RF barrier, e.g., an additional EM resonant between the panels. In the case of FR2-1 26 GHz, since each panel can perform more directive beamforming in analog domain than possible in FR1 using mMIMO panels, the FR2-1 antenna isolation performance is better than what is achievable in FR1.

Figure 9.5.1.2.1-1: FR2-1 testbed and SIC performance when varying the operating frequency

Similar as FR1, an important design consideration for increased spatial isolation provided by the RF barrier is whether such stopband performance is stable over a wide enough frequency range. Our FR2-1 26 GHz testbeds have achieved isolation performance that show almost uniform antenna and panel isolation performance with respect to frequency for 100 MHz CC BW in 26 GHz. Figure 9.5.1.2.1-1 shows measurement results from the FR2-1 testbed with respect to achievable antenna isolation as a function of the operating frequency.

According to the applied mechanisms and measurement results, the achievable level for TX and RX spatial isolation without impact on radiation pattern based on compact antenna size is around 87dB for FR2.

Frequency isolation at TX

In the case of FR2-1, frequency-domain isolation for SBFD is of particular importance. Non-linear characteristics of mmWave PAs are worse than those of FR1 mid-band PAs. RAN4 ACLR requirements are more relaxed in FR2-1 when compared to FR1. This is due to beamforming providing isolation in FR2-1, implying that the probability of a blocker coming from the same direction is much lower than in FR1. Another consideration is that in FR1, the difference between the out-of-channel requirements like the ACLR and in-channel requirements like EVM is large. The PA linearity requirement is therefore dominated by out-of-channel requirements, e.g., ACLR. In FR2-1, these are at comparable levels. Spectral regrowth due to IM3 is dominant for in-channel requirements and as such, PA linearity requirements are rather driven by EVM and possibly in-band emissions. Another design challenge for DPD in FR2-1 is that PA characteristics must be carried through a feedback link from the output of the PA. In the case of mmWave, it is more difficult than in FR1 to create such a feedback link due to signal attenuation. Therefore, it is significantly more challenging to exploit DPD in FR2-1 such as done for FR1.

Despite these design challenges for gNB-side SBFD operation, our FR2-1 26 GHz testbed measurement results in Figure 9.5.1.2.1-2 show that 28 dBc leakage ratio between DL and UL subband (or component carriers) are still possible, e.g., similar to ACLR as existing out-of-channel requirement for FR2-1.

Figure 9.5.1.2.1-2: FR2-1 testbed and PSD for SBFD DL and UL subbands after antenna isolation and filtering

Frequency isolation at RX and RF SIC

Considering the input power at LNA for FR2 BS could be much less than FR1 WA BS, because of the better spatial isolation and lower BS output power, there is no necessity to have subband filtering to improve the linearity of LNA.

Digital IC

Similar as FR1, the digital SIC performance is helped when synchronization to accurately remove the Tx signal from the Rx signal can be obtained. As described in our analysis for FR1 WA BS, two methods exist to estimate the interference channel. One approach is to store information on a Tx signal that has passed through the PA with a feedback link and then estimate the interference channel over-the-air to remove the interference from the Rx signal. Another approach is to use only over-the-air estimation. Without a feedback link, the whole combined channel can still be estimated through the Rx panel. We used the second approach in the FR2-1 26 GHz testbed.

### 9.2.1.3	Conclusion

### 9.5.2	Co-channel inter-sub-band co-site inter-sector interference analysis

Editor's note: This section captures the typical assumption of RF requirements and the analysis results

### 9.5.2.1	Summary table for co-channel inter-sub-band co-site inter-sector interference analysis

Table 9.2.2.1-1: FR2 BS Co-site Inter-sector Co-channel Interference Analysis Summary

| FR1 (or FR2-1) |  |  |  | Samsung |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BS class |  |  |  | FR2-1 BS |  |  |  |
| BS TX Power dBPTX = ① dBm |  |  |  | 30 dBm |  |  |  |
| Number of co-site co-channel sectors considered |  |  |  | 1 |  |  |  |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαCSSI-frequency, TX = ② dBc |  | 28 dBc |  |  |  |
|  |  |  |  | Frequency isolation techniques used |  |  | Without DPD |  |  |  |  |  |  |  |
|  | Spatial isolation | Co-channel Co-site Inter-sector Spatial isolation capability dBαCSSI-spatial = ③ dBc |  | 87 dBc |  |  |  |
|  |  |  |  | Co-channel Co-site Inter-sector Spatial isolation techniques used |  |  | Based on 87dB for typical spatial isolation |  |  |  |  |  |  |  |
|  | TX Beam nulling /isolation of inter-sector interference in TX sub-band dBαCSSI-beam = ④ dBc |  |  | 20 dBc |  |  |  |
|  | DL EIRP impact due to beam nulling in TX sub-band (considering all nulling for self- and inter-sector interference) |  |  | Neglectable |  |  |  |
|  | Interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPCSSI_leakage due to inter-sector interference (Note 1) |  |  | -105 dBm |  |  |  |
|  | Interference signal in gNB TX subband, measured at the input of LNA (Note 1) due to inter-sector interference |  |  | -77 dBm |  |  |  |
|  | Blocker Suppression at RX | Frequency isolation capability dBαCSSI-frequency, RX= ⑥ dBc |  | 24 dBc |  |  |  |
|  |  | Frequency isolation techniques |  | Filtering |  |  |  |
|  |  | RX IMD | Rx IIP3 capability (dBm) | IM3 contribution is Neglectable |  |  |  |
|  |  |  | Rx IM3 contribution (dBm) |  |  |  |  |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | N/A |  |  |  |
|  | Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized due to co-site inter-sector co-channel interference only dBPCSSI_Selectivity(Note 1, 2) |  |  | -101 dBm |  |  |  |
|  | RX Beam nulling /isolation in RX sub-band dBαCSSI-beam = ⑨ dBc |  |  | 20 dB |  |  |  |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | Neglectable |  |  |  |
|  | Digital processing interference supression capability |  |  | 0 dB |  |  |  |
| Total interference in RX SB (dBm) (Note 2) |  |  |  | -99.5 dBm |  |  |  |
| Noise floor ⑩dBm |  |  |  | -83 dBm/100MHz |  |  |  |
| Calculated Desensitization (dB) |  |  |  | 0.1dB |  |  |  |
| SBFD configuration |  |  |  | DU (100MHz-100MHz) |  |  |  |
| Guardband assumption (if exist) |  |  |  | 5PRB |  |  |  |
| bandwidth over which suppression is achieved |  |  |  | 100MHz |  |  |  |
| Others |  |  |  |  |  |  |  |
| Note 1: Relevant metrics are derived from other parameters for checking purpose. Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics. Note 4: The abbreviation CSSI refers to co-site co-channel inter-sector interference in this table |  |  |  |  |  |  |  |

### 9.5.2.2	Feasibility study on co-channel inter-sub-band co-site inter-sector interference

### 9.2.2.2.1	Samsung

Different from the self-interference, the digital IC is not of necessity for the co-channel interference from co-site inter-sector BS. Furthermore, the isolation achieved by TX and RX beamforming nulling will be larger due to the different beamforming directions from different sectors.

### 9.5.2.3	Conclusion

### 9.5.3	Co-channel inter-sub-band inter-site interference analysis

Editor's note: This section captures the typical assumption of RF requirements and the analysis results

Similar as FR1, on the feasibility and how to model inter-site gNB-gNB CLI modelling considering unwanted emission and receiver selectivity, RAN4 agree that

- The same transmitter leakage and receiver impairment model as used for investigating gNB self-interference, but antenna isolation is replaced with inter-site isolation.

  - TX leakage baseline: gNB ACLR

  - Receiver impairment can be studied with gNB ACS as baseline for system level simulation and feasibility study, and further study on the possibility of improved receiver impairment performance compared to gNB ACS shall not be precluded in future RAN4 works.

### 9.5.4	Summary

Editor's note: This section captures the conclusion of BS SBFD feasibility.

## < END OF Text Proposal>