3GPP TSG-RAN4 Meeting #108	        R4-2313548 Toulouse, France, 21 – 25 August 2023

Source: 	Qualcomm Inc.

Title: 	PC2 A-MPR and RSD for bands n7 and n70

Agenda Item:	7.22.2

Document for:	Approval

# Introduction

In this contribution A-MPR analysis is provided for n7 and RSD analysis are provided for both n7 and n70. Proposals for A-MPR and RSD requirements for PC2 FDD operation are done.

# Discussion

## A-MPR for n7 PC2

Operating band n7 is subject to additional emission requirement captured under NS_46. This requirement is re-produced below from TS 38.101-1

When "NS_46" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.25-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.25-1: Additional requirements for “NS_46”

| Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency range | 2570 | - | 2575 | +1.6 | 5 | 1, 2 |
| Frequency range | 2575 | - | 2595 | -15.5 | 5 | 1, 2 |
| Frequency range | 2595 | - | 2620 | -40 | 1 | 1 |
| NOTE 1: This requirement is applicable for all carriers confined in 2500-2570 MHz. Sepcial restrictions apply for channel bandwidths up to 20MHz: For carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB with the minimum supported SCS of 15KHz. NOTE 2: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |

The corresponding A-MPR has been specified for channel bandwidths of 25 MHz and wider. For 15 and 20 MHz channel bandwidths an RB restriction to 54RB is applicable. A-MPR for PC3 is reproduced below

Table 6.2.3.17-1: A-MPR regions for NS_46

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 MHz | 2534.5 ≤ FC ≤ 2557.5 |  | Note 1 | A3 |
| 30 MHz | 2515 ≤ FC ≤ 2555 | ≥0, <1.44 | >0 | A4 |
|  |  | ≥1.44, <13.5 | >max (0, 12*SCS*RBend -1.8) | A5 |
|  |  | ≥13.5, <19.8 | >11.52 | A6 |
|  |  | ≥19.8, <25.92 | >6.3 | A7 |
|  |  | ≥25.92 | >0 | A8 |
| 35 MHz | 2517.5 ≤ FC ≤ 2552.5 | ≥0, <3.42 | >0 | A4 |
|  |  | ≥3.42, <15.84 | >max (0, 12*SCS*RBend - 3.06) | A5 |
|  |  | ≥15.84, <22.68 | >12.6 | A6 |
|  |  | ≥22.68, <28.8 | >9.0 | A7 |
|  |  | ≥28.8 | >0 | A8 |
| 40 MHz | 2520 ≤ FC ≤ 2550 | ≥0, <4.14 | >0 | A4 |
|  |  | ≥4.14, <18 | >max (0, 12*SCS*RBend - 4.5) | A5 |
|  |  | ≥18, <25.74 | >13.5 | A6 |
|  |  | ≥25.74, <32.4 | >12.6 | A7 |
|  |  | ≥32.4 | >0 | A8 |
| 50 MHz | 2525 ≤ FC ≤ 2545 | ≥0, <9 | >0 | A4 |
|  |  | ≥9, <21.6 | >max (0, 12*SCS*RBend - 7.2) | A5 |
|  |  | ≥21.6, <31.5 | >18 | A6 |
|  |  | ≥31.5, <39.6 | >16.2 | A7 |
|  |  | ≥39.6 | >0 | A8 |
| NOTE 1: > 9.72 MHz for DFT-s-OFDM, > 16.02 MHz for CP-OFDM. |  |  |  |  |

Table 6.2.3.17-2: A-MPR for NS_46

| Modulation/Waveform |  | A3 | A4 | A5 | A6 | A7 | A8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | 4.5 | 5 | 2 | 3.5 | 6 | 10 |
|  | QPSK | 4.5 | 5 | 2 | 3.5 | 6 | 10 |
|  | 16 QAM | 4.5 | 5 | 2 | 3.5 | 6 | 10 |
|  | 64 QAM | 4.5 | 5 |  | 3.5 | 6 | 10 |
|  | 256 QAM |  |  |  |  | 6 | 10 |
| CP-OFDM | QPSK | 6 | 5 | 3.5 | 5.5 | 7 | 11 |
|  | 16 QAM | 6 | 5 | 3.5 | 5.5 | 7 | 11 |
|  | 64 QAM | 6 | 5 | 3.5 | 5.5 | 7 | 11 |
|  | 256 QAM | 6 |  |  |  | 7 | 11 |

A-MPR was evaluated for PC2 using simulations and results for some individual simulated RB allocations were also validated by measurements. Simulation assumptions were as follows:

- PA calibration point: ACLR of 31 dBc is met with DFT-s-OFDM StartRB = 0, L_CRB = 100 waveform

- IQ-image: -28 dBc

- LO-leakage: -28 dBc

- CIM3: -60 dBc

- Requirements considered: NR ACLR, SEM, Spurious emissions, EVM, NS_46 additional requirements.

The simulation results are included in the appendix of this document. Based on the simulations the A-MPR requirements in Table 1 and Table 2 are proposed for n7 PC2 to meet NS_46 requirements.

Table 1: A-MPR regions for NS_46 for PC2

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 MHz | 2512.5 ≤ FC ≤ 2557.5 | < 14.4 | >max (0, 12*SCS*RBend - 2.7) | A5 |
|  |  | ≥ 14.4, <18.9 | > 9.9 | A7 |
|  |  | ≥ 18.9 | ≥ 7.2 | A9 |
|  |  | ≥ 17.64 | < 7.2 | A10 |
| 30 MHz | 2515 ≤ FC ≤ 2555 | ≥0, <1.44 | >0 | A4 |
|  |  | ≥1.44, <13.5 | >max (0, 12*SCS*RBend -1.8) | A5 |
|  |  | ≥13.5, <19.8 | >11.52 | A6 |
|  |  | ≥19.8, <25.92 | >6.3 | A7 |
|  |  | ≥25.92 | >0 | A8 |
|  |  | > 20.7 , < 25.92 | ≤ 6.3 | A10 |
| 35 MHz | 2517.5 ≤ FC ≤ 2552.5 | ≥0, <3.42 | >0 | A4 |
|  |  | ≥3.42, <15.84 | >max (0, 12*SCS*RBend - 3.06) | A5 |
|  |  | ≥15.84, <22.68 | >12.6 | A6 |
|  |  | ≥22.68, <28.8 | >9.0 | A7 |
|  |  | ≥28.8 | >0 | A8 |
|  |  | > 24.3 , < 28.8 | ≤ 9.0 | A10 |
| 40 MHz | 2520 ≤ FC ≤ 2550 | ≥0, <4.5 | >0 | A4 |
|  |  | ≥4.5, <18 | >max (0, 12*SCS*RBend - 4.5) | A5 |
|  |  | ≥18, <25.74 | >13.5 | A6 |
|  |  | ≥25.74, <32.4 | >12.6 | A7 |
|  |  | ≥32.4 | >0 | A8 |
|  |  | > 27.9 , < 32.4 | ≤ 12.6 | A10 |
| 50 MHz | 2525 ≤ FC ≤ 2545 | ≥0, <9 | >0 | A4 |
|  |  | ≥9, <21.6 | >max (0, 12*SCS*RBend - 7.2) | A5 |
|  |  | ≥21.6, <31.5 | >18 | A6 |
|  |  | ≥31.5, <39.6 | >16.2 | A7 |
|  |  | ≥39.6 | >0 | A8 |
|  |  | > 33.84, < 39.6 | ≤ 19.8 | A10 |

Table 2: A-MPR for NS_46 for PC2

| Modulation/Waveform |  | A4 | A5 | A6 | A7 | A8 | A9 | A10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | 6.5 | 2.5 | 3.5 | 6 | 10 | 6.5 | 3.5 |
|  | QPSK | 6.5 | 2.5 | 3.5 | 6 | 10 | 6.5 | 3.5 |
|  | 16 QAM | 6.5 | 2.5 | 3.5 | 6 | 10 | 6.5 | 3.5 |
|  | 64 QAM | 6.5 | 2.5 | 3.5 | 6 | 10 | 6.5 | 3.5 |
|  | 256 QAM | 6.5 |  |  | 6 | 10 | 6.5 |  |
| CP-OFDM | QPSK | 6.5 | 3.5 | 5.5 | 7 | 11 | 8 | 4.5 |
|  | 16 QAM | 6.5 | 3.5 | 5.5 | 7 | 11 | 8 | 4.5 |
|  | 64 QAM | 6.5 | 3.5 | 5.5 | 7 | 11 | 8 | 4.5 |
|  | 256 QAM | 6.5 |  |  | 7 | 11 | 8 |  |

Proposal 1: Adopt the A-MPR regions and A-MPR values in Table 1 and Table 2 for NS_46 for PC2 operation with channel bandwidths wider than 20 MHz.

Compared to PC3, the largest differences are in 25 MHz channel bandwidth, for which new A-MPR regions are defined. For all of the channel bandwidths a new region A10 is applied, and in for 40 MHz channel bandwidth the boundary between A4 and A5 has been slightly shifted.

For A-MPR values, in addition to the new regions, the largest difference to PC3 is in region A4 for SC-FDMA, for which A-MPR is increased by 2 dB.

For channel bandwidths ranging from 10 to 20 MHz our evaluation shows that the existing RB restriction to 54 RB is not sufficient. In addition to RB number restriction, RB placement restriction could be needed, which would effectively make some RB positions unusable. Another potential approaches would be to introduce A-MPR or a combination of A-MPR and RB restriction.

We see a need to evaluate the best approach further, to make sure that no issues rise in a case where UE would fall back to PC3. If A-MPR is introduced for all RB allocations, a scenario is possible where RB allocation which is valid for PC2 would not be allowed to be transmitted in PC3.

Proposal 2: Further evaluate general approach for NS_46 for PC2 for 20 MHz and narrower channel bandwidths.

## RSD

We measured PA noise for RSD analysis. PA was calibrated using conventional 3GPP calibration assumptions and RX band noise measured for the analysis for PC3 and PC2.

In the 2 TX case the impact of reverse-IM was addressed by adding 1dB into measured PC3 RX band noise.

For n70, we analyzed that no RSD is needed for 1TX PC2 and 2TX PC2 cases. This is well in line with already agreed 0dB RSD for n1 which has same baseline REFSENS and smaller TX-RX separation than n70 (190MHz vs 300MHz).

Proposal 3: Use 0dB RSD for n70 PC2 1TX and 2TX cases

For n7 we got the following results. For 1TX case the impact of TX IP2 is slightly visible for smaller BW’s.

Proposal 4: Use the following RSD numbers for n7 PC2 RSD

| n7 1TX PC2 RSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5MHz | 10MHz | 15MHz | 20MHz | 25MHz | 30MHz | 35MHz | 40MHz | 50MHz |
| 0.6dB | 0.4dB | 0.4dB | 0.4dB | 0.3dB | 0.3dB | 0.3dB | 0.3dB | 1.8dB |

| n7 2TX PC2 RSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5MHz | 10MHz | 15MHz | 20MHz | 25MHz | 30MHz | 35MHz | 40MHz | 50MHz |
| 1.2dB | 1.2dB | 1.3dB | 1.3dB | 1.2dB | 1.2dB | 1.2dB | 1.4dB | 3.8dB |

Snapshots of analysis are shown below.

Figure 1 n7 5MHz 1 TX PC2 analysis

Figure 2 n7 5MHz 2TX PC2 RSD analysis

# Conclusions

In this contribution A-MPR for band n7 and RSD analysis for bands n7 and n70 were presented. Following proposals were made for A-MPR for NS_46:

Proposal 1: Adopt the A-MPR regions and A-MPR values in Table 1 and Table 2 for NS_46 for PC2 operation with channel bandwidths wider than 20 MHz.

Proposal 2: Further evaluate general approach for NS_46 for PC2 for 20 MHz and narrower channel bandwidths.

Table 1: A-MPR regions for NS_46 for PC2

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 MHz | 2512.5 ≤ FC ≤ 2557.5 | < 14.4 | >max (0, 12*SCS*RBend - 2.7) | A5 |
|  |  | ≥ 14.4, <18.9 | > 9.9 | A7 |
|  |  | ≥ 18.9 | ≥ 7.2 | A9 |
|  |  | ≥ 17.64 | < 7.2 | A10 |
| 30 MHz | 2515 ≤ FC ≤ 2555 | ≥0, <1.44 | >0 | A4 |
|  |  | ≥1.44, <13.5 | >max (0, 12*SCS*RBend -1.8) | A5 |
|  |  | ≥13.5, <19.8 | >11.52 | A6 |
|  |  | ≥19.8, <25.92 | >6.3 | A7 |
|  |  | ≥25.92 | >0 | A8 |
|  |  | > 20.7 , < 25.92 | ≤ 6.3 | A10 |
| 35 MHz | 2517.5 ≤ FC ≤ 2552.5 | ≥0, <3.42 | >0 | A4 |
|  |  | ≥3.42, <15.84 | >max (0, 12*SCS*RBend - 3.06) | A5 |
|  |  | ≥15.84, <22.68 | >12.6 | A6 |
|  |  | ≥22.68, <28.8 | >9.0 | A7 |
|  |  | ≥28.8 | >0 | A8 |
|  |  | > 24.3 , < 28.8 | ≤ 9.0 | A10 |
| 40 MHz | 2520 ≤ FC ≤ 2550 | ≥0, <4.5 | >0 | A4 |
|  |  | ≥4.5, <18 | >max (0, 12*SCS*RBend - 4.5) | A5 |
|  |  | ≥18, <25.74 | >13.5 | A6 |
|  |  | ≥25.74, <32.4 | >12.6 | A7 |
|  |  | ≥32.4 | >0 | A8 |
|  |  | > 27.9 , < 32.4 | ≤ 12.6 | A10 |
| 50 MHz | 2525 ≤ FC ≤ 2545 | ≥0, <9 | >0 | A4 |
|  |  | ≥9, <21.6 | >max (0, 12*SCS*RBend - 7.2) | A5 |
|  |  | ≥21.6, <31.5 | >18 | A6 |
|  |  | ≥31.5, <39.6 | >16.2 | A7 |
|  |  | ≥39.6 | >0 | A8 |
|  |  | > 33.84, < 39.6 | ≤ 19.8 | A10 |

Table 2: A-MPR for NS_46 for PC2

| Modulation/Waveform |  | A4 | A5 | A6 | A7 | A8 | A9 | A10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | 6.5 | 2.5 | 3.5 | 6 | 10 | 6.5 | 3.5 |
|  | QPSK | 6.5 | 2.5 | 3.5 | 6 | 10 | 6.5 | 3.5 |
|  | 16 QAM | 6.5 | 2.5 | 3.5 | 6 | 10 | 6.5 | 3.5 |
|  | 64 QAM | 6.5 | 2.5 | 3.5 | 6 | 10 | 6.5 | 3.5 |
|  | 256 QAM | 6.5 |  |  | 6 | 10 | 6.5 |  |
| CP-OFDM | QPSK | 6.5 | 3.5 | 5.5 | 7 | 11 | 8 | 4.5 |
|  | 16 QAM | 6.5 | 3.5 | 5.5 | 7 | 11 | 8 | 4.5 |
|  | 64 QAM | 6.5 | 3.5 | 5.5 | 7 | 11 | 8 | 4.5 |
|  | 256 QAM | 6.5 |  |  | 7 | 11 | 8 |  |

Following proposals were made for RSD:

Proposal 3: Use 0dB RSD for n70 PC2 1TX and 2TX cases

Proposal 4: Use the following RSD numbers for n7 PC2 RSD

| n7 1TX PC2 RSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5MHz | 10MHz | 15MHz | 20MHz | 25MHz | 30MHz | 35MHz | 40MHz | 50MHz |
| 0.6dB | 0.4dB | 0.4dB | 0.4dB | 0.3dB | 0.3dB | 0.3dB | 0.3dB | 1.8dB |

| n7 2TX PC2 RSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5MHz | 10MHz | 15MHz | 20MHz | 25MHz | 30MHz | 35MHz | 40MHz | 50MHz |
| 1.2dB | 1.2dB | 1.3dB | 1.3dB | 1.2dB | 1.2dB | 1.2dB | 1.4dB | 3.8dB |

# Appendix: A-MPR simulation results

Figure 1: 25 MHz CP-OFDM 64QAM

Figure 2: 25 MHz DFT-s-OFDM 64QAM

Figure 3: 30 MHz CP-OFDM 64QAM

Figure 4: 30 MHz DFT-s-OFDM 64QAM

Figure 5: 35 MHz CP-OFDM 64QAM

Figure 6: 35 MHz DFT-s-OFDM 64QAM

Figure 7: 40 MHz CP-OFDM 64QAM

Figure 8: 40 MHz DFT-s-OFDM 64QAM

Figure 9: 50 MHz CP-OFDM 64QAM

Figure 10: 50 MHz DFT-s-OFDM 64QAM