3GPP TSG-RAN WG4 Meeting #108                          			R4-2313585

# Toulouse, France, August 21 – August 25, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.104 | CR | 0517 | rev |  | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [NR_RAIL_EU_900MHz-Core, NR_RAIL_EU_1900MHz_TDD-Core] CR to TS 38.104: corrections of RMR-specific BS requirements for band n100 and n101, Rel-17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RAIL_EU_900MHz-Core, NR_RAIL_EU_1900MHz_TDD-Core |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Referring to the LS received from ECC WG FM in R4-2311003 and related discussion paper in R4-2313584, this CR corrects RMR specific BS RF requirements for n100 and n101, to fix the antenna gain assumption, and to remove redundant requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add new symbols for n100 and n101 antenna gains. - Correction of BS output power requirement, OBUE and TX spur limits to remove antenna gain assumption and use manufacturer declaration instead. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RMR specific requirements would not follow ECC WG FM feedback, and the agreed corrections. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 6.2.4, 6.6.4.2.5.7, 6.6.5.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.141-1 CR#0376 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------ Modified section ------------------------------

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Physical resource block number

f	Separation between the channel edge frequency and the nominal -3 dB point of the measuring filter closest to the carrier frequency

fmax	f_offsetmax minus half of the bandwidth of the measuring filter

BeWθ,REFSENS	Beamwidth equivalent to the OTA REFSENS RoAoA in the θ-axis in degrees. Applicable for FR1 only.

BeWφ,REFSENS	Beamwidth equivalent to the OTA REFSENS RoAoA in the φ-axis in degrees. Applicable for FR1 only.

BWChannel	BS channel bandwidth

BWChannel,block	Sub-block bandwidth, expressed in MHz. BWChannel,block = Fedge,block,high- Fedge,block,low.

BWChannel_CA	Aggregated BS Channel Bandwidth, expressed in MHz. BWChannel_CA = Fedge,high- Fedge,low.

BWConfig	Transmission bandwidth configuration, where BWConfig = NRB x SCS x 12

BWContiguous	Contiguous transmission bandwidth, i.e. BS channel bandwidth for single carrier or Aggregated BS channel bandwidth for contiguously aggregated carriers. For non-contiguous operation within a band the term is applied per sub-block.

BWGB,high	The minimum guard band defined in clause 5.3.3 for highest assigned component carrier

BWGB,low	The minimum guard band defined in clause 5.3.3 for lowest assigned component carrier

EISminSENS	The EIS declared for the minSENS RoAoA

EISREFSENS	OTA REFSENS EIS value

EISREFSENS_50M	Declared OTA reference sensitivity basis level for FR2 based on a reference measurement channel with 50MHz BS channel bandwidth

f_BE_offset	Separation between the edge of the last transmitted channel of the channels assigned for NR-U channel bandwidth and the centre of the measuring

f_offset	Separation between the channel edge frequency and the centre of the measuring

f_offsetmax	The offset to the frequency ΔfOBUE outside the downlink operating band

FC	RF reference frequency on the channel raster, given in table 5.4.2.2-1

FC,block,high	Fc of the highest transmitted/received carrier in a sub-block.

FC,block,low	Fc of the lowest transmitted/received carrier in a sub-block.

FC,high	The Fc of the highest carrier, expressed in MHz.

FC,low	The Fc of the lowest carrier, expressed in MHz.

FDL,high	The highest frequency of the downlink operating band

FDL,low	The lowest frequency of the downlink operating band

Fedge,block,high	The upper sub-block edge, where Fedge,block,high = FC,block,high + Foffset,high.

Fedge,block,low	The lower sub-block edge, where Fedge,block,low = FC,block,low - Foffset,low.

Fedge,high	The upper edge of Aggregated BS Channel Bandwidth, expressed in MHz. Fedge,high = FC,high + Foffset,high.

Fedge,low	The lower edge of Aggregated BS Channel Bandwidth, expressed in MHz. Fedge,low = FC,low - Foffset,low.

FFBWhigh	Highest supported frequency within supported operating band, for which fractional bandwidth support was declared

FFBWlow	Lowest supported frequency within supported operating band, for which fractional bandwidth support was declared

Ffilter	Filter centre frequency

Foffset,high	Frequency offset from FC,high to the upper Base Station RF Bandwidth edge, or from F C,block,high to the upper sub-block edge

Foffset,low	Frequency offset from FC,low to the lower Base Station RF Bandwidth edge, or from FC,block,low to the lower sub-block edge.

FREF	RF reference frequency

FREF,shift	RF reference frequency for Supplementary Uplink (SUL) bands

FREF-Offs	Offset used for calculating FREF

Fstep,X	Frequency steps for the OTA transmitter spurious emissions (Category B)

FUL,high	The highest frequency of the uplink operating band

FUL,low	The lowest frequency of the uplink operating band

GBChannel	Minimum guard band defined in clause 5.3.3

GmaxRMR1900	Declared maximum antenna gain value for band n100

GmaxRMR900	Declared maximum antenna gain value for band n101

Ncells	Declared number corresponding to the minimum number of cells that can be transmitted by an BS type 1-H in a particular operating band

------------------------------ Next modified section ------------------------------

### 6.2.4	Additional requirements (regional)

In certain regions, additional regional requirements may apply.

For Band n41 and n90 operation in Japan, the rated output power, Prated,c.sys for BS type 1-H or sum of Prated,c,AC over all antenna connectors for BS type 1-C declared by the manufacturer shall be equal to or less than 20 W per 10 MHz bandwidth.

For band n100 in CEPT countries, Prated,c,AC of the WA BS shall not exceed (64.5 – GmaxRMR900) dBm/5MHz + (fDL-922.1) x 40/3 dB, where fDL is the centre frequency in MHz, and GmaxRMR900 is the declared antenna gain value for band n100. This limit is derived from ECC Decision(20)02 [21] assuming one antenna connector.

NOTE: for more details refer to TR 38.853 [23].

For band n101 in CEPT countries, Prated,c,AC shall not exceed (65 – GmaxRMR1900) dBm/10MHz, or (62 – GantRMR1900) dBm/5MHz, where the GmaxRMR1900 is the declared antenna gain value for band n101. This limit is derived from ECC Decision(20)02 [21] assuming one antenna connector.

NOTE: for more details refer to TR 38.852 [22].

------------------------------ Next modified section ------------------------------

6.6.4.2.5.7	Additional operating band unwanted emissions limits for Band n100

In CEPT countries where ECC Decision(20)02 [21] applies, the emissions of the WA BS shall not exceed the maximum levels specified in table 6.6.4.5.7-1.

Table 6.6.4.2.5.7-1: Additional operating band unwanted emission limits for Band n100

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (Note) (dBm) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.2 MHz | 0.1 MHz f_offset < 0.3 MHz | 32.5 - GmaxRMR900 | 200 kHz |
| 0.2 MHz f < 1 MHz | 0.6MHz f_offset <1.4 MHz | 14 - GmaxRMR900 | 800 kHz |
| 1 MHz f 10 MHz | 1.5 MHz f_offset < 10.5 MHz | 5 - GmaxRMR900 | 1 MHz |
| NOTE: The basic limits are derived from ECC Decision(20)02 [21] assuming one antenna connector. For more details, refer to TR 38.853 [23]. GmaxRMR1900 is the declared antenna gain value for band n100. |  |  |  |

------------------------------ Next modified section ------------------------------

The following requirement shall apply to BS operating in Band n101 in CEPT countries. The power of any spurious emission shall not exceed:

Table 6.6.5.2.3-12: Additional BS Spurious emissions limits for Band n101

| Frequency range | Maximum level (dBm) | Measurement bandwidth | Note |
| --- | --- | --- | --- |
| 1920 MHz – 1980 MHz | - 43 - GmaxRMR1900 | 5 MHz | This limit is derived from ECC Decision(20)02 [21] assuming one antenna connector. For more details, refer to TR 38.852 [23]. GmaxRMR1900 is the declared antenna gain value for band n101. |

The following requirement shall apply to BS operating in Band n100 in CEPT countries. The power of any spurious emission shall not exceed:

Table 6.6.5.2.3-13: Additional BS Spurious emissions limits for Band n100

| Frequency range | Maximum level (dBm) | Measurement bandwidth | Note |
| --- | --- | --- | --- |
| 880 MHz – 915 MHz | - 49 - GmaxRMR900 | 5 MHz | This limit is derived from ECC Decision(20)02 [21] assuming one antenna connector. For more details, refer to TR 38.853 [23]. GmaxRMR900 is the declared antenna gain value for band n100. |

------------------------------ End of modified section ------------------------------