# 3GPP TSG-RAN WG4 Meeting # 108                                                            R4-2313632

Toulouse, France, 21th August 2023 - 25th August 2023

Source:	China Unicom

Title:	Discussions on 4Rx at low frequency band (<1GHz)

Agenda item:			7.29.2

Document for:	Approval

1. Introduction

In RAN4-107 meeting, a WF [1] on 4Rx at low frequency band (<1GHz) was agreed as below:

Issue 1-1-1: Is it feasible to support low band 4Rx in handheld UE (<1GHz)

Agreement online:

- The feasibility to support low band 4Rx in handheld UE is confirmed at least for some UE.

Issue 1-2-2:  ΔRIB,4R

- Proposals

  - Option 1: -2.7dB (ZTE, Ericsson)

  - Option 2: Some relaxation is needed/preferred (Samsung, vivo, Xiaomi, Sony, Google, Huawei)

    - Option 2a: -2.2dB (Samsung, Sony, Google, Huawei)

    - Option 2b: -2.5dB (Xiaomi)

      - The current delta RIB,4R defined in Ts38.101-1 could not be used in OTA test.

WF: Value between [-2.7dB, -2.2dB] will be decided in next meeting.

Issue 1-2-1: How to specify requirements for handheld UE low band 4Rx (<1GHz)

WF: FFS whether below changes are agreeable in next meeting

Table 7.3.2-2: Four antenna port reference sensitivity allowance ΔRIB,4R

| Operating band | ΔRIB,4R (dB) |
| --- | --- |
| n5, n8, n28, n71, n85, n105 | -2.71 |
| n1, n2, n3, n25, n30, n40, n7, n34, n38, n39, n41, n66, n70 | -2.7 |
| n48, n77, n78, n79, n104 | -2.2 |
| NOTE 1: 4 Rx operation is targeted for FWA form factor, and for handheld UE ΔRIB,4R = TBD is applied |  |

In this contribution,  we share our considerations on enhancements for 4Rx at low frequency band (<1GHz).

2. Discussion

For Issue 1-2-2 on ΔRIB,4R requirement, it is preferable to keep the same set of requirements for 4Rx handheld UE and FWA without relaxations, for better network DL coverage performance.

Proposal 1:  From network performance perspective, it is preferable to keep the same requirement for 4Rx FWA and handheld UE.

For Issue 1-2-1 on  how to specify requirements for handheld UE low band 4Rx (<1GHz), if same set of requirements are confirmed for 4Rx handheld UE and FWA, it is then proposed to just remove Note 1 for simplicity.

Proposal 2: To remove ‘Note 1’ if same set of requirements are applied for 4Rx handheld UE and FWA.

3. Conclusion

In this contribution, we shared our views on enhancements for 4Rx at low frequency band, and made the following proposals:

Proposal 1:  From network performance perspective, it is preferable to keep the same requirement for 4Rx FWA and handheld UE.

Proposal 2: To remove ‘Note 1’ if same set of requirements are applied for 4Rx handheld UE and FWA.

3. Reference

[1] R4-2310409	WF on UE RF requirements for 3Tx and 4Rx, OPPO