3GPP TSG-RAN WG4 Meeting #108	R4-2313635

Toulouse, France, 21st August 2023-25th August 2023

Title:	Clarifications for Non-Terrestrial Networks LS response to RAN5

Source:	Keysight Technologies UK Ltd, THALES

Agenda item:	10.2.3

Document for:	Discussion and endorsement

# 1.	Introduction

A RAN5 LS has been received requiring clarifications for Non-Terrestrial Networks [1].

This discussion paper reviews concerns received and proposes both responses and changes in corresponding core requirements to fully solve issues detected (in case any) according to our view.

# 2. 	Discussion

## 2.1	Requirements applicability to different types of satellites

The first set of questions received are related to requirements applicability to different types of satellites:

Q1a: Are all the section 6 and section 7 RF Tx/Rx requirements defined in TS 38.101-5 applicable to both GSO and NGSO? Q1b: Are there any NR NTN demod performance requirements applicable to GSO (even if not defined in TS 38.101-5)? Q1a also applies to section 6 and section 7 requirements defined in TS 36.102. Please answer in the context of TS 36.102 also.

### 2.1.1	Types of satellites used per WID or SID and specification.

In Rel-14, TR 38.913 section 6.1.12 included the following examples to describe satellite deployment scenarios that could be used to extend terrestrial networks with satellites:

In Rel-15, the study item defining channel model for non-terrestrial networks ([3]), in its justification section considered the following types of satellites:

The outcome of such Study item was included in TR 38.811 ([4]), which considered initially the following air/space borne vehicle characteristics (GEO vs Non-GEO):

although LEO, MEO, Non-GEO, GSO and NGSO nomenclature is used along this technical report ([4]).

In Rel-16 the study item on solutions for NR to support non-terrestrial networks included in its Study item definition ([5]) the following text in the objectives section:

The outcome of this study item was included in TR 38.821 ([6]), that despite describing all different types of NTN platforms as:

,

it focused on six reference scenarios as (only GEO and LEO):

LEO, MEO, HEO, GSO and NGSO nomenclature is used along this technical report ([6]).

In Rel-17, solutions for NR to support non-terrestrial networks (NTN) Work Item was defined ([7]). The following statement was included in the justification section:

Some of the outcomes of this Work item were:

- The definition of the following capabilities in TS 38.306 ([8]):

Observation 1: GSO vs NGSO were the satellite types used to define UE capabilities related to non-terrestrial networks. There could be devices supporting only GSO satellites, only NGSO satellites or both GSO and NGSO satellites.

- In TS 38.101-5 ([9]), except for the applicability of demodulation performance requirements, types of satellites are only used in definition of terms and abbreviations section (GSO, GEO, NGEO, LEO and MEO are used). Regarding demodulation performance requirements, the only reference to a type of satellites is to NGSO as:

- In TS 38.108 ([10]), SAN classes have been defined as LEO and GEO and these are the types of SANs mentioned along the whole specification:

- In TS 38.133 ([11]), there is a mix of using GEO/LEO vs GSO/NGSO, being GEO/LEO the majority of the occurrences while defining the requirements (exceptions in clauses 9.3C.4 and 9.3C.7) and GSO/NGSO the majority when defining requirements in the Annexes (exceptions in Annex A3.36.4 and in Annex B).

Additionally, in Rel-17, NB-IoT/eMTC support for Non-Terrestrial Networks was defined ([12]). No clarification regarding types of satellites was included in work item definition.

Some of the outcomes of this Work item was:

- The definition of the following capabilities (among others) in TS 36.306 ([13]):

In Rel-18, NB-IoT/eMTC core & performance requirements for Non-Terrestrial Networks (NTN) was defined ([14]). No clarification regarding types of satellites was included in work item definition.

Some of the outcomes of this Work item are:

- In TS 36.102 ([9]), except for the applicability of demodulation performance requirements and some clarifications in Output RF spectrum emissions, types of satellites are only used in definition of terms and abbreviations section (GSO, GEO, NGSO, LEO and MEO are used). Regarding demodulation performance requirements and Output RF spectrum emissions, the only reference to a type of satellites is to NGSO as:

- In TS 36.108 ([16]), SAN classes have been defined as LEO and GEO and these are the types of SANs mentioned along the whole specification:

- In TS 36.133 ([17]), there is a mix of using GEO/LEO vs GSO/NGSO, being GSO/NGSO the majority of the occurrences in the overall specification.

Observation 2: Types of satellites have not been used uniformly across different 3GPP specifications.

### 2.1.2	Differences between types of satellites used in 3GPP.

The differences between types of satellites considered by 3GPP are the following ones:

- NGSO (Non – Geosynchronous Orbit)

The main characteristics of NGSO satellites are:

- NGSO satellites includes Low-Earth Orbit (LEO) and Medium-Earth Orbit (MEO) satellites:

  - LEO satellites are in orbits in an altitude range between 300 km and 1500 km [6].

  - MEO satellites are in orbits in an altitude range between 7000 km and 25000 km [6].

- High relative satellite velocity with respect to the UE.

-GSO (Geosynchronous Orbit)

The main characteristics of GSO satellites are:

- GSO satellites are located in an Earth-centred orbit at a constant altitude of 35,786 km with an orbital period that matches Earth's rotation on its axis (23 hours, 56 minutes, and 4 seconds).

  - For an observer on Earth's surface, a satellite in a geosynchronous orbit returns to exactly the same position in the sky after a period of one sidereal day.

- Over the course of a day, the object's position in the sky may remain still or trace out a path, typically in a figure-8 form, whose precise characteristics depend on the orbit's inclination and eccentricity.

- A special case of geosynchronous orbit satellites is the geostationary orbit satellite (GEO), which is a circular geosynchronous orbit in Earth's equatorial plane with both inclination and eccentricity equal to 0.

  - A satellite in a geostationary orbit remains in the same position in the sky to observers on the surface.

- Low relative satellite velocity with respect to the UE (compared to NGSO).

Observation 3: GSO/NGSO nomenclature is broader than GEO/LEO nomenclature, being GEO and LEO sub-cases of GSO and NGSO respectively.

### 2.1.3	Applicability of requirements per type of satellite

Considering observation 3 and to provide wider requirements coverage, GSO/NGSO is the preferred satellite categorization to define test requirements applicability unless any exception is identified.

For UE RF transmitter and receiver requirements for either NR NTN and NB-IoT/eMTC NTN:

- Requirements are understood as applicable to all types of satellites. Otherwise (and according to UE capabilities related to non-terrestrial networks mentioned in section 2.1.1), certain devices (those supporting only GSO or only NGSO) would have a very limited set of requirements applicable.

- In case the device supports both types of satellites, worst case requirements testing (NGSO) could be sufficient to demonstrate requirements compliance for both types of satellites for all the requirements.

Regarding demodulation performance requirements for NR NTN, the same approach as the one used for NB-IoT/eMTC NTN ([15]) should be used, i.e. to indicate that legacy demodulation performance requirements are also applicable to the NTN device independently of the type of satellite supported:

Proposal 1: Answer Q1a and Q1b as follows:

Q1a: Are all the section 6 and section 7 RF Tx/Rx requirements defined in TS 38.101-5 applicable to both GSO and NGSO?

Requirements defined in section 6 and 7 in TS 38.101-5 are applicable to both GSO and NGSO. In case UE supports both types of satellites worst case requirements testing (NGSO) could suffice to demonstrate requirements compliance for both types of satellites for all the requirements. Same applies to requirements defined in section 6 and 7 in TS 36.102.

Q1b: Are there any NR NTN demod performance requirements applicable to GSO (even if not defined in TS 38.101-5)?

Legacy requirements defined in TS 38.101-4 sections 5 and 6 are applicable to both GSO and NGSO satellites.

Proposal 2: Add the following paragraph to sections 6.1 and 7.1 in TS 38.101-5 and TS 36.102: “All requirements in this section are applicable to devices supporting GSO and/or NGSO satellites.”

Proposal 3: Add a foot note to Table 8.2.1.1.2-1 in TS 38.101-5 as follows: “NOTE: For UE supporting NTN access (nonTerrestrialNetwork-r17), the requirements in TS 38.101-4 Clause 5 and Clause 6 also apply to UE following applicability statements in those clauses.”.

## 2.2	Zero Doppler conditions

The second set of questions received are related to zero Doppler conditions:

Q2a: With regards to zero Doppler conditions indicated in section 6 and section 7 requirements in TS 38.101-5: Q2a1: Specifically, for NGSO where satellite orbit introduces a time varying Doppler shift and time varying propagation delay, is it expected to emulate zero Doppler condition in conformance testing of these section 6 and section 7 requirements? Q2a2: For GSO (different from GEO), do we need to emulate any Doppler shift/propagation delay in conformance testing? Q2a3: For GEO, do we need to emulate any Doppler shift/propagation delay in conformance testing? Q2a questions also apply to section 6 and section 7 requirements defined in TS 36.102. Please answer in the context of TS 36.102 also. Q2b: Under the zero Doppler conditions defined in section 6/7 of TS 38.101-5 and TS 36.102, what are RAN4 assumptions for UE Doppler and delay pre-compensation mechanisms for conformance testing: activated or deactivated? Q2c: Are the zero Doppler or time varying assumptions applicable for conformance testing of RRM test cases in TS 38.133 Annex A.14 and in TS 36.133 Annexes A.13 and A.14? Q2d: Are the zero Doppler or time varying assumptions applicable for conformance testing of demod performance requirements in section 8 in TS 38.101-5 and 36.102?

### 2.2.1	Doppler and delay in actual NTN deployments.

Based on the discussions so far in 3GPP, Doppler and delays in real NTN deployments are as specified in the following table:

|  | GSO |  | NGSO |
| --- | --- | --- | --- |
|  | GEO | GSO (different from GEO) |  |
| Doppler | Close to zero / negligible | Small although variable (TR 38.821 [6] considered a maximum of 0.93 ppm for Scenario A) | High (variable) (TR 38.821 [6] considered a maximum of 24 ppm for Scenario C with 600 km orbit altitude) |
| Delay (Maximum propagation delay from UE to Reference point) | High (≈constant) Max=270.73ms (according to max value of nta-Common-r17 in E-UTRA or ta-Common-r17 in NR) | High (slightly variable) | Low (variable) (TR 38.821 [6] considered a maximum of 12.88ms for Scenario C with 600 km orbit altitude) |

### 2.2.2 Zero Doppler conditions in Tx/Rx core requirements.

In TS 38.101-5 [9], RAN4 decided to add the following statement in sections 6.1 and 7.1 indicating that all requirements in sections 6 and 7, except for frequency error, shall be verified when zero Doppler conditions are set to zero:

The rationale behind such statements was somehow discussed in RAN4#104-e ([17]), where it was mentioned that those statements were required because no Doppler pre-compensation error had been considered when defining core requirements.

Despite those statements have not reached yet 36.102 [15], there was an interim agreement that they would be also applicable to NB-IoT/eMTC NTN in [18]:

For frequency error requirement, core specifications indicate that they need to be verified in 2 cases, one of them should be zero Doppler conditions:

| 38.101-5 ([9]) |  |
| --- | --- |
| 36.102 ([15]) |  |
| 36.102 ([15]) |  |

In figure 1, an example of Doppler and delay of a GSO satellite orbit with 7º inclination is shown:

| - 24 hours period | - 1 hour around zero Doppler instant |
| --- | --- |

Figure 1. Doppler and delay of a GSO satellite orbit with 7º inclination

Zero Doppler conditions could be understood as:

- Option 1: Instants where Doppler is exactly 0 Hz as shown by red squares in Figure 1a (2 instants along a 24 hours period).

- Option 2: Period of time where Doppler is very small. Figure 1b shows one hour zoom around zero Doppler instant. Doppler boundaries will depend on the duration of the test.

Although Option 2 is more realistic than Option1 (as it makes use of a satellite propagator model so the device can exercise the Doppler pre-compensation mechanism), RAN4 approach was to totally remove any impact of Doppler pre-compensation mechanism into RF Tx/Rx requirements (except for one case of frequency error discussed in section 2.3), i.e. Option1. Option 1 can be seen as a static snapshot of the satellite orbit in a concrete instant, i.e., ephemeris will remain constant along time in a satellite position providing zero Doppler conditions.  Ephemeris value will depend on the satellite orbit and the UE location.

As UE is supposed to estimate and pre-compensate Doppler shift based on UE location and satellite ephemeris values signalled in corresponding SIB (SIB19 for NR NTN, SIB31 for IoT NTN) according to 3GPP TS 38.300 ([19]) section 16.14.2.2 and TS 36.300 ([20]) section 23.21.2.2 as represented in Figure 2:

Figure 2. Doppler emulation and pre-compensation mechanism overview

it will be required to deactivate the Doppler pre-compensation mechanism in the UE side so no pre-compensation error is added into the measurement results independently of the value of:

- ul-SyncValidityDuration-r17 (36.331 [21])/ntn-UlSyncValidityDuration (38.331 [22]), indicating maximum time duration (from epochTime) during which the UE can apply the satellite ephemeris/assistance information without acquiring new satellite ephemeris/assistance information.

- SIB19/SIB31 periodicity configured.

- Test time (in case of UE conformance testing).

Zero Doppler conditions can then be depicted as shown in Figure 3:

Figure 3. Zero Doppler emulation and deactivation of associated pre-compensation mechanism overview

To be noticed that deactivating the Doppler pre-compensation mechanism is not strictly needed for GEO satellites as actual Doppler is already negligible.

No statement has been added to NTN RAN4 specifications regarding the implications of zero Doppler conditions in terms of delays.

Observation 4: No statement was agreed to be added to NTN RAN4 specifications indicating any implications of zero Doppler conditions in terms of delays.

However, the UE is supposed to compute the RTT between the UE and RP based on GNSS position, the ephemeris and the Common TA parameters:

| 38.300 ([19]) |  |
| --- | --- |
| 36.300 ([20]) |  |

Hence, under zero Doppler conditions, if UE position is fixed, Common TA remains constant and ephemeris of the satellite is constant, then delay should be constant. The exact value of the delay will depend on the satellite orbit, the UE location and the satellite ephemeris value.

Observation 5: Delays shall be consistent with satellite ephemeris.

As the UE might compute a variable delay (at least between SIB19/SIB31 updates) based on the satellite orbit, the UE location and the satellite ephemeris value, when deactivating the Doppler pre-compensation mechanism, it will be required to deactivate the pre-compensation of the variable part of the delay.

Same rationale as described in this section for GSO (with inclination different from zero) is applicable to NGSO satellites and to NR or IoT NTN.

### 2.2.3	Doppler conditions in RRM

No statement has been added to NTN RAN4 specifications regarding zero Doppler conditions for RRM requirements. In fact, for NR RRM tests Annex B.5 was added to TS 38.133 ([11]):

This annex is assumed to be applicable to all RRM tests.

Highlighted text in annex B.5 implies:

-  The use of a satellite propagator model, which implies periodical updates of the ephemeris in SIB19 according to satellite movement in the orbit.

- The design of a UE location in such a way that the elevation angle relative to the UE position is higher than 30º for the whole duration of the test.

With regards to IoT NTN, according to R4-2306370 ([23]), the following agreement was achieved:

As a conclusion, zero Doppler conditions are not applicable to RRM tests for NR NTN nor IoT NTN.

### 2.2.4	Doppler conditions in demodulation and CSI reporting requirements

No statement has been added to NTN RAN4 specifications regarding zero Doppler conditions for demodulation or CSI reporting requirements.

However, when discussing simulation assumptions for IoT NTN demodulation and CSI requirements koffset values, koffset value agreed was 8 slots (refer to R4-2220278 Issue 1-1-1 and Issue 1-2-4 ([24])) after considering the analysis in R4-2213860 ([25]) where it was shown the dependency of the koffset value with the satellite orbit and the elevation angle.

Similar discussion took place when defining koffset simulation assumption value for NR NTN demod and CSI reporting (refer to R4-2214386 ([26]) and R4-2217480 Issue 2-1 ([27])).

If elevation angle was considered in those assumptions, it could be thought that satellite propagator model is applicable to demodulation and CSI reporting performance requirements for NR and IoT NTN and hence zero Doppler conditions are not applicable.

However, in R4-2214386 ([26]), the agreement in Issue 1-4 was as follows:

Which implies that first block (New satellite link model) in Figure 6.2-3 in TR 38.811 ([4]) has not been considered when defining demodulation and CSI reporting requirements for NTN communications.

However, adding similar statement to the one added in sections 6.1 and 7.1 indicating that “All requirements in section 8 shall be verified when Doppler conditions are set to zero” would have been confusing as terrestrial model based on TR38.901 (second block in Figure 6.2-3 in TR 38.811 ([4]) already considers some Doppler different from zero).

### 2.2.5	Proposals

Proposal 4: Respond Q2 questions in the LS as follows:

Q2a1: Specifically, for NGSO where satellite orbit introduces a time varying Doppler shift and time varying propagation delay, is it expected to emulate zero Doppler condition in conformance testing of these section 6 and section 7 requirements?

Q2a2: For GSO (different from GEO), do we need to emulate any Doppler shift/propagation delay in conformance testing?

Q2a3: For GEO, do we need to emulate any Doppler shift/propagation delay in conformance testing?

Zero Doppler conditions are applicable to all RF requirements specified in sections 6 and 7 in 38.101-5 and 36.102. Consequently, constant delay shall be emulated independently of the type of satellite. This will represent realistic testing conditions for GEO satellites and a static snapshot of the satellite orbit in a concrete instant for GSO satellites (with inclination different from 0º) and NGSO satellites.

Q2b: Under the zero Doppler conditions defined in section 6/7 of TS 38.101-5 and TS 36.102, what are RAN4 assumptions for UE Doppler and delay pre-compensation mechanisms for conformance testing: activated or deactivated?

For all types of satellites, the assumptions are that:

- Doppler pre-compensation mechanism is deactivated

- Delay pre-compensation mechanism only compensates for a constant delay

To be noticed that these assumptions are not strictly required for GEO satellites.

Q2c: Are the zero Doppler or time varying assumptions applicable for conformance testing of RRM test cases in TS 38.133 Annex A.14 and in TS 36.133 Annexes A.13 and A.14?

No, zero Doppler conditions are not applicable to RRM test cases in TS 38.133 Annex A.14 and in TS 36.133 Annexes A.13 and A.14.

Q2d: Are the zero Doppler or time varying assumptions applicable for conformance testing of demod performance requirements in section 8 in TS 38.101-5 and 36.102?

Zero Doppler conditions related to satellite motion for DL in service link are applicable to demodulation or CSI reporting test cases in section 8 in TS 38.101-5 and TS 36.102. However, Doppler related to terrestrial model based on TR 38.901 is not zero.

Proposal 5: Update section 6.1 in TS 38.101-5 as follows: “All requirements in this section, other than frequency error in clause 6.4.1, shall be verified when Doppler conditions are set to zero and delay conditions are set to constant for all types of satellites.

Proposal 6: Update section 7.1 in TS 38.101-5 as follows: “All requirements in this section shall be verified when Doppler conditions are set to zero and delay conditions are set to constant for all types of satellites.

Proposal 7: Update section 6.1 in TS 36.102 adding the following sentence: “All requirements in this section, other than frequency error in clauses 6.4A.1 and 6.4B.1, shall be verified when Doppler conditions are set to zero and delay conditions are set to constant for all types of satellites.”

Proposal 8: Update section 7.1 in TS 36.102 adding the following sentence: “All requirements in this section shall be verified when Doppler conditions are set to zero and delay conditions are set to constant for all types of satellites.”

Proposal 9: Update section 8 in TS 38.101-5 adding the following sentence: “All requirements in this section shall be verified when Doppler conditions related to satellite motion for DL in service link are set to zero and delay conditions are set to constant for all types of satellites.”

Proposal 10: Update section 8 in TS 36.102 adding the following sentence: “All requirements in this section shall be verified when Doppler conditions related to satellite motion for DL in service link are set to zero and delay conditions are set to constant for all types of satellites.”

## 2.3 Other than zero Doppler conditions

The third set of questions received are related to other than zero Doppler conditions:

Q3a: For the NTN frequency error requirements defined in section 6.4.1 of TS 38.101-5, what is RAN4 assumption in terms of constant/variable Doppler and delay conditions for the other than zero Doppler conditions for GSO (different from GEO), GEO and NGSO? TS 38.101-5 snippet of clause 6.4.1 is captured below for reference. Q3b: In case of constant Doppler conditions, does RAN4 assume the UE Doppler and delay pre-compensation mechanisms only apply to the constant Doppler while they don’t apply to any time-varying Doppler or time delay introduced by satellite model in conformance testing? Q3a and Q3b also apply to frequency error requirements defined in TS 36.102 section 6.4A.1 and 6.4B.1. Please answer in the context of TS 36.102 also.

As described, NTN RAN4 specifications indicate that frequency error requirement will be verified for at least two cases of which one has zero Doppler conditions.

As the only RF Tx/Rx core requirement that includes the impact of Doppler pre-compensation error is the frequency error in the case of non-zero Doppler conditions, the assumption should be that such verification is done under worst realistic conditions as described in section 2.2.1, using the same satellite propagator model required for RRM tests.

Proposal 11: Respond Q3 questions in the LS as follows:

Q3a: For the NTN frequency error requirements defined in section 6.4.1 of TS 38.101-5, what is RAN4 assumption in terms of constant/variable Doppler and delay conditions for the other than zero Doppler conditions for GSO (different from GEO), GEO and NGSO?

The assumption for the second case of frequency error verification in section 6.4.1 of TS 38.101-5 and in sections 6.4A.1 and 6.4B.1 of TS 36.102 is to test that second case under worst realistic Doppler and delay testing conditions, i.e.:

- GSO satellite (with inclination different form 0º): Small and slightly variable Doppler with high and slightly variable delay, using Eckstein-Hechler satellite propagator model.

- NGSO satellite: High and variable Doppler with low and variable delay, using Eckstein-Hechler satellite propagator model.

Q3b: In case of constant Doppler conditions, does RAN4 assume the UE Doppler and delay pre-compensation mechanisms only apply to the constant Doppler while they don’t apply to any time-varying Doppler or time delay introduced by satellite model in conformance testing?

N/A

Proposal 12: Update section 6.4.1 in TS 38.101-5 as follows: “Requirement will be verified for at least two cases of which one has zero Doppler conditions: one with zero Doppler and constant delay conditions and the other one with realistic Doppler and delay conditions if different from the first one (using Eckstein-Hechler satellite propagator model).”

Proposal 13: Update section 6.4A.1 in TS 36.102 as follows: “Requirement will be verified for at least two cases of which one has zero Doppler conditions: one with zero Doppler and constant delay conditions and the other one with realistic Doppler and delay conditions if different from the first one (using Eckstein-Hechler satellite propagator model).”

Proposal 14: Update section 6.4B.1 in TS 36.102 as follows: “Requirement will be verified for at least two cases of which one has zero Doppler conditions: one with zero Doppler and constant delay conditions and the other one with realistic Doppler and delay conditions if different from the first one (using Eckstein-Hechler satellite propagator model).”

## 2.4 Satellite propagator model

The fourth set of questions received are related to satellite propagator model:

Q4a: For section 6, section 7, section 8 requirements defined in TS 38.101-5, is RAN4 assuming implementation of a satellite propagator model for the service link in conformance testing? This question also applies to section 6, section 7 and section 8 requirements defined in TS 36.102. Please answer in the context of TS 36.102 also. Q4b: Which RRM test cases listed under Annex A.14 are assuming a satellite motion trajectory based on the ephemeris using Eckstein-Hechler model as defined in TS 38.133 Annex B.5 (applicable also to 36.133 as per agreement in R4-2306370)?

As described in previous sections of this document, the understanding is that sections 6, 7 and 8 in TS 38.101-5 and in TS 36.102, except for the non-zero Doppler conditions case in frequency error requirements, are not assuming any satellite propagator model, while

- non-zero Doppler conditions case in frequency error requirements in section 6.4.1 in TS 38.101-5 and in sections 6.4A.1 and 6.4B.1 in TS 36.102

- RRM requirements in 38.133 Annex A.14 and 36.133 Annexes A.13 and A.14

are all assuming Eckstein-Hechler propagator model for the service link in conformance testing.

Proposal 15: Respond Q4 questions in the LS as follows:

Q4a: For section 6, section 7, section 8 requirements defined in TS 38.101-5, is RAN4 assuming implementation of a satellite propagator model for the service link in conformance testing? This question also applies to section 6, section 7 and section 8 requirements defined in TS 36.102. Please answer in the context of TS 36.102 also.

Q4b: Which RRM test cases listed under Annex A.14 are assuming a satellite motion trajectory based on the ephemeris using Eckstein-Hechler model as defined in TS 38.133 Annex B.5 (applicable also to 36.133 as per agreement in R4-2306370)?

All requirements in sections 6, 7 and 8 in TS 38.101-5 and in TS 36.102, except for the non-zero Doppler conditions case in frequency error requirements, are not assuming any satellite propagator model, while

-	Non-zero Doppler conditions case in frequency error requirements in section 6.4.1 in TS 38.101-5 and in sections 6.4A.1 and 6.4B.1 in TS 36.102

-	All RRM requirements in TS 38.133 Annex A.14 and TS 36.133 Annexes A.13 and A.14

are all assuming Eckstein-Hechler propagator model for the service link in conformance testing.

Proposal 16: Add an Annex B.8 in TS 36.133 with similar content to Annex B.5 in TS 38.133:

B.8	High level test procedure for SAN RRM tests

The following high-level steps are conducted for test cases for SAN defined in clauses A.13 and A.14.

-	A set of ephemeris information are pre-defined for each satellite corresponding to different epoch times in [TS TBD].

-	For GEO an altitude of 35,786km is considered. an elevation angle relative to a UE position shall not be smaller than 30 deg during entire test time.

-	For LEO an altitude of 600km on a circular orbit is considered.

-	A motion trajectory is generated for each satellite based on the ephemeris using Eckstein-Hechler model.

- 	UE location is determined for the test. The ephemeris and the UE location should be designed such that elevation angle relative to the UE position shall not be smaller than 30 deg during entire test time.

-	Test equipment adjusts the time and frequency of transmission based on the satellite motion trajectory and UE location during test time to emulate the position and velocity change of the satellite relative to the UE.

## 2.5 UE location updates for multipath fading channels

The last set of questions received are related to UE location updates for multipath fading channels:

Q5a: For conformance testing of TS 38.101-5 section 8 requirements in multipath fading channel, should UE location updates follow UE motion? Q5b: For conformance testing of TS 38.133 Annex A.14 RRM test cases in multipath fading channel, should UE location updates follow UE motion? Q5a and Q5b also apply to section 8 requirements of TS 36.102 and RRM test cases in TS 36.133. Please answer in the context of TS 36.102 and TS 36.133 also.

In a realistic scenario, as the UE moves, it will experience multipath propagation conditions in the NTN communication channel that will depend on the relative movement between the satellite and the UE. The UE will obtain its location through its GNSS receiver and, together with the ephemeris information signalled, it will be able to properly pre-compensate the Doppler effect and the associated delays.

The UE velocity dependent additional Doppler shift could have been included by the UE virtual speed in TDL models if considered at the time of definition of NTN channel propagation conditions.

However, NTN channel propagations conditions have been defined leveraging from TS 38.101-4, which assumes fixed satellite elevation, and therefore the additional TDL-model Doppler shift is not modelling accurately the real satellite trajectory. Anyways, the impact of the UE movement to the Doppler is quite small compared to the satellite movement as shown in [28]:

If in future releases a more accurate Doppler is to be considered modelling UE movement appropriately, discussions will be required in the following aspects:

- how to define the UE direction of travel with respect to satellite trajectory and if it changes during the test scenario

- Once accurate realistic Doppler shift due to the UE movement is included, how to align the delay profile with the Doppler shift profile.

In demodulation and CSI reporting requirements, as satellite motion for DL in service link has not been considered when defining the requirements as described in section 2.2.4, there is no need to provide updated UE location information following UE motion to allow the device to properly pre-compensate Doppler shift and delays. In this case, same assumptions as the ones described in responses to Q2b apply.

Desppite RRM requirements have been defined using more realistic testing conditions through the use of Eckstein-Hechler satellite propagator model as described in section 2.2.3 but with TDL channel propagation conditions assuming fixed satellite elevation, UE location should not follow UE movement in those conformance tests using multipath propagation conditions.

Proposal 17: Respond Q5 questions in the LS as follows:

Q5a: For conformance testing of TS 38.101-5 section 8 requirements in multipath fading channel, should UE location updates follow UE motion?

For those NTN conformance tests in section 8 in TS 38.101-5 and section 8 in TS 36.102 using multipath propagation conditions, there is no need that UE location follows UE movement. Same assumptions as the ones described in responses to Q2b apply.

Q5b: For conformance testing of TS 38.133 Annex A.14 RRM test cases in multipath fading channel, should UE location updates follow UE motion?

For those NTN conformance tests in, section A.14 in TS 38.133 and sections A.13 and A.14 in TS 36.133 using multipath propagation conditions, there is no need that UE location follows UE movement.

# 3. 	Conclusion

This discussion paper reviews concerns received on clarifications required for NTN requirements and proposes both responses and changes in corresponding core requirements to fully solve issues detected (in case any) according to proponent’s view.

The following observations and proposals are made:

Observation 1: GSO vs NGSO were the satellite types used to define UE capabilities related to non-terrestrial networks. There could be devices supporting only GSO satellites, only NGSO satellites or both GSO and NGSO satellites.

Observation 2: Types of satellites have not been used uniformly across different 3GPP specifications.

Observation 3: GSO/NGSO nomenclature is broader than GEO/LEO nomenclature, being GEO and LEO sub-cases of GSO and NGSO respectively.

Proposal 1: Answer Q1a and Q1b as follows:

Q1a: Are all the section 6 and section 7 RF Tx/Rx requirements defined in TS 38.101-5 applicable to both GSO and NGSO?

Requirements defined in section 6 and 7 in TS 38.101-5 are applicable to both GSO and NGSO. In case UE supports both types of satellites worst case requirements testing (NGSO) could suffice to demonstrate requirements compliance for both types of satellites for all the requirements. Same applies to requirements defined in section 6 and 7 in TS 36.102.

Q1b: Are there any NR NTN demod performance requirements applicable to GSO (even if not defined in TS 38.101-5)?

Legacy requirements defined in TS 38.101-4 sections 5 and 6 are applicable to both GSO and NGSO satellites.

Proposal 2: Add the following paragraph to sections 6.1 and 7.1 in TS 38.101-5 and TS 36.102: “All requirements in this section are applicable to devices supporting GSO and/or NGSO satellites.”

Proposal 3: Add a foot note to Table 8.2.1.1.2-1 in TS 38.101-5 as follows: “NOTE: For UE supporting NTN access (nonTerrestrialNetwork-r17), the requirements in TS 38.101-4 Clause 5 and Clause 6 also apply to UE following applicability statements in those clauses.”.

Observation 4: No statement was agreed to be added to NTN RAN4 specifications indicating any implications of zero Doppler conditions in terms of delays.

Observation 5: Delays shall be consistent with satellite ephemeris.

Proposal 4: Respond Q2 questions in the LS as follows:

Q2a1: Specifically, for NGSO where satellite orbit introduces a time varying Doppler shift and time varying propagation delay, is it expected to emulate zero Doppler condition in conformance testing of these section 6 and section 7 requirements?

Q2a2: For GSO (different from GEO), do we need to emulate any Doppler shift/propagation delay in conformance testing?

Q2a3: For GEO, do we need to emulate any Doppler shift/propagation delay in conformance testing?

Zero Doppler conditions are applicable to all RF requirements specified in sections 6 and 7 in 38.101-5 and 36.102. Consequently, constant delay shall be emulated independently of the type of satellite. This will represent realistic testing conditions for GEO satellites and a static snapshot of the satellite orbit in a concrete instant for GSO satellites (with inclination different from 0º) and NGSO satellites.

Q2b: Under the zero Doppler conditions defined in section 6/7 of TS 38.101-5 and TS 36.102, what are RAN4 assumptions for UE Doppler and delay pre-compensation mechanisms for conformance testing: activated or deactivated?

For all types of satellites, the assumptions are that:

- Doppler pre-compensation mechanism is deactivated

- Delay pre-compensation mechanism only compensates for a constant delay

To be noticed that these assumptions are not strictly required for GEO satellites.

Q2c: Are the zero Doppler or time varying assumptions applicable for conformance testing of RRM test cases in TS 38.133 Annex A.14 and in TS 36.133 Annexes A.13 and A.14?

No, zero Doppler conditions are not applicable to RRM test cases in TS 38.133 Annex A.14 and in TS 36.133 Annexes A.13 and A.14.

Q2d: Are the zero Doppler or time varying assumptions applicable for conformance testing of demod performance requirements in section 8 in TS 38.101-5 and 36.102?

Zero Doppler conditions related to satellite motion for DL in service link are applicable to demodulation or CSI reporting test cases in section 8 in TS 38.101-5 and TS 36.102. However, Doppler related to terrestrial model based on TR 38.901 is not zero.

Proposal 5: Update section 6.1 in TS 38.101-5 as follows: “All requirements in this section, other than frequency error in clause 6.4.1, shall be verified when Doppler conditions are set to zero and delay conditions are set to constant for all types of satellites.

Proposal 6: Update section 7.1 in TS 38.101-5 as follows: “All requirements in this section shall be verified when Doppler conditions are set to zero and delay conditions are set to constant for all types of satellites.

Proposal 7: Update section 6.1 in TS 36.102 adding the following sentence: “All requirements in this section, other than frequency error in clauses 6.4A.1 and 6.4B.1, shall be verified when Doppler conditions are set to zero and delay conditions are set to constant for all types of satellites.”

Proposal 8: Update section 7.1 in TS 36.102 adding the following sentence: “All requirements in this section shall be verified when Doppler conditions are set to zero and delay conditions are set to constant for all types of satellites.”

Proposal 9: Update section 8 in TS 38.101-5 adding the following sentence: “All requirements in this section shall be verified when Doppler conditions related to satellite motion for DL in service link are set to zero and delay conditions are set to constant for all types of satellites.”

Proposal 10: Update section 8 in TS 36.102 adding the following sentence: “All requirements in this section shall be verified when Doppler conditions related to satellite motion for DL in service link are set to zero and delay conditions are set to constant for all types of satellites.”

Proposal 11: Respond Q3 questions in the LS as follows:

Q3a: For the NTN frequency error requirements defined in section 6.4.1 of TS 38.101-5, what is RAN4 assumption in terms of constant/variable Doppler and delay conditions for the other than zero Doppler conditions for GSO (different from GEO), GEO and NGSO?

The assumption for the second case of frequency error verification in section 6.4.1 of TS 38.101-5 and in sections 6.4A.1 and 6.4B.1 of TS 36.102 is to test that second case under realistic Doppler and delay testing conditions, i.e.:

- GSO satellite (with inclination different form 0º): Small and slightly variable Doppler with high and slightly variable delay, using Eckstein-Hechler satellite propagator model.

- NGSO satellite: High and variable Doppler with low and variable delay, using Eckstein-Hechler satellite propagator model.

Q3b: In case of constant Doppler conditions, does RAN4 assume the UE Doppler and delay pre-compensation mechanisms only apply to the constant Doppler while they don’t apply to any time-varying Doppler or time delay introduced by satellite model in conformance testing?

N/A

Proposal 12: Update section 6.4.1 in TS 38.101-5 as follows: “Requirement will be verified for at least two cases of which one has zero Doppler conditions: one with zero Doppler and constant delay conditions and the other one with realistic Doppler and delay conditions if different from the first one (using Eckstein-Hechler satellite propagator model).”

Proposal 13: Update section 6.4A.1 in TS 36.102 as follows: “Requirement will be verified for at least two cases of which one has zero Doppler conditions: one with zero Doppler and constant delay conditions and the other one with realistic Doppler and delay conditions if different from the first one (using Eckstein-Hechler satellite propagator model).”

Proposal 14: Update section 6.4B.1 in TS 36.102 as follows: “Requirement will be verified for at least two cases of which one has zero Doppler conditions: one with zero Doppler and constant delay conditions and the other one with realistic Doppler and delay conditions if different from the first one (using Eckstein-Hechler satellite propagator model).”

Proposal 15: Respond Q4 questions in the LS as follows:

Q4a: For section 6, section 7, section 8 requirements defined in TS 38.101-5, is RAN4 assuming implementation of a satellite propagator model for the service link in conformance testing? This question also applies to section 6, section 7 and section 8 requirements defined in TS 36.102. Please answer in the context of TS 36.102 also.

Q4b: Which RRM test cases listed under Annex A.14 are assuming a satellite motion trajectory based on the ephemeris using Eckstein-Hechler model as defined in TS 38.133 Annex B.5 (applicable also to 36.133 as per agreement in R4-2306370)?

All requirements in sections 6, 7 and 8 in TS 38.101-5 and in TS 36.102, except for the non-zero Doppler conditions case in frequency error requirements, are not assuming any satellite propagator model, while

-	Non-zero Doppler conditions case in frequency error requirements in section 6.4.1 in TS 38.101-5 and in sections 6.4A.1 and 6.4B.1 in TS 36.102

-	All RRM requirements in TS 38.133 Annex A.14 and TS 36.133 Annexes A.13 and A.14

are all assuming Eckstein-Hechler propagator model for the service link in conformance testing.

Proposal 16: Add an Annex B.8 in TS 36.133 with similar content to Annex B.5 in TS 38.133:

B.8	High level test procedure for SAN RRM tests

The following high-level steps are conducted for test cases for SAN defined in clauses A.13 and A.14.

-	A set of ephemeris information are pre-defined for each satellite corresponding to different epoch times in [TS TBD].

-	For GEO an altitude of 35,786km is considered. an elevation angle relative to a UE position shall not be smaller than 30 deg during entire test time.

-	For LEO an altitude of 600km on a circular orbit is considered.

-	A motion trajectory is generated for each satellite based on the ephemeris using Eckstein-Hechler model.

- 	UE location is determined for the test. The ephemeris and the UE location should be designed such that elevation angle relative to the UE position shall not be smaller than 30 deg during entire test time.

-	Test equipment adjusts the time and frequency of transmission based on the satellite motion trajectory and UE location during test time to emulate the position and velocity change of the satellite relative to the UE.

.Proposal 17: Respond Q5 questions in the LS as follows:

Q5a: For conformance testing of TS 38.101-5 section 8 requirements in multipath fading channel, should UE location updates follow UE motion?

For those NTN conformance tests in section 8 in TS 38.101-5 and section 8 in TS 36.102 using multipath propagation conditions, there is no need that UE location follows UE movement. Same assumptions as the ones described in responses to Q2b apply.

Q5b: For conformance testing of TS 38.133 Annex A.14 RRM test cases in multipath fading channel, should UE location updates follow UE motion?

For those NTN conformance tests in, section A.14 in TS 38.133 and sections A.13 and A.14 in TS 36.133 using multipath propagation conditions, there is no need that UE location follows UE movement.

[29]- [32] show associated CRs to 38.101-5, 36.102 and 36.133 if proposals 1-17 are agreed. [33] includes the Reply LS to RAN5 proposed.

# 4. 	References

- R4-2311011, “LS on clarifications for Non-Terrestrial Networks”, RAN4 #108

- 3GPP TR 38.913, “Study on Scenarios and Requirements for Next Generation Access Technologies”

- RP-171450, “Study on NR to support Non-Terrestrial Networks Study Item Definition”, RAN#76, Thales, Dish network, HUGHES Network Systems Ltd, ESA

- 3GPP TR 38.811, " Solutions for NR to support non-terrestrial networks (NTN)"

- RP-190710, “Study on solutions for NR to support Non-Terrestrial Networks Study Item Definition”, RAN#83, Thales

- 3GPP TR 38.821, " Solutions for NR to support non-terrestrial networks (NTN)"

- RP-220208, “Solutions for NR to support non-terrestrial networks (NTN) Work Item Definition”, RAN#95-e, Thales

- 3GPP TS 38.306, " NR; User Equipment (UE) radio access capabilities"

- 3GPP TS 38.101-5, " NR; User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements"

- 3GPP TS 38.108, " NR; Satellite Access Node radio transmission and reception"

- 3GPP TS 38.133, "Requirements for support of radio resource management"

- RP-211601, “NB-IoT/eMTC support for Non-Terrestrial Networks work item definition”, RAN#92, MediaTek Inc., Eutelsat S.A.

- 3GPP TS 36.306, " E-UTRA; User Equipment (UE) radio access capabilities"

- RP-223437, “WID on NB-IoT/eMTC core & performance requirements for Non-Terrestrial Networks”, RAN#98, MediaTek Inc.

- 3GPP TS 36.102, " E-UTRA; User Equipment (UE) radio transmission and reception for satellite access"

- 3GPP TS 36.108, " E-UTRA; Satellite Access Node radio transmission and reception"

- R4-2214298, “Email discussion summary for [104-e][309] NTN_Solutions_UERF_Maintenance”, RAN4#104-e, Moderator (ZTE)

- R4-2303538, “WF on IoT NTN UE RF requirement”, RAN4#106, MediaTek Inc.

- 3GPP TS 38.300, “NR; NR and NG-RAN Overall Description; Stage 2”

- 3GPP TS 36.300, “Evolved Universal Terrestrial Radio Access (E-UTRA); and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2 (Release 17)”

- 3GPP TS 36.331, “Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification (Release 17)”

- 3GPP TS 38.331, “NR; Radio Resource Control (RRC) protocol specification (Release 17)”

- R4-2306370, “WF on NB-IoT/eMTC RRM requirements for NTN”, Moderator (MediaTek inc.), RAN4#106bis-e

- R4-2220278, “WF on IoT-NTN UE demodulation and CQI reporting requirements”, MediaTek, RAN4#105

- R4-2213860, “Discussion on UE NTN demod PDSCH”, Huawei, HiSilicon, RAN4#104-e

- R4-2214386, “WF for NTN demodulation requirements - general and PDSCH”, Qualcomm Incorporated, RAN4#104-e

- R4-2217480, “Email discussion summary for [104-bis-e][317] NR_NTN_Demod_Part1”, Moderator (Qualcomm incorporated), RAN4-bis-e

- “Space-Terrestrial Radio Network Integration for IoT”, Dr. Gilles Charbit, Dr. Debby Lin, Dr. Kader Medles, Linda Li, and Dr. I-Kang Fu, MediaTek Inc.

- R4-2313636, “Clarifications to 38.101-5 (Rel-17)”, Keysight Technologies UK Ltd, THALES, RAN4#108

- R4-2313637, “Clarifications to 38.101-5 (Rel-18)”, Keysight Technologies UK Ltd, THALES, RAN4#108

- R4-2313638, “Clarifications to 36.102”, Keysight Technologies UK Ltd, THALES, RAN4#108

- R4-2313639, “New Annex B.8 definition for High level test procedure for SAN RRM tests”, Keysight Technologies UK Ltd, THALES, RAN4#108

- R4-2313640x, “Reply LS on clarifications for Non-Terrestrial Networks”, Keysight Technologies UK Ltd, THALES, RAN4#108