3GPP TSG-RAN WG4 Meeting #108	R4-2313647

# Toulouse, 21st - 25th Aug, 2023

Title: 	Discussion on NR UE ATG demodulation requirements

Source: 	Huawei, HiSilicon

Agenda item:	8.13.5.2

Document for:	Discussion

Background

During RAN4#107 meeting, WF [1] for ATG demodulation requirements was approved. In this contribution, we share our views about NR UE ATG demodulation requirements.

Discussion

PDSCH

Test scope for PDSCH

- New PDSCH test cases for ATG with FDD slot pattern can be as follows. FDD 10 MHz 15kHz SCS; 2T2R; 16QAM (MCS [13] in table 1). 64QAM (MCS [22] in table 1). 256QAM (MCS [24] in table 2). 2T4R; 16QAM (MCS [13] in table 1). 64QAM (MCS [22] in table 1). 256QAM (MCS [24] in table 2). - New PDSCH test cases with static TDD slot pattern can be as follows (if introduced): TDD 40MHz 30kHz SCS; FFS on new TDD pattern: 30D4S6U; 2T2R; 16QAM (MCS [13] in table 1). 64QAM (MCS [22] in table 1). 256QAM (MCS [24] in table 2). 2T4R; 16QAM (MCS [13] in table 1). 64QAM (MCS [22] in table 1). 256QAM (MCS [24] in table 2). 7D1S2U; 2T2R; 16QAM (MCS [13] in table 1). 64QAM (MCS [22] in table 1). 256QAM (MCS [24] in table 2). 2T4R; 16QAM (MCS [13] in table 1). 64QAM (MCS [22] in table 1). 256QAM (MCS [24] in table 2). - For existing PDSCH test cases: - Choose 1 test case per Modulation Order for PDSCH Mapping Type A only, for FDD and TDD, FFS on which test cases to choose. - Consider Single Carrier requirements only

For the new test cases, based on our simulation results, we confirm that the proposed MCSs in the WF (MCS13, MCS22 in Table 1 and MCS24 in Table 2) are feasible.

For the existing PDSCH test case, we propose to select the “yellow-marked” cases in the following Table 2.1.1-1 and Table 2.1.1-2 for ATG UE requirements. Considering that there is no 64QAM rank 1 case in Rel-15 TS 38.101-4, so how to reuse 64QAM cases can be further discussed, one possible method is to define new requirements for 64QAM by reusing the existing 64QAM rank2 case but change the rank to rank1.

Table 2.1.1-1 FDD rank1 requirements in TS 38.101-4

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value Fraction of maximum throughput (%) |
| --- | --- | --- | --- | --- | --- | --- |
| 1-1 | R.PDSCH.1-1.1 FDD | 10 / 15 | QPSK, 0.30 | TDLB100-400 | 2x2, ULA Low 2x4, ULA Low | 70 |
| 1-2 | R.PDSCH.1-1.2 FDD | 10 / 15 | QPSK, 0.30 | TDLC300-100 | 2x2, ULA Low 2x4, ULA Low | 70 |
| 1-3 | R.PDSCH.1-4.1 FDD | 10 / 15 | 256QAM, 0.82 | TDLA30-10 | 2x2, ULA Low 2x4, ULA Low | 70 |
| 1-4 | R.PDSCH.1-2.1 FDD | 10 / 15 | 16QAM, 0.48 | TDLC300-100 | 2x2, ULA Low 2x4, ULA Low | 30 |
| 1-5 | R.PDSCH.1-8.1 FDD | 10 / 15 | 16QAM, 0.48 | HST-750 | 1x2 1x4 | 70 |

Table 2.1.1-2 TDD rank1 requirements in TS 38.101-4

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Fraction of maximum throughput (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-1 | R.PDSCH.2-1.1 TDD | 40 / 30 | QPSK, 0.30 | FR1.30-1A | TDLB100-400 | 2x2, ULA Low 2x4, ULA Low | 70 |
| 1-2 | R.PDSCH.2-1.2 TDD | 40 / 30 | QPSK, 0.30 | FR1.30-1 | TDLC300-100 | 2x2, ULA Low 2x4, ULA Low | 70 |
| 1-3 | R.PDSCH.2-4.1 TDD | 40 / 30 | 256QAM, 0.82 | FR1.30-1 | TDLA30-10 | 2x2, ULA Low 2x4, ULA Low | 70 |
| 1-4 | R.PDSCH.2-2.1 TDD | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLC300-100 | 2x2, ULA Low 2x4, ULA Low | 30 |
| 1-5 | R.PDSCH.2-5.1 TDD | 40 / 30 | QPSK, 0.30 | FR1.30-2 | TDLA30-10 | 2x2, ULA Low 2x4, ULA Low | 70 |
| 1-6 | R.PDSCH.2-6.1 TDD | 40 / 30 | QPSK, 0.30 | FR1.30-3 | TDLA30-10 | 2x2, ULA Low 2x4, ULA Low | 70 |
| 1-7 | R.PDSCH.2-10.1 TDD | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | HST-1000 | 1x2 1x4 | 70 |
| 1-8 | R.PDSCH.2-11.1 TDD | 40 / 30 | QPSK, 0.30 | FR1.30-5 | TDLB100-400 | 2x2, ULA Low 2x4, ULA Low | 70 |
| 1-9 | R.PDSCH.2-12.1 TDD | 40 / 30 | QPSK, 0.30 | FR1.30-6 | TDLB100-400 | 2x2, ULA Low 2x4, ULA Low | 70 |

Confirm that the proposed MCSs in the WF (MCS13, MCS22 in Table 1 and MCS24 in Table 2) are feasible for the new test cases.

Select the “yellow-marked” cases in the following Table 2.1.1-1 and Table 2.1.1-2 for ATG UE requirements.

Considering that there is no 64QAM rank 1 case in Rel-15 TS 38.101-4, define new requirements for 64QAM by reusing the existing 64QAM rank2 case but change the rank to rank1.

Applicability rule

- Option 1: New UE feature for ATG scenario and corresponding applicability rule shall be introduced, for instance UE capability; Test type; Test list. ATG with 2RX; FR1 FDD; PDSCH; All tests in Clause 5.2.1.1.x. PDCCH; All tests in Clause 5.3.2.1.2. FR1 TDD; PDSCH; All tests in Clause 5.2.1.2.x. PDCCH; All tests in Clause 5.3.2.2.2. ATG with 4RX; FR1 FDD; PDSCH; All tests in Clause 5.2.2.1.x. PDCCH; All tests in Clause 5.3.3.1.2. FR1 TDD; PDSCH; All tests in Clause 5.2.1.2.x. PDCCH; All tests in Clause 5.3.3.2.2. - Option 2: For the applicability of the ATG UE Demod requirements, follow ongoing discussion in RF on related ATG UE capabilities;

The existing applicability rule for different number of Rx antenna can be reused with some modification.

The existing applicability rule for different number of Rx antenna can be reused with some modification as following:

Table xxx: Requirements applicability for ATG

| UE capability | Test type |  | Test list |
| --- | --- | --- | --- |
| UE supports only 2RX | FR1 FDD | PDSCH | All tests in Clause 5.2.1.1.x |
|  |  | PDCCH | All tests in Clause 5.3.2.1.2 |
|  | FR1 TDD | PDSCH | All tests in Clause 5.2.1.2.x |
|  |  | PDCCH | All tests in Clause 5.3.2.2.2 |
| UE supports only 4Rx or both 2RX and 4RX | FR1 FDD | PDSCH | All tests in Clause 5.2.2.1.x |
|  |  | PDCCH | All tests in Clause 5.3.3.1.2 |
|  | FR1 TDD | PDSCH | All tests in Clause 5.2.1.2.x |
|  |  | PDCCH | All tests in Clause 5.3.3.2.2 |

PDCCH requirements

Test scope for PDCCH

Agreement: - No new test cases for PDCCH under the assumption that new PDSCH test cases will be specified. - FFS on the selected legacy test cases for ATG UEs

For the existing PDSCH test case, it is better to cover all aggregation level and select one case per aggregation level. We propose to select the “yellow-marked” cases in the following Table 2.2.1-1 and Table 2.2.1-2 for ATG UE requirements.

Table 2.2.1-1 FDD requirements in TS 38.101-4

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Pm-dsg (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 10 | 24 | 2 | 2 | R.PDCCH. 1-2.1 FDD | TDLA30-10 | 1x2 Low 1x4 Low | 1 |
| 2 | 10 | 24 | 2 | 2 | R.PDCCH. 1-2.3 FDD | TDLC300-100 | 1x2 Low 1x4 Low | 1 |
| 3 | 10 | 48 | 2 | 4 | R.PDCCH. 1-2.4 FDD | TDLA30-10 | 1x2 Low 1x4 Low | 1 |
| 4 | 10 | 48 | 1 | 4 | R.PDCCH.1-1.1 FDD | TDLA30-10 | 1x2 Low 1x4 Low | 1 |
| 5 | 10 | 48 | 2 | 16 | R.PDCCH. 1-2.6 FDD | TDLA30-10 | 1x2 Low 1x4 Medium A | 1 |
| 1 | 10 | 24 | 2 | 4 | R.PDCCH. 1-2.2 FDD | TDLC300-100 | 2x2 Low 2x4 Low | 1 |
| 2 | 10 | 48 | 2 | 8 | R.PDCCH. 1-2.5 FDD | TDLC300-100 | 2x2 Low 2x4 Low | 1 |
| 3 | 10 | 48 | 1 | 8 | R.PDCCH.1-1.3 FDD | TDLA30-10 | 2x2 Low 2x4 Low | 1 |

Table 2.2.1-2 TDD requirements in TS 38.101-4

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Pm-dsg (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 40 | 102 | 1 | 2 | R.PDCCH. 2-1.1 TDD | TDLA30-10 | 1x2 Low 1x4 Low | 1 |
| 2 | 40 | 102 | 1 | 4 | R.PDCCH. 2-1.2 TDD | TDLC300- 100 | 1x2 Low 1x4 Low | 1 |
| 3 | 40 | 48 | 2 | 16 | R.PDCCH. 2-2.1 TDD | TDLC300- 100 | 1x2 Low 1x4 Medium A | 1 |
| 1 | 40 | 90 | 1 | 8 | R.PDCCH. 2-1.3 TDD | TDLC300-100 | 2x2 Low 2x4 Low | 1 |

Select the “yellow-marked” cases in the following Table 2.2.1-1 and Table 2.2.1-2 for ATG UE requirements.

CSI reporting requirements

Test scope for CSI reporting

Agreement: - FFS: The feasibility of the CSI reporting given the large propagation delay in the ATG scenario.

As per WID, the maximum cell coverage is about 300km when the plane is above the sea, the maximum RTT delay is about 2ms. For more typical scenario, the ISD is 200km that is corresponding to 0.77ms RTT delay. We don’t think such propagation delay is  large comparing to the CQI/RI/PMI delay in the current CSI reporting cases (8ms or 9.5ms for AWGN case). In addition, there is no large Doppler in ATG scenario after UE compensation that means the channel is rather stable during tens of microsecond. So we think the CSI reporting is still useful for ATG scenario. We propose to reuse the CSI reporting cases for ATG scenario from the existing legacy CSI reporting cases.

The CSI reporting is still useful for ATG scenario.

Reuse the CSI reporting cases for ATG scenario from the existing legacy CSI reporting cases.

Proposals

In this contribution, we discuss on NR UE ATG demodulation requirements. Our observations and proposals are:

- Confirm that the proposed MCSs in the WF (MCS13, MCS22 in Table 1 and MCS24 in Table 2) are feasible for the new test cases.

Select the “yellow-marked” cases in the following Table 2.1.1-1 and Table 2.1.1-2 for ATG UE requirements.

Considering that there is no 64QAM rank 1 case in Rel-15 TS 38.101-4, define new requirements for 64QAM by reusing the existing 64QAM rank2 case but change the rank to rank1.

The existing applicability rule for different number of Rx antenna can be reused with some modification as following:

Table xxx: Requirements applicability for ATG

| UE capability | Test type |  | Test list |
| --- | --- | --- | --- |
| UE supports only 2RX | FR1 FDD | PDSCH | All tests in Clause 5.2.1.1.x |
|  |  | PDCCH | All tests in Clause 5.3.2.1.2 |
|  | FR1 TDD | PDSCH | All tests in Clause 5.2.1.2.x |
|  |  | PDCCH | All tests in Clause 5.3.2.2.2 |
| UE supports only 4Rx or both 2RX and 4RX | FR1 FDD | PDSCH | All tests in Clause 5.2.2.1.x |
|  |  | PDCCH | All tests in Clause 5.3.3.1.2 |
|  | FR1 TDD | PDSCH | All tests in Clause 5.2.1.2.x |
|  |  | PDCCH | All tests in Clause 5.3.3.2.2 |

Select the “yellow-marked” cases in the following Table 2.2.1-1 and Table 2.2.1-2 for ATG UE requirements.

- The CSI reporting is still useful for ATG scenario.

Reuse the CSI reporting cases for ATG scenario from the existing legacy CSI reporting cases.

Reference

R4-2309796, WF for ATG demodulation requirements, RAN4#107, CMCC