3GPP TSG-RAN WG4 Meeting #108	R4-2313648

# Toulouse, 21st - 25th Aug, 2023

Title: 	Simulation results on NR UE ATG demodulation requirements

Source: 	Huawei, HiSilicon

Agenda item:	8.13.5.2

Document for:	Discussion

- Background

During RAN4#107 meeting, WF [1] for ATG demodulation requirements was approved. In this contribution, we provide our updated simulation results about NR UE ATG demodulation requirements.

- Simulation

The ideal simulation results for NR UE ATG demodulation requirements are shown as following Table 2-1.

Table 2-1 Ideal simulation results for NR UE ATG demodulation requirements

| Case number | SCS | Bandwidth | TDD pattern | Antenna configuration | Propagation condition | Rank | SNR@70% max TP |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | MCS13 | MCS22 | MCS20 Table2 | MCS24 Table2 | MCS27 Table2 |
| 1 | 15kHz | 10MHz | / | 2x2 | AWGN + 220Hz Doppler | 1 | 3.6 | 10.9 | 16.0 | 19.7 | 23.4 |
| 2 | 15kHz | 10MHz | / | 2x4 | AWGN + 220Hz Doppler | 1 | 1.1 | 8.2 | 13.1 | 16.7 | 20.4 |
| 3 | 30kHz | 40MHz | 7DS2U S:6D+4G+4U | 2x2 | AWGN + 500Hz Doppler | 1 | 3.6 | 10.9 | 15.7 | 19.8 | 23.5 |
| 4 | 30kHz | 40MHz | 7DS2U S:6D+4G+4U | 2x4 | AWGN + 500Hz Doppler | 1 | 1.1 | 8.2 | 12.8 | 16.9 | 20.5 |
| 5 | 30kHz | 40MHz | 30D4S6U S:40G | 2x2 | AWGN + 500Hz Doppler | 1 | 3.6 | 10.9 | 15.7 | 19.8 | 23.5 |
| 6 | 30kHz | 40MHz | 30D4S6U S:40G | 2x4 | AWGN + 500Hz Doppler | 1 | 1.1 | 8.2 | 12.8 | 16.9 | 20.5 |

- Conclusion

In this contribution, we provide our updated simulation results about NR UE ATG demodulation requirements.

- Reference

[1]	R4-2309796, WF for ATG demodulation requirements, RAN4#107, CMCC

- Appendix: Simulation assumption

Simulation assumption for PDSCH

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Frequency range |  |  | FR1 | FR1 |
| Bandwidth |  | MHz | 10 | 40 |
| Subcarrier spacing |  | kHz | 15 | 30 |
| Duplex Mode |  |  | FDD | TDD |
| TDD Slot Configuration |  |  | N/A | 7D1S2U S:6D+4G+4U |
|  |  |  |  | 30D4S6U S:40G |
| Propagation channel |  |  | AWGN+220Hz doppler | AWGN+500Hz doppler |
| Antenna configuration |  |  | 2x2 | 2x2 |
|  |  |  | 2x4 | 2x4 |
| Beamforming Model |  |  | As defined in Annex B.4.1 in TS 38.101-4 | As defined in Annex B.4.1 in TS 38.101-4 |
| MCS/Rank |  |  | Rank 1 16QAM Table 1 MCS13 64QAM Table 1 MCS 22 256QAM Table 2 MCS 20/24/27 | Rank 1 16QAM Table 1 MCS13 64QAM Table 1 MCS 22 256QAM Table 2 MCS 20/24/27 |
| Receiver type |  |  | MMSE-IRC | MMSE-IRC |
| PDSCH configuration | Mapping type |  | Type A | Type A |
|  | Starting symbol (S) |  | 2 | 2 |
|  | Length (L) |  | 12 | 12 |
|  | PRB bundling size |  | 2 | 2 |
|  | PRB bundling type |  | Static | Static |
|  | VRB-to-PRB mapping interleaver bundle size |  | Non-interleaved | Non-interleaved |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 | Type 1 |
|  | Number of additional DMRS |  | 1 | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 | 1 |
| NZP CSI-RS for CSI acquisition | Row index (Note 3) |  | 3 | 3 |
|  | First subcarrier index in the PRB used for CSI-RS |  | k0 = 0 | k0 = 0 |
|  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 | l0 = 12 |
|  | Number of CSI-RS ports (X) |  | 2 | 2 |
|  | CDM Type |  | 'FD-CDM2' | 'FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 |
|  | CSI-RS periodicity | Slots | 20 | 40 |
|  | CSI-RS offset | Slots | 0 | 0 |
|  | Frequency Occupation |  | Start PRB 0 | Start PRB 0 |
|  |  |  | Number of PRB = ceil(BWP size/4)*4 | Number of PRB = ceil(BWP size/4)*4 |
|  | QCL info |  | TCI state #1 | TCI state #1 |
| ZP CSI-RS for CSI acquisition | Row index (Note 3) |  | 5 | 5 |
|  | First subcarrier index in the PRB used for CSI-RS |  | k0 = 4 | k0 = 4 |
|  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 | l0 = 12 |
|  | Number of CSI-RS ports (X) |  | 4 | 4 |
|  | CDM Type |  | 'FD-CDM2' | 'FD-CDM2' |
|  | Density (ρ) |  | 1 | 1 |
|  | CSI-RS periodicity | Slots | 20 | 40 |
|  | CSI-RS offset | Slots | 0 | 0 |
|  | Frequency Occupation |  | Start PRB 0 | Start PRB 0 |
|  |  |  | Number of PRB = ceil(BWP size/4)*4 | Number of PRB = ceil(BWP size/4)*4 |
| Maximum number of HARQ transmission |  |  | 4 | 4 |
| Number of HARQ Processes |  |  | 4 | 8 for 7DS2U 32 for 30D4S6U |
| Test metric |  |  | 70% of max throughput at target SNR. |  |