3GPP TSG-RAN WG4 Meeting #108	R4-2313652

# Toulouse, 21st - 25th Aug, 2023

Title: 	Discussion on UE PDSCH demodulation requirements for NR FR2 multi-Rx chain DL reception

Source: 	Huawei, HiSilicon

Agenda item:	8.7.4.2

Document for:	Discussion

Background

During RAN4#107 meeting, WF [1] on UE demodulation and CSI requirements for FR2 multi-Rx DL reception was approved. In this contribution, we share our views about UE PDSCH demodulation requirements for NR FR2 multi-Rx chain DL reception.

Discussion

PTRS port for sDCI schemes

<Way forward > - Proposals - Option 1: One PTRS port per TRP for sDCI schemes - Option 2: One PTRS port for sDCI SDM based scheme.

UE supporting one port per TRP for PTRS for single-DCI SDM scheme is up to UE capability, so we prefer to only configure one port PTRS for single-DCI SDM scheme for FR2 multi-Rx demodulation requirements.

| supportTwoPortDL-PTRS-r16 Indicates whether UE supports 2-port DL PT-RS. UE supports this feature should indicate support singleDCI-SDM-scheme-r16 for the band. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |

Only configure one port PTRS for single-DCI SDM scheme for FR2 multi-Rx demodulation requirements.

PDSCH rate matching in mTRP transmission

<Way forward > - Proposals - Option 1: PT-RS allocation does not overlap with PDSCH allocation per TRP. - Option 2: PT-RS allocation does not overlap with any PDSCH allocation.

From our understanding, in PDSCH rate matching, a PDSCH TB is not transmitted on the REs that PT-RS for the same PDSCH TB is transmitted. We confirm that PT-RS allocation does not overlap with PDSCH allocation per TRP.

Confirm that PT-RS allocation does not overlap with PDSCH allocation per TRP.

Test cases and simulation parameters

<Way forward > - Proposals - Option 1: Consider PDSCH test cases and simulation parameter for sDCI based SDM scheme as presented in Tables 1 and 2. Tables can be found in R4-2308945. <Way forward > - Proposals - Option 1: Consider PDSCH test cases and simulation parameter for mDCI based SDM scheme as presented in Tables 3 and 4. Tables can be found in R4-2308945. <Way forward > - Proposals - Option 1: Consider PDSCH test cases and simulation parameter for mDCI based non-overlapping scheme as presented in Tables 5 and 6. Tables can be found in R4-2308945.

For the sDCI SDM, mDCI fully overlapping and mDCI non-overlapping cases, it is better to configure PDSCH starting symbol 1 length 13 that is same as the existing FR2 requirements.

For the sDCI SDM, mDCI fully overlapping and mDCI non-overlapping cases, configure PDSCH starting symbol 1 length 13.

Proposals

In this contribution, we discuss on UE PDSCH demodulation requirements for NR FR2 multi-Rx chain DL reception. Our observations and proposals are:

- Only configure one port PTRS for single-DCI SDM scheme for FR2 multi-Rx demodulation requirements.

Confirm that PT-RS allocation does not overlap with PDSCH allocation per TRP.

For the sDCI SDM, mDCI fully overlapping and mDCI non-overlapping cases, configure PDSCH starting symbol 1 length 13 .

Reference

R4-2309824, WF on UE demodulation and CSI requirements for FR2 multi-Rx DL reception, RAN4#107, Qualcomm