3GPP TSG-RAN WG4 Meeting #108	R4-2313653

# Toulouse, 21st - 25th Aug, 2023

Title: 	Discussion on UE CSI reporting requirements for NR FR2 multi-Rx chain DL reception

Source: 	Huawei, HiSilicon

Agenda item:	8.7.4.3

Document for:	Discussion

Background

During RAN4#107 meeting, WF [1] on UE demodulation and CSI requirements for FR2 multi-Rx DL reception was approved. In this contribution, we share our views about UE CSI reporting requirements for NR FR2 multi-Rx chain DL reception.

Discussion

Simulation parameters for PMI reporting requirement for sDCI SDM scheme

<Way forward > - Proposals - Option 1: - Consider the parameters in Table 7 to define PMI reporting requirements for sDCI SDM based scheme. Table 7 can be found in R4-2308946. - Consider PDSCH Reference Channel for TDD PMI reporting requirements with UL-DL pattern FR2.120-1 as given by TS 38.101-4 Table A.3.2.2.5-8, with MCS Table 64QAM and MCS index MCS13 (16QAM with CR = 0.48).

Firstly, we think FR2.120-1 (DDDSU) TDD pattern is more typically used in FR2, and the corresponding CQI/RI/PMI delay should be 1.75ms. Also, the new reference channel should be defined rather than reuse the existing one since total two layer (1+1) is expected. For the propagation channel, TDLA30-35 can be used that is same as the existing cases. For the antenna configuration, considering that we have achieve the agreements to use 2 port per TRP, it is reasonable to use 2x2 ULA low antenna configuration.

Use FR2.120-1 (DDDSU) TDD pattern with CQI/RI/PMI delay 1.75ms for PMI reporting requirement for sDCI SDM scheme.

Define new reference channel for PMI reporting requirement for sDCI SDM scheme as following:

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | Modified from R.PDSCH.5-7.1 TDD |
| Channel bandwidth | MHz | 100 |
| Subcarrier spacing | kHz | 120 |
| Allocated resource blocks | PRBs | 66 |
| Number of consecutive PDSCH symbols |  | 12 |
| Allocated slots per 2 frames |  | 63 |
| MCS table |  | 64QAM |
| MCS index |  | 13 |
| Modulation |  | 16QAM |
| Target Coding Rate |  | 0.48 |
| Number of MIMO layers |  | 2 |
| Number of DMRS REs (Note 3) |  | 24 |
| Overhead for TBS determination |  | 6 |
| Information Bit Payload per Slot |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | Bits | N/A |
| For CSI-RS Slot i, if mod(i,5) =1 for i from {0,…,159} | Bits | N/A |
| For Slot i = 80 | Bits | 28680 |
| For Slot i, if mod(i, 5) = {0,2} for i from {1,…,79,82,…,159} | Bits | 28680 |
| Transport block CRC per Slot |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | Bits | N/A |
| For CSI-RS Slot i, if mod(i,5) =1 for i from {0,…,159} | Bits | N/A |
| For Slot i = 80 | Bits | 24 |
| For Slot i, if mod(i, 5) = {0,2} for i from {1,…,79,82,…,159} | Bits | 24 |
| Number of Code Blocks per Slot |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | CBs | N/A |
| For CSI-RS Slot i, if mod(i,5) =1 for i from {0,…,159} | CBs | N/A |
| For Slot i = 80 | CBs | 4 |
| For Slot i, if mod(i, 5) = {0,2} for i from {1,…,79,82,…,159} | CBs | 4 |
| Binary Channel Bits Per Slot |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | Bits | N/A |
| For CSI-RS Slot i, if mod(i,5) =1 for i from {0,…,159} | Bits | N/A |
| For Slot i = 80 | Bits | 57648 |
| For Slot i, if mod(i, 5) = {0,2} for i from {1,…,79,82,…,159} | Bits | 60720 |
| Max. Throughput averaged over 2 frames | Mbps | 90.342 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data |  |  |

Use TDLA30-35 propagation channel for PMI reporting requirement for sDCI SDM scheme.

Use 2x2 ULA low antenna configuration for PMI reporting requirement for sDCI SDM scheme.

Proposals

In this contribution, we discuss on UE CSI reporting requirements for NR FR2 multi-Rx chain DL reception. Our observations and proposals are:

- Use FR2.120-1 (DDDSU) TDD pattern with CQI/RI/PMI delay 1.75ms for PMI reporting requirement for sDCI SDM scheme.

Define new reference channel for PMI reporting requirement for sDCI SDM scheme as following:

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | Modified from R.PDSCH.5-7.1 TDD |
| Channel bandwidth | MHz | 100 |
| Subcarrier spacing | kHz | 120 |
| Allocated resource blocks | PRBs | 66 |
| Number of consecutive PDSCH symbols |  | 12 |
| Allocated slots per 2 frames |  | 63 |
| MCS table |  | 64QAM |
| MCS index |  | 13 |
| Modulation |  | 16QAM |
| Target Coding Rate |  | 0.48 |
| Number of MIMO layers |  | 2 |
| Number of DMRS REs (Note 3) |  | 24 |
| Overhead for TBS determination |  | 6 |
| Information Bit Payload per Slot |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | Bits | N/A |
| For CSI-RS Slot i, if mod(i,5) =1 for i from {0,…,159} | Bits | N/A |
| For Slot i = 80 | Bits | 28680 |
| For Slot i, if mod(i, 5) = {0,2} for i from {1,…,79,82,…,159} | Bits | 28680 |
| Transport block CRC per Slot |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | Bits | N/A |
| For CSI-RS Slot i, if mod(i,5) =1 for i from {0,…,159} | Bits | N/A |
| For Slot i = 80 | Bits | 24 |
| For Slot i, if mod(i, 5) = {0,2} for i from {1,…,79,82,…,159} | Bits | 24 |
| Number of Code Blocks per Slot |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | CBs | N/A |
| For CSI-RS Slot i, if mod(i,5) =1 for i from {0,…,159} | CBs | N/A |
| For Slot i = 80 | CBs | 4 |
| For Slot i, if mod(i, 5) = {0,2} for i from {1,…,79,82,…,159} | CBs | 4 |
| Binary Channel Bits Per Slot |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | Bits | N/A |
| For CSI-RS Slot i, if mod(i,5) =1 for i from {0,…,159} | Bits | N/A |
| For Slot i = 80 | Bits | 57648 |
| For Slot i, if mod(i, 5) = {0,2} for i from {1,…,79,82,…,159} | Bits | 60720 |
| Max. Throughput averaged over 2 frames | Mbps | 90.342 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data |  |  |

Use TDLA30-35 propagation channel for PMI reporting requirement for sDCI SDM scheme.

Use 2x2 ULA low antenna configuration for PMI reporting requirement for sDCI SDM scheme.

Reference

R4-2309824, WF on UE demodulation and CSI requirements for FR2 multi-Rx DL reception, RAN4#107, Qualcomm