3GPP TSG-RAN WG4 Meeting #108	R4-2313654

# Toulouse, 21st - 25th Aug, 2023

Title: 	Discussion on deployment and channel modelling for HST FR2

Source: 	Huawei, HiSilicon

Agenda item:	8.12.5.1

Document for:	Discussion

Background

During RAN4#107 meeting, WF [1] for FR2 HST demodulation requirements was approved. In this contribution, we share our views about deployment and channel modelling for HST FR2.

Discussion

Simultaneous Rx reception in open space scenario

Deployment Scenario

Agreement: - RAN4 will consider both scenario A and scenario B as starting point for initial evaluation with simultaneous multi-Rx reception - FFS whether single requirement or separate requirements need to be introduced for scenario A and scenario B in Bi-directional deployment scenario for PDSCH demodulation requirements

Based on our simulation results in our simulation paper, there is negligible performance difference between scenario A and scenario B, so only defining one set of requirements is more reasonable. From our understanding, the large frequency jump handling is the most important aspect to be verified in the HST demodulation test. Here we list the maximum frequency jump value for different scenarios as following Table 2.1.1-1.

Table 2.1.1-1 Maximum frequency jump value for different scenarios

| Parameter | Description | Scenario A | Scenario B-1 | Scenario B-2 |
| --- | --- | --- | --- | --- |
| Ds | Inter-RRH distance | 700m | 700m | 700m |
| Dmin | Distance between rail track and RRH | 10m | 150m | 150m |
| v | Train velocity | 350km/h | 350km/h | 350km/h |
| fd | Maximum Doppler frequency shift | 9722Hz | 9722Hz | 9722Hz |
| fc | Carrier frequency | 30GHz | 30GHz | 30GHz |
| Drx_panel | Distance between two UE Rx panels | 0m | 0m | 0m |
| Ds_offset | Switching transmission point between adjacent RRHs | 10m | 100m | 350m |
| Maximum frequency jump |  | 2847Hz | 4163Hz | 688Hz |

Since that Scenario B-1 provide the largest maximum frequency jump value, we propose to only consider Scenario B-1 for Bi-directional deployment scenario for PDSCH demodulation requirements.

Only consider Scenario B-1 for Bi-directional deployment scenario for PDSCH demodulation requirements.

Channel Model

Agreement: - No need to model the relative propagation delay from the visible RRH into the channel modelling - FFS whether need to relative power for channel modelling - The following model on doppler shift trajectory can be considered for PDSCH requirement with multi-Rx reception in Bi-directional deployment scenario as baseline - FFS on the starting point used - Doppler shift fst=fd cosθt (Hz) from the left Tx panel of RRH for PDSCH received at right Rx panel of UE is given by cosθt=Ds_offset +Ds-vtDmin2+Ds_offset+Ds-vt2, 0<t≤Dsv cosθt=cosθt modDsv, t>Ds/v - Doppler shift fst=fd cosθt (Hz) from the right Tx panel of RRH for PDSCH received at left Rx panel of UE is given by cosθt=-(Ds-Dsoffset +vt)Dmin2+Ds-Dsoffset +vt2, 0<t≤2*Ds_offsetv cosθt=-(Dsoffset +vt)Dmin2+Dsoffset +vt2, 2*Ds_offsetv<t≤Dsv cosθt=cosθt modDsv, t>Ds/v.

Whether to consider power modelling is related to the detailed UE implementation, e.g. FFT processing. In the real scenario, when UE is near one TRP, there is large received power difference between two Rx panels. In this case, not considering power modelling means two independent FFT window is assumed and therefore the UE implementation is limited. From our understanding, the UE implementation should not be limited, so we propose to model the relative power for HST FR2 multi-Rx channel modelling. The starting point t=0 is corresponding to the location where UE is right under the TRP#0.

Model the relative power for HST FR2 multi-Rx channel modelling as following:

- Power level Pk (dB) for the signal from kth RRH, normalized to the total power received from all visible RRHs, is given by:

  - Pk=-20lgy-xk-10lgi∈i|i*Ds±Ds-Ds_offset≤a<i*Ds±Ds+Ds_offset1y-xi2 for k*Ds±Ds-Ds_offset≤a<k*Ds±Ds+Ds_offset

  - Pk=-20lgy-xk-10lgi∈i|i*Ds+Ds_offset≤a+0, 1*Ds<i*Ds+Ds-Ds_offset1y-xi2 for k*Ds+Ds_offset≤a+0, 1*Ds<k*Ds+Ds-Ds_offset

Proposals

In this contribution, we discuss on deployment and channel modelling for HST FR2. Our observations and proposals are:

- Only consider Scenario B-1 for Bi-directional deployment scenario for PDSCH demodulation requirements.

Model the relative power for HST FR2 multi-Rx channel modelling as following:

- Power level Pk (dB) for the signal from kth RRH, normalized to the total power received from all visible RRHs, is given by:

  - Pk=-20lgy-xk-10lgi∈i|i*Ds±Ds-Ds_offset≤a<i*Ds±Ds+Ds_offset1y-xi2 for k*Ds±Ds-Ds_offset≤a<k*Ds±Ds+Ds_offset

  - Pk=-20lgy-xk-10lgi∈i|i*Ds+Ds_offset≤a+0, 1*Ds<i*Ds+Ds-Ds_offset1y-xi2 for k*Ds+Ds_offset≤a+0, 1*Ds<k*Ds+Ds-Ds_offset

Reference

R4-2309825, WF for FR2 HST demodulation requirements, RAN4#107, Samsung