3GPP TSG-RAN WG4 Meeting #108	R4-2313657

# Toulouse, 21st - 25th Aug, 2023

Title: 	Discussion on UE multi-Rx demodulation requirements for HST FR2

Source: 	Huawei, HiSilicon

Agenda item:	8.12.5.3

Document for:	Discussion

Background

During RAN4#107 meeting, WF [1] for FR2 HST demodulation requirements was approved. In this contribution, we share our views about UE multi-Rx demodulation requirements for HST FR2.

Discussion

UE processing assumption for the FFT window

Way Forward - Proposals - Option 1: RAN4 shall consider that the CPE will have independent FFT per panel - Option 2: - Do not specify baseline UE processing assumption for the FFT window and leave it to UE implementation for FR2 HST performance requirements definition. In case large span is observed, additional margin should be added. - Option3 - Independent FFT window for each panel for the scenario the reception time difference between different TRPs is larger than one CP.

From our understanding, both UE processing assumption for the FFT window are feasible under HST scenario with MRTD larger than CP that is considered in RRM session. So we propose to not specify baseline UE processing assumption for the FFT window and leave it to UE implementation for FR2 HST performance requirements definition. In case large span is observed, additional margin should be added.

Do not specify baseline UE processing assumption for the FFT window and leave it to UE implementation for FR2 HST performance requirements definition. In case large span is observed, additional margin should be added.

Transmission scheme & applicability rule

Agreement: - FFS on sDCI schemes Way forward: - FFS whether to introduce FR2 HST Demodulation tests for simultaneous reception with sDCI, and related applicability rule to skip sDCI testing if the device supports mDCI

Considering that there is large power difference between the signal received from two TRP, we propose to only consider multi-DCI based multi-TRP with full-overlapping resources allocation as high priority and not consider single-DCI based multi-TRP scheduling.

Do not consider single-DCI based multi-TRP scheduling.

Proposals

In this contribution, we discuss on UE multi-Rx demodulation requirements for HST FR2. Our observations and proposals are:

- Do not specify baseline UE processing assumption for the FFT window and leave it to UE implementation for FR2 HST performance requirements definition. In case large span is observed, additional margin should be added.

Do not consider single-DCI based multi-TRP scheduling.

Reference

R4-2309825, WF for FR2 HST demodulation requirements, RAN4#107, Samsung