3GPP TSG-RAN WG4 Meeting #108	R4-2313659

# Toulouse, 21st - 25th Aug, 2023

Title: 	Discussion on reference tunnel deployment scenario

Source: 	Huawei, HiSilicon

Agenda item:	8.12.5.4

Document for:	Discussion

Background

During RAN4#107 meeting, WF [1] on demodulation requirements for Rel-18 FR2 HST was approved. In this contribution, we share our views about HST FR2 reference tunnel deployment scenario.

Discussion

Test scope of demodulation requirements of tunnel scenario

UE demodulation requirements

Way forward: - Option 1 - Introduce the UE demodulation requirements with DPS transmission scheme with 1a and 1b for tunnel scenario. - Uni-directional with DPS scheme 1b - Bi-directional with DPS scheme 1a - Option 2 - RAN4 to define new demodulation requirements for tunnel deployment scenario only if a new channel model in the tunnel is introduced. - Option 3 - RAN4 should agree to not introduce dedicated Demodulation requirements for Tunnel Deployment, considering this scenario covered by existing test cases; - Option 4 - UE demodulation requirements can be defined only if new channel model with multi-path propagation introduced.

From our understanding, the large frequency jump handling is the most important aspect to be verified in the HST demodulation test. For the UE side, in Rel-17 FR2 HST WI, various deployments are studied and one requirement are defined using Bi-directional Scenario-B model considering that Bi-directional Scenario-B model provide largest frequency jump among all deployments (i.e. Uni-directional Scenario-A, Uni-directional Scenario-B and Bi-directional Scenario-B). From the test coverage point of view, another requirement is defined in Scenario-A.

In the tunnel scenario, if the existing FR2 HST-DPS channel model is reused with some modification based on agreed Ds and Dmin, the performance is similar as the Scenario-A since that the maximum frequency jump is almost same for two scenarios, as shown in the following Figure 2.1.1-1. So we proposed that UE demodulation requirements can be defined only if new channel model with multi-path propagation introduced.

Figure 2.1.1-1 Doppler trajectory for Uni-directional Scenario-A and tunnel deployment

The maximum frequency jump is almost same for two scenarios if the existing FR2 HST-DPS channel model Scenario is reused with some modification based on agreed Ds and Dmin.

UE demodulation requirements can be defined only if new channel model with multi-path propagation introduced.

BS demodulation requirements

Way forward: - Option 1 - Introduce single set requirement for PUSCH in tunnel scenario based on Bi-directional scenario in the tunnel scenario. - Option 2 - RAN4 to define new demodulation requirements for tunnel deployment scenario only if a new channel model in the tunnel is introduced. - UE demodulation requirements can be defined only if new channel model with multi-path propagation introduced. - Option 3 - UE demodulation requirements can be defined only if new channel model with multi-path propagation introduced.

From our understanding, the large frequency jump handling is the most important aspect to be verified in the HST demodulation test. For the BS side, in Rel-17 FR2 HST WI, various deployments are studied and only one set of requirements are defined using Bi-directional Scenario-B model considering that Bi-directional Scenario-B model provide largest frequency jump among all deployments (i.e. Uni-directional Scenario-A, Uni-directional Scenario-B and Bi-directional Scenario-B).

In the tunnel scenario, if the existing FR2 HST-DPS channel model is reused with some modification based on agreed Ds and Dmin, the performance is similar as the Scenario-A since that the maximum frequency jump is almost same for two scenarios, as shown in the following Figure 2.1.2-1. So we proposed that BS demodulation requirements can be defined only if new channel model with multi-path propagation introduced.

Figure 2.1.2-1 Doppler trajectory for Uni-directional Scenario-A and tunnel deployment

The maximum frequency jump is almost same for two scenarios if the existing FR2 HST-DPS channel model Scenario is reused with some modification based on agreed Ds and Dmin.

BS demodulation requirements can be defined only if new channel model with multi-path propagation introduced.

Proposals

In this contribution, we discuss on HST FR2 reference tunnel deployment scenario. Our observations and proposals are:

- The maximum frequency jump is almost same for two scenarios if the existing FR2 HST-DPS channel model Scenario is reused with some modification based on agreed Ds and Dmin.

- UE demodulation requirements can be defined only if new channel model with multi-path propagation introduced.

The maximum frequency jump is almost same for two scenarios if the existing FR2 HST-DPS channel model Scenario is reused with some modification based on agreed Ds and Dmin.

BS demodulation requirements can be defined only if new channel model with multi-path propagation introduced.

Reference

R4-2309825, WF for FR2 HST demodulation requirements, RAN4#107, Samsung