3GPP TSG-RAN WG4 Meeting #108	R4-2313660

# Toulouse, 21st - 25th Aug, 2023

Title: 	Discussion on demodulation requirements for NR network-controlled repeaters

Source: 	Huawei, HiSilicon

Agenda item:	8.28.6

Document for:	Discussion

Background

During RAN4#107 meeting, WF [1] on NCR_MT demodulation requirements was approved. In this contribution, we share our views about demodulation performance requirements for NR network-controlled repeaters.

Discussion

General

FR1 requirements

Agreement: - 10MHz for 15kHz SCS and 40MHz for 30kHz SCS for NCR-MT demodulation requirements. - FFS whether need to consider 5MHz/15kHz

For the channel bandwidth, we think the only one typical value is enough, i.e. 10MHz for 15kHz SCS and 40MHz for 30kHz SCS that is the same value used for the legacy UE requirements and IAB-MT requirements.

Only consider 10MHz for 15kHz SCS and 40MHz for 30kHz SCS for NCR-MT demodulation requirements.

    - FR2 requirements

Agreement: - Option 1: New FR2 requirements shall be defined on PDCCH for signaling of the indication of changing the Access link beam. - Option 2: Do not define new requirements on PDCCH for signaling of Access link beam change indication.

RAN4 do not define performance requirements for all new signaling but just focus on the UE baseband processing. From demodulation point of view, we don’t see any UE algorithm change for the new signaling of Access link beam change indication. So we prefer to not define new requirements on PDCCH for signaling of Access link beam change indication.

Do not define new requirements on PDCCH for signaling of Access link beam change indication.

PDSCH

MCS

Agreement: - MCS 4 (QPSK) for FR1 - QPSK is the same as legacy UE for FR2

As per TS 38.101-4, QPSK with CR 0.30 (MCS4) is used for FR2 legacy UE case, so we confirm that MCS4 is used for FR2 NCR PDSCH requirements.

Confirm that MCS4 is used for FR2 NCR PDSCH requirements.

PDCCH

PDCCH requirements

Agreement: - Reuse IAB-MT and legacy UE requirements as much as possible as baseline. - FFS for adapt PDCCH requirement for testing of DCI type 2_8 used for access link beam change indication. - Interesting companies can bring simulation results for DCI type 2_8.

Same as we discussed in section 2.1.2, RAN4 do not define performance requirements for all new signaling but just focus on the UE baseband processing. From demodulation point of view, we don’t see any UE algorithm change for the new signaling of Access link beam change indication. Therefore, to save simulation effort, we propose to reuse same DCI type from IAB-MT and legacy UE requirements for NCR PDCCH requirements.

Reuse same DCI type from IAB-MT and legacy UE requirements for NCR PDCCH requirements.

CSI

PMI requirements

Agreement: - Option1: Define PMI requirements for MCR-MT. - Option 1A: Testing of performance requirements for PMI reporting is optional (Reuse from IAB-MT). - Option 2: No

For the NCR-MT CSI reporting requirements, the only issue is that whether to consider PMI reporting cases. From our understanding, the NCR has the similar deployment scenario as the IAB. The PMI reporting cases for the IAB-MT are already defined in the specification TS 38.174, so we prefer to define PMI reporting cases for the NCR-MT with the test parameters reused from IAB. Since that there is applicability rule defined for the IAB that “Testing of performance requirements for RI and PMI reporting is optional”, we propose to reuse the same applicability for the NCR PMI reporting case to allow different BS implementation.

Define PMI reporting cases for the NCR-MT with the test parameters reused from IAB.

Define applicability rule that “Testing of performance requirements for PMI reporting is optional”.

List for NCR requirements

Overall, we give the following list for NCR requirements.

FR1 PDSCH (4 new simulation is needed)

| Case number | Bandwidth (MHz) | SCS (kHz) | Propagation condition | Antenna configuration | Test metric |
| --- | --- | --- | --- | --- | --- |
| 1 | 10 | 15 | TDLA30-10 | 2x2 | 70% max throughput 1% BLER |
| 2 | 10 | 15 | TDLA30-10 | 2x4 | 70% max throughput 1% BLER |
| 3 | 40 | 30 | TDLA30-10 | 2x2 | 70% max throughput 1% BLER |
| 4 | 40 | 30 | TDLA30-10 | 2x4 | 70% max throughput 1% BLER |

| Parameter | Unit | Case 1/2 | Case 3/4 |
| --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 40 |
| Subcarrier spacing | kHz | 15 | 30 |
| Allocated resource blocks | PRBs | 52 | 106 |
| Number of consecutive PDSCH symbols |  | 12 | 12 |
| MCS table |  | 64QAM | 64QAM |
| MCS index |  | 4 | 4 |
| Modulation |  | QPSK | QPSK |
| Target Coding Rate |  | 0.30 | 0.30 |
| Number of MIMO layers |  | 1 | 1 |
| Number of DMRS REs |  | 12 | 12 |
| Overhead for TBS determination |  | 0 | 0 |
| Information Bit Payload per Slot |  | 4096 | 8456 |
| Transport block CRC per Slot |  | 24 | 24 |
| Number of Code Blocks per Slot |  | 1 | 2 |
| Binary Channel Bits Per Slot |  | 13728 | 27984 |

FR1 PDCCH (1 new simulation is needed)

| Case number | Bandwidth (MHz) | SCS (kHz) | CORESET RB | CORESET duration | Aggregation level | Propagation condition | Antenna configuration | Test metric | SNR reused (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 10 | 15 | 24 | 2 | 2 | TDLA30-10 | 1x2 | 1% BLER | 8.1 |
| 2 | 10 | 15 | 24 | 2 | 2 | TDLA30-10 | 1x4 | 1% BLER | 2.2 |
| 3 | 10 | 15 | 48 | 2 | 4 | TDLA30-10 | 1x2 | 1% BLER | 5.5 |
| 4 | 10 | 15 | 48 | 2 | 4 | TDLA30-10 | 1x4 | 1% BLER | 0.2 |
| 5 | 10 | 15 | 48 | 1 | 8 | TDLA30-10 | 2x2 | 1% BLER | -0.2 |
| 6 | 10 | 15 | 48 | 1 | 8 | TDLA30-10 | 2x4 | 1% BLER | x |
| 7 | 40 | 30 | 102 | 1 | 2 | TDLA30-10 | 1x2 | 1% BLER | 7.0 |
| 8 | 40 | 30 | 102 | 1 | 2 | TDLA30-10 | 1x4 | 1% BLER | 2.1 |
| 9 | 40 | 30 | 102 | 1 | 4 | TDLA30-10 | 1x2 | 1% BLER | 4.9 |
| 10 | 40 | 30 | 102 | 1 | 4 | TDLA30-10 | 1x4 | 1% BLER | 0.7 |
| 11 | 40 | 30 | 90 | 1 | 8 | TDLA30-10 | 2x2 | 1% BLER | -0.7 |
| 12 | 40 | 30 | 90 | 1 | 8 | TDLA30-10 | 2x4 | 1% BLER | -4.1 |

| Parameter | Unit | Case 1/2 | Case 3/4 | Case 5/6 | Case 7/8 | Case 9/10 | Case 11/12 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 30 | 30 | 30 |
| CORESET frequency domain allocation |  | 24 | 48 | 48 | 102 | 102 | 90 |
| CORESET time domain allocation |  | 2 | 2 | 1 | 1 | 1 | 1 |
| Aggregation level |  | 2 | 4 | 8 | 2 | 4 | 8 |
| DCI Format |  | 1_0 | 1_1 | 1_1 | 1_0 | 1_1 | 1_1 |
| Payload (without CRC) | Bits | 39 | 52 | 52 | 41 | 53 | 53 |

FR1 CQI (no new simulation is needed)

| Parameter |  | Unit | Test1 |  | Test2 |  | Test3 |  | Test4 |  | Test5 |  | Test6 |  | Test7 |  | Test8 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |  |  | 10 |  |  |  | 40 |  |  |  | 40 |  |  |  |
| Subcarrier spacing |  | kHz | 15 |  |  |  | 15 |  |  |  | 30 |  |  |  | 30 |  |  |  |
| Default TDD UL-DL pattern (Note 1) |  |  | 3D1S1U, S=10D:2G:2U |  |  |  | 3D1S1U, S=10D:2G:2U |  |  |  | 7D1S2U, S=6D:4G:4U |  |  |  | 7D1S2U, S=6D:4G:4U |  |  |  |
| SNR |  | dB | 8 | 9 | 14 | 15 | 5 | 6 | 11 | 12 | 8 | 9 | 14 | 15 | 5 | 6 | 11 | 12 |
| Propagation channel |  |  | AWGN |  |  |  | AWGN |  |  |  | AWGN |  |  |  | AWGN |  |  |  |
| Antenna configuration |  |  | 2x2 |  |  |  | 2x4 |  |  |  | 2x2 |  |  |  | 2x4 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic |  |  |  | Periodic |  |  |  | Periodic |  |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  | 2 |  |  |  | 2 |  |  |  | 2 |  |  |  | 2 |  |  |  |
|  | CDM Type |  | FD-CDM2 |  |  |  | FD-CDM2 |  |  |  | FD-CDM2 |  |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  | 1 |  |  |  | 1 |  |  |  | 1 |  |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1) |  | Row 3,(6,-) |  |  |  | Row 3,(6,-) |  |  |  | Row 3,(6,-) |  |  |  | Row 3,(6,-) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 13 |  |  |  | 13 |  |  |  | 13 |  |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset | slot | 5/1 |  |  |  | 5/1 |  |  |  | 10/1 |  |  |  | 10/1 |  |  |  |
| ReportConfigType |  |  | Periodic |  |  |  | Periodic |  |  |  | Periodic |  |  |  | Periodic |  |  |  |
| CQI-table |  |  | Table 2 |  |  |  | Table 2 |  |  |  | Table 2 |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |  |  | cri-RI-PMI-CQI |  |  |  | cri-RI-PMI-CQI |  |  |  | cri-RI-PMI-CQI |  |  |  |
| cqi-FormatIndicator |  |  | Wideband |  |  |  | Wideband |  |  |  | Wideband |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  | Wideband |  |  |  | Wideband |  |  |  | Wideband |  |  |  | Wideband |  |  |  |
| Sub-band Size |  | RB | 8 |  |  |  | 8 |  |  |  | 16 |  |  |  | 16 |  |  |  |
| Csi-ReportingBand |  |  | 1111111 |  |  |  | 1111111 |  |  |  | 1111111 |  |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  | slot | 5/0 |  |  |  | 5/0 |  |  |  | 10/9 |  |  |  | 10/9 |  |  |  |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |  |  |  | typeI-SinglePanel |  |  |  | typeI-SinglePanel |  |  |  | typeI-SinglePanel |  |  |  |
|  | Codebook Mode |  | 1 |  |  |  | 1 |  |  |  | 1 |  |  |  | 1 |  |  |  |
|  | (CodebookConfig-N1, CodebookConfig-N2) |  | Not configured |  |  |  | Not configured |  |  |  | Not configured |  |  |  | Not configured |  |  |  |
|  | CodebookSubsetRestriction |  | 010000 |  |  |  | 010000 |  |  |  | 010000 |  |  |  | 010000 |  |  |  |
|  | RI Restriction |  | N/A |  |  |  | N/A |  |  |  | N/A |  |  |  | N/A |  |  |  |
| CQI/RI/PMI delay |  | ms | 8 |  |  |  | 8 |  |  |  | 9.5 |  |  |  | 9.5 |  |  |  |
| Maximum number of HARQ transmission |  |  | 1 |  |  |  | 1 |  |  |  | 1 |  |  |  | 1 |  |  |  |
| Note 1: The same requirements are applicable for FDD and TDD with different UL-DL pattern. Note 2: If the NCR-MT reports in an available uplink reporting instance at slot #n based on CQI estimation at a downlink slot not later than slot#(n-4), this reported CQI cannot be applied at the gNB downlink before slot#(n+4). Note 3: SSB, TRS, CSI-RS, and/or other unspecified test parameters with respect to TS 38.101-4 are left up to test implementation, if transmitted or needed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

FR1 PMI (no new simulation is needed)

| Parameter |  | Unit | Test1 | Test2 | Test3 | Test4 | Test5 | Test6 | Test7 | Test8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  | 10 |  | 40 |  | 40 |  |
| Subcarrier spacing |  | kHz | 15 |  | 15 |  | 30 |  | 30 |  |
| TDD DL-UL configuration (Note 1) |  |  | 3D1S1U, S=10D:2G:2U |  | 3D1S1U, S=10D:2G:2U |  | 7D1S2U, S=6D:4G:4U |  | 7D1S2U, S=6D:4G:4U |  |
| Propagation channel |  |  | TDLA30-5 |  | TDLA30-5 |  | TDLA30-5 |  | TDLA30-5 |  |
| Antenna configuration |  |  | High XP 4 x 2 (N1,N2) = (2,1) | High XP 8 x 2 (N1,N2) = (4,1) | High XP 4 x 4 (N1,N2) = (2,1) | High XP 8 x 4 (N1,N2) = (4,1) | High XP 4 x 2 (N1,N2) = (2,1) | High XP 8 x 2 (N1,N2) = (4,1) | High XP 4 x 4 (N1,N2) = (2,1) | High XP 8 x 4 (N1,N2) = (4,1) |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic |  | Periodic |  | Periodic |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  | 4 | 8 | 4 | 8 | 4 | 8 | 4 | 8 |
|  | CDM Type |  | FD-CDM2 | CDM4 (FD2, TD2) | FD-CDM2 | CDM4 (FD2, TD2) | FD-CDM2 | CDM4 (FD2, TD2) | FD-CDM2 | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 |  | 1 |  | 1 |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1) |  | Row 4, (0,-) | Row 8, (4,6) | Row 4, (0,-) | Row 8, (4,6) | Row 4, (0,-) | Row 8, (4,6) | Row 4, (0,-) | Row 8, (4,6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (13,-) | (5,-) | (13,-) | (5,-) | (13,-) | (5,-) | (13,-) | (5,-) |
|  | NZP CSI-RS-timeConfig periodicity and offset | slot | 5/1 |  | 5/1 |  | 10/1 |  | 10/1 |  |
| ReportConfigType |  |  | Periodic |  | Periodic |  | Periodic |  | Periodic |  |
| CQI-table |  |  | Table 1 |  | Table 1 |  | Table 1 |  | Table 1 |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  | cri-RI-PMI-CQI |  | cri-RI-PMI-CQI |  | cri-RI-PMI-CQI |  |
| cqi-FormatIndicator |  |  | Wideband |  | Wideband |  | Wideband |  | Wideband |  |
| pmi-FormatIndicator |  |  | Wideband |  | Wideband |  | Wideband |  | Wideband |  |
| Sub-band Size |  | RB | 8 |  | 8 |  | 16 |  | 16 |  |
| csi-ReportingBand |  |  | 1111111 |  | 1111111 |  | 1111111 |  | 1111111 |  |
| CSI-Report interval and offset |  | slot | Not configured |  | Not configured |  | Not configured |  | Not configured |  |
| CSI-Report periodicity and offset |  |  | 5/0 |  | 5/0 |  | 10/9 |  | 10/9 |  |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |  | typeI-SinglePanel |  | typeI-SinglePanel |  | typeI-SinglePanel |  |
|  | Codebook Mode |  | 1 |  | 1 |  | 1 |  | 1 |  |
|  | (CodebookConfig-N1, CodebookConfig-N2) |  | (2,1) | (4,1) | (2,1) | (4,1) | (2,1) | (4,1) | (2,1) | (4,1) |
|  | (CodebookConfig-O1, CodebookConfig-O2) |  | (4,1) |  | (4,1) |  | (4,1) |  | (4,1) |  |
|  | CodebookSubsetRestriction |  | 11111111 | 0x FFFF | 11111111 | 0x FFFF | 11111111 | 0x FFFF | 11111111 | 0x FFFF |
|  | RI Restriction |  | 00000001 | 00000010 | 00000001 | 00000010 | 00000001 | 00000010 | 00000001 | 00000010 |
| CQI/RI/PMI delay |  | ms | 5.5 | 6.5 | 5.5 | 6.5 | 5.5 | 6.5 | 5.5 | 6.5 |
| Maximum number of HARQ transmission |  |  | 4 |  | 4 |  | 4 |  | 4 |  |
| Note 1: The same requirements are applicable for TDD with different UL-DL pattern. Note 2: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (0.5 ms granularity) with equal probability of each applicable i1, i2 combination. Note 3: If the NCR-MT reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4) for test 1 and not later than slot#(n-6) for test 2, this reported PMI cannot be applied at the gNB downlink before slot#(n+4) for test 1 and before slot#(n+6) for test 2. Note 4: Randomization of the principle beam direction shall be used. Note 5: SSB, TRS, CSI-RS, and/or other unspecified test parameters with respect to TS 38.101-4 are left up to test implementation, if transmitted or needed. |  |  |  |  |  |  |  |  |  |  |

| Parameter | Test1 | Test2 | Test3 | Test4 | Test5 | Test6 | Test7 | Test8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.3 | 1.5 | 1.3 | 1.5 | 1.3 | 1.5 | 1.3 | 1.5 |

FR2 PDSCH (2 new simulation is needed)

| Case number | Bandwidth (MHz) | SCS (kHz) | Propagation condition | Antenna configuration | Test metric |
| --- | --- | --- | --- | --- | --- |
| 1 | 100 | 120 | TDLA30-75 | 2x2 | 70% max throughput 1% BLER |
| 2 | 100 | 120 | TDLA30-75 | 2x4 | 70% max throughput 1% BLER |

| Parameter | Unit | Case 1/2 |
| --- | --- | --- |
| Channel bandwidth | MHz | 100 |
| Subcarrier spacing | kHz | 120 |
| Allocated resource blocks | PRBs | 66 |
| Number of consecutive PDSCH symbols |  | 13 |
| MCS table |  | 64QAM |
| MCS index |  | 4 |
| Modulation |  | QPSK |
| Target Coding Rate |  | 0.30 |
| Number of MIMO layers |  | 1 |
| Number of DMRS REs |  | 12 |
| Overhead for TBS determination |  | 0 |
| Information Bit Payload per Slot |  | 5504 |
| Transport block CRC per Slot |  | 24 |
| Number of Code Blocks per Slot |  | 1 |
| Binary Channel Bits Per Slot |  | 18282 |

FR2 PDCCH (no new simulation is needed)

| Case number | Bandwidth (MHz) | SCS (kHz) | CORESET RB | CORESET duration | Aggregation level | Propagation condition | Antenna configuration | Test metric | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 100 | 120 | 60 | 1 | 2 | TDLA30-75 | 1x2 | 1% BLER | 6.4 |
| 2 | 100 | 120 | 60 | 1 | 4 | TDLA30-75 | 1x2 | 1% BLER | 2.9 |
| 3 | 100 | 120 | 60 | 1 | 8 | TDLA30-75 | 2x2 | 1% BLER | 0.1 |

| Parameter | Unit | Case 1 | Case 2 | Case 3 |
| --- | --- | --- | --- | --- |
| Subcarrier spacing | kHz | 30 | 30 | 30 |
| CORESET frequency domain allocation |  | 102 | 102 | 90 |
| CORESET time domain allocation |  | 1 | 1 | 1 |
| Aggregation level |  | 2 | 4 | 8 |
| DCI Format |  | 1_0 | 1_1 | 1_1 |
| Payload (without CRC) | Bits | 41 | 53 | 53 |

FR2 CQI (no new simulation is needed)

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 100 |  |  |  |
| Subcarrier spacing |  |  | kHz | 120 |  |  |  |
| Duplex Mode |  |  |  | TDD |  |  |  |
| Default TDD UL-DL pattern (Note 1) |  |  |  | 3D1S1U |  |  |  |
| Special Slot Configuration |  |  |  | 10D+2G+2U |  |  |  |
| SNRBB |  |  | dB | 8 | 9 | 14 | 15 |
| Propagation channel |  |  |  | AWGN |  |  |  |
| Antenna configuration |  |  |  | 2×2 with static channel specified in Annex I.1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 2 |  |  |  |
|  | CDM Type |  |  | fd-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | 6 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 5/1 |  |  |  |
| ReportConfigType |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  | Table 1 |  |  |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  | RB | 8 |  |  |  |
| csi-ReportingBand |  |  |  | 111111111 |  |  |  |
| CSI-Report periodicity and offset |  |  | slot | 5/0 |  |  |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  | 010000 |  |  |  |
|  |  | RI Restriction |  | N/A |  |  |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |  |  |
| Note 1: The same requirements are applicable to with different UL-DL patterns. Note 2: If the NCR-MT reports in an available uplink reporting instance at slot #n based on CQI estimation at a downlink slot not later than slot#(n-4), this reported CQI cannot be applied at the gNB downlink before slot#(n+4). Note 3: SSB, TRS, CSI-RS, and/or other unspecified test parameters with respect to TS 38.101-4 are left up to test implementation, if transmitted or needed. |  |  |  |  |  |  |  |

FR2 PMI (no new simulation is needed)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 100 |
| Subcarrier spacing |  | kHz | 120 |
| Default TDD UL-DL pattern (Note 1) |  |  | 3D1S1U |
| Special Slot Configuration |  |  | 10D+2G+2U |
| Propagation channel |  |  | TDLA30-35 |
| Antenna configuration |  |  | 2 x 2 ULA Low |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic |
|  | Number of CSI-RS ports (X) |  | 2 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 3, (6,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (13,-) |
|  | CSI-RS interval and offset | slot | 5/1 |
| ReportConfigType |  |  | Periodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| Sub-band Size |  | RB | 8 |
| csi-ReportingBand |  |  | 111111111 |
| CSI-Report interval and offset |  | slot | 5/0 |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | N/A |
|  | CodebookSubsetRestriction |  | 001111 |
|  | RI Restriction |  | N/A |
| CQI/RI/PMI delay |  | ms | 1.75 |
| Maximum number of HARQ transmission |  |  | 4 |
| Note 1: The same requirements are applicable for FDD and TDD with different UL-DL pattern. Note 2: For random precoder selection, the precoder shall be updated in each slot (0.125 ms granularity). Note 3: If the NCR-MT reports in an available uplink reporting instance at slot #n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 4: Randomization of the principle beam direction shall be used. Note 5: SSB, TRS, CSI-RS and/or other unspecified test parameters with respect to TS 38.101-4 are left up to test implementation, if transmitted or needed |  |  |  |

| Parameter | Test 1 |
| --- | --- |
|  | 1.05 |

Use above list for NCR requirements.

Proposals

In this contribution, we discuss on demodulation performance requirements for NR network-controlled repeaters. Our observations and proposals are:

- Only consider 10MHz for 15kHz SCS and 40MHz for 30kHz SCS for NCR-MT demodulation requirements.

Do not define new requirements on PDCCH for signaling of Access link beam change indication.

Confirm that MCS4 is used for FR2 NCR PDSCH requirements.

Reuse same DCI type from IAB-MT and legacy UE requirements for NCR PDCCH requirements.

Define PMI reporting cases for the NCR-MT with the test parameters reused from IAB.

Define applicability rule that “Testing of performance requirements for PMI reporting is optional”.

Use above list for NCR requirements.

Reference

R4-2309832, WF on NCR_MT demodulation requirements, RAN4#107, ZTE Corporation