3GPP TSG-RAN WG4 Meeting #108	R4-2313661

# Toulouse, 21st - 25th Aug, 2023

Title: 	Simulation results on demodulation performance requirements for NR network-controlled repeaters

Source: 	Huawei, HiSilicon

Agenda item:	8.28.6

Document for:	Discussion

Background

During RAN4#107 meeting, WF [1] on NCR_MT demodulation requirements was approved. In this contribution, we provide our initial simulation results about NR network-controlled repeaters.

Simulation

Based on our discussion Tdoc in the same topic, here we only list the cases that need to perform new simulation.

FR1 PDSCH

| Case number | Bandwidth (MHz) | SCS (kHz) | Propagation condition | Antenna configuration | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 70% max throughput | 1% BLER |
| 1 | 10 | 15 | TDLA30-10 | 2x2 | -3.4 | 1.8 |
| 2 | 10 | 15 | TDLA30-10 | 2x4 | -6.5 | -2.9 |
| 3 | 40 | 30 | TDLA30-10 | 2x2 | -3.3 | 1.3 |
| 4 | 40 | 30 | TDLA30-10 | 2x4 | -6.4 | -3.2 |

FR1 PDCCH

| Case number | Bandwidth (MHz) | SCS (kHz) | CORESET RB | CORESET duration | Aggregation level | Propagation condition | Antenna configuration | SNR@1% BLER (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 10 | 15 | 48 | 1 | 8 | TDLA30-10 | 2x4 | -7.9 |

FR2 PDSCH

| Case number | Bandwidth (MHz) | SCS (kHz) | Propagation condition | Antenna configuration | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 70% max throughput | 1% BLER |
| 1 | 100 | 120 | TDLA30-75 | 2x2 | -3.3 | 0.1 |
| 2 | 100 | 120 | TDLA30-75 | 2x4 | -6.3 | -4.1 |

Conclusion

In this contribution, we provide our updated simulation results about NR network-controlled repeaters.

Reference

R4-2309832, WF on NCR_MT demodulation requirements, RAN4#107, ZTE Corporation