3GPP TSG-RAN WG4 Meeting #108                                                            R4-2313704

Toulous,FR, Aug 21 – Aug 25, 2023

Agenda Item:	8.18.1.2

Source:	Spreadtrum

Title:	Simulation results for phase I study for MU-MIMO advanced receiver

Document for:	Discussion

1	Introduction

The R18 WI “NR demodulation performance evolution” [2] has been revised at RAN#100 meeting to introduce network assistant signaling to support advanced receiver to cancel inter-user interference for MU-MIMO. Meantime, test cases were identified for demodulation performance investigation. In this contribution, we share our simulation results to investigate the R-ML receiver, comparison has been done on the demodulation performance among the R-ML receivers aided with 3 levels of information: aided with genie information, aided with modulation order (MO in short) information (i.e. need to blind detect the FDRA and DMRS port allocation of the co-scheduled UE), no aided information (i.e. need to blind detect MO, FDRA and DMRS port allocation information of the co-scheduled UE). The test cases are based on the agreed parameter configurations and assumptions for information in [1][3]

2	Discussion

## 2.1 Assumptions

Following assumptions are assumed:

- Maximum transmission bandwidth configuration for all UEs

- Assume all scheduled DMRS ports have same QCL assumptions

- Assume the scrambling ID for DMRS sequence is the same for the target UE the co-scheduled UE(s)

- Random precoder for rank 1+1, orthogonal precoder for rank 2+2

- Same CDM group for rank 1+1, different CDM groups for rank 2+2

Table 2.1-1 shows the general parameter configurations, which are basically using what was agreed in the previous meetings.

Table 2.1-1 General parameter configurations

| Parameter |  | Unit | Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | Target UE | Co-scheduled UE |
| Number |  |  | 1 | 1 |
| Channel Bandwidth/SCS |  | MHz/kHz | 10/15 for FDD |  |
| Duplex mode |  |  | FDD |  |
| MCS |  |  | 13/17 | QPSK/16QAM/64QAM |
| Allocation for interference UE and target UE | Rank allocation |  | 1 | 1 |
|  |  |  | 2 | 2 |
|  | DMRS port allocation |  | For rank 1+1: Port 1000 For rank 2+2: Port 1000 and 1001 | For rank 1+1: Port 1001 For rank 2+2: Port 1002 and 1003 |
| MIMO configuration |  |  | 2T2R, 4T4R with different correlations for verification |  |
| CDM groups without data |  |  | Same for rank 1+1 / different for rank 2+2, 1+1+1 |  |
| Precoding model | Target UE |  | Random precoding with Single panel Type 1 per PRB bundling size per slot | - For case with rank 1+1: Random - For case with rank 2+2: Orthogonal |
| PDSCH configuration | Mapping type |  | Type A |  |
|  | Starting symbol (S) |  | 2 |  |
|  | Length (L) |  | 12 |  |
|  | PRB bundling size |  | 2 |  |
|  | PRB bundling type |  | Static |  |
| PDSCH DMRS configuration | DMRS Type |  | DMRS Type 1 |  |
|  | Number of additional DMRS |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |
| Propagation conditions |  |  | TDLC300-100 for rank combination 1+1 TDLA30-10 for rank combination 2+2 |  |

## 2.2 Test cases and results summary

We’ve evaluated the following test cases and summarized the results in table 2.2-1

T

Table 2.2-1 Simulation cases and results

| Test num | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format |  | Propagation condition | Rank | Correlation matrix and antenna configuration | Reference value SNR @ 70% Tput (dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Target UE | Co-scheduled UE |  | (target UE + co-scheduled UE) |  | R-ML (DMRS and FDRA blind detection) | R-ML (DMRS and FDRA and MO blind detection) | R-ML (genie) | Total loss (dB) | Loss due to DMRS port and FDRA blind detection (dB) | Loss due to MO blind detection (dB) |
| Rank 1+1 |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 10 / 15 | MCS13 16QAM, | QPSK | TDLC300-100 | 1+1 | 2x2, ULA medium | 13.2 | 13.2 | 12.5 | 0.7 | 0.7 | 0.1 |
| 2 | 10 / 15 | MCS13 16QAM, | 16QAM | TDLC300-100 | 1+1 | 2x2, ULA medium | 17.1 | 17.6 | 15.9 | 1.7 | 1.2 | 0.5 |
| Rank 2+2 |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | 10/15 | MCS13 16QAM, | 64QAM | TDLA30-10 | 2+2 | 4x4, ULA Low | 11.4 | 12.1 | 10.9 | 1.2 | 0.5 | 0.7 |
| 4 | 10.15 | MCS 13 16QAM, | QPSK | TDLA30-10 | 2+2 | 4x4, ULA Low | 10.2 | 10.2 | 9.8 | 0.3 | 0.3 | 0.0 |
| 5 | 10/15 | MCS17 64QAM | 64QAM | TDLA30-10 | 2+2 | 4x4, ULA Low | 17.5 | 18.4 | 15.4 | 3.0 | 2.1 | 0.9 |
| 6 | 10/15 | MCS17 64QAM | 16QAM | TDLA30-10 | 2+2 | 4x4, ULA Low | 16.6 | 17.2 | 15.1 | 2.1 | 1.5 | 0.6 |
| 7 | 10/15 | MCS17 | QPSK | TDLA30-10 | 2+2 | 4x4, ULA Low | 14.0 | 14.1 | 13.1 | 1.1 | 1.0 | 0.1 |

Based on performance statistics collected in the simulation, we calculate the total SNR loss due to blind detection, SNR loss due to blind detection of MO, and SNR loss due to blind detection of FDMA and DMRS port allocation respectively, by comparing the demodulation performance of the R-ML receiver aided with 3 levels of information: aided with genie information, aided with MO information, no aided information. We furtherly reach the following observations.

Observation 1: R-ML SNR loss due to modulation order detection error increases with the modulation order increment, it‘s observed that 64QAM shows the highest SNR loss among the three MOs of 64QAM, 16QAM and QPSK.

Observation 2: The higher SNR point is more sensitive to the DMRS port detection error, it’s observed that DMRS port detection error leads to higher performance loss in the higher SNR case, e.g test number 5 shows 2.1dB loss due to FDRA and DMRS port blind detection error, while the corresponding loss in test number 4 is 0.3dB, while noting the two test cases show the highest difference of SNRs.

3	Summary

In this contribution, we shared our simulation results for R-ML receiver based on the parameter configurations and assumptions agreed in the last RAN4 #107 meeting [1]. Looking into the simulation results, we get the following observations.

Observation 1: R-ML SNR loss due to modulation order detection error increases with the modulation order increment, it‘s observed that 64QAM shows the highest SNR loss among the three MOs of 64QAM, 16QAM and QPSK.

Observation 2: The higher SNR point is more sensitive to the DMRS port detection error, it’s observed that DMRS port detection error leads to higher performance loss in the higher SNR case, e.g test number 5 shows 2.1dB loss due to FDRA and DMRS port blind detection error, while the corresponding loss in test number 4 is 0.3dB, while noting the two test cases show the highest difference of SNRs.

References

[1] R4-2309892 WF on advanced receiver for MU-MIMO scenario, China Telecom, RAN4 #107

[2] RP-231188 Revised WID on NR demodulation performance evolution,China Telecom, TSGRAN #100

[3] R4-2307632	Simulation result collection for advanced receiver for MU-MIMO, China Telecom, RAN4 #107