3GPP TSG-RAN WG4 Meeting #108	R4-2313716

Toulouse, France, Aug 21 – Aug 25, 2023

Agenda Item:	8.25.2.1

Source:	MediaTek Inc.

Title:	Discussion on the general aspects of MUSIM gaps

Document for:	Discussion

# Introduction

In NR Rel-17 specification, RAN4 has introduced gap patterns particularly for MUSIM purpose. However, their corresponding RRM requirements were not specified. In Rel-18 MUSIM WI, RAN4 has discussed the RRM requirements for MUSIM and the open issues are captured in the WF [1]. In this paper, some general aspects of MUSIM are further discussed.

# Discussion

The following general issues in MUSIM WI are still to be discussed in RAN4 as captured in the WF [1].

Issue 1-1-3: Mandatory MUSIM gap patterns - Proposals - P1: No need to discuss further whether to introduce mandatory MUSIM gap patterns (Apple oppo Huawei Nokia Qualcomm MTK) - P2: RAN4 to define the mandatory MUSIM gap patterns (CMCC Ericsson Nokia) - P3: No more discussion if there is no consensus (vivo)

For Issue 1-1-3, we share same view as in P1, i.e., no need to discuss further whether to introduce mandatory MUSIM gap. MUSIM gap patterns are recommended by UE based on its calculation to facilitate required network B activities. Introducing mandatory gaps does not mean UE can expect a unified network B configurations (SSB, paging, …). Therefore, we do not see a strong need here.

Proposal 1: No need to discuss further whether to introduce mandatory MUSIM gap patterns.

Issue 1-1-5: Others - Proposals - P1: UE shall not request MUSIM gaps beyond the UE capacity considering the UEs current configuration (Nokia) - P2: UE shall not request more MUSIM gaps than it is capable of handling with the current measurement gap allocation (Nokia)

For Issue 1-1-5, P1 and P2 are related to UE capability. If UE is capable to support multiple MUSIM gaps, then it is up to UE how/when to request such gaps, which can be based on the UE observations for NW B activities. Therefore, for the above issue we don’t think there is something to be captured in the specs.

# Summary

In this contribution, the following proposal is concluded:

Proposal 1: No need to discuss further whether to introduce mandatory MUSIM gap patterns.

# References

- R4-2310165, WF on NR Dual TxRx Multi-SIM, vivo.