# 3GPP TSG-RAN WG4 Meeting # 108                                                            R4-2313729

Toulouse, France, 21 - 25 Aug 2023

Source:	T-Mobile USA

Title:	TP for TR 38.896 to add PC2 for n71

Agenda item:			7.22.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 38.896 to add PC2 for n71 including RSD requirements based on the WF in [1]. NS_35 A-MPR for PC2 is TBD.

# 2. Reference

[1] R4-2306544	WF on FDD HPUE Basket WIs, China Unicom

# 3. Text Proposal

<Start of changes>

## 5.x	NR band n71

### 5.x.1	UE maximum output power

Table 5.x.1-1: UE output power for PC2

| NR band | Class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| n71 | 26 | +2/-3 |

### 5.x.2	A-MPR requirements

[NS_35 A-MPR TBD]

### 5.x.3	Reference sensitivity requirements

For PC2 single band n71, both 1Tx and 2Tx requirements are considered. Therefore, for the Reference Sensitivity Degradation requirements, not supporting Tx diversity (i.e. 1Tx) and supporting Tx diversity (i.e. 2Tx) are needed to be defined.

For PC2 single band n71 not supporting Tx diversity (i.e. 1Tx), the Reference Sensitivity Degradation from PC3 to PC2 are defined in Table:

Table 5.x.3-1: Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE not supporting Tx Diversity

| Operating Band | Source | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n71 | Skyworks(R4-2300652) | 0.1 | 0.1 | 0.9 | 2.0 | 2.2 | 2.3 | 2.4 |  |  |  |
|  | Apple(R4-2300362) | 0 | 1.6 | 0.6 | 1.6 | 1.8 | 2.0 | 2.8 | N/A | N/A | N/A |
|  | Huawei, HiSilicon(R4- 2300715) | 1.2 | 1.2 | 1.4 | 3.2 | 3.4 | 3.5 | 3.6 | - | - | - |
|  | Murata(R4-2305393) | 0.7 | 0.8 | 0.6 | 2.1 | 2.1 | 2.0 | 2.8 | - | - | - |
| Average |  | 0.5 | 0.9 | 0.9 | 2.2 | 2.4 | 2.5 | 2.9 |  |  |  |

For PC2 single band n8 supporting Tx diversity (i.e. 2Tx), several companies shared the analysis, and we summarize the required sensitivity degradation levels from companies as shown in 5.x.3-2:

Table 5.x.3-2: Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE supporting Tx Diversity

| Operating Band | Source | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n71 | Skyworks(R4-2300652) | 0.3 | 0.4 | 1.5 | 4.7 | 5.1 | 5.3 | 5.6 |  |  |  |
|  | Apple(R4-2305364) | 0.5 | 0.5 | 1.0 | 6.2 | 6.7 | 7.1 | 7.5 | N/A | N/A | N/A |
|  | Huawei, HiSilicon(R4- 2300715) | 2.9 | 2.9 | 3.6 | 6.2 | 6.4 | 6.6 | 6.8 | - | - | - |
|  | Murata(R4-2305393) | 1.6 | 1.8 | 2.0 | 5.6 | 6.0 | 6.5 | 6.8 | - | - | - |
|  | MediaTek(R4-2302353) | 0 | 0 | 0.5 | 4.8 | 5.5 | 5.6 | 5.6 |  |  |  |
| Average |  | 1.1 | 1.1 | 1.7 | 5.5 | 5.9 | 6.2 | 6.5 |  |  |  |

<End of changes>