3GPP TSG-RAN4 Meeting #108	R4-2313782

# Toulouse , France, 21st – 25th August 2023

Agenda item:	8.2.4

Source:	Keysight Technologies

Title:	On Multi-RX UE demod topics

Document for:	Approval

# Introduction

This contribution briefly addresses multi-RX UE demod topics, specifically the request for multi-RX SNR reference numbers.

# Multi-RX Demod

In the last meeting, important discussions about impairments due isolation for demod testing were held [1]

Issue 3-3-1: Test method for demodulation requirements for FR2 multi-Rx (Issue 1-1-3 in thread [322] R4-2310459) - Observations - Observation 1 (Qualcomm): - Real OTA testing may include impairment due to RF, hence demod requirements may not be truly tested in the presence of such impairments. - Observation 2 (MediaTek): - The split to emulator and simulator channels has not been captured in the earlier proposals - In OTA testing, we employ channel model Hemulator (4x4) between TRPs and TX-probes - Chamber channel Hchamber is not fading, but rather constant and deterministic for a given physical TX-probe and UE-panel setup (depending on AoA). - Proposals - Option 1 (Qualcomm): RAN4 to adopt virtual cable setup to test the FR2 multi-Rx demodulation requirements as discussed in NR FR2 OTA enhancements. - Option 2 (MediaTek): - Confirm understanding of test setup in chamber - Option 1: Install probes in test chamber according to defined AoA and keep emulator channel independent of the AoA - Option 2: Install probes in test chamber in 90 angle and model AoA in channel emulator. - Other options are not precluded - In corresponding simulations, we employ the same model Hemulator concatenated with a model of the chamber channel Hchamber (4x4) → Hsimulator = Hchamber ∙ Hemulator. - We need to specify the properties of both Hchamber and Hemulator explicitly. - In Hemulator set H12 and H21 as all zeros, and model H11 and H22 as mutually independent 2x2 fading channels that can follow some existing 3GPP correlation model (for example 2x2 Xpol high) - Define Hchamber=I1γI1γII, where γ depends on AoA. Figure 3.2.3-1: Illustration of virtual cable approach for multi-Rx demodulation test (R4-2309244) - Agreements: - RAN4 to adopt virtual cable setup to test the FR2 multi-Rx demodulation requirements as discussed in NR FR2 OTA enhancements. - RAN4 to further study the number of AoA pairs satisfying the [12dB] min. isolation based on the simulation assumptions in UE RF with the following options: - Option 1: Pure isolation - Option 2: Inverse channel approach - Other option is not precluded - All the channel model parameters need to be modelled in the channel emulator.

It was concluded, similar to legacy FR2 demod testing, for companies to study minimum isolation requirements and the corresponding MU impact.

Issue 3-2-1: Minimum isolation requirements - Proposals - Option 1 (R4-2309244): It is encouraged companies provide the simulation results to verify the minimum isolation requirements of [12dB] for multi-Rx demodulation test. Figure 3.2.2-1: Illustration of isolation implementation in the simulation (R4-2309244) - Option 2: TBA - Agreements: - Option 1 agreed

Keysight has led most of the legacy minimum isolation and MU discussions [2][3] and is, in principle, willing to repeat some of the work for multi-Rx demod testing. Previous contributions were using Keysight SystemVue, an electronic design automation environment for electronic system-level design as well as NR end-to-end system simulations which can be used to for standard-compliant 5G NR signal generation and advanced receiver modelling for EVM and throughput simulations. It supports 3D MIMO channel emulation using either 3GPP 38.901 channel models for FR1 and FR2 or user-defined channel models. The goal here would be to leverage SystemVue again to study the effect of isolation on the throughput vs SNR results. For 2x2 DL, we checked our SystemVue performance against reference SNR numbers stated in [4], e.g., Tables 7.2.2.2.1-3 through 7.2.2.2.1-5 to make sure that our simulations are within an expected and acceptable range. For us to get started with multi-Rx demod simulations, we require similar SNR reference numbers for FR2 Multi-Rx together with the underlying system assumptions.

Proposal 1: OEM/Chipset vendors to make multi-Rx SNR reference numbers together with the underlying system assumptions available.

# Conclusion

The following observations and conclusions were made in this contribution.

Error! Reference source not found.

Proposal 1: OEM/Chipset vendors to make multi-Rx SNR reference numbers together with the underlying system assumptions available.

# References

- R4-2309811, WF for FR2 OTA test SI, Qualcomm Incorporated, 3GPP TSG-RAN WG4 Meeting # 107, May 2023

- R5-199575, End-to-End System Simulations for Min. Isolation Requirements, Keysight Technologies, 3GPP TSG-RAN WG5 #85, November 2019

- R5-201157, End-to-End System Simulations for Min. Isolation Requirements, Keysight Technologies, 3GPP TSG-RAN5 Meeting #86-e, February 2020

- TS 38.101-4, User Equipment (UE) radio transmission and reception; Part 4: Performance requirements