3GPP TSG-RAN4 Meeting #108	R4-2313799

# Toulouse , France, 21st – 25th August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.151 | CR | 0018 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | FR2 CM Validation Corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_OTA_enh-Core |  |  |  |  |  | Date: |  |  | 2023-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | CM validations for cross-polarizaiton and power valdiation were not captured properly and/or clearly |  |  |  |  |  |  |  |  |
| Summary of change: |  | CM validations were clarified, e.g, separate V and H measurements for each probe antenna are required. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CM validations are not performed properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | D.3.4, D.3.5, D.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

## D.3.4	FR2 PAS similarity percentage (PSP)

The PSP validation measurements aim at evaluating PAS similarity percentage (PSP), which is one of the validation metrics for characterizing FR2 channel model under test in the quite zone of 3D-MPAC. For PSP validation measurement, only vertical polarization validation is required.

The measurement array is essentially a virtual array configuration realized in 3D-MPAC through a -θ positioning system. The measurement array is a semi-circle and sectored array configuration illustrated in Figure D.3.4-1 where complex channel frequency response is measured at each antenna location 0.5 λ apart using a vector network analyser (VNA) setup.  The vertical sectors of the measurement array are limited to 60 (±30) and the horizontal sector to 180 (±90) with the broad side direction points towards the probes. Depending of the turntable architecture/implementation, the virtual array configuration for the PSP validation is composed of two alternative semi-circle arrangements (1 x horizontal and either 2 x crossed vertical or 2 x parallel vertical). The radius of the array element locations with respect to the centre of the test zone is 5 cm, which is equivalent to the half of the test zone radius at 28 GHz. For different frequency bands, the radius of the measurement array sectored semi-circles remains fixed at 5 cm while the spatial sampling of the array varies. This measurement validates the proper angular behaviour in the test zone.

Figure D.3.4-1: Semi-circle measurement array configurations with K = 37 elements (at 28 GHz). On the left with two crossed vertical sectors, on the right with two parallel vertical sectors.

Figure D.3.4-2: Setup for FR2 PSP validation measurements

The PSP validation is measured with a vector network analyser as shown in Figure D.3.4-2 illustrating the PSP measurement setup. Port 1 of the VNA transmits signals through the fading emulator and radiate them through L probes within the anechoic chamber. The radiated signals are then received at the test antenna that is positioned inside the test zone. The test antenna is mounted on a -θ positioner which is capable of moving the antenna to pre-defined spatial locations on a fixed radius from the centre of the quiet zone according the measurement array configuration. Finally, the signal is received at port 2 of the VNA. The most suitable approach for the PSP validation is based on an omnidirectional antenna (omnidirectional pattern in AZ and wide BW in EL) as the test can be automated easily. Alternatively, a directional antenna could be used but requires frequent re-positioning.

The measurement and analysis procedure are given as follows:

1.	Set the target channel model in the Channel Emulator.

2.	For each position of the test antenna on the measurement array configuration in the test zone, step & pause the emulator to different time instances. Measure the complex frequency responses Hf,t=Hm∆f,n∆T, m=0,..., M-1 for all stepped channel snapshots n=0,..., N-1 , where the interval between frequency and time samples is ∆f and ∆T, respectively. The number of channel snapshots N and frequency samples M.

3.	Move the measurement antenna with a positioner to another location k and repeat step 2 to record frequency responses Hk(m∆f,n∆T)   of all stepped channel snapshots.

4.	Repeat step 3 to record frequency responses at all k=0,..., K  spatial sample points.

5.	Estimate the measured PAS through the following two- stage processing:

a.		In the first stage, calculate the discrete azimuth and elevation angles (DoA) for the measurement array configuration by applying the MUSIC algorithm. Estimate the powers from the DoA and auto-covariance matrix of the received signal acquired through VNA complex frequency response data.

i) Compose an estimate of the covariance matrix RH∈CK×K. The entry on the kth row and k'th column of RH is

RHk,k'=n=1Nm=1MHkm∆f,n∆T Hk'm∆f,n∆T,

where  is the complex conjugate operator.

ii) Compose pseudo-PAS of the chamber environment and channel model using the MUSIC algorithm. The intention of this step is to estimate DoAs of signals radiated by probes. Perform first the eigen decomposition of RH and pick the noise-subspace matrix V. The matrix V is obtained from the eigen decomposition by removing eigenvectors, i.e. columns, that correspond to X strongest eigenvalues, where X is the number of active probes in the MPAC setup. The pseudo-PAS is [9]

PMUSICβ=1ωβ VVH ωHβ ,

where H is the matrix Hermitean operator and the near-field array factor of the virtual array composed by K spatial measurement antenna locations is

ωβ=λ04πrk-R uβexpj2πrk-R uβλ0,

where λ0 is the wavelength at the carrier centre frequency,  is the norm of a vector, rk is a location vector of the kth virtual array element, uβ is the unit vector to direction β, and R is the a priori known approximate range length [9]. Both vectors rk and uβ are defined with respect to same origin, which is the centre of the test zone.

iii) Find local maxima of PMUSICβ and pick directions βx, x=1,…,X of the X  highest peaks.

iv) Perform Bartlett beamforming as defined in step 6 with the steering vector ωβx (instead of a(β)) to the X directions identified in iii). The output is X power estimates Pestβx.

b.	In the second stage, use the angle and power estimates, i.e. the discrete PAS of X azimuth and elevation directions and power values in conjunction with a 4x4 DUT sampling array for beamforming with the conventional Bartlett beamformer to estimate the “measured PAS seen by DUT” for PSP calculation. This is

Poβ=aHβx=1XaβxPestβxaHβxaβ,

where a(β) is the array steering vector of the 4x4 DUT array.

6.	Evaluate the reference OTA PAS for the 4x4 DUT array by applying the conventional Bartlett beamformer. The theoretical PAS as reference is calculated for the ideal channel model as

Prβ=aHβ ∮a(β) P(β)aH(β)dβ aβ,

where a(β) is the array steering vector of the 4x4 DUT array and P(β) is the PAS of the reference channel model.

7.	Calculate total variation distance (Dp) from the reference and measured PAS. Mathematically,

8.	Calculate PSP values as PSP = (1-Dp) x 100%.

VNA settings:

Table D.3.4-1: VNA settings for FR2 PSP measurements

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | Downlink centre frequency in Table D.3.1-1 |
| Span | MHz | 0, or the minimum (Note 1) |
| Number of traces |  | 1000 |
| Number of points |  | 1 (Note 1) |
| NOTE 1: Span and number of points may be increased to estimate reliably. |  |  |

Channel model specification:

Table D.3.4-2: Channel model specification for FR2 PSP measurements

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | Downlink centre frequency in Table D.3.1-1 |
| Distance between traces in channel model | wavelength (Note) | > 2 |
| Channel model |  | As specified in Annex D.1 |
| NOTE: Time [s] = distance [l] / MS speed [l/s] MS speed [l/s] = MS speed [m/s] / Speed of light [m/s] * Centre frequency [Hz] |  |  |

Time Domain Alternative Method:

PSP validation can also be implemented using time-domain techniques using the testing setup presented in Figure D.3.4-3. The VNA is substituted by a signal generator, and a signal analyser.

Figure D.3.4-3: Setup for FR2 PSP validation measurements based on time domain

Table D.3.4-3: Signal Generator Settings for FR2 PSP measurements based on time domain

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | Downlink centre frequency in Table D.3.1-1 |
| Output power | dBm | Function of the CE. Sufficiently above Noise Floor |

Table D.3.4-4: Signal Analyser Settings for FR2 PSP measurements based on time domain

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | Downlink centre frequency in Table D.3.1-1 |
| Sampling | Hz | At least 10 times bigger than the max Doppler spread (fd=v/λ) |
| Observation time | s | At least 32s |

The measurement and analysis procedure are given as follows:

Follow the same procedure as before, but M is set to 1. The Channel Emulator is not stepped, but it is allowed to play in free run mode for each of the K spatial points.

## D.3.5	FR2 Cross-polarization

This measurement checks how well the measured vertically or horizontally polarized power levels follow expected values. The test setup for cross-polarization is the same as PDP validation in Figure D.3.2-1.

Method of measurement: 		Step the emulation and store traces from VNA.

VNA settings:

Table D.3.5-1: VNA settings for FR2 cross-polarization

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | Downlink Centre Frequency in Table D.3.1-1 |
| Span | MHz | 40 |
| Number of traces |  | 1000 |
| Number of points |  | 802 |
| Averaging |  | 1 |

Channel model specification:

Table D.3.5-2: Channel model specification for FR2 cross-polarization.

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | Downlink centre frequency in Table D.3.1-1 |
| Distance between traces in channel model | wavelength (Note) | > 2 |
| Channel model |  | As specified in Annex D.1 |
| Mobile speed | km/h | 30 |
| NOTE: Time [s] = distance [l] / MS speed [l/s] MS speed [l/s] = MS speed [m /s] / Speed of light [m/s] * Centre frequency [Hz] |  |  |

Measurement Procedure:

1.	Play or step through the channel model listed in Annex D.1.

2.	Measure the absolute power received at the centre of the test zone, averaged over a statistically significant number of fades.

a.	Perform separate V-polarization and H-polarization measurements sequentially, e.g., by rotating a single-polarized horn antenna accordingly or with a use of a dual-polarized horn where the un-used polarization is terminated.

b.

b.	Measure input 1+2 first and then enable input at a time to measure input 1 and 2 separately.

c. 	Disable all CE outputs and enable one output at a time. Get the VNA trace pointing the measurement horn antenna towards the active probe. Record frequency sweeps through the channel model. Repeat for all the probes.

3.	Calculate the V/H ratio.

- Measured VNA traces (frequency responses over time and probes HV(t,f,kV) and HHt,f,kH are saved into a hard drive. The data is read into, e.g., Matlab. The frequency responses are summed in power over time k, frequency f  and probes k and the V/H ratio calculated as follows:

PV=tfkVHV(t,f,kV)2

PH=tfkHHH(t,f,kH)2

PV/H[dB]=10log10PVPH

4.	Compare it with the theory value.

Reference target for FR2 CDL-C UMi:

-	V/H FR2 CDL-C-UMi, X2+, fc=28 GHz

-	Beam 1, Input 1:  V/H = -0.45 dB

-	Beam 1, Input 2:  V/H = 0.49 dB

-	Beam 1, Input 1+2:  V/H = 0 dB

## D.3.6	FR2 Power validation

This measurement checks the total power in the centre of the test zone. The power validation is measured with a spectrum analyser as shown in Figure D.3.6-1.

Figure D.3.6-1: Setup for FR2 power validation measurements

Spectrum analyser settings:

Table D.3.6-1: Spectrum analyser settings for FR2 power validation measurements

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | Downlink centre frequency in Table D.3.1-1 |
| Integrated Channel Span | Hz | 20MHz |
| RBW | Hz | 30 kHz |
| VBW | Hz | ≥10MHz |
| Number of points |  | ≥400 |
| Averaging |  | ≥100 |
| Detector |  | RMS |

Measurement Procedure:

1.	Place a horn antenna in the centre of the test zone connected to a spectrum analyser (or power meter) via a cable. Point the horn to the first probe and initially select the V-polarization.

2.	Record the cable and horn antenna gains.

3.	Load the target channel model into the channel emulator.

4.	Start the NR FR2 signalling in the base station emulator with the required parameter identical to the measurements conditions.

5.	Disable all CE outputs and enable only the CE output that points towards the active probe with the polarization matching the polarization of the reference horn antenna.

6.	Average the power received by the spectrum analyser to get ≥ 100 averages to account for the fading channel.

7.	Repeat steps 5 to 6 for all six probe directions and perform separate V and H polarization measurements while matching the polarization of the horn antenna in the centre of the test zone with the polarization of the probe.

8.	Calculate the total power received at the test area as the sum of the power over k=6 probes in the two polarizations as follows:

PV=10log10kV10PkV[dB]10

PH=10log10kH10PkH[dB]10

PV/H[dB]=10log10PVPH

<<< END OF CHANGES >>>