3GPP TSG-RAN WG4 Meeting # 108	R4-2313815

# Toulouse, France, August 21 – August 25, 2023

Source: 	Skyworks Solutions, Inc.

Title: 	On SL-U MPR

Agenda Item:	8.30.2.1.2	Tx requirements	[NR_SL_enh2-Core]

Document for:	Discussion

# Introduction

In RAN4#107, some Tx requirements for SL-U were captured in [1]. The SL-U MPR study may need some insight on the NR-U MPR specification.

# Discussion

## NR-U MPR specification

In RAN4#107, in the context of SL-U, MPR was discussed and NR-U MPR was pointed as an input to the study. For those not familiar with the NR-U MPR work there might be some aspects that are easy to miss.

In this paper we will try to provide some insights that we hope can be valuable to the SL-U work.

One aspect is that the work started with PC5 at 20dBm to be able to reuse available Wi-Fi PAs which lead to a slightly different calibration point for MPR:

- NR PC3 uses NR SEM mask and 30dB ACLR at 1dB MPR for 100RB0 DFT-s-OFDM QPSK

- NR-U PC5 uses NR-U SEM mask and 27dB ACLR at 1dB MPR for 100RB3 DFT-s-OFDM QPSK

- For 1Tx PC3 NR-U, the NR PC3 calibration is used, however the ACLR requirement is 27dB

- For 2Tx PC3 NR-U, each PA is calibrated with PC5 calibration point and reverse IMD contribution should be considered

- For NR-U the NR-U SEM which is relative to the in-band PSD, is the limiting factor and behaves as ACLR.

  - It is limiting especially for some wideband operation cases where the image and the spectral regrowth overlap in a 28dBR region

- In NR a 28dBc carrier and image is specified but in NR-U, to meet the wideband operation mask of 28dBr outside the allocated sub-band, there are case which would be marginal even without added spectral regrowth.

  - The 28dBR plateau was a hard negotiation with different parties at the time and was justified by the existence of the image leakage and IEEE specification of 20dBR

  - However, for MPR studies the assumption was that image leakage will be better the 28dBR to leave some room for the non-linearities. Unfortunately, there was no formal agreement on the image level assumption to align all MPR values. In our case, we used the level that must be met to support UL 256QAM: 34dB leaving some room for non-linearities.

- NR-U MPR is only valid for a set of waveforms:

  - For single CC: Fully allocated and interlaces only (NR partial allocation is not supported)

  - For wideband operation, the allocated sub-bands must be contiguous and use the same type of waveform (fully allocated or interlaces)

  - NR partial contiguous allocations or quasi contiguous allocations are not supported by NR-U MPR or A-MPR

- For intra-band contiguous ULCA PC5, the two CCs are supporting the same type of waveform (full or interlace) and when wideband operation is used the allocated sub-bands must be contiguous.

## NR-U A-MPR specification

In NR-U band n96 and n102 there are specific protection for the V2-X bands in n47, while n46 finishes far away from V2-X band except for the US in UNII-4. All these are protection levels for UE-to-UE coexistence, in SL-U though it may be important to also consider in-UE coexistence between n46, n96 and n102 channels and band 47 V2X signals.

For the SL-U work, we encourage companies to agree on a comprehensive set of assumption for MPR and we stay available for any help in understanding the status of the NR-U MPR and A-MPR.

# Conclusions

In this contribution, we share insight on the NR-U MPR and A-MPR work which, we hope, will be valuable for the SL-U work.

For the SL-U work, we encourage companies to agree on a comprehensive set of assumption for MPR and we stay available for any help in understanding the status of the NR-U MPR and A-MPR.

# References

- R4-2310308 TP to TR on the SL-e TX requirement, Oppo, RAN4#107