3GPP TSG-RAN WG4 Meeting #108	R4-2313819

Toulouse, France, 21st - 25th August 2023

Agenda item:			8.26.5.1

Source:	Inmarsat, Viasat

Title:	NTN UE types above 10 GHz and beam steering impact on RRM

Document for:	Discussion

# Introduction

This document discusses aspects related to NTN UE operating above 10 GHz, specifically with the aim of helping to define the scope of RAN4 RRM work.

Per RAN guidance as of RAN#100:

“TSG RAN tasks RAN4 to scope the work needed to define the RRM requirements (i.e. beam steering related) necessary for NR-NTN UEs operating in the NTN example band (i.e. Ka band) based on associated assumptions of the UE architecture (including beam steering capabilities).”

Furthermore, during RAN4#107, the following agreement was reached:

Agreement:

- At least consider the differentiation between mechanical and electronic steering;

  - FFS on phased array or parabolic;

  - Encourage satellite companies to provide the data to show the beam switching delay.

- As the baseline, assume that UE has the single beam towards one single satellite at a given time.

The Moderator summary included more recommendations on how to address open points:

Moderator’s Recommendation

- Wait for RF conclusion on RF/antenna structure (e.g. parabolic antenna and phased array) and restrictions on satellite type for different antenna structure.

- Collect views on the following aspects from RF and interested parties/companies to get more clarity on the scope of RRM requirements:

  - The maximum (recommended) interruption time (switching time) between 2 consecutive satellites during HO

  - The (minimum) number of satellites to which a VSAT/NTN UE will connect in Ka-band

  - Initial Satellite search time

  - Tracking accuracy

  - Others

This contribution is hence specifically targeted at providing input on differences between terminal architectures in terms of mechanical vs electronic steering, how that relates to parabolic vs phased array aperture, applicability to fixed, mobile and “on-the-pause” or “nomadic” UE, to GEO/GSO vs NGSO/LEO and lastly some input guidance on beam steering delay.

# VSAT UE Types

## Overview

Two of the points that have been brought to discussion in RAN4 and RAN numerous times during the discussion on NTN UE support above 10 GHz:

- Parabolic vs phased array antenna;

- Mechanical vs electronic beam steering and tracking.

Many companies seem to conflate the two things, assuming that all mechanical terminals make use of parabolic antennas, and that all terminals making us of phased array antennas are fully capable of electronic beam steering.  Furthermore, there seems to be an implicit assumption that NGSO systems, in particular LEO, make always use of phased array terminals, and that GEO/GSO systems make generally use of parabolic terminals.  However, neither of these assumptions are coorect.

In this paper, we aim to try and clarify those aspects, the necessary distinctions, and will attempt to show that for practical purposes, aspect (1), i.e. whether the antenna is implemented as parabolic or phased array, does not necessarily have any bearing on the beam steering capabilities.  What really matters instead are the actual beam steering and tracking capabilities, and whether they are based on mechanical or electronic steering.

By looking at current Very Small Aperture Terminal (VSAT) implementations available in the SATCOM market, we can summarize their broad types into 4 buckets:

- Fixed beam - Do not have any automatic beam steering or satellite tracking capability, and instead rely on human pointing.

- Mechanically-steered - Predominantly (or exclusively) rely on a mechanical multi-axis gimbal for beam steering and tracking, usually to  compensate vessel, vehicle or aircraft movement, or for auto-deploy in the case of land-portable terminals.

- Electronically-steered;

- Hybrid (mechanical + electronic) - Combine the capabilities of electronic beam steering via phased arrays with mechanical gimbals.  These architectures are becoming more and more common in latest satcom applications.

All these four types are used interchangeably for GEO/GSO and NGSO/LEO, often irrespective of whether they are used for fixed, mobile or ”on-the-pause” (aka nomadic, movable or transportable) applications.

Throughout the section, we will use the term ”VSAT” irrespectively to identify any satellite user terminal with a directional antenna, irrespective of the antenna implementation or intended application.

Another factor that can be taken into account is whether the VSAT is capable of forming one single beam at any given instant of time, or multiple beams, whether through a single or multiple antenna apertures.

Some example terminals are shown below – note that we are not taking into account multi-aperture terminals.

| # | Terminal | Application / Use Case | Antenna Type | Beam Steering | Supported Orbit(s) |
| --- | --- | --- | --- | --- | --- |
| 1 |  | Residential Broadband | Parabolic | Fixed | GEO |
| 2 |  | Land Portable | Phased Array | Fixed + Electronic (fine adjustment) | GEO |
| 3 |  | Land Portable | Parabolic | Mechanical | GEO |
| 4 |  | Maritime | Parabolic | Mechanical | GEO, MEO, LEO |
| 5 |  | Aeronautical | Parabolic | Mechanical | GEO, MEO |
| 6 |  | Aeronautical | Phased Array | Mechanical | GEO, MEO, LEO? |
| 7 |  | Land Mobile | Phased Array | Hybrid | GEO, MEO, LEO |
| 8 |  | Residential Broadband | Phased Array | Hybrid | LEO |
| 9 |  | Aeronautical | Phased Array | Hybrid | GEO, MEO, LEO |
| 10 |  | Land Mobile | Phased Array | Electronic | GEO, MEO, LEO |

From the survey above, we can see that the line between mechanical, electronic and hybrid beam steering is often blurred, with mechanical steering often used also for phased array implementations, and that there is no clear cut distinction between which one is used for which orbit (GEO, MEO, LEO). Therefore we propose the following:

Proposal 1: Adopt the term ”Directional NTN UE” to identify an NTN UE that has directional antenna, typically known in satellite communications as ”Very Small Aperture Terminal (VSAT)”, irrespective of whether it is fixed or mobile, mechanically-steered or electronically-steered, based on parabolic reflector or phased array.

Proposal 2: Consider mechanically-steered or electronically-steered NTN UE irrespective of whether it is based on parabolic of phased array aperture and irrespective of the orbit (GEO, MEO, LEO).

Proposal 3: Do not consider hybrid mechanical-electronically steered NTN UE as a separate case

# Beam Steering

## Mechanical Beam Steering

Typically, VSAT terminals in high-frequency bands (e.g. Ku, Ka, etc) are characterized in their tracking capabilities including tracking velocity and acceleration, as well as range of motion with 6 parameters:

- Range of motion:

  - Azimuth Range in degrees (min, max deg or continuous)

  - Elevation Range in degrees (min, max deg)

- Azimuth Tracking:

  - Azimuth Velocity in degrees per second (deg/s or °/s)

  - Azimuth Acceleration in degrees per second per second (deg/s2 or °/s2)

- Elevation Tracking:

  - Elevation Velocity in degrees per second (deg/s or °/s)

  - Elevation Acceleration in degrees per second per second (deg/s2 or °/s2)

Based on internal (non-public) system specifications from the Inmarsat and Viasat systems, we can extract the following minimum specifications for GEO mobile VSAT terminals applicable to land mobile, maritime and aeronautical. Aeronautical terminals drive the most stringent requirements, therefore, the target specification values are generally the following (or better):

- Azimuth Range = 0 to 260 deg, or 360 deg continuous

- Elevation Range = 0 to 90 deg

- Azimuth Velocity = 20 deg/s

- Azimuth Acceleration = 40 deg/s2

- Elevation Velocity = 15 deg/s

- Elevation Acceleration = 30 deg/s2

Proposal 4: Consider the following parameters related to beam steering and tracking for mechanically-steered VSAT as a worst case reference to derive inter-satellite beam steering time and corresponding timer and RRM requirements:

- Azimuth Velocity = 20 deg/s

- Azimuth Acceleration = 40 deg/s2

- Elevation Velocity = 15 deg/s

- Elevation Acceleration = 30 deg/s2

In practice however, actual values range depending on implementation and tend to be better than the specification requirement.  A small number of commercial terminals were surveyed, and summarized in the following table.  However, these specifications are not consistent across the industry and often incomplete, and it should further be noted that very high tracking speeds for mechanical antennas generally drive very high power motors and power consumption, therefore they should not be considered the norm.

| # | Terminal | Type/Size | Angular Az/El Velocity (deg/s) | Angular Az/El Accel (deg/s2) |
| --- | --- | --- | --- | --- |
| 1 | Smiths KaStream® 5000 MK II https://www.smithsinterconnect.com/CMSPages/GetFile.aspx?guid=56ef9e62-502c-421e-aff2-b5d3747ee795 | Aero VSAT, 30 cm | >30-60 | >60 |
| 2 | SeaTel 4009 https://www.bluesat.com/amfile/file/download/file/2124/product/3361/ | Maritime, 1 m | 90 | ? |
| 3 | https://www.digisat.org/cpi-3.7aebp-2-axis-positioner | Earth Station, 3.7 m | 60 | 60 |
| 4 | SatPro GA280 http://www.satpro.com/upfiles/GA280.pdf | Phased Array | >60 | >100 |
| 5 | https://www.navarino.co.uk/wp-content/uploads/2022/02/SAILOR-60-GX-R2-product-sheet_a4_lowres.pdf | Maritime, 65 cm | 15 | 15 |
| 6 | https://www.viasat.com/content/dam/us-site/maritime/documents/1380722_Sailor_600_Co-Branded_Datasheet_2021_002.pdf | Maritime, 60 cm | 15 | 15 |
| 7 | https://www.viasat.com/content/dam/us-site/government/documents/1060540_Viasat_GMT-6524_datasheet_031_web.pdf | Maritime, 1m | 15 | 15 |
| 8 | https://www.jst-group.com/wp-content/uploads/2020/06/Inmarsat_Global-Government_GX_Land_Catalogue_May_2020_EN_LowRes.pdf | Military, phased array hybrid | 200 | 200 |

## Inter-satellite beam steering time

If we consider the Azimuth and Elevation Velocity values discussed earlier, we can then derive an estimation of the beam steering time during an inter-satellite handover or re-selection.  That is, the time required for a mechanically-steered terminal to repoint between satellites.

The actual inter-satellite beam steering time can be observed to depend on three main factors:

- Beam steering and tracking velocity of the antenna;

- Angular separation between satellites;

- Elevation angle.

We can consider two different scenarios for GEO/GSO and NGSO (e.g. LEO).   For a baseline GEO system capable of global sub-polar coverage and composed of three satellites, we have a maximum angular separation between the three satellites of 360/3=120 degrees.

For NGSO/LEO scenario, if we take a baseline assumption, in line with TR 38.821, that the minimum elevation supported by the user link will be 30 degrees, and for a constellation with minimal coverage overlap between satellites, the maximum angular separation between satellites, as seen by the UE, will be of about 120 degrees, driven by the elevation threshold.  This would put the LEO case in a similar ballpark as the baseline GEO case.

One important aspect to note is that Azimuth and Elevation ranges of motion and freedom of motion might not be the same, as rotations along the elevation axis are more constrained compared with azimuth axis, which usually is either 0 to 360 or, more typically, 360 degrees of continuous motion.

In case of GEO, the closest the UE is to the Equator (which is where the GEO arc is aligned), the more the antenna movement requires a combination between elevation and azimuth motion.  By comparison, if the UE is located in high latitudes, the movement across the GEO arc is primarily driven by Azimuth change.

Similarly, for LEO, the Elevation component of the movement becomes more prevalent the closer the UE is to the satellite Nadir point for each orbital plane.

For an examplary case, with GEO/GSO and 20 degrees elevation, the antenna movement can be approximated with an Azimuth rotation. We can simply divide the angular separation by the Angular velocity value:

- 120 / 20 = 6 seconds

- 120 / 15 = 8 seconds

Proposal 5: Consider 8 seconds as reference beam steering time between two satellites for NTN UE with mechanical aperture, for both GEO and LEO.

In practice however, it should be noted that in real-world networks, angular separation between satellites is usually less than 120 degrees, as can be seen from the example GEO constellation below.

Figure 1- Reference GEO network coverage #1 showing separation between GEO satellite orbital slots

| Network #1 Satellite Name/Designation | Orbital Slot | Separation from West-adjacent satellite |
| --- | --- | --- |
| Inmarsat-5 F2 (GX-2) | 55° West | 125.4° |
| Inmarsat-5 F5 (GX-5) | 11° East | 66° |
| Inmarsat-5 F4 (GX-4) | 56.5° East | 45.5° |
| Inmarsat-5 F1 (GX-1) | 62.6° East | 6.1° |
| Inmarsat-5 F3 (GX-3) | 179.6° East | 117° |
| Note: orbital slots are for representative purpose and may not be accurate. |  |  |

Figure 2- Reference GEO network coverage #2 showing separation between GEO satellite orbital slots

| Network #2 Satellite Name/Designation | Orbital Slot | Separation from West-adjacent satellite |
| --- | --- | --- |
| WildBlue-1 | 111° West | 94° |
| Viasat-3 Americas | 88.9 West | 21.1° |
| Viasat-2 | 69.9° West | 19° |
| KA-SAT | 9° East | 78.9° |
| Viasat-3 EMEA | 13.8 East | 4.8° |
| ChinaSat-16 | 110° East | 96.2° |
| NBN SkyMuster I | 140° East | 30° |
| NBN SkyMuster II | 145° East | 5° |
| Viasat-3 | 155° East* | 10° |
| Note: orbital slots are for representative purpose and may not be accurate. |  |  |

# Scoping of RRM Requirements for NTN UE

## Impact on Inter-satellite Mobility

Based on the discussion above, we identify that multiple scenarios are possible in practice, that may have different implications in terms of RRM requirements for inter-satellite measurement, handover and re-selection.

These could be summarized to the following scenarios, in principle irrespective of whether the antenna is implemented as a parabolic reflector or as a phased array:

- NTN UE capable of Single beam (at any given time), via single, mechanically-steered aperture

- NTN UE capable of Single beam (at any given time), via dual mechanically-steered aperture

- NTN UE capable of Single beam (at any given time), via single electronically-steered aperture

- NTN UE capable of Multiple beams (analog or digital, simultaneous at any given time), via dual mechanically-steered apertures (i.e. 2 antennas)

- NTN UE capable of Multiple beams (analog or digital, simultaneous at any given time), via single or dual electronically-steered aperture.

However, it is very easy to conclude that the multi-beam cases can be considered optimizations on the baseline case, which is that the UE will only be able to support one simultaneous analog beam at any given instant of time.  Multi-beam scenarios could be considered at a later phase.

Observation 1: Scenarios in which a directional NTN UE can support multiple simultaneous analogue beam at any given time can be considered as optimizations on the baseline case with single beam.

The remaining point which has a key impact on RRM requirement scoping is how fast the beam can be re-traced steered during either an inter-satellite handover case, or during the cell/satellite search phase (which includes spatial scan to identify a target satellite for initial access).  The main driving factor here is whether the beam is electronically or mechanically-steered.

Observation 2: We can simplify the initial assumption by assuming that the NTN UE is capable of sustaining a single beam at any given instant of time and differentiate between whether the beam is mechanically- or electronically-steered.

In practice however, even the electronically-steered case could be seen as a “high-performance” optimization of the baseline case with mechanical steering, which would allow us to potentially reduce the scope of RRM requirement specification.

Proposal 6: Agree on either of these two options as baseline assumption for directional NTN UE, for the purpose of defining any RRM and mobility requirements:

- Option 1: Assume that the NTN UE is capable of sustaining a single beam at any given instant of time, and differentiate between whether the beam is mechanically- or electronically-steered.

- Option 2: Assume that the NTN UE is capable of sustaining a single beam at any given instant of time and only differentiate beam steering time, irrespective of electronic vs mechanical steering implementation (can be abstracted away as left to implementation).

Proposal 7: De-scope or at least de-prioritize scenarios with UEs capable of forming multiple beams simultaneously, whether via single or multiple apertures.

This gives us two different architectures to study and specify:

- Architecture A) Single beam, Mechanically-steered UE – UE does not support beam sweeping, and beam steering/tracking between satellites requires up to a few seconds.

- Architecture B) Single beam - Electronically-steered UE – UE can support beam sweeping, similar to existing TN FR2 UE or Base Station and beam re-trace takes on the order of us to ms.

Proposal 8: Study the two following NTN UE architecture options:

- Architecture A) Single beam, Mechanically-steered UE – UE does not support beam sweeping, and beam steering/tracking between satellites requires up to a few seconds.

- Architecture B) Single beam - Electronically-steered UE – UE can support beam sweeping, similar to existing TN FR2 UE or Base Station and beam re-trace takes on the order of us to ms.

Our expectation is that a number of RRM related requirements will be impacted, but we think the electronically-steered UE (Architecture B) should be close to being able to meet existing FR2 UE RRM requirements.  Some of the key aspects are related to target satellite measurements during inter-satellite CONNECTED mode HO and IDLE mode re-selection.

The standard procedure for example for inter-satellite HO would require the UE to re-point towards the target satellite to perform a measurement, re-point back to the serving satellite to report the measurement and later re-point yet again towards the target satellite upon reception of the HO trigger from the Network.

With a mechanically-steered UE taking several seconds to re-point its antenna, this is not feasible.  Therefore, we propose to remove the requirement for target satellite measurement before a HO or re-selection, and assume the NTN UE will perform a ”blind” HO or re-selection, as a baseline, which is more aligned to how satellite systems usually work.

Proposal 9: Remove requirement for target cell/satellite measurement before an inter-satellite mobility event, for both inter-satellite HO and inter-satellite cell re-selection.

The underlying baseline assumption should be that the UE has knowledge of neighbour satellite position via constellation Ephemeris and will be able to use that information, alongside with its own coarse position, to re-point towards the correct target satellite, either independently (satellite re-selection) or when instructed by the Network.

Proposal 10: The NTN UE should use knowledge of neighbor satellite ephemeris and its own position to point towards a target satellite during inter-satellite mobility.

Furthermore, for NGSO systems with either Earth-fixed or Earth-moving beams, in both in IDLE and CONNECTED mode, the inter-satellite HO or Re-selection events can be very predictable.

Outside of the RRM requirements, there is also an open point as to whether and which higher layer mobility aspects might be impacted by the beam steering capability, one of such being mobility timers. We propose that RAN4 ask RAN2 to clarify any such impacts.

Proposal 11: FFS impact on higher layer mobility aspects.  RAN4 should send an LS to RAN2 to clarify whether beam steering time for mechanically-steered NTN UE may impact mobility timers (e.g. such as t304) during inter-satellite mobility in IDLE and CONNECTED modes.

Lastly, when we remove the requirement for measurement of target satellite before inter-satellite mobility, it is possible that a number of corner conditions may arise, due to the lack of knowledge by the UE on the signal from the target satellite ahead of the mobility event.  One possibility is that the UE may re-point towards a target satellite which may be subject to temporary or permanent line-of-sight obstruction.  In such scenario, the HO or re-selection may fail.

For example, when inter-satellite mobility is triggered without prior measurement, there can be two possible outcomes:

- The UE finds the target satellite and HO is successful;

- The UE does not find the target satellite due to temporary or permanent LoS obstruction or absence of the satellite:

  - Temporary obstruction

  - Permanent obstruction or absence of target satellite

How the UE should behave in such scenarios may need to be studied.  One approach could be that the UE is simply instructed to perform a HO without any knowledge of the target satellite signal level.   This might mean that if the HO fails, the UE will eventually declare BFD, RLF and lose connection to the network.

Another approach could be that the UE should perform both target satellite measurement and HO at the same time, after having steered towards the target satellite, as a form of Conditional HO (CHO).  The procedure could assume that the UE should point to the new target satellite, but autonomously decide to only perform HO in case the target satellite measurement succeeds (condition is met), without first reporting the measurement result to the source gNB.  If the condition is not met, the UE could attempt HO to another satellite if known, or re-point towards the last known satellite (if available).

Proposal 12: Consider the following situations when handling inter-satellite mobility for mechanically-steered NTN UE when the UE does not find the target satellite due to temporary or permanent LoS obstruction or absence of the satellite:

  - Case 1: Temporary obstruction

  - Case 2: Permanent obstruction or absence of target satellite

# Conclusion

Proposal 1: Adopt the term ”Directional NTN UE” to identify an NTN UE that has directional antenna, typically known in satellite communications as ”Very Small Aperture Terminal (VSAT)”, irrespective of whether it is fixed or mobile, mechanically-steered or electronically-steered, based on parabolic reflector or phased array.

Proposal 2: Consider mechanically-steered or electronically-steered NTN UE irrespective of whether it is based on parabolic of phased array aperture and irrespective of the orbit (GEO, MEO, LEO).

Proposal 3: Do not consider hybrid mechanical-electronically steered NTN UE as a separate case

Proposal 4: Consider the following parameters related to beam steering and tracking for mechanically-steered VSAT as a worst case reference to derive inter-satellite beam steering time and corresponding timer and RRM requirements:

- Azimuth Velocity = 20 deg/s

- Azimuth Acceleration = 40 deg/s2

- Elevation Velocity = 15 deg/s

- Elevation Acceleration = 30 deg/s2

Proposal 5: Consider 8 seconds as reference beam steering time between two satellites for NTN UE with mechanical aperture, for both GEO and LEO.

Observation 1: Scenarios in which a directional NTN UE can support multiple simultaneous analogue beam at any given time can be considered as optimizations on the baseline case with single beam.

Observation 2: We can simplify the initial assumption by assuming that the NTN UE is capable of sustaining a single beam at any given instant of time and differentiate between whether the beam is mechanically- or electronically-steered.

Proposal 6: Agree on either of these two options as baseline assumption for directional NTN UE, for the purpose of defining any RRM and mobility requirements:

- Option 1: Assume that the NTN UE is capable of sustaining a single beam at any given instant of time, and differentiate between whether the beam is mechanically- or electronically-steered.

- Option 2: Assume that the NTN UE is capable of sustaining a single beam at any given instant of time and only differentiate beam steering time, irrespective of electronic vs mechanical steering implementation (can be abstracted away as left to implementation).

Proposal 7: De-scope or at least de-prioritize scenarios with UEs capable of forming multiple beams simultaneously, whether via single or multiple apertures.

Proposal 8: Study the two following NTN UE architecture options:

- Architecture A) Single beam, Mechanically-steered UE – UE does not support beam sweeping, and beam steering/tracking between satellites requires up to a few seconds.

- Architecture B) Single beam - Electronically-steered UE – UE can support beam sweeping, similar to existing TN FR2 UE or Base Station and beam re-trace takes on the order of us to ms.

Proposal 9: Remove requirement for target cell/satellite measurement before an inter-satellite mobility event, for both inter-satellite HO and inter-satellite cell re-selection.

Proposal 10: The NTN UE should use knowledge of neighbor satellite ephemeris and its own position to point towards a target satellite during inter-satellite mobility.

Proposal 11: FFS impact on higher layer mobility aspects.  Consider sending an LS to RAN2 to clarify whether beam steering time for mechanically-steered beam may impact mobility timers (e.g. such as t304) during inter-satellite mobility in IDLE and CONNECTED modes.

Proposal 12: Consider the following situations when handling inter-satellite mobility for mechanically-steered NTN UE when the UE does not find the target satellite due to temporary or permanent LoS obstruction or absence of the satellite:

  - Case 1: Temporary obstruction

  - Case 2: Permanent obstruction or absence of target satellite

# Reference

- Eutelsat TYPE APPROVAL AND CHARACTERIZATION PROCEDURES ESOG 120 Issue 8 – Rev. 1, May 2021;  https://www.eutelsat.com/files/contributed/satellites/pdf/esog120.pdf

- Inmarsat Global Xpress Release 2 System Definition Manual (SDM) – Not Public.

- Viasat G-12 Ka Antenna System Specification – Not Public.

- https://www.inmarsat.com/en/about/technology/satellites.html

- https://en.wikipedia.org/wiki/Inmarsat

- https://www.viasat.com/space-innovation/satellite-fleet/global-satellite-internet/