3GPP TSG-RAN WG4 Meeting #108	R4-2313855

Toulouse, France, August 21 – 25, 2023

Source: 	Huawei

Title: 	WF for FR2-2 MU

Agenda Item:	5.4

Document for:	Approval

# WF

The following WF is agreed for the remaining issues on the FR2-2 MU budget for NR BS:

- C1-7 contributor

  - To merge C1-7 and C1-7_mixer rows for in-band measurement, with the updated UID description:

C1-7 RF power measurement equipment (e.g. spectrum analyzer, power meter, mixer) – low power (UEM, absolute ACLR)

- LNA MU

  - RAN4 aims to conclude MU for FR2-2 by RAN#108 meeting. No further discussion on LNA usage assumption after RAN#108 meeting.

  - LNA MU contributor of 0.16 dB to be added to the Absolute ACLR, relative ACLR, and to the OBUE MU calculation spreadsheets.

- Switching uncertainty MU

  - A2-11 (Switching uncertainty) for CATR inband TRP measurement; 52.6 < f < 71GHz: 0.25 dB

- Tx OFF requirement

  - No LNA for TX OFF MU budget.

- EVM MU

  - Keep as 1%

- inband TRP requirement

  - Option 1 agreed as one possible procedure; further work on the CR drafting, i.e.

For inband TRP, in order to use reduced MU for power measurement equipment, calibration procedure described in this document should be used.

- Spurious emissions requirement

  - C1-7 for [60 < f ≤ 71 GHz]: 2.0 dB (1 sigma)