3GPP TSG-RAN WG4 Meeting #108                                                               R4-2313867

Toulouse, France, August 21 – August 25 , 2023

Agenda item:			8.19.4

Source:	Samsung

Title:	WF for feasibility and BS requirements impact on SBFD operation

Document for:	Approval

# Impacts on BS RF requirements

## Agreements achieved in Tuesday BS RF Session

Issue 2-1-3: Multi-carrier BS analysis - Agreement: - During Rel-18 SI, RAN4 will only discuss the interpretation-1 of multi-carrier support for SBFD-capable BS, i.e., SBFD operates in only one BS carrier, and legacy TDD operates in other intra-band BS carrier(s) contiguous or non-contiguous to the SBFD carrier. - RAN4 didn’t study the feasibility for the case on interpretation-2 of multi-carrier support for SBFD-capable BS i.e., SBFD operates in more than one BS carriers, and legacy TDD operates in the other intra-band BS carrier(s) (if any), which is contiguous or non-contiguous to the SBFD carriers. Issue 3-1-3: Time-domain configuration for SBFD-capable BS RF requirement - Agreement: - Proposal 1: For SBFD-capable BS, Existing RF requirements shall be applied in the OFDM symbols/slots others than SBFD symbols/slots; RAN4 discussion shall only be focused on RF requirement impacts in the SBFD symbols/slots. - Proposal 2: For SBFD-capable BS, RF requirement impacts for SBFD operation in symbols/slots configured as UL in TDD-UL-DL-ConfigCommon shall be treated as 2nd priority. Issue 3-1-4: Frequency-domain configuration for SBFD-capable BS RF requirement - Agreement: For SBFD-capable BS, RAN4 RF requirement study shall be discussed by restricting the maximum number of UL subbands for SBFD operation in an SBFD symbol/slot within a TDD carrier to be one. Issue 3-2-2: Output power dynamics - Agreement: - RE power control dynamic range: Same requirements can be applied. - Total dynamic range: Requirements applicable for SBFD slots - FFS for the requirements limit and conformance testing Issue 3-2-4: Transmitted signal quality - Agreement: - For transmitted signal quality: - All the existing requirements for frequency error, modulation quality (EVM) and time alignment error (TAE) shall also be applied to BS in SBFD symbols. - Further discuss the joint measurement for normal DL symbols/slots and SBFD DL symbols/slots during WI phase. Issue 3-2-5: Unwanted emissions - Agreement: - OBW: For occupied bandwidth requirement: Apply the existing OBW requirement for the whole BS channel bandwidth in SBFD symbols/slots. - OBUE: The RF bandwidth edge from which OBUE is defined is the edge of the carrier (same for both SBFD and non-SBFD symbols/slots). - Transmitter spurious emission: All the existing requirements shall also be applied to SBFD-capable BS in SBFD symbols, - Note: The requirement of protection of the BS receiver of own or different BS is not applicable for TDD operation. - Co-location/co-existence: - Option 1: Co-location requirement can’t use 30 dB coupling loss as the coupling loss assumption for SBFD capable gNB co-location related requirement. - Option 2: No update on existing requirements, it’s declaration basis whether BS need to follow the requirements. FFS whether applicable for SBFD symbols/slots. Issue 3-2-6: Transmitter intermodulation - Agreement: - FFS whether Tx IMD requirements applicable for SBFD slots/symbols Issue 3-3-1: Reference sensitivity level and OTA sensitivity - Agreement: - For BS type 1-H if supported: The existing requirement for conducted reference sensitivity level shall also be applied to BS in SBFD symbols, i.e, no degradation allowed. - For BS type 1-C: FFS whether supported for SBFD capable BS, FFS for the requirements and conformance testing Issue 3-3-2: Dynamic range - Agreement: Dynamic range requirements applicable for SBFD symbols/slots - FFS for IoT level, and wanted signal power level Sub-topic 3-4: Potentially new requirements for SBFD operation Sub-topic description: Issue 3-4-1: Transmitter transient period between SBFD and non-SBFD - Agreement: - For transmitter transient period between SBFD and non-SBFD or SBFD reconfigurations if needed: requirement shall be introduced to BS in SBFD symbols/slots, by defining the transient period as the time period which the transmitter is changing from the SBFD operation to non-SBFD operation or vice versa, or during SBFD reconfigurations. Issue 3-4-2: In-channel adjacent subband leakage ratio, In-channel adjacent subband Blocking and adjacent subband selectivity - Agreement: - FFS whether new requirements need to be specified for co-site inter-sector gNB and inter-site gNB with candidate list as following - in-channel adjacent subband leakage ratio, - in-channel adjacent subband blocking and - in-channel adjacent subband selectivity.

## Other issues not yet discussed on Tuesday BS RF Session

#### Issue 3-1-5: TP for detailed TR skeleton for impact on BS RF requirements

- Agreement:

  - Use the TP as the baseline for the TP drafting for the section 10.1 for BS requirement impact.

## 10.1 Impact on BS RF requirements Editor's note: This section captures the impact on BS RF requirements ### 10.1.1 General Editor's note: This section captures the general analysis for BS RF requirements, and also some general assumption which shall be based for the detailed analysis for BS RF requirements. ### 10.1.2 Impact on BS TX requirements Editor's note: This section captures the RF requirement impact analysis for all existing BS TX requirements, which has been specified in TS38.104 already. #### 10.1.2.1 Base Station output power and radiated transmit power #### 10.1.2.2 Output power dynamics #### 10.1.2.3 Transmit ON/OFF power #### 10.1.2.4 Transmitted signal quality #### 10.1.2.5 Unwanted emissions #### 10.1.2.6 Transmitter intermodulation ### 10.1.3 Impact on BS RX requirements Editor's note: This section captures the RF requirement impact analysis for all existing BS RX requirements, which has been specified in TS38.104 already. #### 10.1.3.1 Reference sensitivity level and OTA sensitivity #### 10.1.3.2 Dynamic range #### 10.1.3.3 In-band selectivity and blocking #### 10.1.3.4 Out-of-band blocking #### 10.1.3.5 Receiver spurious emissions #### 10.1.3.6 Receiver intermodulation #### 10.1.3.7 In-channel selectivity ### 10.1.4 Potentially new requirements for SBFD operation Editor's note: This section captures the analysis for potentially new requirements for SBFD operation, which has not been specified in TS38.104 for non-SBFD operation. #### 10.1.4.1 Transmitter transient period #### 10.1.4.2 In-channel adjacent subband leakage ratio, #### 10.1.4.3 In-channel adjacent subband Blocking and adjacent subband selectivity

#### Issue 3-3-3: In-band selectivity and blocking

- Agreement:

  - ACS requirement and the interference level shall be determined by RAN4 co-existence study, and for the definition of ACS requirement:

    - Conducted ACS: Take the existing wanted signal of ACS requirement by using the existing reference sensitivity level.

    - OTA ACS: The OTA sensitivity degradation shall be taken into account to determine the level of wanted signal and interference signal mean power.

  - In-band blocking requirement and the interference level shall be determined by RAN4 co-existence study, and for the definition of In-band blocking requirement:

    - Conducted In-band blocking: Take the existing wanted signal of In-band blocking requirement by using the existing reference sensitivity level.

    - OTA In-band blocking: The OTA sensitivity degradation shall be taken into account to determine the level of wanted signal and interference signal mean power.

#### Issue 3-3-4: Out-of-band blocking

- Agreement:

  - For out-of-band blocking requirement: no change is needed for out-of-band blocking, except.

    - OTA out-of-band blocking: The OTA sensitivity degradation shall be taken into account to determine the level of wanted signal mean power.

#### Issue 3-3-5: Receiver spurious emissions

- Agreement:

  - For receiver spurious emissions (for both conducted and OTA tests): Apart from existing requirements for normal reception on UL symbols, no need to specify additional receiver spurious emissions requirement for SBFD operation in SBFD symbols.

#### Issue 3-3-6: Receiver intermodulation

- Agreement:

  - RX intermodulation requirement and the interference levels shall be determined by RAN4 co-existence study, and for the definition of RX intermodulation requirement:

    - Conducted RX intermodulation: Take the existing wanted signal of RX intermodulation requirement by using the existing reference sensitivity level.

    - OTA RX intermodulation: The OTA sensitivity degradation shall be taken into account to determine the level of wanted signal and interference signal mean power.

  - FFS whether an additional requirement based on a single input signal placed to cause IM with the RX sub-band provides any additional robustness, and whether such a requirement is anyhow implicitly captured by the SBFD RX blocking requirement.

#### Issue 3-3-7: In-channel selectivity

- Agreement:

  - In-channel selectivity

    - The requirement shall be studied based on that the wanted signal and UL interfering signal shall be located in the configured UL subband.

    - FFS the wanted signal and interfering signal levels in future work item.