3GPP TSG-RAN WG4 Meeting #108						R4-2313872

Toulouse, France, 21 – 25 August 2023

# Title: 	WF for SBFD co-existence study

# Agenda Item:	8.19.4

# Source: 	CMCC

# Document for:	Approval

- Introduction

This document provides way-forwards on adjacent channel co-existence simulation of non-overlapping subband fullduplex (SBFD) based on the outcomes of “Email discussion summary for [108][308] FS_NR_duplex_evo_Part3”.

Moderator note: it’s highly appreciated for companies to show whether simulation results are updated or not on top of #108 meeting to help moderator reduce workload of collecting simulation results in #108bis meeting.

- Way forward on adjacent channel simulation assumption for SBFD

  - Sub-topic 1-2 propagation loss or penetration loss model

    - Issue 1-2-1: principle for penetration loss for hotspot scenario

Agreements:

  - For pairs of indoor UEs deployed in the same cluster area

    - Option 1:Indoor-to-outdoor penetration losses are not calculated for pairs of indoor UEs deployed in the same cluster area

    - Option 2: RAN1 approach in TR

Note: Option 1 is baseline. No comparison between options are expected.

  - For indoor UEs in different clusters, a single penetration loss component is calculated. The indoor distance used for the calculation depends on the 2D distance between the pair of UEs.

    - Issue 1-2-2: how to  generate d2D-in for a UE-UE link associated with an indoor UE

Agreements:

  - Follow same approach as RAN1. The following is used to generate  d2D-in for a UE-UE link associated with an indoor UE (the other UE could be an outdoor UE or an indoor UE in a different building) in order to calculate the inside loss component (PLin) of the UE-UE O2I building penetration loss

d2D-in =Uniform(0,min⁡{25m,d2D2})

  - Sub-topic 1-3 ACLR/ACS modeling

    - Issue 1-3-1: adjacent channel selectivity for each base station type

Agreements:

- For co-existence simulation purpose (The ACSBS/ICSBS which refer to baseband suppression):

- Baseline assumption: 50dB for all BS classes

- Other values with in the range [46dB ~62dBc] not precluded pending on companies’ input ‘ for all BS classes

- Above agreement no impact on RAN1 evaluation

  - Sub-topic 1-5 Scenario

    - Issue 1-5-1: priority of scenarios and cases

Agreements:

- For scenario 4, high priority.

- For other scenarios, maintain previous priority.

    - Issue 1-5-2: Tx power and bandwidth for UMi-to-UMi scenario

Proposals

  - Option 1: 46dBm/100MHz

  - Option 2: 38dBm/100MHz

Agreements:

  - both option 1 and option 2 and companies choose either one or both in their simulation.

    - Issue 1-5-3: except for above parameters, other parameters for UMi-to-UMi scenario

Agreements:

the same as used in Uma-to-UMi scenario, i.e. defined in R4-2305922

    - Issue 1-5-6: scenario 7 FR2 Urban Hotspot -> Urban Hotspot

Agreements:

Down-select this scenario 7 from the study given the FR2

  - Sub-topic 1-8 sub-band filter

Proposals

  - Option 1: it is proposed to consider the sub-band filter per operator for the new deployment band which can enable more deployment scenarios. (Huawei)

Agreements:

- Companies are encouraged to bring more analysis on possible solution to enable more deployment scenarios which not reflected by existing co-existence simulation e.g. the sub-band filter per operator.

- Template to collect simulation results

Agreements:

It’s approved to use following template for final simulation results collection.

- Simulation results

Agreements:

All following agreements are based on collected simulation results until RAN4#108.

The deadline for collecting simulation results is RAN4#108bis.

The numbers and/or ranges of performance degradation will be captured in TR.

Blocking probability (receiver is being blocked) is not mandatory to be provided, and can be captured for information.

## 4.1 Scenario 1 FR1 Urban Macro -> Urban Macro (high priority)

Case 1: aggressor SBFD DU victim NR TDD DL (high priority)

Agreements:

- For 100% grid shift and baseline assumption, SINR/throughput degradation is acceptable for both SBFD antenna configuration 1 and 2.

Case 2: aggressor SBFD DU victim NR TDD UL (low priority)

Agreements:

- For 100% grid shift and baseline assumption, higher than 5% throughput loss is observed.

- The degradation is even worse for less grid shift.

Case 3: aggressor NR TDD DL victim SBFD DU (high priority)

Agreements:

- For aggressor NR TDD DL interfere SBFD UL:

  - For 100% grid shift, interference is higher than 5% throughput loss is observed.

    - The degradation is even worse for less grid shift.

    - For 100% grid shift , no 100% UL throughput loss is observed at SBFD receiver

  - [Regarding blocking probability, only one company show the results e.g. 2% blocking from Ericsson]

- For aggressor NR TDD DL interfere SBFD DL: TBD

Case 4: aggressor NR TDD UL victim SBFD DU  (low priority)

Agreements:

For 100% grid shift and baseline assumption, interference is acceptable for cell average, FFS for cell edge.

## 4.2 Scenario 2 FR1 Urban Hotspot -> Urban Hotspot (2nd priority)

Draw conclusion on simulation results in next meeting.

## 4.3 Scenario 3 FR1 Indoor -> Indoor (2nd priority)

Agreements:

- For 100% grid shift and baseline assumption, the interference between legacy TDD and SBFD using adjacent channel is acceptable, i.e. less than 5% for both SBFD antenna configuration 1 and configuration 2.

## 4.4 Scenario 4 FR1 UMa-to-UMi (high priority)

Case 1: aggressor SBFD DU victim NR TDD DL (high priority)

Agreements:

- For 100% grid shift and baseline assumption, SINR/throughput degradation is acceptable.

Case 2: aggressor SBFD DU victim NR TDD UL (low priority)

Agreements:

- For 100% grid shift and baseline assumption, higher than 5% throughput loss is observed.

  - It’s observed that the degradation is even worse with 50% grid shift based on the results in R4-2313631 submitted in RAN4#108

## 4.5 Scenario 5 FR1 UMi -> FR1 UMi (2nd priority)

Case 1: aggressor SBFD DU victim NR TDD DL (high priority)

Agreements:

- At cell-average, for 100% grid shift, SINR/throughput degradation is acceptable with the gNB Tx power is 46dBm/100M or 38dBm/100MHz.

- Further discuss the cell edge impact

Case 2: aggressor SBFD DU victim NR TDD UL (low priority)

Agreements:

- For 100% grid shift, simulation results are much sensitive to gNB Tx power. For lower power, i.e. 38dBm/100MHz, interference is acceptable whereas for high power, i.e. 46dBm/100MHz interference is higher than 5% throughput loss.

Case 3: aggressor NR TDD DL victim SBFD DU (high priority)

Agreements:

- For aggressor NR TDD DL interfere SBFD UL:

  - For 100% grid shift, simulation results are much sensitive to gNB Tx power. For lower power, i.e. 38dBm/100MHz, interference is acceptable with current or enhanced ACIR whereas for high power, i.e. 46dBm/100MHz interference is higher than 5% throughput loss.

- For aggressor NR TDD DL interfere SBFD DL: interference is acceptable

Case 4: aggressor NR TDD UL victim SBFD DU  (low priority)

Agreements:

- For 100% grid shift and baseline assumption, interference is acceptable.

## 4.6 Scenario 6 FR2 Urban Macro -> Urban Macro (high priority)

Case 1: aggressor SBFD DU victim NR TDD DL (high priority)

Agreements:

- For 100% grid shift, SINR/throughput degradation is acceptable with current or enhanced ACIR.

Case 2: aggressor SBFD DU victim NR TDD UL (low priority)

Agreements:

- For 100% grid shift and baseline assumption, SINR/throughput degradation is acceptable with current or enhanced ACIR

- Some companies show simulation results is extremely sensitive to parameters like the GS and the BS transmission power.

Case 3: aggressor NR TDD DL victim SBFD DU (high priority)

Agreements:

- For aggressor NR TDD DL interfere SBFD UL:

  - For 100% grid shift, interference is acceptable with current or increased ACIR

  - For less than 100% grid shift or BS power is increased, interference maybe larger than 5% loss and one company show 4% blocking probability at cell edge

- For aggressor NR TDD DL interfere SBFD DL: interference is acceptable for 100% grid shift.

Case 4: aggressor NR TDD UL victim SBFD DU  (low priority)

Agreements:

- For 100% grid shift and baseline assumption, interference is acceptable for the case when NR TDD UL interfere SBFD UL and DL

## 4.7 Scenario 7 FR2 Urban Hotspot -> Urban Hotspot (down-select)

Agreements:

- Down-select this scenario 7 from the study given the FR2

## 4.8 Scenario 8 FR2 Urban Micro -> Urban Micro (2nd priority)

Draw conclusion on simulation results in next meeting.

## 4.9 Scenario 9 FR2 Indoor -> Indoor (2nd priority)

Draw conclusion on simulation results in next meeting.