3GPP TSG-RAN WG4 Meeting # 108      	R4-2313875

Toulouse, France, August 21 – August 25 , 2023

Title: 	WF for ATG demodulation requirements

Agenda Item:	8.13.6

Source: 	CMCC

Document for:	Approval

- General

  - Agreements

Issue 1-2: TDD pattern 30D4S6U

Agreement:

- Wait RAN1 response. If RAN1 response that “Increasing the number of HARQ processes” and “K1 range extension” issue can be solved by existing NTN solution, consider to define corresponding demodulation requirements with the new TDD pattern and note should be added in the specification that this pattern is for ATG scenario only, these requirements should be optional based on UE capability

Issue 1-3: Applicability rule for TDD pattern

- Option 1: the legacy TDD pattern can be skipped if the test of new TDD pattern is passed;

- Other options not precluded

Issue 1-4: Applicability rule for antenna configuration

Agreement:

- 2Rx can be skipped if the test of 4Rx is passed for the ATG CPE supporting both 2Rx and 4Rx

Issue 1-5: Specification impact

For ATG UE demodulation requirements

- Option 1: Introduce a new section under each physical channel for ATG UE demodulation requirements.

- Option 2:

  - New clause for ATG applicability of requirements should be introduced

  - New clauses for ATG new incremental PDSCH requirements should be introduced

For ATG BS demodulation requirements

- Option 1: Add a new section for ATG PUSCH demodulation requirements. In this section, clarifications for how to reuse legacy applicability rules and requirements should be added, and new defined PUSCH demodulation requirements could be captured.

- Option 2:

  - New clause for ATG new incremental PUSCH requirements should be introduced in 38.104

  - New clause for ATG applicability of PUSCH requirements should be introduced in 38.141-1 and 38.141-2

  - New clause for ATG applicability of PUCCH requirements should be introduced in 38.141-1 and 38.141-2

  - New clause for ATG applicability of PUCCH requirements should be introduced in 38.141-1 and 38.141-2

- Option 3: Capture ATG demodulation requirement into the same section with legacy requirement to minimize the effort of specification modification with adding the referring statement as “The following requirements in sections of 8.2.1 and 8.2.3 can be applied for BS declared to support ATG scenario”. New dedicated requirement can be added into the corresponding table in section 8.2.1

Issue 1-6: Manufactory declaration for ATG BS

Agreement:

- Introduce a new manufactory declaration for ATG BS. I.e., in TS38.141-1

| D.xxx | Air-to-ground scenario | Declaration of air-to-ground scenario support, i.e. ATG support or no ATG support | x | x |
| --- | --- | --- | --- | --- |

- UE demodulation

  - PDSCH requirements

Issue 2-1-1: MCS

Agreement:

- Consider MCS24 (Table-2) for defining requirements for 256QAM

Issue 2-1-2: Test scope for PDSCH

Agreement:

- For new incremental requirements:

  - introduces following test cases for new PDSCH requirements

| FDD 10 MHz 15kHz SCS | 2T2R, 2T4R | 16QAM (MCS [13] in table 1) |
| --- | --- | --- |
|  |  | 64QAM (MCS [22] in table 1) |
|  |  | 256QAM (MCS [24] in table 2) |

| TDD 40MHz 30kHz SCS | 7D1S2U & New TDD pattern for ATG specific scenario e.g. flexible TDD configuration or 30D4S6U(if introduced) | 2T2R, 2T4R | 16QAM (MCS [13] in table 1) |
| --- | --- | --- | --- |
|  |  |  | 64QAM (MCS [22] in table 1) |
|  |  |  | 256QAM (MCS [24] in table 2) |

  - PDCCH requirements

Issue 2-2-1: Test scope for PDCCH

- Option 1: To consider legacy PDCCH requirements for ATG PDCCH requirements.

  - 2T2R FDD: All test cases in 5.3.2.1.2

  - 2T4R FDD: All test cases in 5.3.3.1.2

  - 2T2R TDD: All test cases in 5.3.2.2.2

  - 2T4R TDD: All test cases in 5.3.3.2.2

- Option 2: Select following legacy test cases for ATG UEs

  - 1T2R FDD: Test number 2, 3 and 4 in 5.3.2.1.1

  - 2T2R FDD: All test cases in 5.3.2.1.2

  - 1T2R TDD: Test number 1 and 2 in 5.3.2.2.1

  - 2T2R TDD: All test cases in 5.3.2.2.2

  - 1T4R FDD: Test number 2, 3 and 4 in 5.3.3.1.1

  - 2T4R FDD: All test cases in 5.3.3.1.2

  - 1T4R TDD: Test number 1 and 2 in 5.3.3.2.1

  - 2T4R TDD: All test cases in 5.3.3.2.2

- Option 3: Select following legacy test cases for ATG UEs

  - 1T2R FDD: Test number 1, 3 and 5 in 5.3.2.1.1

  - 2T2R FDD: Test number 3 in 5.3.2.1.2

  - 1T2R TDD: All test cases in 5.3.2.2.1

  - 2T2R TDD: All test cases in 5.3.2.2.2

  - 1T4R FDD: Test number 1, 3 and 5 in 5.3.3.1.1

  - 2T4R FDD: Test number 3 in 5.3.3.1.2

  - 1T4R TDD: All test cases in 5.3.3.2.1

  - 2T4R TDD: All test cases in 5.3.3.2.2

- Option 4: Do not consider in the ATG scope legacy PDCCH requirements

  - CSI reporting requirements

Issue 2-3-1: Test scope for CSI reporting

Agreement:

- For CQI requirements, reuse the following requirements:

  - 2T2R FDD: CQI requirements in 6.2.2.1.1.1

  - 2T2R TDD: CQI requirements in 6.2.2.2.1.1

  - 2T4R FDD: CQI requirements in 6.2.3.1.1.1

  - 2T4R TDD: CQI requirements in 6.2.3.2.1.1

- Not introduce RI requirements for ATG UE

- FFS for PMI requirements

- BS demodulation

  - PUSCH requirements

Issue 3-1: MCS

Agreement:

- For the test case which reusing existing requirements, cover 16QAM, 64QAM and 256QAM, whether to test 256QAM based on the manufactory declaration.

- For the new dedicated ATG requirements, cover [64QAM MCS [28] (Table 1)] and 256QAM MCS [22] (Table 2), whether to test 256QAM based on the manufactory declaration.

Issue 3-2: Test metric

Agreement:

- For the test case which reusing existing requirements, the test metric cover SNR at 70% and 30% TP.

- Only consider 70% throughput requirements for new dedicated requirements.

Issue 3-3: Test scope for PUSCH

Agreement:

- For new incremental requirements:

  - The same requirements are applicable to TDD with different UL-DL pattern

- For legacy requirements reusing:

  - Consider normal PUSCH demodulation and UCI multiplexing on PUSCH as mandatory

- Simulation assumption

Simulation assumption for PDSCH

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Frequency range |  |  | FR1 | FR1 |
| Bandwidth |  | MHz | 10 | 40 |
| Subcarrier spacing |  | kHz | 15 | 30 |
| Duplex Mode |  |  | FDD | TDD |
| TDD Slot Configuration |  |  | N/A | 7D1S2U S:6D+4G+4U |
|  |  |  |  | 30D4S6U S:40G |
| Propagation channel |  |  | AWGN+220Hz doppler | AWGN+500Hz doppler |
| Antenna configuration |  |  | 2x2 | 2x2 |
|  |  |  | 2x4 | 2x4 |
| Beamforming Model |  |  | As defined in Annex B.4.1 in TS 38.101-4 | As defined in Annex B.4.1 in TS 38.101-4 |
| MCS/Rank |  |  | Rank 1 16QAM Table 1 MCS13 64QAM Table 1 MCS 22 256QAM Table 2 MCS 24 | Rank 1 16QAM Table 1 MCS13 64QAM Table 1 MCS 22 256QAM Table 2 MCS 24 |
| Receiver type |  |  | MMSE-IRC | MMSE-IRC |
| PDSCH configuration | Mapping type |  | Type A | Type A |
|  | Starting symbol (S) |  | 2 | 2 |
|  | Length (L) |  | 12 | 12 |
|  | PRB bundling size |  | 2 | 2 |
|  | PRB bundling type |  | Static | Static |
|  | VRB-to-PRB mapping interleaver bundle size |  | Non-interleaved | Non-interleaved |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 | Type 1 |
|  | Number of additional DMRS |  | 1 | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 | 1 |
| NZP CSI-RS for CSI acquisition | Row index (Note 3) |  | 3 | 3 |
|  | First subcarrier index in the PRB used for CSI-RS |  | k0 = 0 | k0 = 0 |
|  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 | l0 = 12 |
|  | Number of CSI-RS ports (X) |  | 2 | 2 |
|  | CDM Type |  | 'FD-CDM2' | 'FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 |
|  | CSI-RS periodicity | Slots | 20 | 40 |
|  | CSI-RS offset | Slots | 0 | 0 |
|  | Frequency Occupation |  | Start PRB 0 | Start PRB 0 |
|  |  |  | Number of PRB = ceil(BWP size/4)*4 | Number of PRB = ceil(BWP size/4)*4 |
|  | QCL info |  | TCI state #1 | TCI state #1 |
| ZP CSI-RS for CSI acquisition | Row index (Note 3) |  | 5 | 5 |
|  | First subcarrier index in the PRB used for CSI-RS |  | k0 = 4 | k0 = 4 |
|  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 | l0 = 12 |
|  | Number of CSI-RS ports (X) |  | 4 | 4 |
|  | CDM Type |  | 'FD-CDM2' | 'FD-CDM2' |
|  | Density (ρ) |  | 1 | 1 |
|  | CSI-RS periodicity | Slots | 20 | 40 |
|  | CSI-RS offset | Slots | 0 | 0 |
|  | Frequency Occupation |  | Start PRB 0 | Start PRB 0 |
|  |  |  | Number of PRB = ceil(BWP size/4)*4 | Number of PRB = ceil(BWP size/4)*4 |
| Maximum number of HARQ transmission |  |  | 4 | 4 |
| Number of HARQ Processes |  |  | 4 | 8 for 7DS2U 32 for 30D4S6U |
| Test metric |  |  | 70% of max throughput at target SNR. |  |

Simulation assumption for PUSCH

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Frequency range |  |  | FR1 | FR1 |
| Bandwidth |  | MHz | 5 | 10 |
| Subcarrier spacing |  | kHz | 15 | 30 |
| Duplex Mode |  |  | FDD | TDD |
| TDD Slot Configuration |  |  | N/A | 7D1S2U S:6D+4G+4U |
| Channel model |  |  | AWGN+200Hz | AWGN+500Hz |
| MCS/Rank |  |  | Rank 1 64QAM MCS 28 in Table 1 256QAM MCS 22 in Table 2 | Rank 1 64QAM MCS 28 in Table 1 256QAM MCS 22 in Table 2 |
| Antenna configuration |  |  | 1T2R | 1T2R |
| Transform precoding |  |  | CP-OFDM |  |
| HARQ | Maximum number of HARQ transmissions |  | 4 |  |
|  | RV sequence |  | 0, 2, 3, 1 |  |
| DM-RS | DM-RS configuration type |  | 1 |  |
|  | DM-RS duration |  | single-symbol DM-RS |  |
|  | Additional DM-RS position |  | pos 1 |  |
|  | Number of DM-RS CDM group(s) without data |  | 2 |  |
|  | Ratio of PUSCH EPRE to DM-RS EPRE |  | -3 dB |  |
|  | DM-RS port |  | {0} |  |
|  | DM-RS sequence generation |  | NID0=0, nSCID =0 |  |
| Time domain resource assignment | PUSCH mapping type |  | A |  |
|  | Start symbol |  | 0 |  |
|  | Allocation length |  | 14 |  |
| Frequency domain resource assignment | RB assignment |  | Full applicable test bandwidth |  |
|  | Frequency hopping |  | Disabled |  |
| Code block group based PUSCH transmission |  |  | Disabled |  |
| Test metric |  |  | 70% of max throughput at target SNR. |  |