3GPP TSG-RAN WG4 Meeting #108	R4-2313877

Toulouse, France, 21 – 25 August 2023

Title: 	WF for 8Rx UE performance requirements

Agenda Item:	8.4.4

Source: 	Huawei, HiSilicon

Document for:	Approval

1 General

Issue 1-2: Applicability rules for PDSCH/PDCCH/PBCH tests

- Extend the test applicability rules for PDSCH/PDCCH/PBCH TDD tests agreed in last RAN4#107 [4] to FDD

- The final test applicability rules:

Table 5.1.1.2-1: Requirements applicability

| Supported RX antenna ports | Test type | Test list | Exceptions |
| --- | --- | --- | --- |
| UE supports only 2RX | PDSCH | All tests in Clause 5.2.2 |  |
|  | PDCCH | All tests in Clause 5.3.2 |  |
|  | PBCH | All tests in Clause 5.4.2 |  |
| UE supports only 4RX or both 2RX and 4RX | PDSCH | All tests in Clause 5.2.3 (Note 2) |  |
|  | PDCCH | All tests in Clause 5.3.3 (Note 2) |  |
|  | PBCH | All tests in Clause 5.4.2 or 5.4.3 (Note 1) |  |
| UE supports 8Rx, 4Rx and 2Rx | PDSCH | Tests in Clause 5.2.3.(Note 2, 3) All tests in Clause 5.2.4.(Note 2) | If UE passes tests in Clause 5.2.4, UE can skip Test 2-1 and Test 2-2 in Clause 5.2.3 Table 5.2.3.1.1-4, Table 5.2.3.2.1-4 and Test 4-1 in Clause 5.2.3 Table 5.2.3.1.1-6, Table 5.2.3.2.1-6 |
| or | PDCCH | All tests in Clause 5.3.3.(Note 2,3) |  |
| UE supports only 8Rx and 4Rx | PBCH | All tests in Clause 5.4.3(Note 1) |  |
| UE supports only 8Rx and 2Rx | PDSCH | Tests in Clause 5.2.2.(Note 2, 4) All tests in Clause 5.2.4.(Note 2) | If UE passes tests in Clause 5.2.4, UE can skip Test 2-1 and Test 2-2 in Clause 5.2.2 Table 5.2.2.1.1-4, Table 5.2.2.2.1-4 |
|  | PDCCH | All tests in Clause 5.3.2.(Note 2,4) |  |
|  | PBCH | All tests in Clause 5.4.2 |  |
| UE supports only 8Rx | PDSCH | All tests in Clause 5.2.4.(Note 2) |  |
|  | PDCCH | N/A |  |
|  | PBCH | N/A |  |
| Note 1: Requirements for PBCH with 4Rx is up to UE declaration Note 2: ‘maxMIMO-Layers-r16’ is not configured during the performance requirements testing for UE supporting Release 16 per-BWP MIMO layer adaptation. Note 3: 8Rx capable UEs are tested on any of the 4Rx supported RF bands by connecting 4 out of 8 Rx with data source from system simulator, and the other 4 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Requirements specified with 4Rx should be applied. Note 4: 8Rx capable UEs are tested on any of the 2Rx supported RF bands by connecting 2 out of 8 Rx with data source from system simulator, and the other 6 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Requirements specified with 2Rx should be applied. |  |  |  |

Issue 1-2: Applicability rules for CSI test

Confirm the applicability rules for CSI test agreed in last meeting [4]:

Updated Table 6.1.1.2-1: Requirements applicability

| Supported RX antenna ports | Test type | Test list | Exception |
| --- | --- | --- | --- |
| UE supports only 2RX | CQI | All tests in Clause 6.2.2 |  |
|  | PMI | All tests in Clause 6.3.2 |  |
|  | RI | All tests in Clause 6.4.2 |  |
| UE supports only 4RX or both 2RX and 4RX | CQI | All tests in Clause 6.2.3 |  |
|  | PMI | All tests in Clause 6.3.3 |  |
|  | RI | All tests in Clause 6.4.3 |  |
| UE supports 2Rx, 4Rx and 8Rx, or UE supports 4Rx and 8Rx | CQI | Tests in Clause 6.2.3(Note 1) All tests in Clause 6.2.4 | If UE has passed Tests in Clause 6.2.4, UE can skip tests in Clause 6.2.3.2.1.1 If UE has passed Tests in Clause 6.2.4, UE can skip tests in Clause 6.2.3.1.1.1 |
|  | PMI | All tests in Clause 6.3.3 |  |
|  | RI | All tests in Clause 6.4.3 |  |
| UE supports 2Rx and 8Rx | CQI | Tests in Clause 6.2.2(Note 2) All tests in Clause 6.2.4 | If UE has passed Tests in Clause 6.2.4, UE can skip tests in Clause 6.2.2.2.1.1 If UE has passed Tests in Clause 6.2.4, UE can skip tests in Clause 6.2.2.1.1.1 |
|  | PMI | All tests in Clause 6.3.2 |  |
|  | RI | All tests in Clause 6.4.2 |  |
| UE supports only 8Rx | CQI | All tests in Clause 6.2.4 |  |
|  | PMI | N/A |  |
|  | RI | N/A |  |
| Note 1: 8Rx capable UEs are tested on any of the 4Rx supported RF bands by connecting 4 out of 8 Rx with data source from system simulator, and the other 4 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Requirements specified with 4Rx should be applied. Note 2: 8Rx capable UEs are tested on any of the 2Rx supported RF bands by connecting 2 out of 8 Rx with data source from system simulator, and the other 6 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Requirements specified with 2Rx should be applied. |  |  |  |

Issue 1-3: Test applicability rules for CA demodulation requirements

- Option 1: For FR1 8Rx CA applicability rules of demodulation performance requirements, extend Table 5.1.1.7.2-1 to include tests cases in new Clause 5.2A.4.1.

- Other options not precluded

Issue 1-4: Applicability rules for different number of RX antenna ports for CA demodulation requirements

- Option 1:

  - Within the CA configuration if any of the PCell and/or the SCells is a 4Rx supported RF band, 4 out of the 8Rx should be connected with data source from system simulator, depending on UE’s declaration and AP configuration. Requirements from Clause 5.2A.3.1 are applied.

  - Within the CA configuration if any of the PCell and/or the SCells is a 2Rx supported RF band, 2 out of the 8Rx should be connected with data source from system simulator, depending on UE’s declaration and AP configuration. Requirements from Clause 5.2A.2.1 are applied.

  - Within the CA configuration if any of the PCell and/or the SCells is a 8Rx supported RF band, all 8Rx should be connected with data source from system simulator. Requirements from Clause5.2A.4.1 are applied.

  - For 8Rx capable UEs, the 2Rx supported RF bands, 4Rx supported RF bands and 8Rx supported RF bands are up to UE’s declaration.

- Other options not precluded

Issue 1-5: Applicability rules for different number of RX antenna ports for CA CSI requirements

- Option 1: For FR1 8Rx CA applicability rules and antenna connection of CSI reporting requirements, below requirements should be specified.

  - All the requirements specified in clause 6.2A for CA with 4 RX are applied for 8 RX capable UEs by connecting all 8 RX with data source from system simulator and reducing the signal power density by 3 dB compared to the signal power density for 4 RX in the test configurations.

  - All the requirements specified in clause 6.2A for CA with 2 RX are applied for 8 RX capable UEs by connecting all 8 RX with data source from system simulator and reducing the signal power density by 6 dB compared to the signal power density for 2 RX in the test configurations.

- Other options not precluded

2 Test scope for CA requirements

Issue 2-1: CA test for PDSCH and SDR

Define PDSCH and SDR 8Rx CA requirements.

Issue 2-2: CA test with power imbalance

Not consider CA test with power imbalance for 8Rx

Issue 2-3: CA test for CQI reporting

- Option 1: Not consider CQI reporting test for CA

- Option 2: Define CQI test for CA

3 PDCCH test

Follow the previous agreements that not define 8Rx PDCCH requirements [3]

4 PDSCH requirements

4.1 TDD requirements

Issue 4-1-1: MCS for Rank 2 test

MCS 19 (Table 1)

Issue 4-1-2: MCS for Rank 4 test

MCS 17 (Table 1)

Issue 4-1-3: MCS for Rank 8 test

MCS 17 (Table 1)

4.2 FDD requirements

RAN4 defines PDSCH FDD requirements with same parameters including MCS values, MIMO layers, antenna configuration, and propagation conditions as TDD requirements.

Annex A can be referred for the details.

4.3 CA requirements

Issue 4-3-1: CBW for PDSCH CA requirements

- Option 1: Follow the existing CA requirements defined for 2Rx and 4Rx, i.e.

  - FDD: 15kHz SCS: 5/10/15/20/25/30/35/40/45/50 MHz

  - TDD: 30kHz SCS: 5/10/15/20/25/30/40/50/60/80/90/100 MHz

- Option 2: Single bandwidth for each CC: 15kHz/10MHz for FDD and 30kHz/40MHz for TDD separately

Issue 4-3-2: Duplex model

- Option 1: Consider the following duplex model for 8Rx CA

  - FDD 15kHz + TDD 30kHz

  - FDD 15kHz + FDD 15kHz

  - TDD 30kHz + TDD 30kHz

- Other options not precluded

Issue 4-3-3: Test scenario

One single test scenario to be defined, with same test configurations as single carrier case except the bandwidth:

- Option 1: Rank8 case

- Option 2: Rank2 case

5 SDR requirements

Issue 5-1: MCS look up Table for FDD

Reuse the MCS look up Table agreed for TDD [4]:

- MCS look up Table for 64QAM

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- |
| 8 | 6 | 1 | 26 |
| 8 | 6 | 0.8 | 24 |
| 8 | 6 | 0.75 | 23 |
| 8 | 6 | 0.4 | 14 |
| 8 | 4 | 1 | 16 |
| 8 | 4 | 0.8 | 16 |
| 8 | 4 | 0.75 | 16 |
| 8 | 4 | 0.4 | 11 |
| 8 | 2 | 1 | 9 |
| 8 | 2 | 0.8 | 9 |
| 8 | 2 | 0.75 | 9 |
| 8 | 2 | 0.4 | 5 |
| Note 1: MCS index for maximum modulation format 8 is based on MCS index Table 1 defined in clause 5.1.3.1 of TS 38.214 |  |  |  |

- MCS look-up Table for 256QAM 8 MIMO layers:

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- |
| 8 | 8 | 1 | 24 |
| 8 | 8 | 0.8 | 23 |
| 8 | 8 | 0.75 | 22 |
| 8 | 8 | 0.4 | 12 |
| Note 2: MCS Index for maximum modulation format 8 is based on MCS index Table 2 defined in clause 5.1.3.1 of TS 38.214 |  |  |  |

- MCS look-up Table for 1024 QAM 2 MIMO layers:

| Supported RX antenna ports | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- | --- |
| 8RX | 2 | 10 | 1 | 24 |
|  | 2 | 10 | 0.8 | 21 |
|  | 2 | 10 | 0.75 | 19 |
|  | 2 | 10 | 0.4 | 9 |
| Note 1: MCS Index for maximum modulation format 10 is based on MCS index Table 4 defined in clause 5.1.3.1 of TS 38.214 [12] |  |  |  |  |

Issue 5-2: SDR CA requirements

- Option 1: Choose single bandwidth combination for SDR requirements definition: 40MHz/30kHz SCS and 10MHz/15kHz SCS

- Option 2: Reuse the existing methodology of Re-15 SDR CA test

6 CSI requirements

6.1 TDD requirements

Issue 6-1-1: Report quantity

Keep the previous agreements [4]

- two-one-TypeI-SinglePanel-Restriction = 00000001

- RI Restriction = 00001000

- TE schedules fixed PMI matrix with i1,1=i1,2= i2=i1,3= 0

Issue 6-1-2: SNR points

[4, 5]dB and [10, 11]dB

6.2 FDD requirements

Issue 6-2-1: Test parameter

- 10 MHz/15 kHz, 52 RBs.

- CQI index for up to 64QAM.

- Rank 4.

Issue 6-2-2: Report quantity

Same as TDD requirements

- two-one-TypeI-SinglePanel-Restriction = 00000001

- RI Restriction = 00001000

- TE schedules fixed PMI matrix with i1,1=i1,2= i2=i1,3= 0

Issue 6-2-3: SNR points

[4, 5]dB and [10,11]dB

Annex A: Simulation assumptions (for information)

A.1 PDSCH

Other common parameters, refer to TS 38.101-4: Table 5.2-1: Common test parameters for PDSCH

Table 1: Simulation assumptions for PDSCH performance

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD, FDD |
| TDD pattern |  |  | 7D1S2U S=6D:4G:4U |
| SCS(kHz)/Bandwidth (MHz) |  |  | TDD: 30/40 FDD: 15/10 |
| MIMO layer and Antenna configuration |  |  | Rank 2: 2x8 Rank 4: 4x8 Rank 8: 8x8 |
| Propagation condition and antenna correlation |  |  | Rank 2: TDLC300-100 ULA Medium B ( = 0.3, = 0.005154) Rank 4: TDLA30-10 Low Rank 8: TDLA30-10 Low |
| MCS |  |  | Rank2: MCS19 Rank 4: MCS17 Rank 8: MCS17 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols (maxLength) for DL |  | Rank2/4: Single-symbol Rank 8: Double-symbol |
| TRS configuration |  |  | Symbol#{5, 9} |
| PT-RS |  |  | Not configured |
| NZP-CSI-RS configuration |  |  | Rank 8: row 6 |
| Overhead |  |  | 0 |
| N1 and N2 configurations for 8Tx cases |  |  | Use (N1,N2) = (4,1), (O1, O2) = (4,1) |
| Coodebook for PDCCH for PDSCH tests with 4Tx and 8Tx |  |  | Keep same number of Tx for PDSCH and PDCCH during PDSCH test. Set “codebookMode” to 1 |
| Special slot (S slot) scheduling |  |  | Not schedule PDSCH in special slot for 8 layers cases |
| Number of HARQ Processes |  |  | TDD: 8 FDD: 4 |
| HARQ ACK/NACK bundling |  |  | Multiplexed |
| Maximum HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity |
| Tx EVM (Explicitly modeled in the simulation) |  |  | 64QAM:6% 256QAM:3% |
| Test metric |  |  | SNR@70% max TP |

A.2 SDR

Other common parameters, refer to TS 38.101-4:

- Table 5.5A-1: Common test parameters for FDD and TDD component carriers

- Table 5.5A-2: Additional test parameters for FDD CC

- Table 5.5A-3: Additional test parameters for TDD CC

Table 2: Simulation assumptions for SDR performance

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| PDSCH configuration | Mapping type |  | Type A |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | wideband |
|  | Resource allocation type |  | Type 0 |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols (maxLength) DL |  | Rank2/4: Single-symbol Rank 8: Double-symbol |
|  | Antenna ports indexes |  | {1000, 1001} for 2 Layers CCs {1000 – 1003} for 4 Layers CCs {1000 – 1007} for 8 Layers CCs |
|  | Number of PDSCH DMRS CDM group(s) without data |  | 1 for 2 layers CCs 2 for others |
| PTRS configuration |  |  | PTRS is not configured |
| Maximum number of HARQ transmission |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination with PRB bundling granularity chosen avoid zeros on Rx |
| Propagation condition |  |  | Static propagation condition No external noise sources are applied |
| Tx EVM |  |  | Tx EVM = 6% for up to 64QAM Tx EVM = 3% for 256QAM Tx EVM = 2.5% for 1024QAM |
| MIMO layers |  |  | 64QAM and 256QAM: 8 1024QAM: 2 |
| Antenna configuration | 2 layers |  | 2x8 |
|  | 4 layers |  | 4x8 |
|  | 8 layers |  | 8x8 |

A.3 CQI test

Other common parameters, refer to TS 38.101-4:

- Table 6.2.2.1.1.1-1: CQI reporting definition test for FDD

- Table 6.2.2.2.1.1-1: CQI reporting definition test for TDD

Table 3: Simulation assumptions for CQI test

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Duplex Mode |  |  |  | TDD, FDD |  |  |  |
| Bandwidth/SCS |  |  | MHz | TDD: 40MHz/30kHz FDD: 10MHz/15kHz |  |  |  |
| TDD UL-DL pattern |  |  |  | 7D1S2U S=6D+4G+4U |  |  |  |
| SNR |  |  | dB | 4 | 5 | 10 | 11 |
| Propagation channel |  |  |  | static channel model with 4T8R |  |  |  |
| Antenna configuration |  |  |  | XP 4×8 |  |  |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 in 38.101-4 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,4 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 |  |  |  |
|  | CSI-RS periodicity and offset |  | slot | TDD: 10/1 FDD: 5/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 4,(0) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | TDD: 10/1 FDD: 5/1 |  |  |  |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  |  |  |
|  | CSI-IM RE pattern |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | TDD: 10/1 FDD: 5/1 |  |  |  |
| ReportConfigType |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  | RB | TDD: 16 FDD: 8 |  |  |  |
| Csi-ReportingBand |  |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  |  | slot | TDD: 10/9 FDD: 5/0 |  |  |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | (2,1) |  |  |  |
|  |  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) |  |  |  |
|  |  | two-one-TypeI-SinglePanel-Restriction |  | 00000001 |  |  |  |
|  |  | RI Restriction |  | N/A |  |  |  |
| Physical channel for CSI report |  |  |  | PUCCH |  |  |  |
| CQI/RI/PMI delay |  |  | ms | TDD: 9.5 FDD: 8 |  |  |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |  |  |
| Number of HARQ Processes |  |  |  | FDD: 4 TDD: 8 |  |  |  |
| Measurement channel |  |  |  | TDD: As specified in Table A.4-3, TBS.3-4 FDD: As specified in Table A.4-1, TBS.1-3 |  |  |  |

Annex B: CR split for TS 38.101-4

| Section | Test cases | Companies |
| --- | --- | --- |
| 5.1.1.2 Applicability of requirements for different number of RX antenna ports | Table PDSCH/PDCCH/PBCH test applicability rules for 8Rx | Nokia |
| 5.2.4 8RX requirements (New) | 5.2.4.1 FDD PDSCH performance requirements | Samsung |
|  | 5.2.4.2 TDD PDSCH performance requirements |  |
| 5.5A Sustained downlink data rate provided by lower layers | SDR tests | Ericsson |
| 6.1.1.2 Applicability of requirements for different number of RX antenna ports | Table 6.1.1.2-1: Requirements applicability for CSI applicabaility rules | Huawei |
| 6.2.4 8RX requirements (New) | 6.2.4.1 FDD CQI requirements | CTC |
|  | 6.2.4.2 TDD CQI requirements |  |
| A.3.2 Reference measurement channels for PDSCH performance requirements | A.3.2.1.1 Reference measurement channels for SCS 15 kHz FR1 | MediaTek |
|  | A.3.2.2.2 Reference measurement channels for SCS 30 kHz FR1 |  |
| B.1 Static propagation condition | B.1.2 UE Receiver with 8Rx (New) | ZTE |
| B.2.3 MIMO Channel Correlation Matrices | MIMO Correlation Matrices using Uniform Linear Array (ULA) | Apple |
| 5.2A.4 8Rx requirements | Demodulation requirements for 8Rx PDSCH CA | Huawei |

Annex C: Reference

[1] R4-2220613, WF on 8RX UE demodulation and CSI requirements, RAN4#105, Huawei, HiSilicon

[2] R4-2302942, WF for 8Rx UE performance requirements, RAN4#106, Huawei, HiSilicon

[3] R4-2305888, WF for 8Rx UE performance requirements, RAN4#106bis-e, Huawei, HiSilicon

[4] R4-2309806, WF for 8Rx UE performance requirements final, RAN4#107, Huawei, HiSilicon