3GPP TSG-RAN WG4 Meeting # 107	R4-2313888

Incheon, KR, May 22 – May 26, 2023

Agenda Item:			8.2.6

Source:					Qualcomm Incorporated

Title:                        WF for FR2 OTA test SI

Document for:		Approval

# Topic #1: Test method for UE RF

Issue 1-1-1: Weighting of the measurement grid

- Proposals

  - Option 1 (Samsung, vivo): Either sin θ or Clenshaw-Curtis weighting can be used in measurement grid analysis. No need for further down-selection.

  - Option 2 (Agreements from RAN4#107): Clenshaw-Curtis weighting is recommended. sin θ weighting is not precluded

- Agreements:

  - Both sin θ and Clenshaw-Curtis weighting can be used in measurement grid analysis. Clenshaw-Curtis weighting is recommended.

Issue 1-1-3: MU analysis of the measurement grid

- Proposals

  - Option 1 (Qualcomm): RAN4 should discuss what uncertainty mechanisms apply for MU analysis for multi-Rx, the parameters that need to change and range of change. For example, the change of θ and φ should be limited to +/- 0.5 * step size along longitude and latitude.

Figure 1: An example of the change for measurement grid analysis (R4-2312886)

  - Option 2: TBA

- Agreements:

  - RAN4 to focus on analyzing the starting point of step size per the candidates of measurement step size as following.

    - The candidates of measurement step size include: 30deg, 15deg, 10deg.

    - The reference step size for the simulation is 1deg as the basis to calculate the gap with candidates of measurement step size.

  - The uncertainty mechanism is limited to the coarseness of the grid and doesn’t depend on the UE random orientations as the legacy approach.

  - Encourage companies to provide suggestions for the MU analysis framework and to agree on a framework at next meeting (RAN4#108bis).

Issue 1-2-1: Absolute probe locations

- Proposals

  - Option 1(Samsung): The probe location should be on the measurement grid w.r.t UE coordination

  - Option 2: (Qualcomm, Keysight): Probe locations should be multiples constant-step grid step size

- Agreement:

  - The probe location should be on the measurement grid, i.e., multiples constant-step grid step size

Issue 1-3: Test procedure

- Proposals

  - Option 1(Samsung): Clarification is needed regarding AoA1 and AoA2 in the test procedure to avoid misunderstanding

Figure 2: Test procedure of UE RF testing

  - Option 2 (OPPO): Update the test procedure diagram for multi-RX DL 2 AoA spherical coverage measurement with the above flow-chart figure. And further updates may be needed based on the core requirement outcomes.

Figure 3: Updated test procedure of UE RF testing

- Agreements:

  - Consider Option 2 as the starting point. Further updates are not precluded to capture the progress in RF requirements.

  - The test procedure implemented in the actual test system may be improved (e.g. to reduce test time) assuming the same test points are executed.

# Topic #2: Test method for UE RRM

Issue 2-1-1: Testability analysis for the RRM testing scenarios

- Proposals

  - Option 1: Time and Frequency multiplexed downlink transmission should be supported by 2AoA measurement setup for multi-Rx RRM testing. The Illustration is shown in the below figure.

Figure 4: Illustration of downlink transmission for 2AoA measurement setup (R4-2312888)

  - Option 2: TBA

- Agreement:

  - RAN4 to further discuss the testability of option 1

Issue 2-1-2: Test directions for 2AoA measurement setup

- Proposals

  - Option 1 (Qualcomm): The spherical coverage requirements defined in the UE RF session should be taken as the baseline of test directions selection in multi-Rx RRM testing.

  - Option 2: TBA

- Agreement:

  - [2 AoA Spherical coverage requirements] defined in the UE RF session should be taken as the baseline of test directions selection in multi-Rx RRM testing.

Issue 2-1-3: Dual TCI switching

- Proposals: Companies to provide the views for the following options for dual TCI switching test

  - Option 1 (Qualcomm): Dual TCI switches simultaneously, probe number for multiple AoA test system is at least 4

Figure 5: Illustration of Dual TCI switches simultaneously with 4 probes

For option 1, in the period of T1, DUT connects TCI state 0 and TCI state 1 via probe#1 and probe#2 respectively. Then in the period of T2, TCI state 0 switches to TCI state 3 via switching between probe#1 and probe#4, and in the meanwhile, TCI state 1 switches to TCI state 2 via switching between probe#2 and probe#3.

  - Option 2: Dual TCI switches sequentially, probe number for multiple AoA test system is at least

Figure 6: Illustration of Dual TCI switches simultaneously with 3 probes

For option 2, in the period of T1, DUT connects TCI state 0 via probe#1. In the period of T2, TCI state 0 (anchor TCI) firstly switches to TCI state 2 via switching between probe#1 and probe#3. Then the TCI state 1 is added via probe#2.

  - Option 3: Dual TCI switches simultaneously, but the beam directions are not changed, probe number for multiple AoA test system is at least 2

Figure 7: Illustration of Dual TCI switches simultaneously with 2 probes

For option 3, in the period of T1, DUT connects TCI state 0 and TCI state 1 via Pol.H of probe#1 and Pol.H of probe#2, respectively. Then in the period of T2, TCI state 0 switches to TCI state 3 via switching between Pol.H and Pol.V of probe 1, and in the meanwhile, TCI state 1 switches to TCI state 2 via switching between Pol.H and Pol.V of probe 2. Note that in option 3, different SSB IDs are transmitted from two polarizations in T1 and T2.

  - Option 4: TBA

- Agreement:

  - Capture all three candidate options into TR38.871 listing pros and cons, and further discuss based on the conclusion from RRM session for introduced RRM requirements.

Issue 2-2-1: SINR control for rough beam

- Proposals

  - Option 1 (Qualcomm): For rough beam, the lower bound of G1/G2 is the gain difference from legacy REFSENS and legacy EIS spherical coverage

  - Option 2: TBA

- Agreement:

  - Option 1 agreed

# Topic #3: Test method for UE Demodulation

Issue 3-1-1: X value in Noc level configuration

- Proposals

  - Option 1 (Qualcomm): As a baseline, RAN4 to use X = 2 for multi-Rx demodulation test directions selection

  - Option 2: (Huawei): Clarify the simulation assumptions before specifying X

- Agreement:

  - Further align the simulation assumption and make decision in RAM4#108bis for X value within the range [2~5].

Issue 3-2-1: Assumptions for minimum isolation simulation and corresponding MU

- Proposals

  - Option 1 (Qualcomm): The following assumptions are adopted for simulation of minimum isolation requirements.

    - For the reference SNR, the following assumptions could be considered:

      - Assume α = 0, β = 0, and γ = good enough isolation, e.g., 100dB

      - Channel model parameters

      - TDLA30-75 is assumed for 100 MHz/120 kHz

      - Time offset values: {0.25us, -0.0625us}/0; Frequency offset: 600Hz/0

      - MCS: MCS13 with 2+2

      - Receiver assumptions: Separate processing per Rx chain.

    - For comparison, to run the simulation with the isolation range of [8dB, 20dB] including both cross-polarizations and cross-talk and then compare the offset between reference SNR and required SNR with different isolation values.

  - Option 2: TBA

- Agreement:

  - The following assumptions are adopted for simulation of minimum isolation requirements.

    - For the reference SNR, the following assumptions could be considered:

      - Assume α = 0, β = 0, and ρ=γ = good enough isolation, e.g., 100dB

      - Channel model parameters

      - TDLA30-75 is assumed for 100 MHz/120 kHz

      - Time offset values: 0; Frequency offset: 0

      - MCS: MCS17 with 1+1

      - Receiver assumptions: Separate processing per Rx chain.

    - For comparison, to run the simulation with the isolation range of [8dB, 20dB] including both cross-polarizations and cross-talk and then compare the offset between reference SNR and required SNR with different isolation values.