3GPP TSG-RAN WG4 Meeting #108	R4-2313890

Toulouse, France, 21 ‒ 25 August, 2023

Agenda item:			8.26.2

Source:	Samsung

Title:	Simulation assumption for NTN co-existence study

Document for:	Approval

- Introduction

This document captures simulation assumptions for the NTN coexistence study in frequency bands above 10 GHz.

Remaining issues for further discussion are highlighted in yellow mark with square brackets.

- Discussion

Co-existence simulation scenarios

| No. | Combination | Aggressor | Victim | Frequency band | Scope of Coexistence Simulation |
| --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | NTN UL | TN UL | 27 GHz | ACLR NTN UE to be varied/defined ACS TN gNB fixed |
| 2 | TN with NTN | TN UL | NTN UL | 27 GHz | ACLR TN UE fixed ACS NTN SAN to be varied/defined |
| 3 | TN with NTN | NTN UL | TN DL | 27 GHz | ACLR NTN UE to be varied/defined ACS TN UE fixed |
| 4 | TN with NTN | TN DL | NTN UL | 27 GHz | ACLR TN gNB fixed ACS NTN SAN to be varied/defined |
| 5 | TN with NTN | TN DL | NTN DL | 17 GHz | ACLR TN gNB fixed ACS NTN UE to be varied/defined |
| 6 | TN with NTN | NTN DL | TN DL | 17 GHz | ACLR NTN SAN to be varied/defined ACS TN UE fixed |
| 7 | TN with NTN | NTN DL | TN UL | 17 GHz | ACLR NTN SAN to be varied/defined ACS TN gNB fixed |
| 8 | TN with NTN | TN UL | NTN DL | 17 GHz | ACLR TN UE fixed ACS NTN UE to be varied/defined |
| NOTE 1: For coexistence between Ka-Band DL and adjacent TN bands, there are no 3GPP defined/specified TN bands. NOTE 2: To deprioritize scenario 7 and 8 for SAN elevation angle as 90-degree cases. Scenario 7 and 8 should still be studied for SAN elevation angle as 25-degree cases (computed from the center of the central beam). |  |  |  |  |  |

TN BS and UE ACLR and ACS values

| Frequency band | BS |  | UE |  | ACIR |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ACLR | ACS | ACLR | ACS | BS ACLR UE ACS | UE ACLR BS ACS |
| 17 GHz | [30] | [26] | [19] | [25] | [23.8] | [18.2] |
| 27 GHz | 28 | 24 | 17 | 23 | 21.8 | 16.2 |

- For calibration phase, assuming FRF=1 with single polarization

  - This assumption has no linkage/restriction on real network deployment

- For co-existence simulation, assume FRF=2 with two polarizations (RHCP, LHCP)

- For NTN SAN Type, consider GEO, LEO@600km & LEO@1200km.

- For NTN UE Type, following assumptions are used to be in line with the WID:

  - Fixed and mobile VSAT for GSO

  - Fixed VSAT for NGSO

- Use TN urban macro scenarios (and no TN dense urban) for the calibration and the coexistence analysis with NTN in above 10 GHz.

  - RAN4 should not consider the rural macro deployment scenario for TN.

- Use the same channel BW for TN and NTN.

  - TN 200MHz is the baseline of the study

  - TN 400MHz can be the second priority for the co-existence study.

  - For NTN SAN channel bandwidth, agree to use 200MHz per beam.

- For NTN UE antenna type

  - Focus on NTN UE with parabolic antenna in this release in co-ex study.

  - FFS whether UE with phased-array antenna would not be supported in this Release.

Network layout model

Co-existence between NTN and TN

Cellular cell structure is considered for both NTN and TN network layout.

Coordination System

Referring to TR 38.811 Section 6.3 and Annex A, a 3D global coordinate system is considered (Earth-Centred Earth Fixed) for simulating NTN beams direction and location on the earth surface. It means the NTN beam location, TN randomly dropping location are generated with a set of three parameters (x,y,z).

Simulation Methodology

Following simulation steps can be used for NTN-TN co-existence study.

- Generate aggressor and victim networks.

- FRF=2 with two polarizations (RHCP, LHCP)

  - Do not consider interference leakage from adjacent beams as starting point for co-ex study.

    - For example for the figure above, do not consider the interference leakage from green, blue and purple beams when study SINR for the red beam.

- Deployment of TN network (19 cells with wraparound) refers to Annex 2

- UE associations

- TN UE are generated randomly inside the TN network, make sure enough TN UEs are associated to each TN sectors based on coupling loss.

- Deployment of NTN UE refers to Annex 2.

Consider following approaches for the deployment of NTN UE and satellite - The Satellite should be generated in the visible area/sky of NTN UE; - NTN UEs point to the satellite accurately; - The position of the satellite should guarantee that NTN UE vertical angle towards the satellite; - For calibration, use 30 degree and 90 degree instead of the range. - SAN elevation angle value, 25 & 90 degree (computed from the centre of the beam), for co-existence simulation. - Horizontal angle of NTN UEs should be calculated based on the satellite position - Use 35m as minimum distance assumed between VSAT and TN BS for co-ex study.

- Once association is done, round robin scheduling is used. BF weights are adjusted to point to the LOS direction between BS-UE. This is done for both victim and aggressor networks.

- Throughput is computed in the victim systems without considering ACI as below:

- ThputNO ACIbpshz=fSINRICI=fSN+IICI, where IICI is the inter-cell interference.

For TN-NTN SINR calculation, the satellite receiver off angle should be considered in the satellite receiver gain calculation when calculating SINR. Note that such angle is not considered in TR 38.821 section 6.1.3 equations. Thus those equations should be used for SINR calculation.

- Throughput is computed considering ACI as below:

-	ThputACIbpshz=fSINRICI+ACI=fSN+IICI+IACI, where IACI is the adjacent channel interference.

- RF parameters are determined based on the degradation cause by ACI as below:

-	LossACI=1-ThputACIThputSINGLE.

To simplify the simulation of interference from TN to NTN UL in Case 2 and 6, following method can be used. Consider the active TN cells from central NTN beam for the ACI evaluation from TN to NTN UL. There is a view that simplifying such coexistence simulation work for Case 2 may even not be required.

- Step 1: to drop NTN UE per NTN beamprint randomly;

- Step 2: to drop N terrestrial clusters consisting of 57 sectors per NTN beamprint randomly

- Step 3: to calculate the total ACI per beam to NTN UL by following scaling factor:

where:

active_TN = active_factor*round (the area per beam/the area of 57 sectors)

active_factor = 20% (or lower, particularly for urban scenarios)

- Step 4: to calculate the total ACI from all beams (e.g. M=7) for NTN:

Simulation parameters

NTN parameters

Satellite parameters

Set-1 satellite parameters are shown in Table 2.3.1-2 according to TR 38.821.

The satellite max Tx power can be calculated by the equation as below:

maxTx powerdBm=EIRP densitydBWMHz+30+10 log10NRB*SCSMHz*12-Max GaindBi

Table 2.3.1-1 NRB configuration per BandWidth size and SCS

| Configuration FR2 Ka-band | NRB (200MHz BW) |
| --- | --- |
| SCS 120 kHz | 132 |

Table 2.3.1-2 Set-1 satellite parameters for co-existence study

| Satellite orbit |  |  | GEO |  | LEO-1200 |  | LEO-600 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Satellite altitude |  |  | 35786 km |  | 1200 km |  | 600 km |  |
| Payload characteristics for DL transmissions |  |  |  |  |  |  |  |  |
| Satellite EIRP density |  | Ka-band (i.e. 20 GHz for DL) | 40 dBW/MHz |  | 10 dBW/MHz |  | 4 dBW/MHz |  |
| Satellite max TX power in dBm | BW (MHz) |  | 200 | 400 | 200 | 400 | 200 | 400 |
|  | SCS kHz |  | 120 | 120 | 120 | 120 | 120 | 120 |
|  | NRB |  | 132 | 264 | 132 | 264 | 132 | 264 |
| Satellite Tx max Gain |  |  | 58.5 dBi |  | 38.5 dBi |  | 38.5 dBi |  |
| 3dB beamwidth or HPBW (Half-Power BandWidth) of main central beam |  |  | 0.1765 deg |  | 1.7647 deg |  | 1.7647 deg |  |
| ABS (Adjacent Beam Spacing) of adjacent beams from the central beam |  |  | Note 1 |  | Note 1 |  | Note 1 |  |
| Satellite beam diameter |  |  | 110 km |  | 40 km |  | 20 km |  |
| Payload characteristics for UL transmissions |  |  |  |  |  |  |  |  |
| Satellite Rx max Gain |  |  | 58.5 dBi |  | 38.5 dBi |  | 38.5 dBi |  |
| Note 1: ABS[rad] = sqrt(3) x sin(HPBW[degrees]/2) or ABS[rad] = sqrt(3) x sinr(HPBW[rad]/2) - with ABS [degree]=180/pi x ABS[rad] and - with HPBW the Half-Power BandWidth of the main lobe from the satellite antenna pattern. |  |  |  |  |  |  |  |  |

Table 2.3-2 Other parameters for NTN

| Parameters | NTN | Remark |
| --- | --- | --- |
| Carrier frequency | 27GHz(UL) / 17GHz(DL) |  |
| The number of active UE (UL) | 10 UEs and 13RBs per UE for GEO and LEO |  |
| The number of active UE (DL) | 1 | Same with TN |
| Traffic model | Full buffer |  |
| DL power control | NO |  |
| UL power control | See Session 2.7.2 |  |
| NTN satellite Noise figure in dB | See Table 2.3.1-3 |  |
| Handover margin | 3dB |  |

Table 2.3.1-3 NTN satellite Noise figure in dB

| Satellite | GEO | LEO 600 | LEO 1200 |
| --- | --- | --- | --- |
| G_Rx (dBi) | 58.5 | 38.5 | 38.5 |
| Noise figure for calibration is 5.9 for all satellite types. |  |  |  |

UE parameters

UE parameters are shown in Table 2.3.1-4

[Table 2.3.1-4 UE characteristics for system level simulations

| Characteristics | fixed VSAT | L-ESIM |
| --- | --- | --- |
| Frequency band | 27GHz | 27GHz |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter | Directional (M,N,P,Mg,Ng) = (TBD,TBD,2,1,1); (dV,dH) = (TBD, TBD)λ with directional antenna element (HPBW=65 deg) |
| Polarisation | Circular | circular |
| Rx Antenna gain | 39.7 dBi | TBD dBi per element |
| Antenna temperature | 150 K | TBD K |
| Tx transmit power | 2 W (33 dBm) | [TBD W (TBD dBm)] |
| Tx antenna gain | 43.2 dBi | TBD dBi per element |

]

- FFS whether and how to consider phased array antenna pattern, not preclude to consider phased array antenna pattern in the future if needed.

- For calibration, the number of active NTN UE (UL) is 10 UEs with divided RBs per UE by channel bandwidth.

- For calibration, NTN UE antenna elevation angle is 30 and 90 degree.

- For calibration, NTN UE NF is 4dB

| NTN UE scenario | Fixed VSAT (on roof) | M-ESIM | A-ESIM | L-ESIM |
| --- | --- | --- | --- | --- |
| Altitude | 22.5m | 22.5m | 3-14km | 1.5m |

Nosie Figure for NTN SAN and UE

Use the following NF sets in co-ex study based on agreement in #108 main and BSRF session.

|  | SAN NF | UE NF |
| --- | --- | --- |
| For GEO | 3.5 dB | 2.5 dB |
| For LEO (option 1) | 3.5 dB | 2.5 dB |
| For LEO (option 2) | 5.9 dB | 6 dB |

TN parameters

Table 2.3.2-1 Simulation assumptions of TN NR

| Carrier frequency in GHz | 27 |
| --- | --- |
| Size of each nominal channel BW in MHz | 200 |
| Transmission bandwidth in MHz | 190.8 |
| Environment | Urban macro |
| Network layout | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |
| Inter-site distance in meter | 300m |
| Radius of UE dropping within a micro cell | [TBD] |
| Number of micro BSs per macro cell | [TBD] |
| UE number per cell | Same as the number of BS beam |
| System loading and activity | 20% |
| Network location | See Annex 2 |
| DL subcarrier spacing | 120kHz |
| UL | OFDMA |
| DL power control | No |
| UL power control | See Section 2.7.1 |
| Frequency reuse | 1 |
| Number of scheduled UE per cell (DL) | Same as the number of BS beam |
| Number of scheduled UE per cell (UL) | Same as the number of BS beam |
| BS height | 25 |
| UE antenna height in meter | 1.5 |
| UE TX power in dBm | -40 to 23 |
| Building penetration loss | In path loss model, TR 38.901 |
| Cell selection margin in dB | 3 |
| BS-MS min distance in meters | 35 |
| BS noise figure in dB | 10 |
| UE noise figure in dB | 10 |
| BS-UE path-loss model | TR 38.901 |
| Standard deviation of BS-UE log-normal shadow fading in dB | Deployment scenario related, referring to TR 38.901. |
| Shadowing correlation | Between cells: 1.0 Between sites: 0.5 |
| Link-level performance model | See Section 2.10 |
| UE distribution | Uniform |
| Evaluation metrics | See Section 2.9 |

Table 2.3.2-3 ACLR/ACS for TN (30GHz)

|  |  | NR |
| --- | --- | --- |
| BS | ACLR | 28dB |
|  | ACS | 24dB |
| UE | ACLR | 17dB |
|  | ACS | 22dB |

Antenna and beam forming pattern modelling

Satellite and UE Antenna and beam forming pattern modelling

Satellite and UE Antenna and beam forming pattern modelling of satellite could be referred to section 6.4.1 in TR 38.811.

Satellite antenna pattern

Parabolic antenna pattern for SAN and UE:

Reusing Antenna pattern in section 6.4.1 of TR38.811 for NTN SAN and NTN UE with parabolic antenna. FFS what pattern /modelling can be applied to NTN UE with phased array antenna.

The following normalized antenna gain pattern, corresponding to a typical reflector antenna with a circular aperture, is considered.

1                for θ=0

4J1kasinθkasinθ2        for 0<θ≤90°

where:

-	J1(x) is the Bessel function of the first kind and first order with argument;

-	x, a is the radius of the antenna's circular aperture;

-	k = 2f/c is the wave number;

-	f is the frequency of operation;

-	c is the speed of light in a vacuum and  is the angle measured from the bore sight of the antenna's main beam.

Note that ka equals to the number of wavelengths on the circumference of the aperture and is independent of the operating frequency.

Satellite and UE beam forming pattern

The following table is agreed for the beam layout definition for a single satellite simulation in S-Band.

Table 2.4.1-1: Beam layout definition for single satellite simulation

| Beam layout definition | Baseline: Hexagonal mapping of the beam bore sight directions on UV plane defined in the satellite reference frame. Only the 3dB beam width parameters should be used. The beam diameter and beam spacing values can be computed directly from the 3 dB beam width assumptions and should be considered as informative. |
| --- | --- |
| Number of beams | Baseline: 7-beam layout (i.e. 6 co-frequency beams surrounding the central beam) |
| UV plane illustration (extracted from [19]) |  |
| UV plane convention | U axis is defined as the perpendicular line to the satellite-earth line on the orbital plane as illustrated here after: The straight line being orthogonal to UV plane is pointing towards the Earth centre. UV coordinates of the nadir of the reference satellite is (0,0) |
| Adjacent beam spacing on UV plane | Baseline: Adjacent beam spacing computation based on 3dB beam width of the satellite antenna pattern: ABS[rad] = sqrt(3) x sin(HPBW[degrees]/2) or ABS[rad] = sqrt(3) x sinr(HPBW[rad]/2) with ABS [degree]=180/pi x ABS[rad] and with HPBW the Half-Power BandWidth of the main lobe from the satellite antenna pattern. |
| Central beam bore sight direction definition | Baseline: [Case 1: Central beam center is considered at nadir point [Case 2: TBD° for GEO and LEO] Interested companies can bring analysis and results for other values. |
| Frequency-reuse factor | FRF=2 |
| Polarization re-use | Enable Note: Polarization re-use should apply only if circular polarization for terminal antenna is considered |
| UEs outdoor/indoor distribution | 100% outdoor distribution for UEs |
| UE distribution | The cell area associated to a given beam is defined as the Voronoi cell associated with the corresponding beam centers. |
| UE configuration | Fixed and mobile VSAT for GSO Fixed VSAT for NGSO |
| UE orientation | Ideal Tracking serving beam |
| UE attachment | RSRP |
| NOTE 1: Typical impairment values (additional frequency error, SNR loss) due to the feeder link except for delay can be considered to be negligible. When available, specific values can be considered in the evaluation and should be reported. NOTE 2: For the calibration purpose, the ionospheric scintillation loss shall be considered equal to zero (i.e., the UEs are located between 20 and 60 degrees of latitude). |  |

TN BS and UE antenna and beam forming pattern modelling

BS antenna

|  | Macro urban |
| --- | --- |
| Antenna pattern | TR 38.803 |
| Element gain GE,max (dBi) | 5.5 |
| Horizontal 3dB /vertical 3dB 3 dB beam width of single element (degree) | 90º for H 90º for V |
| Horizontal/vertical front-to-back ratio Am (dB) | 30 for both H/V |
| Side lobe suppression SLAv (dB) | 30 |
| Antenna polarization | Linear ±45º |
| Antenna array configuration (Row × Column) | 16 × 8 elements |
| Horizontal/Vertical radiating element spacing | dh = 0.5 dv = 0.5 |
| Array Ohmic loss LE (dB) | 2 |
| Conducted power (before Ohmic loss) per antenna element (dBm) | 22 |
| Mechanical downtilt (degrees) | 10 |

Table 2.4.2-1 BS antenna modelling for Urban macro scenario

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) | AE,Vθ"=-min12θ"-90°θ3dB2, SLAV, θ3dB=90°， SLAV=30dB |
| Antenna element horizontal radiation pattern (dB) | AE,Hφ"=-min12φ"φ3dB2, Am, φ3dB=90°， Am=30dB |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 5.5 dBi |
| (Mg, Ng, M, N, P) note | For 30GHz: (1, 1, 8, 16, 2) |
| (dv, dh) | (0.5λ, 0.5λ) |
| Note: An additional 3dB gain is added to the total beamforming gain to account for the two polarization directions. Boresight direction is horizontal. |  |

UE antenna

Refer to TR 38.803.

Table 2.4.2-2 Antenna parameters for 30GHz

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 5 dBi |
| (Mg, Ng, M, N, P) | (1, 1, 2, 2, 2) |
| (dv, dh) | (0.5λ, 0.5λ) |
| UE orientation | Random orientation in the azimuth domain: uniformly distributed between -90 and 90 degrees* Fixed elevation: 90 degrees |
| NOTE: This is done to emulate two panels: the configuration is equivalent to 2 panels with 180 shift in horizontal orientation and UE orientation uniformly distributed in the azimuth domain between -180 and 180 degrees. |  |

ACIR model

- To use 1 UE for both TN and NTN in the DL

- To use 10 UEs for NTN and 1 UE for TN in the UL

- Use the frequency flat ACLR/ACS model for both NTN and TN as starting point.

  - FFS the step-model ACLR/ACS

Propagation model

Following table is to be used for calibration.

| Link | Propagation model |
| --- | --- |
| TN BS to Fixed VSAT on roof | Free space path loss |
| TN BS to L-ESIM at 1.5 m | UMa as in 3GPP TR 38.803 |
| TN BS to TN UE | UMa as in 3GPP TR 38.803 |
| TN UE to Fixed VSAT on roof | UMa as in 3GPP TR 38.803 (BS is to be replaced with VSAT) |
| TN UE to L-ESIM | Umi |
| Satellite to TN BS/UE | 3GPP TR 38.821 |
| Satellite to VSAT/ESIM | 3GPP TR 38.821 |
| TN BS to Satellite | 3GPP TR 38.821 |
| Note1: For the propagation models which use the 3GPP TR 38.821 [3], to use same assumptions as in [3] and consider the atmospheric losses and the scintillation losses. Note2: TN BS height is 25m |  |

Models for co-existence simulation need further discussion.

Transmission power control model

UL TPC

Use following UL power control model for both NTN and TN as starting point.

Where,

Pmax = maximum UE Tx power, i.e. 23 dBm for TN UE, and 33 dBm for fixed VSAT NTN UE.

Rmin = minimum power reduction ratio, i.e. -63 dB for TN UE, and [-63 dB by assuming NTN UE min Tx power as -30dBm as starting point] for NTN UE.

γ = 1

CLx-ile = –SNR_target + Pmax – ThermalNoise – BS/SAN_NoiseFigure - 10*log10(BW), considering SNRtarget is 15dB and BW is actual UL BW.

DL TPC

For downlink scenario, no power control scheme is applied.

Received power model

The received power in downlink and uplink scenarios is defined as below:

RX_PWR = TX_PWR – Path loss + G_TX + G_RX

Where,

RX_PWR is the received power

TX_PWR is the transmitted power

G_TX is the transmitter antenna gain (directional array gain)

G_RX is the receiver antenna gain (directional array gain).

Performance metric

For NR,

The average throughput loss and 5%-ile throughput loss should be less than 5%.

For NTN,

The average throughput loss and 5%-ile throughput loss should be less than 5%.

Throughput ~ SNR mapping

Adopt Section 5.2.7 of TR 38.803[20] as the SINR-Throughput performance metrics for both NTN and TN.

  - FFS the applicability of the mapping for NTN link

- Conclusion

It is proposed to use the simulation assumptions in this paper for NTN co-existence study in bands above 10GHz.

- Reference

[1] R4-2305981 Summary_311_2nd round

[2] R4-2305928 WF on [311] NR_NTN_enh_Part3

Appendix 1. Interference Chart

Coexistence scenarios for NTN-TN scenarios are shown in Figure A.1.

Figure 1. Coexistence scenarios for use cases 1-4 (in above 10 GHz)

Figure 2. Coexistence scenarios for use cases 5-8 (in above 10 GHz)

Appendix 2. Consideration of Network and UE depolyment

| No. | Combination | Aggressor | Victim | Which NTN cell/UE to observe? | Which TN/UE to observe? | Which TN cells in a TN to observe? |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | NTN UL | TN UL | NTN cell: Nadir point. NTN UE: NTN UEs randomly dropped in TN clusters. | TN randomly placed in this NTN beam | Only the active TN clusters which contain NTN UE(s) . |
| 2 | TN with NTN | TN UL | NTN UL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs randomly dropped in TN clusters | Consider an active rate of 20% for TN. | Only the active TN cells in central NTN beam |
| 3 | TN with NTN | NTN UL | TN DL | NTN cell: Nadir point NTN UE: NTN UEs randomly dropped in TN clusters. | TN clusters randomly placed in this NTN beam | All active TN clusters which contain NTN UE(s) |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (to be further discussed for GEO and LEO，interested companies can bring analysis and results for other values). NTN UE: NTN UEs randomly dropped in TN clusters. | TN clusters randomly placed in this NTN beam | Only the active TN clusters which contain NTN UE(s). |
| 4 | TN with NTN | TN DL | NTN UL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE NTN UEs randomly dropped in TN clusters | Consider the active rate of 20% for TN. | All active TN cells in central NTN beam |
| 5 | TN with NTN | TN DL | NTN DL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs randomly dropped in TN clusters. | One cluster with 19 TN cells (57 sectors) randomly placed in the central NTN beam | Only the active TN clusters which contain the NTN UE(s). |
| 6 | TN with NTN | NTN DL | TN DL | NTN cell: Nadir point. NTN UE: NTN UEs randomly dropped in TN clusters. | TN clusters randomly placed in this NTN beam | All in central NTN beam |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (to be further discussed for GEO and LEO，interested companies can bring analysis and results for other values) NTN UE: NTN UEs randomly dropped in TN clusters | TN clusters randomly placed in this NTN beam |  |
| 7 | TN with NTN | NTN DL | TN UL | NTN cell: Nadir point NTN UE: NTN UEs randomly dropped in TN clusters. | TN clusters randomly placed in this NTN beam | Only the active TN clusters which contain NTN UE(s) |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (to be further discussed for GEO and LEO，interested companies can bring analysis and results for other values). NTN UE: NTN UEs randomly dropped in TN clusters. | TN clusters randomly placed in this NTN beam | Only the active TN clusters which contain NTN UE(s). |
| 8 | TN with NTN | TN UL | NTN DL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs randomly dropped in TN clusters | Consider the active rate of 20% for TN. | All active TN cells in central NTN beam |