3GPP TSG-RAN WG4 Meeting #108																	 R4-2313901

Toulouse, France, August 21 – August 25, 2023

Agenda item:			8.33.5

Source:	Moderator (Qualcomm Incorporated)

Title:	WF for mobile IAB RF requirements and co-existence study

Document for:	Approval

# Introduction

This document provides way-forward on mobile IAB (mIAB) based on the outcomes of “Topic summary for [108][314] NR_mobile_IAB_RF [1].

# Simulation parameters

O2I penetration loss

- O2I car penetration loss -model in 38.901 can be used for mIAB co-existence studies.

IAB-MT/DU antenna modelling

- Reuse IAB-MT antenna modelling for mIAB as starting point and suggested to further study and specify the antenna modelling for mobile IAB-DU.

# Coexistence results

Simulation results in R4-2311558 (Nokia), R4-2313218 (Qualcomm) and R4-2313473 (Ericsson) have been submitted in RAN4#108 with the following observations:

- The Uplink SINR degradation due to the mIAB interference can be marginal.

- mIAB-MT transmission power of 33 dBm (TRP) is fine to protect the legacy NR BS.

- mIAB-MT maximum transmission power EIRP needs to comply the regulatory requirement.

- Legacy requirements can be reused for mobile IAB.

The following proposals have been tentatively agreed (to be confirmed in RAN4#108-bis):

IAB-MT RF requirements

- Reuse the legacy ACLR and ACS for mobile IAB-MT for both FR1 and FR2

IAB-MT dynamic range

- UE type of Tx dynamic range is needed for mobile IAB-MT.

Mobile IAB requirements

- Use the local area IAB-MT to specify the mobile IAB feature.

# Timeline

Agree on the following timeline for mobile IAB study in RAN4.

| date | RAN4 meeting | RAN4 target |
| --- | --- | --- |
| 2023-08 | RAN4#108 | - First set of simulation results with tentative agreements. - Stable set of assumptions for coexistence simulations. |
| 2023-10 | RAN4#108 bis | - Conclude co-existence related RF requirements. - Discussion on how to include mobile IAB requirements in TS 38.174. |
| Note 1: Simulation parameters can be found in R4-2309863 (RAN4#107) and R4-2305935 (RAN4#106-bis-e). The agreed list of coexistence scenarios can be found in R4-2305935. |  |  |

# References

- R4-2314250, “Topic summary for [108][314] NR_mobile_IAB_RF” Qualcomm Incorporated.