3GPP TSG-RAN WG4 Meeting # 108                                                   R4-2313904

# Toulouse, France, Aug 21 – Aug 25, 2023

Agenda item:			8.28.7

Source:	Moderator (ZTE)

Title:	Ad-hoc minutes for NCR RF requirement

Document for:	Information

- 1Topic #2: RF requirements for NCR-Fwd

  - 1.1 Companies’ contributions summary

(Cat A CRs are not listed)

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2311562 | Nokia, Nokia Shanghai Bell | Discussion of Spurious Emissions requirements for NCR-Fwd |
| R4-2311635 | CATT | Further discussion on RF requirements for NCR-Fwd |
| R4-2313006 | Ericsson | NCR TX RF requirements |
| R4-2313179 | ZTE Corporation | Discussion on RF requirements for NCR-Fwd |

    - 1.1.1. Sub-topic 1-1  RF requirement for NCR-Fwd type 1-H

| For NCR-Fwd type 1-H: |  |
| --- | --- |
| RF requirements |  |
| Transmitter spurious emissions | Agreement: For the simultaneous transmission of NCR-Fwd and NCR-MT part in FR1, use repeater uplink transmitter spurious emission requirement FFS for for the simultaneous transmission of NCR-Fwd and NCR-MT part in FR2 |
| Receiver spurious emissions | Ericsson: There are no direction regulatory reference for the spurious emission requirement. No agreement: FFS for scaling factors for receiver spurious emission of NCR-Fwd type 1-H. |
| Transmit ON/OFF power and transition period | Agreement: For shared antenna connector for NCR-MT and NCR-Fwd backhual link, then transition period and transmit ON-OFF power could be defined together and tested together. For transition period and OFF power requirement for NCR-Fwd type 1-H, propose to follow Rel-17 repeater requirement. |

    - 1.1.2 Sub-topic 1-2  RF requirement for NCR-Fwd Type 1-O

| For NCR-Fwd type 1-O |  |
| --- | --- |
| RF requirements |  |
| Repeater output power | Agreement: repeater radiated output power in the UL part: - For wide area NCR-fwd link, X=9dB; - For local area NCR-fwd link, X=6dB; |
| Adjacent Channel Leakage Power Ratio | Agreement: for the absolute ACLR limit for NCR type 1-O, the scaling factor should be - For wide area NCR-fwd link, X=[9]dB; - For local area NCR-fwd link, X=[6]dB; |
| Operating band unwanted emissions | Agreement: for OBUE requirement of uplink transmission of NCR-Fwd type 1-O, propose to have the the same scaling factor for NCR-Fwd uplink transmission output power: - For wide area NCR-fwd link, X=9dB; - For local area NCR-fwd link, X=6dB; |
| Transmitter spurious emissions | Agreement: for transmitter spurious emission requirements for NCR-Fwd type 1-O, propose to have the the same scaling factor for NCR-Fwd uplink transmission output power: for DL part: - To follow the option 2 with 9dB scaling factor; For UL part: - For wide area NCR-fwd link, X=9dB; - For local area NCR-fwd link, X=6dB; |
| Receiver spurious emissions | Agreement: FFS |
| Error Vector Magnitude | Agreement for EVM: G_Rx_Ant: based on the sub-array: 3dB contour of sub-array is left up to the declaration and other approaches are not precluded. |
| Input intermodulation | Agreement for Input intermodulation: Agreement: G_Rx_Ant: based on the whole antenna array: - 3dB contour of whole antenna array is left up to the declaration - EIRP and TRP approach - other approaches are not precluded. Agreement: Use the co-location reference antenna for other Co-location with BS/repeater in other systems and Co-existence with other systems requirement. |
| Output intermodulation | Agreement: Use the co-location reference antenna for the output intermodulation requirement for NCR Fwd type is type 1-O |
| Adjacent Channel Rejection Ratio (ACRR) |  |
| Transmit ON/OFF power and transition period | Agreement: For transition period for NCR-Fwd type 1-O, propose to follow Rel-17 repeater requirement and OFF-power requirement could be based on co-location reference antenna with its OFF power as -106dBm/MHz. Requirement could be applicable for both individual NCR-MT and NCR-Fwd testing and joint testing for NCR-MT and NCR-Fwd when they are switched ON-OFF together. |

    - 1.1.3 Sub-topic 1-3  Dynamic beamforming for NCR

- Agreement:

- spec updates for NCR dynamic beamforming as following

| Beam related requirements | Requirements for repeater which may need to be updated for NCR | NCR |
| --- | --- | --- |
| A set of declaration for each beam | For each beam, the requirement is based on declaration of a beam identity, reference beam direction pair, beamwidth, rated beam EIRP, OTA peak directions set, the beam direction pairs at the maximum steering directions and their associated rated beam EIRP and beamwidth(s). | Sufficient for NCR beam related declaration |
| Minimum requirement of output power | The AoA of input signal shall be the same as the reference direction for the OTA peak directions set when operating in the opposite DL/UL direction. | For NCR, input signal arrives from any direction within the OTA REFSENS RoAoA |
| EVM | OTA modulation quality requirement is defined as a directional requirement at the RIB and shall be met within the OTA coverage range on the transmit side and the AoA of the incident wave of the received signal is in the reference direction at the receive side. | For NCR, input signal arrives from any direction within the OTA REFSENS RoAoA |
| Input intermodulation | The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction: | From the receiver of NCR perspective, it should be okay to have same direction for received signals within pass-band and interfering signal outside pass-band. |
| ACRR | The requirement shall apply at the RIB when the AoA of the incident wave of a received signal in the passband and a received signal on an adjacent channel outside repeater passband is from the same direction and are the same as the TX reference direction for the opposite DL/UL setting. | The last sentence ”the same as the TX reference direction for the opposite DL/UL setting” is not applicable for NCR |