TSG-RAN Working Group 4 (Radio) meeting #108	R4-2313911

Toulouse, EU, 21st to 25th August 2023

Source:	Ericsson

Title:	TP to TR 38.858: Addition of coexistence simulation assumptions to Annex E

Agenda item:	8.19.2.1

Document for:	Approval

# Introduction

At the last RAN4 meeting (RAN4#107 in Incheon) simulation assumptions related to the adjacent carrier coexistence evaluation part of SBFD SI (FS_NR_duplex_evo) was discussed extensively. The outcome from the discussion was collected in a way-forward contribution in [5].

At previous meetings detailed information relevant for the simulation assumptions have been collected in numerous contributions [1, 2, 3, 4].

As technical background for the SBFD adjacent carrier coexistence evaluation is essential for the correct conclusion to be made an Annex in TR 38.858 [7] have been created to collect simulation assumptions including relevant models and parameters. To further stimulate discussion on how to include simulation assumptions into the technical report a text proposal was drafted to previous meeting in [6] and feedback received from all participating companies have been included in this version.

In this contribution a text proposal to TR 38.858, Annex E has been created based on input from previous agreements. The text proposal is attached at the end of this contribution, and it is presented for approval. Feedback from all participating companies is appreciated.

This is a revised version of R4-2312377.

# Discussion

The SBFD coexistence evaluation relies on modelling two network deployed in the same geographical area operating at adjacent carriers. The victim impact of DL and UL performance is studied for different deployment scenarios defined by RAN4. The currently defined deployment scenarios to consider ranges from FR1 to FR2-1 including macro, micro and indoor scenarios as listed in Table 2-1.

Table 2-1: Network deployment scenarios

| Scenario | FR | Aggressor | Victim | Priority |
| --- | --- | --- | --- | --- |
| 1 | FR1 | Urban Macro | Urban Macro | High |
| 2 | FR1 | Urban Hotspot | Urban Hotspot | Low |
| 3 | FR1 | Indoor | Indoor | Low |
| 4 | FR1 | Urban Macro | Micro | High |
| 5 | FR1 | Micro | Micro | Low |
| 6 | FR2-1 | Urban Macro | Urban Macro | High |
| 7 | FR2-1 | Urban Hotspot | Urban Hotspot | Low |
| 8 | FR2-1 | Urban Micro | Urban Micro | Low |
| 9 | FR2-1 | Indoor | Indoor | Low |

The impact is studied both for SBFD and TDD as victim system using different configuration cases. For each case the victim is either DL or UL for TDD and SBFD. Which gives four different cases to study. All cases are listed in Table 2-2.

Table 2-2: Coexistence cases

| Case | Aggressor | Victim | Slot allocation Aggressor Victim | Priority |
| --- | --- | --- | --- | --- |
| 1 | SBFD | TDD DL |  | High |
| 2 | SBFD | TDD UL |  | Low |
| 3 | TDD DL | SBFD |  | High |
| 4 | TDD UL | SBFD |  | Low |

The coexistence evaluation is based on simulation results produced by an extensive coexistence model build around numerous sub-models (e.g., deployment, antenna, propagation, blocking, BS, UE, etc.) requiring an extensive set of parameter value assumptions. For simplicity the parameter has been divided into following categories:

- For each deployment scenario and FR, parameter assumptions related to the network deployment.

- For each deployment scenario and FR, parameter assumptions related to the carrier configuration.

- For each BS class (Wide area, Medium Range, Local Area) and FR, parameter values related to BS RF characteristics.

- For each FR, parameters values related to UE RF characteristics.

In addition to parameter assumptions and models used for previous coexistence evaluations in RAN4, new aspects unique for SBFD needs to be modelled. For this purpose, new models are defined for power scaling, receiver blocking, UE ACLR versus UE power, etc. The details related to these new modes needs to be captured in TR 38.858. In this contribution a text proposal to TR 38.858, Annex E is provided for discussion with the intention to be approved at the end of the meeting. Since the existence evaluation have started and simulation results have been presented by several companies it is essential to capture the technical background information in TR 38.858.

The technical information to this text proposal have been collected from agreed documents from previous RAN4 meeting as listed below:

- Way-forward contribution from RAN4#106-bis-E meeting in [1].

- Parameter overview from RAN4#106-bis-E meeting in [2].

- Way-forward contribution from RAN4#106-bis-E meeting on Macro to Micro simulation parameters in [3].

- LS to RAN1 [4] agreed at RAN4#106-bis-E meeting.

- In BS RF chairman notes it’s also agreed to use the blocking model and parameters from LS in [4] for coexistence evaluation.

- Description of how to implement BS receiver blocking model in coexistence simulator from way-forward contribution [5] from RAN4#107.

# Conclusion

In contribution we have collected all essential technical background relevant for the simulation assumptions used for the SBFD adjacent carrier coexistence evaluation. The source for the information is tied to references to agreements at previous RAN4 meetings.

A text proposal with simulation assumptions to TR 38.858, Annex E have been created. The text proposal includes deployment scenarios, SBFD coexistence evaluation cases, BS and UE RF characteristics, essential models and general description of how the simulation is conducted.

The text proposal is attached at the end of this contribution, and it is presented for approval.

# References

[1]	R4-2305921, “WF for SBFD adjacent channel co-existence study”, CMCC

[2] 	R4-2305923, “Summary of co-existence simulation parameters”, CMCC, CableLabs, Samsung

[3]	R4-2305922, “WF on Uma- Umi scenario simulation parameters”, CableLabs

[4]	R4-2305920, “Reply LS on interference modelling for duplex evolution”, Samsung

[5]	R4-2309793, “WF for SBFD co-existence study”, CMCC

[6]	R4-2307701, “TP to TR 38.858: Addition of coexistence simulation assumptions to Annex D”, Ericsson

[7]	TR 38.858, “Study on Evolution of NR Duplex Operation”, V0.4.1, 3GPP

TEXT PROPOSAL:

Annex <E>:
Adjacent channel co-existence evaluation

# E.1	RAN4 co-existence simulation scenarios

The coexistence evaluation is conducted considering the different scenarios listed in Table E.1-1.

Table E.1-1: Network deployment scenarios

| Scenario | FR | Aggressor | Victim | Priority |
| --- | --- | --- | --- | --- |
| 1 | FR1 | Urban Macro | Urban Macro | High |
| 2 | FR1 | Urban Hotspot | Urban Hotspot | Low |
| 3 | FR1 | Indoor | Indoor | Low |
| 4 | FR1 | Urban Macro | Micro | High |
| 5 | FR1 | Micro | Micro | Low |
| 6 | FR2-1 | Urban Macro | Urban Macro | High |
| [7 | FR2-1 | Urban Hotspot | Urban Hotspot | Low] |
| 8 | FR2-1 | Urban Dense | Urban Dense | Low |
| 9 | FR2-1 | Indoor | Indoor | Low |

The Urban Hotspot reuses most parameter assumption as Urban Macro, except that Urban Macro uses random dropping method for UE while Urban Hotspot uses cluster-based dropping method for UE as described in Table E.2.1-1 and Table E.2.1-2 and Table E.2.1-3.

The coexistence evaluation captures cases where TDD and SBFD are both victim and aggressor networks. This to evaluate impact on legacy TDD networks if SBFD is introduced in a neighbouring channel, also to understand impact on SBFD network due to the legacy TDD network, as described in Table E.1-2. It is worth noting that RAN4 has only considered the case of {D, U} as an SBFD configuration as it is comparable in terms of performance (based on RAN4 models and parameters) to the {D, U, D} SBFD configuration.

Table E.1-2: Coexistence cases

| Case | Aggressor | Victim | Slot allocation Aggressor Victim | Priority |
| --- | --- | --- | --- | --- |
| 1 | SBFD | TDD DL |  | High |
| 2 | SBFD | TDD UL |  | Low |
| 3 | TDD DL | SBFD |  | High |
| 4 | TDD UL | SBFD |  | Low |

# E.2	RAN4 co-existence simulation assumptions

## E.2.1	Deployment

Simulation assumptions related to network layout is captured for FR1 in Table E.2.1-1 and for FR2-1 in Table E.2.1-2.

Simulation assumptions related to cluster-based UE dropping are captured in Table E.2.1-3.

Table E.2.1-1: FR1 deployment parameters

| Parameter | Urban Macro (Note 1) | Micro | Indoor |
| --- | --- | --- | --- |
| Layout | Single layer with 19 hexagonal cells with wrap around. | Single layer with 19 hexagonal cells with wrap around. | Total 12 BSs (Operator A: 6 BSs & Operator B: 6 BSs) 120 x 50 m |
| Inter-BS distance | 500 m | 289 m | 20 m |
| Grid shift | Baseline: 100% 2nd priority: Other values, e.g., 50%, 10% | Baseline: 100% The victim network (UMi) in the center, the aggressor network moved by the grid offset. 100% is relative to the micro ISD. 2nd priority: Other values | N/A |
| Carrier frequency | 4 GHz | 4 GHz | 4 GHz |
| Path-loss model | BS-to-UE: UMa see TR 38.803 BS-to-BS: 1st priority: Reuse the same model as in TR 38.828 with h_UT equals to 25m. 2nd priority: If the 2D distance between two BS are less than or equal to the ISD, set the LOS probability to X; Otherwise, reuse BS-to-UE LOS probability equation in TR 38.828. X = 0.75. For other cases, reuse BS-to-UE LOS probability equation in TR 38.803. UE-to-UE: UMi (h_BS=1.5m) + penetration loss, see TR 38.803. UMi model is not applicable when 2D distance is less than 10 m, instead FSPL is applicable. | BS-to-UE: UMi see TR 38.803 BS-to-BS: for the UMa-to-UMi scenario: UMa (h_UE = 10 m) see TR 38.803. For LoS probability for Macro-to-Micro case: Reuse the same model as in TR 38.828 with h_UT = 10 m for the UMi-to-UMi scenario: UMa (h_UE = 10 m) see TR 38.803. UE-to-UE: Outdoor UE – Outdoor UE see TR 36.828 + penetration loss see TR 38.803. UMi model is not applicable when 2D distance is less than 10m, instead free space model is applicable | BS-to-BS: InH-office BS-to-UE: InH-office UE-to-UE: InH-office As described in TR 38.803, subclause 5.2.2.1 |
| Site-interference level | Baseline: 6 dB below noise floor (corresponding to 1 dB desens) Optional: A reported value | Baseline: 6 dB below noise floor (corresponding to 1 dB desens) Optional: A reported value | N/A |
| BS height | 25 m | 10 m | 3 m |
| UE height | 1.5 m | 1.5 m | 1.5 m |
| UE distribution | Random dropping (Note 1) Baseline: 20% indoor and 80% outdoor Optional: 80% indoor and 20% outdoor | Uniformly UE distribution in the cell 20% indoor and 80% outdoor | 100% indoor |
| O-to-I penetration loss model | 80% low-loss model and 20% high-loss model | 80% low-loss model and 20% high-loss model | N/A |
| Minimum BS-UE (2D) distance | 35 m | 5 m | 0 m |
| Minimum UE-UE (2D) distance | 3 m | 3 m | 1 m |
| DL power control | No | No | No |
| UL power control | Yes | Yes | Yes |
| Handover margin | 3 dB | 3 dB | 3 dB |
| BS mechanical down-tilt angle | 6 degrees | 6 degrees | 90 degrees |
| Note 1: The Urban Hotspot reuses the assumptions of Urban Macro, except Urban Hotspot assumes Cluster-based drop. See Table E.2.1-3 for Cluster-based dropping. |  |  |  |

Table E.2.1-2: FR2-1 deployment parameters

| Parameter | Urban Macro (Note 1) | Urban Dense | Indoor |
| --- | --- | --- | --- |
| Layout | Single layer with 19 hexagonal cells with wrap around. | Fixed cluster circle within a macro cell. Number of micro BSs per macro cell is 3. Radius of UE dropping within a micro cell is < 28.9 m. Minimum distance between micro BSs in different operator is 10 m. Shadowing correlation is 0.5. | Total 12 BSs (Operator A: 6 BSs & Operator B: 6 BSs) 120 x 50 m |
| Inter-BS distance | 200 m | 10 m Minimum distance between micro BSs in different operator | 20 m |
| Grid shift | Baseline: 100% 2nd priority: Other values, e.g., 50%, 10% | N/A | N/A |
| Carrier frequency | 30 GHz | 30 GHz | 30 GHz |
| Path-loss model | BS-to-UE: UMa + penetration loss see TR 38.803 BS-to-BS: 1st priority: Reuse the same model as in TR 38.828 with h_UT equals to 25m. 2nd priority: If the 2D distance between two BS are less than or equal to the ISD, set the LOS probability to X; Otherwise, reuse BS-to-UE LOS probability equation in TR 38.828. X = 0.75. For other cases, reuse BS-to-UE LOS probability equation in TR 38.803. UE-to-UE: UMi (h_BS=1.5m) + penetration loss, see TR 38.803. UMi model is not applicable when 2D distance is less than 10 m, instead FSPL is applicable. | BS-to-UE: UMi + penetration loss BS-to-BS: UMi UE-to-UE: UMi (h_BS=1.5m~22.5m) + penetration loss, see TR 38.803. UMi model is not applicable when 2D distance is less than 10 m, instead FSPL is applicable. | BS-to-BS: InH-office BS-to-UE: InH-office UE-to-UE: InH-office As described in TR 38.803, subclause 5.2.2.1 |
| Site-interference level | Baseline: 6 dB below noise floor (corresponding to 1 dB desens) Optional: A reported value | Baseline: 6 dB below noise floor (corresponding to 1 dB desens) Optional: A reported value | N/A |
| BS height | 25 m | 10 m | 3 m |
| UE height | 1.5 m | 1.5 m ≦ hUT ≦ 22.5 m | 1.5 m |
| UE distribution | Indoor UE ratio: 0% Uniform UE distribution | Indoor UE ratio: 80 % Uniform UE distribution | 100% indoor |
| O-to-I penetration loss model | N/A | 50% low-loss model and 50% high-loss model | N/A |
| Minimum BS-UE (2D) distance | 35 m | 3 m | 0 m |
| Minimum UE-UE (2D) distance | 3 m | 3 m | 1 m |
| DL power control | No | No | No |
| UL power control | Yes | Yes | Yes |
| Handover margin | 3 dB | 3 dB | 3 dB |
| BS mechanical down-tilt angle | 6 degrees | 10 degrees | 90 degrees |
| Note 1: The Urban Hotspot reuses the assumptions of Urban Macro, except Urban Hotspot assumes Cluster-based drop. See Table E.2.1-3 for Cluster-based dropping. |  |  |  |

[Table E.2.1-3: Cluster-based UE dropping parameters

| Parameter | Urban Hotspot |
| --- | --- |
| Cluster number per macro cell | 1 |
| Cluster area | Circular area with 25 m radius |
| UE distribution | UEs dropped within a cluster are indoor and UEs dropped outside a cluster are outdoor |
| Indoor and outdoor ratio | 80% indoor and 20% outdoor |
| Minimum (2D) distance | UE-UE: 1 m Cluster center – cluster center: 50 m Macro TRP – cluster center: 60 m (Note 1) |
| UE dropping | Evenly random dropping into the cluster with minimum UE-UE distance limitation |
| Note 1: Consider the hexagonal grid of one of the two operators as the reference when dropping the cluster. The minimum distance between macro TRP to cluster centre should be respected also for TRPs belonging to the other operator |  |

]

## E.2.2	Traffic

Assumptions related to network traffic load configuration for FR1 is captured in Table E.2.2-1 and for FR2-1 in Table E.2.2-2.

Table E.2.2-1: FR1 network traffic parameters

| Parameter | Urban Macro | Micro | Indoor |
| --- | --- | --- | --- |
| Carrier bandwidth | 100 MHz | 100 MHz | 100 MHz |
| Scheduled channel bandwidth per UE (DL) | For legacy TDD: 100 MHz For SBFD {DU}: 80 MHz | For legacy TDD: 100 MHz For SBFD {DU}: 80 MHz | For legacy TDD: 100 MHz For SBFD {DU}: 80 MHz |
| Scheduled channel bandwidth per UE (UL) | For legacy TDD: 100 MHz For SBFD {DU}: 20 MHz | For legacy TDD: 100 MHz For SBFD {DU}: 20 MHz | For legacy TDD: 100 MHz For SBFD {DU}: 20 MHz |
| SBFD BS PSD | The PSD of SBFD is the same as TDD | The PSD of SBFD is the same as TDD | The PSD of SBFD is the same as TDD |
| Traffic load | Full buffer | Full buffer | Full buffer |

Table E.2.2-2: FR2-1 network traffic parameters

| Parameter | Urban Macro | Urban Dense | Indoor |
| --- | --- | --- | --- |
| Carrier bandwidth | 200 MHz | 200 MHz | 200 MHz |
| Scheduled channel bandwidth per UE (DL) | For legacy TDD: 200 MHz For SBFD {DU}: 160 MHz | For legacy TDD: 200 MHz For SBFD {DU}: 160 MHz | For legacy TDD: 200 MHz For SBFD {DU}: 160 MHz |
| Scheduled channel bandwidth per UE (UL) | For legacy TDD: 200 MHz For SBFD {DU}: 40 MHz | For legacy TDD: 200 MHz For SBFD {DU}: 40 MHz | For legacy TDD: 200 MHz For {DU}: 40 MHz |
| SBFD BS PSD | The PSD of SBFD is the same as legacy TDD | The PSD of SBFD is the same as legacy TDD | The PSD of SBFD is the same as legacy TDD |
| Traffic load | Full buffer | Full buffer | Full buffer |

## E.2.3	BS RF characteristics

The SBFD system is defined for two different antenna configurations:

- SBFD config 1: The total number of antenna elements of the antenna array for SBFD is the same as the total number of antenna elements of the antenna array for legacy TDD.

- SBFD config 2: The total number of antenna elements of the antenna array for SBFD is two times of the total number of antenna elements of the antenna array for legacy TDD.

As baseline the array antenna model, for single element configuration, is described in TR 38.803, subclause 5.2.3. As an option the antenna model extension, required for modelling sub-array configurations, is described in TR 38.803, subclause 5.2.3.2.4.

As an option, power boosting of 3 dB can be considered to compensate for fewer transmitter branches in the case of SBFD config 1.

Assumptions related to BS RF characteristics relevant for different deployment scenarios are captured for FR1 in Table E.2.3-1 and for FR2-1 in Table E.2.3-2.

Table E.2.3-1: FR1 BS parameters

| Parameter | Urban Macro (Wide Area) | Micro (Medium Range) | Indoor (Local Area) |
| --- | --- | --- | --- |
| Transmitter power (Total conducted power) | Option 1: TDD: 49 dBm SBFD config 1: 46 dBm SBFD config 2: 49 dBm Option 2: TDD: 53 dBm SBFD config 1: 50 dBm SBFD config 2: 53 dBm | TDD: 46 dBm SBFD: 46 dBm | TDD: 24 dBm SBFD config 1: 21 dBm SBFD config 2: 24 dBm |
| Antenna configuration | TDD: (Mg,Ng,M,N,P)=(1,1,8,8,2) SBFD config 1: (Mg,Ng,M,N,P)= (1,1,4,8,2) SBFD config 2: (Mg,Ng,M,N,P)= (1,1,8,8,2) (dH,dV)=(0.5,0.8)λ, (3dB,3dB)=(65,65) o SLAv=25 dB, Am=25 dB, GE,max=5 dBi Optional: Extended AAS model in TR 38.803, subclause 5.2.3.2.4. | TDD: (Mg,Ng,M,N,P)=(1,1,2,2,2) SBFD: (Mg,Ng,M,N,P)=(1,1,2,2,2) (dH,dV)=(0.5,0.8)λ, (3dB,3dB)=(65,65) o SLAv=25 dB, Am=25 dB, GE,max=5 dBi | TDD: (Mg,Ng,M,N,P) =(1,1,4,4,2) SBFD config 1: (Mg,Ng,M,N,P) =(1,1,2,4,2) SBFD config 2: (Mg,Ng,M,N,P) =(1,1,4,4,2) (dH,dV)=(0.5,0.5)λ, (3dB,3dB)=(90,90) o SLAv=25 dB, Am=25 dB, GE,max=5 dBi |
| Receiver noise figure model parameters | P1=-33 dBm P2=-15 dBm F1=5 dB F2=14 dB See Figure E.2.3-1 | P1=-38 dBm P2=-20 dBm F1=10 dB F2=19 dB See Figure E.2.3-1 | P1=-35 dBm P2=-17 dBm F1=13 dB F2=22 dB See Figure E.2.3-1 |
| ACLR | 45 dBc | 45 dBc | 45 dBc |
| ACS | Baseline: 50 dBc Optional: A reported value within the range 46 to 62 dBc | Baseline: 50 dBc Optional: A reported value within the range 46 to 62 dBc | Baseline: 50 dBc Optional: A reported value within the range 46 to 62 dBc |
| BS self-interference receiver sensitivity degradation | 1 dB (Corresponds to Mself=6 dB) | 1 dB (Corresponds to Mself=6 dB) | 1 dB (Corresponds to Mself=6 dB) |

Table E.2.3-2: FR2-1 BS parameters

| Parameter | Urban Macro | Urban Dense | Indoor |
| --- | --- | --- | --- |
| Transmitter power (Total conducted power) | 1st priority: TDD: 30 dBm SBFD config 1: 27 dBm SBFD config 2: 30 dBm 2nd priority: TDD: 40 dBm SBFD config 1: 37 dBm SBFD config 2: 40 dBm | TDD: 30 dBm SBFD config 1: 27 dBm SBFD config 2: 30 dBm | TDD: 24 dBm SBFD config 1: 21 dBm SBFD config 2: 24 dBm |
| Antenna configuration | TDD: (Mg,Ng,M,N,P)=(1,1,8,16,2) SBFD config 1: (Mg,Ng,M,N,P)=(1,1,4,16,2) SBFD config 2: (Mg,Ng,M,N,P)=(1,1,8,16,2) (dH,dV)=(0.5,0.5)λ (3dB, 3dB)=(65,65) o, SLAv=30 dB, Am=30 dB, GE,max=3 dBi Optional: (3dB, 3dB)=(90,90) o, SLAv=30 dB, Am=30 dB, GE,max=5.5 dBi | TDD: (Mg,Ng,M,N,P)=(1,1,8,16,2) SBFD config 1: (Mg,Ng,M,N,P)=(1,1,4,16,2) SBFD config 2: (Mg,Ng,M,N,P)=(1,1,8,16,2) (dH,dV)=(0.5,0.5)λ (3dB, 3dB)=(65,65) o, SLAv=30 dB, Am=30 dB, GE,max=3 dBi | TDD: (Mg,Ng,M,N,P)=(1,1,4,8,2) SBFD config 1: (Mg,Ng,M,N,P)=(1,1,2,8,2) SBFD config 2: (Mg,Ng,M,N,P)=(1,1,4,8,2) (dH,dV)=(0.5,0.5)λ (3dB, 3dB)=(90,90) o, SLAv=30 dB Am=30 dB, GE,max=3 dBi |
| Receiver noise figure parameters | Option 1: P1=-63 dBm P2=-45 dBm F1=10 dB F2=19 dB Option 2: P1=-52 dBm F1=10 dB A sloped section rising by 1 dB/dB for Pin>-52 dBm See Figure E.2.3-1 | Option 1: P1=-63 dBm P2=-45 dBm F1=10 dB F2=19 dB Option 2: P1=-52 dBm F1=10 dB A sloped section rising by 1 dB/dB for Pin>-52 dBm See Figure E.2.3-1 | Option 1: P1=-63 dBm P2=-45 dBm F1=10 dB F2=19 dB Option 2: P1=-52 dBm F1=10 dB A sloped section rising by 1 dB/dB for Pin>-52 dBm See Figure E.2.3-1 |
| ACLR | 28 dBc | 28 dBc | 28 dBc |
| ACS | 24 dBc | 24 dBc | 24 dBc |
| BS self-interference receiver sensitivity degradation | 1 dB (Corresponds to Mself=6 dB) | 1 dB (Corresponds to Mself=6 dB) | 1 dB (Corresponds to Mself=6 dB) |

To capture receiver blocking aspects the receiver noise figure (Fblock) will be characterised as a piece-wise linear function of input receiver power (Pin) as shown in Figure E.2.3-1.

# Figure E.2.3-1: Receiver blocking model

Relevant values for F, F2, P1 and P2 for different BS classes are listed in Table E.2.3-1 and Table E.2.3-2. If the total received power is larger than P2 the receiver will be blocked, which correspond to zero throughput.

The input power to the blocking model is defined as average total power in dBm and the output is the noise figure in dB including noise rise due to receiver blocking aspects, such as e.g., receiver intermodulation.

The receiver input total power can be expressed as a sum of wanted signal and all interferer signals in linear scale as:

Pin=Pw+Iintra-SB, inter-UE+Iinter-SB,inter-gNB+IACI+Iself+Ico-site,inter-sector

, where the signal and interferer sources are defined as:

Pw is the wanted signal received from scheduled UE. The wanted signal level is calculated as:
Pw = PTX,UECLUE→serving gNB

Iintra-SB,inter-UE is the interference from UEs transmitting within the UL sub-band in the victim network. The interference level is calculated as:

Iintra-SB,inter-UE = PTX,UECLUE→serving gNB

Iinter-SB,inter-gNB is the interference from gNB transmitting on the adjacent DL sub-band in the victim network. The interference level is calculated as:

Iinter-SB,inter-gNB=PTX,BSCLown network→serving gNB

IACI is the adjacent channel interference from gNB transmitting on the DL channel in the aggressor network. The interference level is calculated as:

IACI=PTX,BSCLadjacent network gNB→serving gNB

Iself is the victim self-interference power. The interference level is calculated as:

Iself=kTBSB FMself

Ico-site,inter-sector is the interference from neighboring sectors in a 3-sector configuration. The interference level is calculated as:

Ico-site,inter-sector=kTBSB FMco-site

, where

PTX,UE is the configured UE transmitter power (see transmission power control in Annex E.3.5).

PTX,BS is the gNB transmitter power.

BSB is the allocated SBFD UL receiver slot bandwidth.

Mself is the noise-floor-to-self interference ratio.

Mco-site is the noise-floor-to-aggregated co-site inter sector interference ratio.

kT is the thermal noise (equal to -174 dBm/Hz in logarithmical scale).

F is the receiver noise figure.

The total interference power can be expressed in linear scale as:

Itot=Iintra-SB,inter-UE+Iinter-SB,inter-gNBASBIRinter-SB,inter-gNB+IACIACIR+Iself+Ico-site,inter-sector

, where additional variables are defined as:

ASBIRinter-SB,inter-gNB is the in-channel Adjacent Sub-Band Interference Ratio, which is derived by applicable BS ACLR and BS ACS.

ACIR is derived by applicable BS ACLR and BS ACS.

The SINR at the victim receiver including total noise and total interference can be expressed in linear scale as:

SINR=PwItot+kTBSBFblock

The receiver noise figure for the receiver exposed for blocking Fblock is produced by the blocking model described in Figure E.2.3-1.

In simulation, the UE power control scheme is only used to compensate path loss and doesn’t consider noise figure increase due to BS receiver blocking. Therefore, the final SINR for UL is less than target SINR.

Note that additional, power scaling of all interference contributions should be considered with respect to aggressor bandwidth and victim bandwidth when ACIR is applied.

## E.2.4	UE RF characteristics

Assumptions relevant for modelling the UE RF characteristics are captured in Table E.2.4-1.

Table E.2.4-1: UE parameters

| Parameter | FR1 | FR2-1 |
| --- | --- | --- |
| Maximum transmitter power | 23 dBm | 22.4 dBm (EIRP) |
| Minimum transmitter power | -33 dBm (TS 38.101-1, Table 6.3.1-1) | -40 dBm |
| Antenna configuration | 0 dBi | (Mg,Ng,M,N,P) = (1,1,2,2,2) (dH,dV)=(0.5,0.5)λ (3dB, 3dB)=(90,90) o, SLAv=25 dB Am=25 dB, GE,max=5.5 dBi |
| Receiver noise figure | 9 dB | 10 dB |
| ACLR | 30 dBc The ACLR is modelled as 30 dB at UE maximum TX power and improves 1 dB per 1 dB backoff TX power up to a maximum 10 dB improvement. i.e., at 10 dB power backoff the ACLR is 40 dB. | 24 dBc The ACLR is modelled as 24 dB at UE maximum TX power and improves 1 dB per 1 dB backoff TX power up to a maximum 10 dB improvement. i.e., at 10 dB power backoff the ACLR is 34 dB. |
| ACS | 33 dBc | 23 dBc |

# E.3	RAN4 co-existence simulation methodology

## E.3.1	Coexistence evaluation methodology

The coexistence evaluation methodology can be summarized as:

- Aggressor and victim network are generated. UEs are distributed as described by parameter assumptions.

- UEs are associated to BS based on coupling loss.

- Once association is done, round robin scheduling is used. BF weights are adjusted to point to the LOS direction between BS and UE. This is done for both victim and aggressor networks.

- Throughput is computed in the victim systems without considering Adjacent Channel Interference (ACI) as:

ThputNO ACI[bit/s/Hz]=f(SINRICI)=f(S/(N+IICI)), where IICI is the inter-cell interference.

- Throughput is computed considering ACI as:

ThputACI[bit/s/Hz]=f(SINRICI+ACI)=f(S/(N+IICI+IACI)), where IACI is the adjacent channel interference.

- RF parameters are determined based on the degradation caused by ACI as:

LossACI=1-ThputACI/ThputSINGLE

The simulation results should be in the form of the throughput with adjacent network  and the relative difference between the two can be compared at 50% and 5% points.

The throughput of a modem with link adaptation can be approximated by an attenuated and truncated form of the Shannon bound. (The Shannon bound represents the maximum theoretical throughput than can be achieved over an AWGN channel for a given SNIR). The following equations approximate the throughput (Tp) in bps/Hz over a channel with a given SNIR, when using link adaptation:

Tp=0∝SSNIR∝SSNIRMAXfor SNIR≤SNIRMIN for SNIRMIN≤SNIR<SNIRMAXfor SNIR≥SNIRMAX

Where:

| S(SNIR) | Shannon bound, S(SNIR) =log2(1+SNIR) bps/Hz |
| --- | --- |
|  | Attenuation factor, representing implementation losses |
| SNIRMIN | Minimum SNIR of the code set, dB |
| SNIRMAX | Maximum SNIR of the code set, dB |

The parameters can be chosen to represent different modem implementations and link conditions. A parameter set relevant for eMBB is listed in Table E.3.1-1.

Table E.3.1-1: Parameters describing baseline Link Level performance for NR

| Parameter | DL | UL | Notes |
| --- | --- | --- | --- |
| , attenuation | 0.6 | 0.4 | Represent implementation losses |
| SNIRMIN | -10 | -10 | Based on QPSK, 1/8 rate (DL) and 1/5 rate (UL) |
| SNIRMAX | 30 | 22 | Based on 256QAM, 0.93 (DL) and 64QAM 0.93 (UL) |

## E.3.2	Received signal power model

The following model is applied:

RX_PWR = TX_PWR – Path loss + G_TX + G_RX

, where

RX_PWR is the received power.

TX_PWR is the transmitted power.

G_TX is the transmitter antenna gain (directional array gain).

G_RX is the receiver antenna gain (directional array gain).

## E.3.3	Network grid shift

In cases with grid shift >0%, the victim network base stations should be placed as shown in Figure E.3.3-1. The victim network grid is shifted along the line between BS and its closest 100% grid-shift BS, where the distance from any BS in victim network to its second and third closest BS in the aggressor network is the same.

# Figure E.3.3-1: Network grid shift definition

## E.3.4	Coupling-loss

The Coupling Loss (CL) is defined as the loss in signal between BS-to-UE, UE-to-UE and BS-to-BS. CL is defined as the loss including propagation loss and antenna gains with BF weights applied measured between antenna connectors.

## E.3.5	Transmission power control

For downlink scenario, no power control scheme is applied.

For uplink scenario, TPC model specified in TR 36.942, subclause 9.1 is applied with following parameters.

-	CLx-ile = –SNR_target + UE_max_eirp– ThermalNoise – BS_NoiseFigure - 10*log10(BW)

-	γ = 1

Where, SNR_target for FR1 and FR2 are 15 dB.

In simulation, power control scheme is only used to compensate path loss. That’s the reason why final SINR for UL is less than assumed target SINR. But commercial UE UL SINR could meet target value according to power control scheme in TS 38.213.