3GPP TSG-RAN WG4 Meeting #108                          			R4-2313913

# Toulouse, France, August 21 – August 25, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.113 | CR | 0088 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 36.113: framework for the EMC-specific manufacturer's declarations, Rel-18 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18, NR_LTE_EMC_enh-Perf |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the WF on the BS EMC test simplification aspects in R4-2309861, it was agreed to use manufacturer declaration for the implementation of the EMC test reduction implementation for MSR BS EMC and AAS BS EMC specification. In order to align all BS EMC specifications, this CR mirrors modifications from TS 37.113 and TS 37.114, introducing new section for the definition of EMC-specific manufacturer declarations. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - 4.7: new section for EMC-specific manufacturer declarations, aligning the approach with RF test specifications. - 6.3, 6.4: correction to add optional declaration for the ancillary equipment performance degradation. - 9.5, 9.6: new cross-references added for EMC-specific declarations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Framework for introduction of the new EMC-specific declaration for EMC testing simplification would be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7, 6.3, 6.4, 9.4, 9.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.113 CR0088, TS37.114 CR0107, TS 37.113 CR0128 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------ Modified section ------------------------------

# 1	Scope

The present document covers the assessment of E-UTRA, E-UTRA with NB-IoT or NB-IoT base stations, repeaters and associated ancillary equipment in respect of Electromagnetic Compatibility (EMC).

The present document specifies the applicable test conditions, performance assessment and performance criteria for E-UTRA, E-UTRA with NB-IoT or NB-IoT base stations, repeaters and associated ancillary equipment in one of the following categories:

-	base stations of E-UTRA, E-UTRA with NB-IoT or NB-IoT meeting the requirements of TS 36.104 [2], with conformance demonstrated by compliance to TS 36.141 [3].

-	repeaters of E-UTRA meeting the requirements of TS 36.106 [4], with conformance demonstrated by compliance to TS 36.143 [5].

Technical requirements related to the antenna port of E-UTRA, E-UTRA with NB-IoT or NB-IoT base stations or repeaters are not included in the present document. These are found in the relevant product standards [2-5].

The environment classification used in the present document refers to the residential, commercial and light industrial environment classification used in IEC 61000-6-1 [6] and IEC 61000-6-3 [7].

The EMC requirements have been selected to ensure an adequate level of compatibility for apparatus at residential, commercial and light industrial environments. The levels, however, do not cover extreme cases which may occur in any location but with low probability of occurrence.

------------------------------ Next modified section ------------------------------

### 4.4.1	Arrangements for test signals for repeaters

For immunity tests of repeaters, the wanted RF input signal shall be coupled to one antenna port at a level which will result, when measured, in the maximum rated RF output power per channel, as declared by the manufacturer in TS 36.141 [3]. The test shall either be repeated with a wanted signal coupled to the other antenna port, or a single test shall be performed with the specified input signals being simultaneously coupled to both antenna ports.

------------------------------ Next modified section ------------------------------

## 4.7	Manufacturer declarations

The following EMC-specific manufacturer’s declarations listed in table 4.7-1 are required to be provided by the manufacturer for E-UTRA BS requirements testing.

NOTE:	The below listed manufacturer’s declarations are supplementary to declarations covered in TS 36.141 [3].

Table 4.7-1 EMC-specific manufacturer declarations for E-UTRA BS test requirements

| Declaration identifier | Declaration | Description |
| --- | --- | --- |
| EMCD.1 | Declaration of ports intended to be used with cables longer than 3 m | Declaration of any ports intended to be used with cables longer than 3 m. |

------------------------------ Next modified section ------------------------------

## 6.3	Performance criteria for continuous phenomena for ancillary equipment

The apparatus shall continue to operate as intended during and after the test. No degradation of performance or loss of function is allowed below the performance level specified by the manufacturer, when the apparatus is used as intended. The performance level may be replaced by a permissible performance loss. If the minimum performance level or the permissible performance loss is not specified by the manufacturer, either of these may be derived from the product description and documentation and what the user may reasonably expect from the apparatus if used as intended.

------------------------------ Next modified section ------------------------------

## 6.4	Performance criteria for transient phenomena for ancillary equipment

The apparatus shall continue to operate as intended after the test. No degradation of performance or loss of function is allowed below the performance level specified by the manufacturer, when the apparatus is used as intended. The performance level may be replaced by a permissible performance loss. During the test, degradation of performance is however allowed. If the minimum performance level or the permissible performance loss is not specified by the manufacturer, either of these may be derived from the product description and documentation and what the user may reasonably expect from the apparatus if used as intended.

------------------------------ Next modified section ------------------------------

## 6.6	Performance criteria for transient phenomena for repeaters

The gain of the EUT shall be measured before the test and after each exposure. At the conclusion of each exposure the gain of the EUT shall not have changed by more than ± 1 dB. At the conclusion of the total test comprising the series of individual exposures, the EUT shall operate as intended with no loss of user control functions or stored data, as specified by the manufacturer, and the gain of the EUT shall not have changed by more than ± 1 dB.

------------------------------ Next modified section ------------------------------

## 9.5	Fast transients common mode

The test shall be performed on AC mains power input ports.

This test shall be performed on signal ports, telecommunication ports, control ports and DC power input/output ports if the cables may be longer than 3 m.

Where this test is not carried out on a port or any other ports because the manufacturer declares in EMCD.1 (see table 4.7-1) that it is not intended to be used with cables longer than 3 m, a list of ports which were not tested for this reason shall be included in the test report.

This test shall be performed on a representative configuration of the equipment, the associated ancillary equipment, or representative configuration of the combination of radio and ancillary equipment.

------------------------------ Next modified section ------------------------------

## 9.6	RF common mode (0,15 MHz - 80 MHz)

The test shall be performed on AC mains power input/output ports.

This test shall be performed on signal ports, telecommunication ports, control and DC power input/output ports, which may have cables longer than 3 m.

Where this test is not carried out on a port or any other ports because the manufacturer declares in EMCD.1 (see table 4.7-1) that it is not intended to be used with cables longer than stated above, a list of ports which were not tested shall be included in the test report.

This test shall be performed on a representative configuration of the equipment, the associated ancillary equipment, or representative configuration of the combination of radio and ancillary equipment.

NOTE:	This test can also be performed using the clamp injection method, where appropriate, see IEC 61000-4-6 [20].

------------------------------ End of modified section ------------------------------