3GPP TSG-RAN WG4 Meeting #108	R4-2313920

Toulouse, France, 21 August ‒ 25 August 2023

Title: 	Simulation assumption for FR2 HST demodulation

Agenda Item:	8.12.6

Source: 	Ericsson

Document for:	Approval

# Simulation assumption for PDSCH demodulation requirements with CA

Interested companies are encouraged to provide the simulation results in RAN4#108bis.

Table 1	Test cases

| Test number | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Number of active PDSCH TCI states | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | FRC 1-1 | 50 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | TBD |
| 1-2 | FRC 1-2 | 100 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | TBD |
| 1-3 | FRC 1-3 | 400 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | TBD |
| 1-4 (For information) | R.PDSCH.5-12.2 TDD | 200 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | 13.9 |
| 2-1 | FRC 2-1 | 50 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | TBD |
| 2-2 | FRC 2-2 | 100 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | TBD |
| 2-3 | FRC 2-3 | 400 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | TBD |
| 2-4 (For information) | R.PDSCH.5-12.1 TDD | 200 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | 13.7 |

Table 2	FRC for test 1 (HST-DPS-FR2-BI-B, single TCI states)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.5-12.2 TDD (For information) | FRC 1-1 | FRC 1-2 | FRC 1-3 |
| Channel bandwidth | MHz | 200 | 50 | 100 | 400 |
| Subcarrier spacing | kHz | 120 | 120 | 120 | 120 |
| Allocated resource blocks | PRBs | 132 | 32 | 66 | 264 |
| Number of consecutive PDSCH symbols |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} |  | 9 | 9 | 9 | 9 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} |  | 13 | 13 | 13 | 13 |
| For Slot i=1,2,3 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |
| Allocated slots per 2 frames |  | 124 | 124 | 124 | 124 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  | 17 | 17 | 17 | 17 |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM |
| Target Coding Rate |  | 0.43 | 0.43 | 0.43 | 0.43 |
| Number of MIMO layers |  | 2 | 2 | 2 | 2 |
| Number of DMRS REs |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} |  | 18 | 18 | 18 | 18 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} |  | 18 | 18 | 18 | 18 |
| For Slot i = 1,2,3 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |
| Overhead for TBS determination |  | 6 | 6 | 6 | 6 |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} | Bits | 57376 | [13832] | [28680] | [114776] |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} | Bits | 90176 | [21504] | [45096] | [180376] |
| For Slot i = 1,2,3 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |
| Transport block CRC per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} | Bits | 24 | 24 | 24 | 24 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} | Bits | 24 | 24 | 24 | 24 |
| For Slot i = 1,2,3 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | CBs | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} | CBs | 7 | [2] | [4] | [15] |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} | CBs | 11 | [3] | [6] | [22] |
| For Slot i = 1,2,3 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |
| Binary Channel Bits Per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |
| For Slots i = 5 and 85 (Note 3) | Bits | 181968 | [33408] | [76632] | [392640] |
| For Slots i = 6 and 86 (Note 3) | Bits | 181968 | [33408] | [76632] | [f392640] |
| For Slot i, if mod(i, 5) = 3 for i from {7,…, 94,87, …, 159} | Bits | 137808 | [33408] | [68904] | [275616] |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {7,…,84,87,…,159} | Bits | 210672 | [51072] | [105336] | [421344] |
| For Slot i = 1,2,3 | Bits | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |
| Max. Throughput averaged over 2 frames | Mbps | 508.251 | [121.433] | [254.15] | [1016.651] |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Binary Channel Bits are calculated under assumption of 52 PRBs TRS allocation when the number of allocated resource blocks are more than 52. Note 4: SS/PBCH block is transmitted in slot #1, slot #2 and slot #3 with periodicity 20ms |  |  |  |  |  |

Table 3	FRC for test 2 (HST-DPS-FR2-UNI-A, two TCI state)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.5-12.1 TDD (For information) | FRC 2-1 | FRC 2-2 | FRC 2-3 |
| Channel bandwidth | MHz | 200 | 50 | 100 | 400 |
| Subcarrier spacing | kHz | 120 | 120 | 120 | 120 |
| Allocated resource blocks | PRBs | 132 | 32 | 66 | 264 |
| Number of consecutive PDSCH symbols |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} |  | 9 | 9 | 9 | 9 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} |  | 13 | 13 | 13 | 13 |
| For slot i = 1 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |
| For slot i = 2 |  | 13 | 13 | 13 | 13 |
| For slot i = 3 |  | 9 | 9 | 9 | 9 |
| Allocated slots per 2 frames |  | 126 | 126 | 126 | 126 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  | 17 | 17 | 17 | 17 |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM |
| Target Coding Rate |  | 0.43 | 0.43 | 0.43 | 0.43 |
| Number of MIMO layers |  | 2 | 2 | 2 | 2 |
| Number of DMRS REs |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} |  | 18 | 18 | 18 | 18 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} |  | 18 | 18 | 18 | 18 |
| For Slot i = 1 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |
| For Slot i = 2 |  | 18 | 18 | 18 | 18 |
| For Slot i = 3 |  | 18 | 18 | 18 | 18 |
| Overhead for TBS determination |  | 6 | 6 | 6 | 6 |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} | Bits | 57376 | [13832-] | [28680] | [114776] |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} | Bits | 90176 | [21504] | [45096] | [180376] |
| For Slot i = 1 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |
| For Slot i = 2 |  | 90176 | [21504] | [45096] | [180376] |
| For Slot i = 3 |  | 57376 | [13832] | [28680] | [114776] |
| Transport block CRC per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} | Bits | 24 | 24 | 24 | 24 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} | Bits | 24 | 24 | 24 | 24 |
| For Slot i = 1 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |
| For Slot i = 2 |  | 24 | 24 | 24 | 24 |
| For Slot i = 3 |  | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | CBs | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} | CBs | 7 | [2] | [4] | [15] |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} | CBs | 11 | [3] | [6] | [22] |
| For Slot i = 1 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |
| For Slot i = 2 |  | 11 | [2] | [4] | [15] |
| For Slot i = 3 |  | 7 | [3] | [6] | [22] |
| Binary Channel Bits Per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A | N/A |
| For Slots i = 5 and 85 (Note 3) | Bits | 195696 | [41856] | [90360] | [406368] |
| For Slots i = 6 and 86 (Note 3) | Bits | 195696 | [41856] | [90360] | [406368] |
| For Slot i, if mod(i, 5) = 3 for i from {7,…, 84, 87, …, 159} | Bits | 137808 | [33408] | [68904] | [275616] |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {7,…,84,87,…,159} | Bits | 210672 | [51072] | [105336] | [421344] |
| For Slot i = 1 | Bits | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |
| For Slot i = 2 |  | 210672 | [51072] | [105336] | [421344] |
| For Slot i = 3 |  | 137808 | [33408] | [68904] | [275616] |
| Max. Throughput averaged over 2 frames | Mbps | 515.629 | [123.2] | [257.839] | [1031.409] |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Binary Channel Bits are calculated under assumption of 52 PRBs TRS allocation when the number of allocated resource blocks are more than 52. Note 4: SS/PBCH block is transmitted in slot #1 with periodicity 20ms |  |  |  |  |  |

Table 4	Test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  |  | TDD |
| Active DL BWP index |  |  |  | 1 |
| PDCCH configuration | TCI state |  |  | Note 1 |
| PDSCH configuration | Mapping type |  |  | Type A |
|  | k0 |  |  | 0 |
|  | Starting symbol (S) |  |  | 1 |
|  | Length (L) |  |  | Specific to each Reference channel |
|  | PDSCH aggregation factor |  |  | 1 |
|  | PRB bundling type |  |  | Static |
|  | PRB bundling size |  |  | 2 |
|  | Resource allocation type |  |  | Type 0 |
|  | RBG size |  |  | Config2 |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |
|  | TCI state |  |  | Note 1 |
| PDSCH DMRS configuration | DMRS Type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | 2 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |
| CSI-RS for tracking | Resource set #1 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 0 for CSI-RS resource 1,2,3,4 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 |
|  |  |  |  | l0 = 9 for CSI-RS resource 2 and 4 |
|  |  | CSI-RS periodicity | Slots | 80 for CSI-RS resource 1,2,3,4 |
|  |  | CSI-RS offset | Slots | 5 for CSI-RS resource 1 and 2 |
|  |  |  |  | 6 for CSI-RS resource 3 and 4 |
|  |  | QCL info |  | TCI state #4 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB =ceil(BWP size/4)*4 |
|  | Resource set #2 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 0 for CSI-RS resource 5,6,7,8 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 4 for CSI-RS resource 5 and 6 |
|  |  |  |  | l0 = 8 for CSI-RS resource 7 and 8 |
|  |  | CSI-RS periodicity | Slots | 80 for CSI-RS resource 5,6,7,8 |
|  |  | CSI-RS offset | Slots | 5 for CSI-RS resource 5 and 6 |
|  |  |  |  | 6 for CSI-RS resource 7 and 8 |
|  |  | QCL info |  | TCI state #5 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB =ceil(BWP size/4)*4 |
|  | Resource set #3 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 1 for CSI-RS resource 9,10,11,12 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 9 and 11 |
|  |  |  |  | l0 = 9 for CSI-RS resource 10 and 12 |
|  |  | CSI-RS periodicity | Slots | 80 for CSI-RS resource 9,10,11,12 |
|  |  | CSI-RS offset | Slots | 5 for CSI-RS resource 9 and 10 |
|  |  |  |  | 6 for CSI-RS resource 11 and 12 |
|  |  | QCL info |  | TCI state #6 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB =ceil(BWP size/4)*4 |
|  | Resource set #4 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 1 for CSI-RS resource 13,14,15,16 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 4 for CSI-RS resource 13 and 14 |
|  |  |  |  | l0 = 8 for CSI-RS resource 15 and 16 |
|  |  | CSI-RS periodicity | Slots | 80 for CSI-RS resource 13,14,15,16 |
|  |  | CSI-RS offset | Slots | 5 for CSI-RS resource 13 and 14 |
|  |  |  |  | 6 for CSI-RS resource 15 and 16 |
|  |  | QCL info |  | TCI state #7 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB =ceil(BWP size/4)*4 |
|  | Resource set #13 (Note2) | First subcarrier index in the PRB used for CSI-RS (k0) |  | 2 for CSI-RS resource 17,18,19,20 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 17 and 19 |
|  |  |  |  | l0 = 9 for CSI-RS resource 18 and 20 |
|  |  | CSI-RS periodicity | Slots | 80 for CSI-RS resource 17,18,19,20 |
|  |  | CSI-RS offset | Slots | 5 for CSI-RS resource 17 and 18 |
|  |  |  |  | 6 for CSI-RS resource 19 and 20 |
|  |  | QCL info |  | TCI state #12 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB =ceil(BWP size/4)*4 |
|  | Resource set #14 (Note2) | First subcarrier index in the PRB used for CSI-RS (k0) |  | 2 for CSI-RS resource 21,22,23,24 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 4 for CSI-RS resource 21 and 22 |
|  |  |  |  | l0 = 8 for CSI-RS resource 23 and 24 |
|  |  | CSI-RS periodicity | Slots | 80 for CSI-RS resource 21,22,23,24 |
|  |  | CSI-RS offset | Slots | 5 for CSI-RS resource 21 and 22 |
|  |  |  |  | 6 for CSI-RS resource 23 and 24 |
|  |  | QCL info |  | TCI state #13 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB =ceil(BWP size/4)*4 |
|  | Resource set #15 (Note2) | First subcarrier index in the PRB used for CSI-RS (k0) |  | 3 for CSI-RS resource 25,26,27,28 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 25 and 27 |
|  |  |  |  | l0 = 9 for CSI-RS resource 26 and 28 |
|  |  | CSI-RS periodicity | Slots | 80 for CSI-RS resource 25,26,27,28 |
|  |  | CSI-RS offset | Slots | 5 for CSI-RS resource 25 and 26 |
|  |  |  |  | 6 for CSI-RS resource 27 and 28 |
|  |  | QCL info |  | TCI state #14 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB =ceil(BWP size/4)*4 |
|  | Resource set #16 (Note2) | First subcarrier index in the PRB used for CSI-RS (k0) |  | 3 for CSI-RS resource 29,30,31,32 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 4 for CSI-RS resource 29 and 30 |
|  |  |  |  | l0 = 8 for CSI-RS resource 31 and 32 |
|  |  | CSI-RS periodicity | Slots | 80 for CSI-RS resource 29,30,31,32 |
|  |  | CSI-RS offset | Slots | 5 for CSI-RS resource 29 and 30 |
|  |  |  |  | 6 for CSI-RS resource 31 and 32 |
|  |  | QCL info |  | TCI state #15 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB =ceil(BWP size/4)*4 |
| NZP CSI-RS for CSI acquisition | Resource set #5 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 0 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #0 |
|  | Resource set #6 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 2 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #1 |
|  | Resource set #7 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 4 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #2 |
|  | Resource set #8 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 6 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #3 |
|  | Resource set #17 (Note2) | First subcarrier index in the PRB used for CSI-RS (k0) |  | 0 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 1 |
|  |  | QCL info |  | TCI state #8 |
|  | Resource set #18 (Note2) | First subcarrier index in the PRB used for CSI-RS (k0) |  | 2 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 1 |
|  |  | QCL info |  | TCI state #9 |
|  | Resource set #19 (Note2) | First subcarrier index in the PRB used for CSI-RS (k0) |  | 4 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 1 |
|  |  | QCL info |  | TCI state #10 |
|  | Resource set #20 (Note2) | First subcarrier index in the PRB used for CSI-RS (k0) |  | 6 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 1 |
|  |  | QCL info |  | TCI state #11 |
| CSI-RS for beam refinement | Resource set #9 | First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 8 for CSI-RS resource 1 l0 = 9 for CSI-RS resource 2 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #0 |
|  | Resource set #10 | First subcarrier index in the PRB used for CSI-RS |  | k0=1 for CSI-RS resource 3,4 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 8 for CSI-RS resource 3 l0 = 9 for CSI-RS resource 4 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #1 |
|  | Resource set #11 | First subcarrier index in the PRB used for CSI-RS |  | k0=2 for CSI-RS resource 5,6 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 8 for CSI-RS resource 5 l0 = 9 for CSI-RS resource 6 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #2 |
|  | Resource set #12 | First subcarrier index in the PRB used for CSI-RS |  | k0=3 for CSI-RS resource 7,8 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 8 for CSI-RS resource 7 l0 = 9 for CSI-RS resource 8 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #3 |
|  | Resource set #21 (Note2) | First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 9,10 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 10 for CSI-RS resource 9 l0 = 11 for CSI-RS resource 10 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 1 |
|  |  | QCL info |  | TCI state #8 |
|  | Resource set #22 (Note2) | First subcarrier index in the PRB used for CSI-RS |  | k0=1 for CSI-RS resource 11,12 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 10 for CSI-RS resource 11 l0 = 11 for CSI-RS resource 12 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 1 |
|  |  | QCL info |  | TCI state #9 |
|  | Resource set #23 (Note2) | First subcarrier index in the PRB used for CSI-RS |  | k0=2 for CSI-RS resource 13,14 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 10 for CSI-RS resource 13 l0 = 11 for CSI-RS resource 14 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 1 |
|  |  | QCL info |  | TCI state #10 |
|  | Resource set #24 (Note2) | First subcarrier index in the PRB used for CSI-RS |  | k0=3 for CSI-RS resource 15,16 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 10 for CSI-RS resource 15 l0 = 11 for CSI-RS resource 16 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 1 |
|  |  | QCL info |  | TCI state #11 |
| TCI state #0 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking Resource set #1' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking Resource set #1' configuration |
|  |  | QCL Type |  | Type D |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 5 from 'CSI-RS for tracking Resource set #2' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 5 from 'CSI-RS for tracking Resource set #2' configuration |
|  |  | QCL Type |  | Type D |
| TCI state #2 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 9 from 'CSI-RS for tracking Resource set #3' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 9 from 'CSI-RS for tracking Resource set #3' configuration |
|  |  | QCL Type |  | Type D |
| TCI state #3 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 13 from 'CSI-RS for tracking Resource set #4' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 13 from 'CSI-RS for tracking Resource set #4' configuration |
|  |  | QCL Type |  | Type D |
| TCI state #8 (Note2) | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 17 from 'CSI-RS for tracking Resource set #13' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 17 from 'CSI-RS for tracking Resource set #13' configuration |
|  |  | QCL Type |  | Type D |
| TCI state #9 (Note2) | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 21 from 'CSI-RS for tracking Resource set #14' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 21 from 'CSI-RS for tracking Resource set #14' configuration |
|  |  | QCL Type |  | Type D |
| TCI state #10 (Note2) | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 25 from 'CSI-RS for tracking Resource set #15' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 25 from 'CSI-RS for tracking Resource set #15' configuration |
|  |  | QCL Type |  | Type D |
| TCI state #11 (Note2) | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 29 from 'CSI-RS for tracking Resource set #16' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 29 from 'CSI-RS for tracking Resource set #16' configuration |
|  |  | QCL Type |  | Type D |
| TCI state #4 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type D |
| TCI state #5 | Type 1 QCL information | SSB index |  | SSB #1 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #1 |
|  |  | QCL Type |  | Type D |
| TCI state #6 | Type 1 QCL information | SSB index |  | SSB #2 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #2 |
|  |  | QCL Type |  | Type D |
| TCI state #7 | Type 1 QCL information | SSB index |  | SSB #3 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #3 |
|  |  | QCL Type |  | Type D |
| TCI state #12 (Note2) | Type 1 QCL information | SSB index |  | SSB #4 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #4 |
|  |  | QCL Type |  | Type D |
| TCI state #13 (Note2) | Type 1 QCL information | SSB index |  | SSB #5 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #5 |
|  |  | QCL Type |  | Type D |
| TCI state #14 (Note2) | Type 1 QCL information | SSB index |  | SSB #6 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #6 |
|  |  | QCL Type |  | Type D |
| TCI state #15 (Note2) | Type 1 QCL information | SSB index |  | SSB #7 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #7 |
|  |  | QCL Type |  | Type D |
| Number of HARQ Processes |  |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  | Specific to each TDD UL-DL pattern and as defined in TS38.101-4 Annex A.1.3 |
| Note 1: For Test 1-x, SSB # (2k mod 8) , CSI-RS (for tracking) resource set # ((k mod 4)+1), CSI-RS (for CSI acquisition) resource set # ((k mod 4) + 5) and CSI-RS (for beam refinement) resource set # ((k mod 4) + 9) are transmitted by kth RRH; SSB # ((2k mod 8)+1) , CSI-RS (for tracking) resource set # ((k mod 4) + 13), CSI-RS (for CSI acquisition) resource set # ((k mod 4) + 17) and CSI-RS (for beam refinement) resource set # ((k mod 4) + 21) are transmitted by kth RRH. TCI state switching command scheduled by MAC CE with MCS 4 is transmitted in slot #i that satisfy modi,n=0 (i≠0). PDCCH and PDSCH associated with TCI # (k mod 4) is transmitted by kth RRH from slot# 0,k=12k-2n+1+THARQ+TMAC proc+TfirstSSB+TSSB proc+TfirstTRSafterSSB+TTRS proc,k=2,3,4… to slot# 2k-1n+THARQ+TMAC proc,k=1,2,3…, PDCCH and PDSCH associated with TCI # ((k mod 4)+8) is transmitted by kth RRH from slot# 2k+1n+1+THARQ+TMAC proc+TfirstSSB+TSSB proc+TfirstTRSafterSSB+TTRS proc,k=0,1,2… to slot# [2k+1n+THARQ+TMAC proc],k=0,1,2…, where k is the RRH number, n = 28800 is half of the number of slots between two RRH, THARQ = 4 is the number of slots between PDSCH and corresponding HARQ-ACK information, TMAC proc = 24 is the number of slots for MAC CE processing, TfirstSSB = 132 is the number of slots to first SSB transmission occasion after MAC CE command is decoded by the UE, TSSB proc= 16 is the number of slots for SSB processing, TfirstTRSafterSSB = 66 is the number of slots to first TRS transmission occasion after first SSB is processed by the UE, TTRSproc= 16 is the number of slots for TRS processing. PDCCH and PDSCH are DTXed in other slots in which throughput statistics are not considered. For Test 1-2, SSB # (k mod 4) , CSI-RS (for tracking) resource set # ((k mod 4)+1), CSI-RS (for CSI acquisition) resource set # ((k mod 4) + 5) and CSI-RS (for beam refinement) resource set # ((k mod 4) + 9) are transmitted by kth RRH. TCI state switching command scheduled by MAC CE with MCS 4 is transmitted in slot #i that satisfy modi,n=0. PDCCH and PDSCH associated with TCI # (k mod 4) is transmitted by kth RRH from slot# 0,k=1k-1n+2+THARQ+TMAC proc,k=2,3,4… to slot# [k * n+THARQ+TMAC proc], k=1,2,3…, where k is the RRH number, n = 57600 is half of the number of slots between two RRH, THARQ = 4 is the number of slots between PDSCH and corresponding HARQ-ACK information, TMAC proc = 24 is the number of slots for MAC CE processing. PDCCH and PDSCH are DTXed in other slots in which throughput statistics are not considered. Note 2: Only configured for Test 1-x. |  |  |  |  |

# Simulation assumption for PDSCH demodulation requirements with multi-Rx simultaneous reception

Interested companies are encouraged to provide the simulation results in RAN4#108bis.

| Parameter |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  | TCI State #1 | TCI State #2 |
|  |  | CORESETPoolIndex |  | 0,1 |  |
| Duplex mode |  |  |  | TDD |  |
| UL-DL configuration |  |  |  | FR2.120-1 |  |
| Active DL BWP index |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  | Type A |  |
|  | k0 |  |  | 0 |  |
|  | Starting symbol (S) |  |  | 1 |  |
|  | Length (L) |  |  | 13 |  |
|  | PRB bundling type |  |  | Static |  |
|  | PRB bundling size |  |  | 2 |  |
|  | Resource allocation type |  |  | Type 1 |  |
|  | RBG size |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  | {1000,1001} | {1002,1003} |
|  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  | 2 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |  |
| Resource allocation |  |  |  | full-overlapping |  |
| Number of HARQ Processes |  |  |  | 8 |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  | As defined in Annex A.1.3 |  |
| Precoding configuration |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity |  |
| Note 1: PDSCH transmission is done from both TRxPs. Transmission from TRxP #1 uses CORESETPoolIndex 0 and transmission from TRxP #2 uses CORESETPoolIndex 1 |  |  |  |  |  |

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | TRxP #1 | TRxP #2-1 | TRxP #2-2 |  |
| Channel bandwidth | MHz | 200 | 200 | 200 |  |
| Subcarrier spacing | kHz | 120 | 120 | 120 |  |
| Allocated resource blocks | PRBs | 132 | 132 | 132 |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} |  | 9 | 9 | 9 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} |  | 13 | 13 | 13 |  |
| For Slot i=1,2,3 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| Allocated slots per 2 frames |  | 124 | 124 | 124 |  |
| MCS table |  | 64QAM | 64QAM | 64QAM |  |
| MCS index |  | 17 | 13 | 19 |  |
| Modulation |  | 64QAM | 16QAM | 64QAM |  |
| Target Coding Rate |  | 0.43 | 0.48 | 0.50 |  |
| Number of MIMO layers |  | 2 | 2 | 2 |  |
| Number of DMRS REs |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} |  | 36 | 36 | 36 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} |  | 36 | 36 | 36 |  |
| For Slot i = 1,2,3 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| Overhead for TBS determination |  | 6 | 6 | 6 |  |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} | Bits | 51216 | 33816 | 53288 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} | Bits | 77896 | 57376 | 92200 |  |
| For Slot i = 1,2,3 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| Transport block CRC per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} | Bits | 24 | 24 | 24 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} | Bits | 24 | 24 | 24 |  |
| For Slot i = 1,2,3 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | CBs | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} | CBs | 7 | 5 | 7 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} | CBs | 10 | 7 | 11 |  |
| For Slot i = 1,2,3 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| For Slots i = 5 and 85 (Note 3) | Bits | 153456 | 102304 | 153456 |  |
| For Slots i = 6 and 86 (Note 3) | Bits | 153456 | 102304 | 153456 |  |
| For Slot i, if mod(i, 5) = 3 for i from {7,…, 94,87, …, 159} | Bits | 125136 | 72864 | 109296 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {7,…,84,87,…,159} | Bits | 182160 | 121440 | 182160 |  |
| For Slot i = 1,2,3 | Bits | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| Max. Throughput averaged over 2 frames | Mbps | 441.601 | 319.213 | 511.326 |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Binary Channel Bits are calculated under assumption of 52 PRBs TRS allocation when the number of allocated resource blocks are more than 52. Note 4: SS/PBCH block is transmitted in slot #1, slot #2 and slot #3 with periodicity 20ms |  |  |  |  |  |