# 3GPP TSG-RAN WG4 Meeting #108	R4-2313922

# Toulouse, France, August 21 – August 25 , 2023

Title: 	WF on NCR_MT demodulation requirements

Agenda Item:	8.28.6

Source: 	ZTE Corporation

Document for:	Approval

Sub-topic 1-1 PDSCH requirements

Issue 1-1-1: Whether to define 5MHz/15kHz for PDSCH FR1 requirements

- Proposals

  - Option 1: Define new FR1 requirements for 5 MHz/15 KHz. (Nokia)

  - Option 2: No. (ZTE, HW)

- Tentative Agreements:

    - No need to define 5MHz/15KHz requirements.

Issue 1-1-2: MCS for PDSCH FR2 requirements

- Proposals

  - Option 1: MCS 4. (HW)

- Tentative Agreements:

  - MCS 4 is for PDSCH FR2.

  - QPSK and 16QAM for PDSCH FR1.

Issue 1-1-3: Test scope for PDSCH FR1

- Proposals

  - Option 1: 8 new simulation is needed.(HW, ZTE)

| Case number | Bandwidth (MHz) | SCS (kHz) | MCS | Propagation condition | Antenna configuration | Test metric |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 10 | 15 | 4 | TDLA30-10 | 2x2 | 70% max throughput 1% BLER |
| 2 | 10 | 15 | 4 | TDLA30-10 | 2x4 | 70% max throughput 1% BLER |
| 3 | 10 | 15 | 13 | TDLA30-10 | 2x2 | 70% max throughput 1% BLER |
| 4 | 10 | 15 | 13 | TDLA30-10 | 2x4 | 70% max throughput 1% BLER |
| 5 | 40 | 30 | 4 | TDLA30-10 | 2x2 | 70% max throughput 1% BLER |
| 6 | 40 | 30 | 4 | TDLA30-10 | 2x4 | 70% max throughput 1% BLER |
| 7 | 40 | 30 | 13 | TDLA30-10 | 2x2 | 70% max throughput 1% BLER |
| 8 | 40 | 30 | 13 | TDLA30-10 | 2x4 | 70% max throughput 1% BLER |

- Recommended WF

  - Please check whether Option 1 is needed for PDSCH FR1 FDD and TDD requirements.

Issue 1-1-4: Test scope for PDSCH FR2

- Proposals

  - Option 1: 2 new simulation is needed.(HW, ZTE)

| Case number | Bandwidth (MHz) | SCS (kHz) | Propagation condition | Antenna configuration | Test metric |
| --- | --- | --- | --- | --- | --- |
| 1 | 100 | 120 | TDLA30-75 | 2x2 | 70% max throughput 1% BLER |
| 2 | 100 | 120 | TDLA30-75 | 2x4 | 70% max throughput 1% BLER |

- Tentative Agreements:

  - Option 1 can be agreed.

Sub-topic 1-2 PDCCH requirements

Issue 1-2-1: Whether to define new requirements on PDCCH for signaling of Access link beam change indication

- Proposals

  - Option 1: Yes. (Nokia)

      - Option 1A: RAN4 shall adapt test parameters for NCR PDCCH requirements following DCI format 5_0/2_8 at least to adapt the max payload size accordingly (i.e., 128 bits).

  - Option 2: No. (HW)

      - Option 2A: Reuse same DCI type from IAB-MT and legacy UE requirements for NCR PDCCH requirements.

- Recommended WF

  - To be discussed

Issue 1-2-2:  Test scope for PDCCH FR1

- Proposals

  - Option 1: 1 new simulation is needed. (HW, ZTE)

| Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Propagation Condition | Antenna configuration and correlation Matrix |
| --- | --- | --- | --- | --- | --- |
| 10 | 48 | 1 | 8 | TDLA30-10 | 2x4 Low |

- Recommended WF

  - Please whether Option1 is needed for PDCCH FR1 FDD requirements.

  - And if RAN4 agrees to define new DCI requirements, please check new test scope for PDCCH FR1.

Issue 1-2-3:  Test scope for PDCCH FR2

- Proposals

  - Option 1: No new simulation is needed. (HW, ZTE)

- Recommended WF

  - Please check whether Option 1 can be considered.

  - And if RAN4 agrees to define new DCI requirements, please check new test scope for PDCCH FR2.

Sub-topic 1-3 CSI requirements

Issue 1-3-1: Whether to define PMI requirements

- Proposals

  - Option 1: Yes. (ZTE, HW)

      - Testing of performance requirements for PMI reporting is optional. (HW)

  - Option 2: No. (Nokia)

- Recommended WF

  - To be discussed

Issue 1-3-2:  Test scope for CQI requirements

- Proposals

  - Option 1: No new simulation is needed for FR1 and FR2. (HW)

- Recommended WF

  - No new simulation is needed FR1 and FR2.

Issue 1-3-3:  Test scope for PMI requirements (if necessary)

- Proposals

  - Option 1: No new simulation is needed for FR1 and FR2. (HW)

- Recommended WF

  - Waiting for the conclusion of Issue 1-3-1.