# 3GPP TSG-RAN WG4 Meeting #108	R4-2313923

# Toulouse, France, August 21 – August 25 , 2023

Title: 	WF on NCR_MT demodulation requirements

Agenda Item:	8.28.6

Source: 	ZTE Corporation

Document for:	Approval

Sub-topic 1-1 PDSCH requirements

Issue 1-1-1: Whether to define 5MHz/15kHz for PDSCH FR1 requirements

- Agreements:

    - No need to define 5MHz/15KHz requirements for PDSCH FR1.

Issue 1-1-2: MCS for PDSCH FR2 requirements

- Agreements:

    - MCS 4 is for PDSCH FR2.

    - QPSK and 16QAM for PDSCH FR1.

Issue 1-1-3: Test scope for PDSCH FR1

- Agreements:

| Case number | Bandwidth (MHz) | SCS (kHz) | MCS | Propagation condition | Antenna configuration | Test metric |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 10 | 15 | 4 | TDLA30-10 | 2x2 | 70% max throughput 1% BLER |
| 2 | 10 | 15 | 4 | TDLA30-10 | 2x4 | 70% max throughput 1% BLER |
| 3 | 10 | 15 | 13 | TDLA30-10 | 2x2 | 70% max throughput 1% BLER |
| 4 | 10 | 15 | 13 | TDLA30-10 | 2x4 | 70% max throughput 1% BLER |
| 5 | 40 | 30 | 4 | TDLA30-10 | 2x2 | 70% max throughput 1% BLER |
| 6 | 40 | 30 | 4 | TDLA30-10 | 2x4 | 70% max throughput 1% BLER |
| 7 | 40 | 30 | 13 | TDLA30-10 | 2x2 | 70% max throughput 1% BLER |
| 8 | 40 | 30 | 13 | TDLA30-10 | 2x4 | 70% max throughput 1% BLER |

Issue 1-1-4: Test scope for PDSCH FR2

- Agreements:

| Case number | Bandwidth (MHz) | SCS (kHz) | Propagation condition | Antenna configuration | Test metric |
| --- | --- | --- | --- | --- | --- |
| 1 | 100 | 120 | TDLA30-75 | 2x2 | 70% max throughput 1% BLER |

Sub-topic 1-2 PDCCH requirements

Issue 1-2-1: Whether to define new requirements on PDCCH for signaling of Access link beam change indication

- Agreements:

    - FFS whether new PDCCH requirements needed or not for DCI format 5_0/2_8 with large payload size.

Issue 1-2-2:  Test scope for PDCCH FR1

- Agreements:

    - Following table used for simulation purpose on the requirements with existing DCI if introduced.

| Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Propagation Condition | Antenna configuration and correlation Matrix |
| --- | --- | --- | --- | --- | --- |
| 10 | 48 | 1 | 8 | TDLA30-10 | 2x4 Low |

Issue 1-2-3:  Test scope for PDCCH FR2

- Agreements:

    - No new simulation for existing DCI , FFS for new DCI if introduced.

Sub-topic 1-3 CSI requirements

Issue 1-3-1: Whether to define PMI requirements

- Agreements:

    - FFS for whether to define PMI requirements.

Issue 1-3-2:  Test scope for CQI requirements

- Agreements:

    - Reusing existing static CQI tests cases, no additional simulation needed.