3GPP TSG-RAN WG4 Meeting #108	R4-2313945

Toulouse, France, August 21 – August 25, 2023

Source:	China Telecom

Title:	Phase I simulation results for the advanced receiver for MU-MIMO

Agenda Item:	8.18.1.2

Document for:	Information

- Introduction

RAN4 #107 meeting we discuss the advanced receiver to cancel inter-user interference for MU-MIMO within the NR_demod_enh3-Perf WI. The WF is approved in [1] as the discussion outcome.

In this paper, our simulation results are provided based on the agreed assumptions in [1-3].

- Simulation Results

| Case Number (Note 1) | Co-scheduled UE number | Rank for the target UE | Rank for the Co-scheduled UE | MCS for the target UE (MCS Table 1) | Modulation order for the co-scheduled UE | MIMO | Channel model | Precoder selection for the Co-scheduled UE | FDRA of the Co-UE | China Telecom |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | R-ML | E-IRC | IRC (baseline) | Gain of R-ML | Gain of E-IRC |
| 1 | 1 | 1 | 1 | 4 | QPSK | 2Tx 2Rx ULA medium | TDLC300-100 | random | Full CHBW allocation (52PRBs) | 5.2 | 6.3 | 7.5 | 2.3 | 1.2 |
| 2 |  |  |  | 13 |  |  |  |  |  | 18.1 | 20.7 | 25.5 | 7.4 | 4.8 |
| 3 |  | 2 | 2 |  | 64QAM | 4Tx 4Rx ULA Low | TDLA30-10 | orthogonal |  | 12.5 | 12.9 | 13.5 | 1 | 0.6 |
| 4 |  |  |  |  | 16QAM |  |  |  |  | 12.3 | 13.1 | 13.4 | 0.9 | 0.3 |
| 5 |  |  |  |  | QPSK |  |  |  |  | 11.3 | 12.8 | 13.5 | 2.2 | 0.7 |
| 6 |  |  |  | 17 | 64QAM |  |  |  |  | 17.2 | 17.3 | 17.9 | 0.7 | 0.6 |
| 7 |  |  |  |  | 16QAM |  |  |  |  | 17 | 17.2 | 17.7 | 0.7 | 0.5 |
| 8 |  |  |  |  | QPSK |  |  |  |  | 13.7 | 17.3 | 17.8 | 4.1 | 0.5 |
| 9 |  | 1 | 1 | 13 | QPSK | 2Tx 2Rx ULA medium | TDLC300-100 | random | Full CHBW allocation (52PRBs) | 18.6 | 20.6 | 25.6 | 7 | 5 |
| 10 |  |  |  |  |  |  |  |  | Partial CHBW allocation (0~25 PRBs) | 11.2 | 11.6 | 13.6 | 2.4 | 2 |
| 11 |  | 2 | 2 |  | 64QAM | 4Tx 4Rx ULA Low | TDLA30-10 | orthogonal | Full CHBW allocation (52PRBs) | 12.4 | 12.8 | 13.5 | 1.1 | 0.7 |
| 12 |  |  |  |  |  |  |  |  | Partial CHBW allocation (0~25 PRBs) | 8 | 8.5 | 9.2 | 1.2 | 0.7 |
| 13 |  | 1 | 1 | 13 | QPSK | 2Tx 2Rx ULA medium | TDLC300-100 | random | Full CHBW allocation (52PRBs) | 18.6 | 20.6 | 25.6 | 7 | 5 |
| 14 |  | 2 | 2 | 17 | 16QAM | 4Tx 4Rx ULA Low | TDLA30-10 | orthogonal |  | 17 | 17.3 | 17.7 | 0.7 | 0.4 |
| 16 (Companies are encouraged to bring simulation results) | 2 | 1 | 1 for each Co-UE | 13 | Co-UE1: QPSK Co-UE2: 16QAM | 2Tx 2Rx ULA medium | TDLC300-100 | random | Co-UE1: 0~25 PRBs Co-UE2: 26~51 PRBs | 20.1 | 20.4 | 26.2 | 6.1 | 5.8 |

- References

- [1] R4-2305914, WF on advanced receiver for MU-MIMO scenario, China Telecom, RAN4 #106bis-e, April, 2023

- [2] R4-2302929, WF on advanced receiver for MU-MIMO scenario, China Telecom, RAN4 #106, March, 2023

- [3] R4-2309892, WF on advanced receiver for MU-MIMO scenario, China Telecom, RAN4 #107, May, 2023