3GPP TSG-RAN WG4 Meeting #108	R4-2313946

Toulouse, France, August 21 – August 25, 2023

Title: 	Way Forward for 256 QAM BS Demod

Agenda Item:	8.6.4.1

Source: 	Nokia, Nokia Shanghai Bell

Document for:	Approval

# 1 General

### Sub-topic 1-1: PUSCH Requirements

Issue 1-1: PUSCH Requirements

Agreement: The demodulation performance requirements for UL 256QAM will be defined only for PUSCH

### Sub-topic 1-2: Bands for FR2 256 QAM

Issue 1-2: Bands for FR2 256 QAM

Agreement: Introduce only one FR2-1 PUSCH with 256QAM demodulation requirements up to 39 GHz (aligned with RF).

Define one set of requirements for frequency range of up to 39GHz, carrier frequency of 39GHz shall be used for the requirements definition.

Interested companies who would like to evaluate the performance for carrier frequency 30GHz compared to 39GHz, are invited to submit simulations for RAN4#108-bis but this is not mandatory.

Issue 1-3: BS Declaration

Agreement: Align with FR1: The PUSCH requirement with FR2-1 UL 256QAM is only applied for BS declared to support it. Introduce the BS declaration for FR2-1 UL 256QAM

# 2 PUSCH requirements

### Sub-topic 2-1: EVM Impact

Issue 2-1: EVM Impact

Agreement: The impact of EVM can be considered in impairment results, and interested companies are encouraged to provide ideal simulations with and without TxEVM for alignment at RAN4#108bis, TxEVM value to be 3.5%.

Note: Test requirement parameters do not capture TxEVM and TEs don’t add additional TxEVM during test.

### Sub-topic 2-2: SNR Limit

Issue 2-2: SNR Limit

RAN4 will attempt to keep simulation parameters to reach an SNR operating point less than 20dB, but will await simulation results from interested companies at RAN4#108-bis.

Request input from TE vendor on maximum testable SNR at RAN4#108bis

### Sub-topic 2-3: Phase Noise Model

Issue 2-3: Phase Noise Model

Interested companies are encouraged to bring simulations to RAN4#108-bis with and without phase noise model, and provide information of phase noise model, with PT-RS enabled and disable.

- Alignment to happen without phase model (therefore with priority)

- Companies encouraged to deliver simulation results for some cases of Phase Noise model, with PT-RS enabled and disabled.

### Sub-topic 2-4: Channel

Issue 2-4: Channel

The following candidate channel models can be considered for simulation at RAN4 #108 bis and down selected at later meetings for requirements.

- TDLA 30-35

- TDLD 30-35

### Sub-topic 2-5: Rank

Issue 2-5: Rank

Agreement: Rank 1

### Sub-topic 2-6: Carrier BW and SCS

Issue 2-6: Carrier BW for 60 kHz SCS and Issue 2-7: Carrier BW for 120 kHz SCS

Interested companies are encouraged to bring simulation on the following config: 60 kHz SCS/50 MHz, 120 kHz SCS/50 MHz, 100 and 200 MHz, discussion for further downselection based on simulation results at for RAN4#108-bis.

### Sub-topic 2-7: DMRS

Issue 2-8: Additional DMRS

Interested companies to provide simulation results with single (1+0) and additional (1+1) DMRS for RAN4#108-bis.

Issue 2-9: DMRS Mapping Type

Agreement: Mapping Type B

### Sub-topic 2-8: PTRS

Issue 2-10: PTRS

Interested companies to provide simulation results with both enabled and disable for RAN4#108-bis.

Configuration: K_PTRS: 2 and L_PTRS =1

### Sub-topic 2-9: Waveform Type

Issue 2.11: Waveform Type

Agreement: CP-OFDM

### Sub-topic 2-10: MCS

Issue 2-12: MCS

RAN4 considers the following candidate MCS before down selection for requirements definition at RAN4#108 bis (all in Table 2): MCS 20, 21 and 22 with a priority on MCS 20

### Sub-topic 2-11: Testing Metric

Issue 2-13: Testing Metric

Agreement: 70% of Max Throughput

### Sub-topic 2-12: Frequency Domain Resource Allocation

Issue 2-14: Frequency Domain Resource Allocation

Agreement: Full RB allocation of the applicable BW

### Sub-topic 2-13: Frequency Hopping

Issue 2-15: Frequency Hopping

Agreement: Frequency hopping disabled

### Sub-topic 2-14: HARQ Transmissions

Issue 2-16: Number of HARQ Transmissions

Agreement: 4

### Sub-topic 2-15: TPMI Index

Agreement: Not applicable, due to Rank 1

### Sub-topic 2-16: Code Block Group

Issue 2-18: Code Block Group

Agreement: disabled

### Sub-topic 2-17: Limited buffer rate matching

Issue 2-19: Limited buffer rate matching

Agreement: disabled

# 3 Work Plan

### Sub-topic 3-1: Work Plan

Issue 3-1: Work Plan

- Proposals

  - Option 1:

| RAN4#108: |  |
| --- | --- |
|  | Discussion and agreement on work plan. Discussion on performance requirements scope. Initial discussion on simulation assumptions |
| RAN4#108-bis |  |
|  | Discussions on performance requirements scope. Discussions on simulation assumptions. Discussions on work split. Initial round of simulation results collection and alignment. |
| RAN4#109 |  |
|  | Finalise discussions on performance requirements scope. Finalise discussions on simulation assumptions. Second round of simulation results collection and alignment. Initial draft CRs. |
| RAN4#110 |  |
|  | Final round of simulation results collection and alignment. CRs submitted. |

Agreement: Option 1

## A.4 Reference

[1] R4-2314257, Topic summary for [108][321] NR_RF_FR2_req_Ph3_Demod, Nokia, Nokia Shanghai Bell, RAN WG4 #108