3GPP TSG-RAN WG4 Meeting # 108	R4-2313960

Toulouse, France, August 21 – August 25, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.181 | CR | 0005 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TS 38.181, Correction on out-of-band emissions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Perf |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The agreement in R4-2302864 (RAN4#106) about removing OBUE concept for TS 38.108 is missing in TS 38.181. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the corresponding conformance test spec from clause 6 to clause 9 and annex C.1 according the agreed CR R4-2302864(RAN4#106) for TS 38.108. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Out-of-band emssions requirment would be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.1, 6.6.4, 6.6.4.1, 6.6.4.5, 6.6.5.1, 6.6.5.4.1, 9.7.1, 9.7.4, 9.7.4.1, 9.7.4.5, 9.7.5.1, 9.7.5.2.4.1, C.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | It is revised from R4-2311599 |  |  |  |  |  |  |  |  |

<Start of Change 1>

6.6	Unwanted emissions

6.6.1	General

Unwanted emissions consist of out-of-band emissions and spurious emissions according to ITU definitions [4]. In ITU terminology, out of band emissions are unwanted emissions immediately outside the SAN channel bandwidth resulting from the modulation process and non-linearity in the transmitter but excluding spurious emissions. Spurious emissions are emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emission, intermodulation products and frequency conversion products, but exclude out of band emissions.

The out-of-band emissions requirement for the SAN transmitter is specified both in terms of Adjacent Channel Leakage power Ratio (ACLR) and out-of-band emissions (OOBE).

Table 6.6.1-1: Void

There is in addition a requirement for occupied bandwidth.

<End of Change 1>

<Start of Change 2>

6.6.4	Out-of-band emissions

6.6.4.1	Definition and applicability

Unless otherwise stated, the out-of-band emission (OOBE) limits for SAN in FR1 are defined from BWSAN channel edge up to frequencies separated from the BWSAN channel edge by 200% of the necessary bandwidth.

The requirements shall apply whatever the type of transmitter considered and for all transmission modes foreseen by the manufacturer’s specification.

Basic limits are specified in the tables below, where:

-	f is the separation between the BWSAN channel edge frequency and the nominal -3dB point of the measuring filter closest to the carrier frequency.

-	f_offset is the separation between the channel edge frequency and the centre of the measuring filter.

-	PSDChannel [dBm/4kHz] represents the Power Spectral Density of the channel(s) operating in BWSAN.

-	BWSAN [MHz] is the considered SAN transponder bandwidth.

-	ΔSat_ClassdB is the SAN class parameter in dB identified to characterize different SAN classes.

<End of Change 2>

<Start of Change 3>

6.6.4.5	Test requirements

For SAN operating in Bands n256, n255, the out-of-band emissions (OOBE) requirements for GEO and LEO classes are defined as described in table 6.6.4.5-1, in line with Annex 5 of ITU recommendation SM.1541-6 [6].

Table 6.6.4.5-1: SAN LEO and GEO out-of-band emission limits

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (dBm) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 2× BWChannel | 0.002 MHz f_offset < 2× BWChannel + 0.002 MHz | maxSE limit, PSDChannel – ΔSat_ClassdB-40×log10 f_offset-0.002BWChannel×2+1dBm | 4 kHz |
| NOTE 1: PSDChannel = Prated,c,sys – 10log10(BWChannel) – 24, unit dBm/4kHz. NOTE 2: SE limit is spurious emission limit specified in spurious emission clause 6.6.5. NOTE 3: PSD attenuation as in ITU-R SM.1541-6 [6], Annex 5 OoB domain emission limits for space services. NOTE 4: ΔSat_ClassdB=0 dB for GEO class and ΔSat_ClassdB=3 dB for LEO class. |  |  |  |

<End of Change 3>

<Start of Change 4>

9.7	OTA unwanted emissions

9.7.1	General

Unwanted emissions consist of so-called out-of-band emissions and spurious emissions according to ITU definitions ITU-R SM.329 [4]. In ITU terminology, out of band emissions are unwanted emissions immediately outside the SAN channel bandwidth resulting from the modulation process and non-linearity in the transmitter but excluding spurious emissions. Spurious emissions are emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emission, intermodulation products and frequency conversion products, but exclude out of band emissions.

The OTA out-of-band emissions requirement for the SAN type 1-O is specified both in terms of Adjacent Channel Leakage power Ratio (ACLR) and out-of-band emissions (OOBE).

Table 9.7.1-1: Void

The unwanted emission requirements are applied per cell for all the configurations.  Requirements for OTA unwanted emissions are captured using TRP, or directional requirements as described per requirement.

For the OTA TRP measurement procedures, refer to TS 38.141-2 [18], annex I.

There is in addition a requirement for occupied bandwidth.

<End of Change 4>

<Start of Change 5>

9.7.4	OTA out-of-band emissions

9.7.4.1	Definition and applicability

The OTA limits for out-of-band emissions are specified as TRP per RIB unless otherwise stated.

For a RIB operating in multi-carrier, the requirements apply to SAN channel bandwidths of the outermost carrier.

<End of Change 5>

<Start of Change 6>

9.7.4.5	Test requirements

The OTA out-of-band emissions requirement for SAN type 1-O shall not exceed each applicable limit in 9.7.4.2 of TS 38.108[2].

The requirements shall apply whatever the type of transmitter considered and for all transmission modes foreseen by the manufacturer's specification. For a RIB operating in multi-carrier, the requirements apply to SAN channel bandwidths of the outermost carrier for the frequency ranges defined in clause 6.6.4.1.

For a multi-carrier single-band RIB the definitions above apply to the lower edge of the carrier transmitted at the lowest carrier frequency and the upper edge of the carrier transmitted at the highest carrier frequency within a specified frequency band.

9.7.5	OTA transmitter spurious emissions

9.7.5.1	General

Unless otherwise stated, all requirements are measured as mean power.

The OTA transmitter spurious emissions limits are specified as TRP per RIB, unless otherwise stated.

The OTA transmitter spurious emission limits for FR1 shall apply from 30 MHz to 11 GHz, excluding the SAN transponder bandwidth BWSAN and the frequency range where the out-of-band emissions apply.

The lower limit and upper limit are as per ITU-R recommendation SM.329, Table 1: For systems operating within 600 MHz and 5.2 GHz, the lower limit is 30 MHz and the upper limit is the 5th harmonic of the higher frequency.

The lower limit of 30 MHz can be replaced as per ITU-R SM.329-12: Systems having an integral antenna incorporating a waveguide section, or with an antenna connection in such form, and of unperturbed length equal to at least twice the cut-off wavelength, do not require spurious domain emission measurements below 0.7 times the waveguide cut-off frequency.

The requirements shall apply whatever the type of transmitter considered (single carrier or multi-carrier). It applies for all transmission modes foreseen by the manufacturer's specification.

SAN type 1-O requirements consists of OTA transmitter spurious emission requirements based on TRP.

<End of Change 6>

<Start of Change 7>

C.1	Measurement of transmitter

Table C.1-1: Derivation of test requirements for conducted transmitter tests

| Test | Minimum requirement in TS 38.108 [2] | Test Tolerance (TT) | Test requirement in the present document |
| --- | --- | --- | --- |
| 6.2 SAN output power | clause 6.2 | Normal and extreme conditions: 0.7 dB | Formula: Upper limit + TT, Lower limit - TT |
| 6.3 Output power dynamics | clause 6.3 | 0.4 dB | Formula: Total power dynamic range – TT (dB) |
| 6.5.1 Frequency error | clause 6.5.1 | 12 Hz | Formula: Frequency Error limit + TT |
| 6.5.2 Modulation quality (EVM) | clause 6.5.2 | 1% | Formula: EVM limit + TT |
| 6.6.2 Occupied bandwidth | clause 6.6.2 | 0 Hz | Formula: Minimum Requirement + TT |
| 6.6.3 Adjacent Channel Leakage Power Ratio (ACLR) | clause 6.6.3 | ACLR: BW ≤ 20MHz: 0.8dB | Formula: ACLR Minimum Requirement - TT |
| 6.6.4 Out-of-band emissions | clause 6.6.4 | 0dB | Formula: Minimum Requirement + TT |
| 6.6.5 Transmitter spurious emissions | clause 6.6.5 | 0dB | Formula: Minimum Requirement + TT |
| NOTE 1: TT values are applicable for normal condition unless otherwise stated. |  |  |  |

Table C.1-2: Derivation of test requirements for OTA transmitter tests

| Test | Minimum requirement in TS 38.108 [2] | Test Tolerance (TTOTA) | Test requirement in the present document |
| --- | --- | --- | --- |
| 9.2 Radiated transmit power | clause 9.2 | Normal conditions: 1.1 dB Extreme conditions: 2.5 dB | Formula: Upper limit + TT, Lower limit – TT |
| 9.3 OTA SAN output power | clause 9.3 | 1.4 dB | Formula: Upper limit + TT, Lower limit – TT |
| 9.4 OTA output power dynamics | clause 9.4 | 0.4 dB | Formula: Total power dynamic range – TT |
| 9.6.1 OTA frequency Error | clause 9.6.1 | 12 Hz | Formula: Frequency Error limit + TT |
| 9.6.2 OTA Modulation quality (EVM) | clause 9.6.2 | 1% | Formula: EVM limit + TT |
| 9.7.2 OTA occupied bandwidth | clause 9.7.2 | 0 Hz | Formula: Minimum Requirement + TT |
| 9.7.3 OTA Adjacent Channel Leakage Power Ratio (ACLR) | clause 9.7.3 | Relative:1 dB | Formula: Relative limit - TT |
| 9.7.4 OTA out-of-band emissions | clause 9.7.4 | 0 dB | Formula: Minimum Requirement + TT |
| 9.7.5.2 General transmitter spurious emissions | clause 9.7.5.2.2 | 0 dB | Formula: Minimum Requirement + TT |
| NOTE: TT values are applicable for normal condition unless otherwise stated. |  |  |  |

<End of Change 7>