3GPP TSG-RAN WG4 Meeting # 108                                                       R4- 2311637

# Toulouse, FR, August 21 – August 25, 2023

Title: 	TP for TR 38.858:“Feasibility of FR1 Local Area BS aspects”

Source: 	CATT

Agenda item:	8.19.2.2.1

Document for:	Approval

Background

This TP provides the TP for FR1 LA BS feasibility from editor point of view. It was agreed in [1] that WA analysis can be broke down to capture several companies’ analysis. Considering the TP inputs for LA may be less than WA. We think the analysis may be combined if there’s not much difference from companies.

The following agreement was agreed in RAN4#107, but is missing in the WF [3],

Issue 1-1-5: MR and LA-BS SBFD with 1dB sensitivity degradation

- Agreement:

  - For implementation feasibility study, RAN4 confirm FR1 MR and LA SBFD gNB with 1dB sensitivity degradation due to self-interference is achievable.

    - FFS on co-site inter-sector interference impact.

So for LA SBFD BS, the conclusion is feasible.

Reference

[1] R4-2306006, “WF on implementation feasibility of SBFD”, RAN4#106bis-e, Samsung

[2] R4-2309790, “WF for RF feasibility study and BS requirements impact on SBFD operation”, RAN4#107, Samsung

[3] R4-2307388, TP for TR 38.858:“Feasibility of FR1 Local Area BS aspects”, RAN4#107, CATT

TP for TR 38.858: Feasibility of FR1 local area BS aspects

9.4	Feasibility of FR1 local area BS aspects

9.4.1	Self-interference analysis

Editor's note: This section captures the typical assumption based on which the RSIC capability is derived and analysis results

### 9.4.1.1	Summary table for self-interference analysis

Table 9.4.1.1-1 summarizes the self-interference analysis results from companies.

Table 9.4.1.1-1: SI feasibility study for FR1 Local Area BS

| FR1 |  |  |  | Ericsson (preliminary) |  | ZTE | CATT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BS class |  |  |  | Local Area BS (3GPP minimum) | Local Area BS (Realistic RX) | LA | LA |
| BS TX Power dBPTX = ① dBm |  |  |  | 24 | 24 | 23 | 24 |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | 45 | 45 | 51 | 45 |
|  |  | Frequency isolation techniques used |  | Digital filtering, CFR, DPD |  | DPD utilized | CFR |
|  | Spatial isolation | Spatial isolation capability dBαSI-spatial = ③ dBc |  | 70 | 70 | 60 | 70 |
|  |  | Spatial isolation techniques used |  | Physical distance, isolation structures |  | TX/RX panel separation | TX/RX panel separation, isolation structures, isolation material, cross polarization |
|  | TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  | 0 | 0 | 0 | 0 |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  | TX beam nulling not assumed due to array size |  | N/A | TX Beam nulling is not assumed due to the array size |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  | -91 | -91 | -101 | -104.01 |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  | 0 | 0 | 20 | 0 |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  | 0 | 0 | 0 | 0 |
|  |  | RF IC techniques and other tech. (before LNA) |  | Analogue IC could be considered for this case, but is restrictive on pre-coding and multi-carrier. Digital IC has instead been assumed. |  | subband filtering |  |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | 0 | 0 |  | 0 |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  | -46 | -46 | -50 | -46 |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | xxx dBc |  | 60 | 55 |
|  |  | Frequency isolation techniques |  | Digital IC of TX. The impact of scattering / reflection in the environment has not been considered. It is assumed that RX ACS is very large due to time alignment and achieving orthogonality between the TX and RX signals in the digital domain. |  | Digital Filtering | Digital filter for ACS |
|  |  | RX IMD | Rx IIP3 capability (dBm) | -24.6 | -14 | -5 | -16 |
|  |  |  | Rx IM3 contribution (dBm) | -88.8 | -110 | -140 | -106 |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | No other significant impacts other than those mentioned above |  | Marginal | No other significant impacts |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | -88.8 | -110 | -110 | -101 |
|  | RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  | 0-3 dBc RX beam nulling is in effect part of the digital baseband combining. Digital baseband combining may improve self-interference suppression, effectively at the cost of suppression of other interferers and RX beamforming gain. Reference scenarios are needed to assess the overall potential from RX baseband combining. |  | 0 | 0 |
|  | RX sensitivity degradation caused by RX beam nulling |  |  |  |  | N/A |  |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | 15 dBc on transmitter |  | 20 | 10 |
| Overall RSIC capability dBRSIC (Note 1) |  |  |  | 112 dBc | 128 dBc | 130.5 | 122.33 |
| Noise floor ⑩dBm |  |  |  | -87 dBm/CBW | -87 dBm/CBW | -91.0 | -87.99 |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | -93 dBm | -93 dBm | -97.0 | -93.99 |
| Required RSIC budget (①-⑪dBc) |  |  |  | 117 dBc | 117 dBc | 127.0 | 117.99 |
| SBFD configuration |  |  |  | 40-20-40 |  | 40-20-40 | 40-20-40 |
| Guardband assumption (if exist) |  |  |  | 5 PRB |  |  |  |
| bandwidth over which suppression is achieved |  |  |  | <300MHz |  |  |  |
| Others |  |  |  | The conclusion does not take into account interference increase due to scattering effects, or the possibility for receiver algorihms to mitigate scattering, inter-sector and inter-site and self-interference (reference scenarios needed). |  |  |  |
| Note 1: Relevant metrics are derived from other parameters for checking purpose. Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics. |  |  |  |  |  |  |  |

### 9.4.1.2	Feasibility study on self-interference

9.4.1.2.1 Ericsson

The self-interference suppression expectations for the LA BS class are provided in table 9.4.1.1-1. Two examples are provided. The first is an LA BS with RF performance just enough to meet 3GPP minimum requirements. The second is an LS BS with improved receiver performance, but still likely sufficient for a commercially viable solution.

The table suggests that, depending on variation in the achievable spatial isolation, some degree of improved receiver and/or digital cancellation may be needed for LA SBFD. However, in general self-interference suppression appears feasible.

Spatial isolation and beam nulling

The spatial isolation for LA is more difficult to quantify, as the array size and form-factor for a LA BS may vary significantly. LA SBFD could even be operated by placing TX and RX LA basestations at some distance. 70dB isolation has been assumed, and obviously the conclusions may vary if a smaller isolation is achieved. Beam nulling is not assumed, since a small array size is assumed.

Analogue interference cancellation

Analogue interference cancellation is more feasible for a LA BS with a smaller array size. The drawback would be the need to beamform on the same way on all RB and all carriers for a LA BS. Potentially analogue IC could be considered if the isolation for a particular design would be substantially less than 70dB.

Analogue filtering

Analogue filtering is not really needed for a LA BS due to the lower power in the TX sub-bands.

Digital interference cancellation and receiver processing

Digital sampling of the TX leakage interference and subtraction at RX is feasible for a LA BS. An alternative to digital cancellation of TX leakage could be an improved PA linearization at the transmitter side. The possibility of either improved PA linearization or digital IC is captured as 15dB digital IC.

Receiver combining could also potentially mitigate some interference although with a smaller array size, the degrees of freedom with which to do so would be lower than other BS classes.

9.4.1.2.2 CATT

In SI capability analysis, the following techniques are used,

- CFR is used to improve equipment efficiency, DPD is used for high power equipment to optimize ACLR.

- Tx antennas panel and Rx antennas panel are separate, there are some isolation material between them, and cross polarization is also used.

- Digital filter is used to resolve the adjacent sub-band (i.e. TX subband) interference issue.

- Digital IC is used to reduce interference in the UL sub-bands.

From the analysis in Table 9.4.1.1-1, the following capabilities are needed,

- Frequency isolation capability of frequency isolation at TX is 45dB

- Spatial isolation is 70dB

- ACS is 55dBc

- IIP3 is -16dBm

- Digital IC is 10dB

The above capabilities except frequency isolation are improved compared with legacy LA BS, but it’s feasible from implementation point of view.

### 9.4.1.3	Conclusion

Editor's note: This section captures the conclusion for feasibility study on self-interference based on RAN4 agreement.

9.4.2	Co-channel inter-sub-band inter-site interference analysis

This section captures the typical assumption of RF requirements and analysis results

9.4.3	Summary

Editor's note: This section captures the conclusion of BS SBFD feasibility.