3GPP TSG-RAN WG4 Meeting #108                                                       	R4-2313861

# Toulouse, France, August 21 – August 25, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.941 | CR | 0049 | rev | 2 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TR 37.941: implementation of FR2-2 MU and TT derivations, Rel-17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz-Perf |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on discussion last meeting and agreements implemented in TS, this CR to TR implements FR2-2 MU and TT derivations. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Excel spreadsheets for FR2 TX and RX MU calculations were updates – all delta highlighted - All FR2 MU and TT tables updated with FR2-2 values |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Calculations of FR2-2 MU and TT values would be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | This CR is to be revised during the meeting, to remove yellow highlights from FR2 Excel spreadsheets in the final version. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------ Modified section ------------------------------

Table 9.2.3.4-2: CATR MU value derivation for EIRP accuracy measurements, Normal test conditions, FR2

| UID | Uncertainty source | Uncertainty value (dB) |  |  | Distribution of the probability | Divisor based on | ci | Standard uncertainty ui (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 24.25 < f <29.5 GHz | 37 < f <43.5 GHz | 52.6 < f ≤ 71 GHz |  | distribution shape |  | 24.25 < f <29.5 GHz | 37 < f <43.5 GHz | 52.6 < f ≤ 71 GHz |
| Stage 2: BS measurement |  |  |  |  |  |  |  |  |  |  |
| A2-1a |  | Misalignment and pointing error of BS (for EIRP) |  | 0.20 |  | 0.20 |  | 0.20 |  | Exp. normal |  | 2.00 |  |  | 1 |  | 0.10 |  | 0.10 |  | 0.10 |
| C1-1 |  | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) - high power |  | 0.50 |  | 0.70 |  |  |  | Normal |  | 1.00 |  |  | 1 |  | 0.50 |  | 0.70 |  |  |
| C1-10 |  | Uncertainty of the RF power measurement equipment (power meter, power sensor) - high power (EIRP) |  |  |  |  |  | 0.98 |  | Gaussian |  | 1 |  |  | 1 |  |  |  |  |  | 0.98 |
| A2-2a |  | Standing wave between BS and test range antenna |  | 0.03 |  | 0.03 |  | 0.21 |  | U-shaped |  | 1.41 |  |  | 1 |  | 0.02 |  | 0.02 |  | 0.15 |
| A2-3 |  | RF leakage (SGH connector terminated & test range antenna connector cable terminated) |  | 0.01 |  | 0.01 |  | 0.00 |  | Normal |  | 1.00 |  |  | 1 |  | 0.01 |  | 0.01 |  | 0.00 |
| A2-4a |  | QZ ripple experienced by BS |  | 0.40 |  | 0.40 |  | 0.40 |  | Normal |  | 1.00 |  |  | 1 |  | 0.40 |  | 0.40 |  | 0.40 |
| A2-12 |  | Frequency flatness of test system |  | 0.25 |  | 0.25 |  | 0.25 |  | Normal |  | 1.00 |  |  | 1 |  | 0.25 |  | 0.25 |  | 0.25 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |
| C1-3 |  | Uncertainty of the network analyzer |  | 0.30 |  | 0.30 |  | 0.85 |  | Normal |  | 1.00 |  |  | 1 |  | 0.30 |  | 0.30 |  | 0.85 |
| A2-5a |  | Mismatch of receiver chain between receiving antenna and measurement receiver |  | 0.43 |  | 0.57 |  | 0.57 |  | U-shaped |  | 1.41 |  |  | 1 |  | 0.30 |  | 0.40 |  | 0.40 |
| A2-6 |  | Insertion loss of receiver chain |  | 0.00 |  | 0.00 |  | 0.18 |  | Rectangular |  | 1.73 |  |  | 1 |  | 0.00 |  | 0.00 |  | 0.10 |
| A2-3 |  | RF leakage (SGH connector terminated & test range antenna connector cable terminated) |  | 0.01 |  | 0.01 |  | 0.01 |  | Normal |  | 1.00 |  |  | 1 |  | 0.01 |  | 0.01 |  | 0.01 |
| A2-7 |  | Influence of the calibration antenna feed cable |  | 0.21 |  | 0.29 |  | 0.29 |  | U-shaped |  | 1.41 |  |  | 1 |  | 0.15 |  | 0.21 |  | 0.21 |
| C1-4 |  | Uncertainty of the absolute gain of the reference antenna |  | 0.52 |  | 0.52 |  | 0.52 |  | Rectangular |  | 1.73 |  |  | 1 |  | 0.30 |  | 0.30 |  | 0.30 |
| A2-8 |  | Misalignment positioning system |  | 0.00 |  | 0.00 |  | 0.00 |  | Exp. normal |  | 2.00 |  |  | 1 |  | 0.00 |  | 0.00 |  | 0.00 |
| A2-1b |  | Misalignment and pointing error of calibration antenna (for EIRP) |  | 0.00 |  | 0.00 |  | 0.00 |  | Exp. normal |  | 2.00 |  |  | 1 |  | 0.00 |  | 0.00 |  | 0.00 |
| A2-9 |  | Rotary joints |  | 0.00 |  | 0.00 |  | 0.00 |  | U-shaped |  | 1.41 |  |  | 1 |  | 0.00 |  | 0.00 |  | 0.00 |
| A2-2b |  | Standing wave between calibration antenna and test range antenna |  | 0.09 |  | 0.09 |  | 0.09 |  | U-shaped |  | 1.41 |  |  | 1 |  | 0.06 |  | 0.06 |  | 0.06 |
| A2-4b |  | QZ ripple experienced by calibration antenna |  | 0.01 |  | 0.01 |  | 0.01 |  | Normal |  | 1.00 |  |  | 1 |  | 0.01 |  | 0.01 |  | 0.01 |
| A2-11 |  | Switching uncertainty |  | 0.10 |  | 0.10 |  | 0.43 |  | Rectangular |  | 1.73 |  |  | 1 |  | 0.06 |  | 0.06 |  | 0.25 |
| Combined standard uncertainty (1σ) (dB) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.89 |  | 1.06 |  | 1.52 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.74 |  | 2.07 |  | 2.98 |

------------------------------ Next modified section ------------------------------

Table 9.2.7-2: OTA test system specific measurement uncertainty values for the EIRP accuracy, FR2, Normal test conditions

|  | Expanded uncertainty (dB) |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | 24.25 < f < 29.5 GHz |  | 37 < f < 43.5 GHz | 43.5 < f < 48.2 GHz | 52.6 < f ≤ 71 GHz |
| Indoor Anechoic Chamber |  | - |  | - | - | - |
| Compact Antenna Test Range |  | 1.74 |  | 2.07 | - | 2.98 |
| One Dimensional Compact Range Chamber |  | - |  | - | - | - |
| Near Field Test Range |  | - |  | - | - | - |
| Plane Wave Synthesizer |  | - |  | - | - | - |
| Common maximum accepted test system uncertainty |  | 1.70 |  | 2.0 | 2.2 (NOTE) | 3.0 |
| NOTE: MU estimation for 43.5 < f < 48.2 GHz was derived based on the linear approximation (based on MU values for lower frequency ranges). MU extrapolation approach was used instead of the typical derivation of the Expanded MU based on the MU budget calculations, as in case of lower frequency ranges. |  |  |  |  |

------------------------------ Next modified section ------------------------------

Table 9.2.8-2: Test Tolerance values for the EIRP accuracy in Normal test conditions, FR2

|  | 24.25 < f < 29.5 GHz | 37 < f < 43.5 GHz | 43.5 < f < 48.2 GHz | 52.6 < f ≤ 71 GHz |
| --- | --- | --- | --- | --- |
| Test Tolerance (dB) | 1.7 | 2.0 | 2.2 | 3.0 |

------------------------------ Next modified section ------------------------------

Table 9.3.3.4-1: CATR MU value derivation for EIRP accuracy measurements in Extreme test conditions, FR2

| UID | Uncertainty source |  | Uncertainty value (dB) |  |  | Distribution of the | Divisor based | ci | Standard uncertainty ui (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 24.25 < f ≤ 29.5 GHz | 37 < f ≤ 43.5 GHz | 52.6 < f ≤ 71 GHz | probability | on distribution shape |  | 24.25 < f ≤ 29.5 GHz | 37 < f ≤ 43.5 GHz | 52.6 < f ≤ 71 GHz |
| Stage 2: BS measurement |  |  |  |  |  |  |  |  |  |  |  |
| A2-1a |  | Misalignment and pointing error of BS (for EIRP) |  |  |  | 0.20 |  |  |  |  | 0.20 |  |  | 0.20 |  |  | Exp. normal |  |  | 2.00 |  |  | 1 |  |  | 0.10 |  |  | 0.10 |  |  | 0.10 |
| C1-1 |  | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) - high power |  |  |  | 0.50 |  |  |  |  | 0.70 |  |  |  |  |  | Gaussian |  |  | 1.00 |  |  | 1 |  |  | 0.50 |  |  | 0.70 |  |  |  |
| C1-10 | Uncertainty of the RF power measurement equipment (power meter, power sensor) - high power (EIRP) |  |  |  |  |  |  |  |  | 0.98 |  |  | Gaussian |  |  | 1.00 |  |  | 1 |  |  |  |  |  |  |  |  | 0.98 |  |  |
| A2-2a |  | Standing wave between BS and test range antenna |  |  |  | 0.03 |  |  |  |  | 0.03 |  |  | 0.21 |  |  | U-shaped |  |  | 1.41 |  |  | 1 |  |  | 0.02 |  |  | 0.02 |  |  | 0.15 |
| A2-3 |  | RF leakage (SGH connector terminated & test range antenna connector cable terminated) |  |  |  | 0.01 |  |  |  |  | 0.01 |  |  | 0.00 |  |  | Gaussian |  |  | 1.00 |  |  | 1 |  |  | 0.01 |  |  | 0.01 |  |  | 0.00 |
| A2-13 |  | QZ ripple with BS (extreme test conditions) |  |  |  | 0.70 |  |  |  |  | 0.70 |  |  | 0.70 |  |  | Gaussian |  |  | 1.00 |  |  | 1 |  |  | 0.70 |  |  | 0.70 |  |  | 0.70 |
| A2-12 |  | Frequency flatness of test system |  |  |  | 0.25 |  |  |  |  | 0.25 |  |  | 0.25 |  |  | Gaussian |  |  | 1.00 |  |  | 1 |  |  | 0.25 |  |  | 0.25 |  |  | 0.25 |
| A2-15 |  | Radome loss variation |  |  |  | 0.50 |  |  |  |  | 0.50 |  |  | 0.50 |  |  | Gaussian |  |  | 1.00 |  |  | 1 |  |  | 0.50 |  |  | 0.50 |  |  | 0.50 |
| A2-14 |  | Wet radome loss variation |  |  |  | 0.90 |  |  |  |  | 0.90 |  |  | 0.90 |  |  | Gaussian |  |  | 1.00 |  |  | 1 |  |  | 0.90 |  |  | 0.90 |  |  | 0.90 |
| A2-16 |  | Change in absorber behavior |  |  |  | 0.50 |  |  |  |  | 0.50 |  |  | 0.50 |  |  | Gaussian |  |  | 1.00 |  |  | 1 |  |  | 0.50 |  |  | 0.50 |  |  | 0.50 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |  |
| C1-3 |  | Uncertainty of the network analyzer |  |  |  | 0.30 |  |  |  |  | 0.30 |  |  | 0.85 |  |  | Gaussian |  |  | 1.00 |  |  | 1 |  |  | 0.30 |  |  | 0.30 |  |  | 0.85 |
| A2-5a |  | Mismatch of receiver chain between receiving antenna and measurement receiver |  |  |  | 0.43 |  |  |  |  | 0.57 |  |  | 0.57 |  |  | U-shaped |  |  | 1.41 |  |  | 1 |  |  | 0.30 |  |  | 0.40 |  |  | 0.40 |
| A2-6 |  | Insertion loss of receiver chain |  |  |  | 0.00 |  |  |  |  | 0.00 |  |  | 0.18 |  |  | Rectangular |  |  | 1.73 |  |  | 1 |  |  | 0.00 |  |  | 0.00 |  |  | 0.10 |
| A2-3 |  | RF leakage (SGH connector terminated & test range antenna connector cable terminated) |  |  |  | 0.01 |  |  |  |  | 0.01 |  |  | 0.01 |  |  | Gaussian |  |  | 1.00 |  |  | 1 |  |  | 0.01 |  |  | 0.01 |  |  | 0.01 |
| A2-7 |  | Influence of the calibration antenna feed cable |  |  |  | 0.21 |  |  |  |  | 0.29 |  |  | 0.29 |  |  | U-shaped |  |  | 1.41 |  |  | 1 |  |  | 0.15 |  |  | 0.21 |  |  | 0.21 |
| C1-4 |  | Uncertainty of the absolute gain of the reference antenna |  |  |  | 0.52 |  |  |  |  | 0.52 |  |  | 0.52 |  |  | Rectangular |  |  | 1.73 |  |  | 1 |  |  | 0.30 |  |  | 0.30 |  |  | 0.30 |
| A2-8 |  | Misalignment positioning system |  |  |  | 0.00 |  |  |  |  | 0.00 |  |  | 0.00 |  |  | Exp. normal |  |  | 2.00 |  |  | 1 |  |  | 0.00 |  |  | 0.00 |  |  | 0.00 |
| A2-1b |  | Misalignment and pointing error of calibration antenna (for EIRP) |  |  |  | 0.00 |  |  |  |  | 0.00 |  |  | 0.00 |  |  | Exp. normal |  |  | 2.00 |  |  | 1 |  |  | 0.00 |  |  | 0.00 |  |  | 0.00 |
| A2-9 |  | Rotary joints |  |  |  | 0.00 |  |  |  |  | 0.00 |  |  | 0.00 |  |  | U-shaped |  |  | 1.41 |  |  | 1 |  |  | 0.00 |  |  | 0.00 |  |  | 0.00 |
| A2-2b |  | Standing wave between calibration antenna and test range antenna |  |  |  | 0.09 |  |  |  |  | 0.09 |  |  | 0.09 |  |  | U-shaped |  |  | 1.41 |  |  | 1 |  |  | 0.06 |  |  | 0.06 |  |  | 0.06 |
| A2-4b |  | QZ ripple experienced by calibration antenna (normal test conditions) |  |  |  | 0.01 |  |  |  |  | 0.01 |  |  | 0.01 |  |  | Gaussian |  |  | 1.00 |  |  | 1 |  |  | 0.01 |  |  | 0.01 |  |  | 0.01 |
| A2-11 |  | Switching uncertainty |  |  |  | 0.10 |  |  |  |  | 0.10 |  |  | 0.43 |  |  | Rectangular |  |  | 1.73 |  |  | 1 |  |  | 0.06 |  |  | 0.06 |  |  | 0.25 |
|  |  |  | Combined standard uncertainty (1σ) (dB) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.56 |  |  | 1.66 |  |  | 1.99 |
|  |  |  | Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.05 |  |  | 3.25 |  |  | 3.89 |

------------------------------ Next modified section ------------------------------

Table 9.3.4-2: Test system specific measurement uncertainty values for the EIRP accuracy in Extreme test conditions, FR2

|  | Expanded uncertainty ue (dB) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 24.25 < f <29.5 GHz | 37 < f ≤ 43.5 GHz | 43.5 < f < 48.2 GHz | 52.6 < f ≤ 71 GHz |
| Compact Antenna Test Range | 3.05 | 3.25 | - | 3.89 |
| Common maximum accepted test system uncertainty | 3.1 | 3.3 | 3.5 (NOTE) | 3.9 |
| NOTE: MU estimation for 43.5 < f < 48.2 GHz was derived based on the linear approximation (based on MU values for lower frequency ranges). MU extrapolation approach was used instead of the typical derivation of the Expanded MU based on the MU budget calculations, as in case of lower frequency ranges. |  |  |  |  |

------------------------------ Next modified section ------------------------------

Table 9.3.5-2: Test Tolerance values for the EIRP accuracy in Extreme test conditions, FR2

|  | 24.25 < f <29.5 GHz | 37 GHz < f ≤ 43.5 GHz | 43.5 < f < 48.2 GHz | 52.6 < f ≤ 71 GHz |
| --- | --- | --- | --- | --- |
| Test Tolerance (dB) | 3.1 | 3.3 | 3.5 | 3.9 |

------------------------------ Next modified section ------------------------------

Table 9.7.7-2: Test Tolerance values for EVM, FR2

|  | 24.25 < f < 29.5 GHz | 37 < f < 43.5 GHz | 43.5 < f < 48.2 GHz | 52.6 < f ≤ 71 GHz |
| --- | --- | --- | --- | --- |
| Test Tolerance (%) | 1 | 1 | 1 | 1 |

------------------------------ Next modified section ------------------------------

Table 9.10.2.3-1: CATR uncertainty assessment for EIRP measurements for transmitter OFF power and transmitter transient period

| UID | Uncertainty source | Uncertainty value |  |  | Distribution of the probability | Divisor based on distribution | ci | Standard uncertainty ui (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 24.25 < f ≤ 29.5 GHz | 37 < f ≤ 43.5 GHz | 52.6 < f ≤ 71 GHz |  | shape |  | 24.25 < f ≤ 29.5 GHz | 37 < f ≤ 43.5 GHz | 52.6 < f ≤ 71 GHz |
| Stage 2: BS measurement |  |  |  |  |  |  |  |  |  |  |
| A2-1a | Misalignment and pointing error of BS (for EIRP) | 0.20 | 0.20 | 0.20 | Exp. normal | 2.00 | 1 | 0.10 | 0.10 | 0.10 |
| C1-9 | Uncertainty of the RF power measurement equipment standard uncertainty σ (dB) of the absolute level for a time domain wideband measurement for FR2 | 1.25 | 1.45 | 2.61 | Gaussian | 1.00 | 1 | 1.25 | 1.45 | 2.61 |
| A2-2a | Standing wave between BS and test range antenna | 0.03 | 0.03 | 0.21 | U-shaped | 1.41 | 1 | 0.02 | 0.02 | 0.15 |
| A2-3 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) | 0.01 | 0.01 | 0.00 | Gaussian | 1.00 | 1 | 0.01 | 0.01 | 0.00 |
| A2-4a | QZ ripple experienced by BS | 0.40 | 0.40 | 0.40 | Gaussian | 1.00 | 1 | 0.40 | 0.40 | 0.40 |
| A2-12 | Frequency flatness of test system | 0.25 | 0.25 | 0.25 | Gaussian | 1.00 | 1 | 0.25 | 0.25 | 0.25 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |
| C1-3 | Uncertainty of the network analyzer |  | 0.30 |  | 0.30 |  | 0.85 |  | Gaussian |  | 1.00 |  | 1 |  | 0.30 |  | 0.30 |  | 0.85 |  |
| A2-5b | Mismatch of receiver chain for low power receiver |  | 0.72 |  | 0.72 |  | 0.72 |  | U-shaped |  | 1.41 |  | 1 |  | 0.51 |  | 0.51 |  | 0.51 |  |
| A2-6 | Insertion loss of receiver chain |  | 0.00 |  | 0.00 |  | 0.18 |  | Rectangular |  | 1.73 |  | 1 |  | 0.00 |  | 0.00 |  | 0.10 |  |
| A2-3 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) |  | 0.01 |  | 0.01 |  | 0.01 |  | Gaussian |  | 1.00 |  | 1 |  | 0.01 |  | 0.01 |  | 0.01 |  |
| A2-7 | Influence of the calibration antenna feed cable |  | 0.21 |  | 0.29 |  | 0.29 |  | U-shaped |  | 1.41 |  | 1 |  | 0.15 |  | 0.21 |  | 0.21 |  |
| C1-4 | Uncertainty of the absolute gain of the reference antenna |  | 0.52 |  | 0.52 |  | 0.52 |  | Rectangular |  | 1.73 |  | 1 |  | 0.30 |  | 0.30 |  | 0.30 |  |
| A2-8 | Misalignment positioning system |  | 0.00 |  | 0.00 |  | 0.00 |  | Exp. normal |  | 2.00 |  | 1 |  | 0.00 |  | 0.00 |  | 0.00 |  |
| A2-1b | Misalignment and pointing error of calibration antenna (for EIRP) |  | 0.00 |  | 0.00 |  | 0.00 |  | Exp. normal |  | 2.00 |  | 1 |  | 0.00 |  | 0.00 |  | 0.00 |  |
| A2-9 | Rotary joints |  | 0.00 |  | 0.00 |  | 0.00 |  | U-shaped |  | 1.41 |  | 1 |  | 0.00 |  | 0.00 |  | 0.00 |  |
| A2-2b | Standing wave between calibration antenna and test range antenna |  | 0.09 |  | 0.09 |  | 0.09 |  | U-shaped |  | 1.41 |  | 1 |  | 0.06 |  | 0.06 |  | 0.06 |  |
| A2-4b | QZ ripple experienced by calibration antenna |  | 0.01 |  | 0.01 |  | 0.01 |  | Gaussian |  | 1.00 |  | 1 |  | 0.01 |  | 0.01 |  | 0.01 |  |
| A2-11 | Switching uncertainty |  | 0.10 |  | 0.10 |  | 0.43 |  | Rectangular |  | 1.73 |  | 1 |  | 0.06 |  | 0.06 |  | 0.25 |  |
| Combined standard uncertainty (1σ) (dB) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.50 |  | 1.68 |  | 2.87 |  |
| Expanded uncertainty (1.96σ – confidence interval of 95 %) (dB) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.95 |  | 3.29 |  | 5.63 |  |

### 9.10.3	Maximum accepted test system uncertainty

The MU assessment was carried out using a CATR chamber only. However other chamber types are not precluded if suitable MU assessment is done.

Table 9.10.3-1: Maximum accepted test system uncertainty values for the EIRP accuracy, FR2

|  | 24.25 < f < 29.5 GHz | 37 < f < 43.5 GHz | 43.5 < f < 48.2 GHz | 52.6 < f ≤ 71 GHz |
| --- | --- | --- | --- | --- |
| Maximum accepted test system uncertainty (dB) | 2.9 | 3.3 | 3.6 (NOTE) | 5.6 |
| NOTE: MU estimation for 43.5 < f < 48.2 GHz was derived based on the linear approximation (based on MU values for lower frequency ranges). MU extrapolation approach was used instead of the typical derivation of the Expanded MU based on the MU budget calculations, as in case of lower frequency ranges. |  |  |  |  |

------------------------------ Next modified section ------------------------------

Table 9.10.4-1: Test Tolerance values for the EIRP accuracy, FR2

|  | 24.25 < f < 29.5 GHz | 37 < f < 43.5 GHz | 43.5 < f < 48.2 GHz | 52.6 < f ≤ 71 GHz |
| --- | --- | --- | --- | --- |
| Test Tolerance (dB) | 2.9 | 3.3 | 3.6 | 5.6 |

------------------------------ Next modified section ------------------------------

Table 10.2.3.4-1: CATR MU value derivation for OTA sensitivity measurement, FR2

| UID | Uncertainty source | Uncertainty value |  |  | Distribution of the | Divisor based on | ci | Standard uncertainty ui (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 24.25 < f < 29.5 GHz |  |  | 37 < f < 43.5 GHz |  |  | 52.6 < f ≤ 71 GHz |  |  | probability |  |  | distribution shape |  |  |  |  |  | 24.25 < f < 29.5 GHz |  |  | 37 < f < 43.5 GHz |  |  | 52.6 < f ≤ 71 GHz |
| Stage 2: BS measurement |  |  |  |  |  |  |  |  |  |  |
| B2-1a |  | Misalignment and pointing error of BS |  |  | 0.2 |  |  | 0.2 |  |  | 0.2 |  |  | Exp. normal |  |  | 2.00 |  |  | 1 |  |  | 0.10 |  |  | 0.10 |  |  | 0.10 |
| B2-2 |  | Standing wave between BS and test range antenna |  |  | 0.21 |  |  | 0.21 |  |  | 0.21 |  |  | U-shaped |  |  | 1.41 |  |  | 1 |  |  | 0.15 |  |  | 0.15 |  |  | 0.15 |
| C1-2 |  | Uncertainty of the RF signal generator |  |  | 0.9 |  |  | 0.9 |  |  |  |  |  | Gaussian |  |  | 1.00 |  |  | 1 |  |  | 0.90 |  |  | 0.90 |  |  |  |
| B2-3 |  | RF leakage & dynamic range, test range antenna cable connector terminated. |  |  | 0.01 |  |  | 0.01 |  |  | 0.01 |  |  | Gaussian |  |  | 1.00 |  |  | 1 |  |  | 0.01 |  |  | 0.01 |  |  | 0.01 |
| B2-4a |  | QZ ripple experienced by BS |  |  | 0.4 |  |  | 0.4 |  |  | 0.4 |  |  | Gaussian |  |  | 1.00 |  |  | 1 |  |  | 0.40 |  |  | 0.40 |  |  | 0.40 |
| B2-9 |  | Miscellaneous uncertainty |  |  | 0 |  |  | 0 |  |  | 0 |  |  | Gaussian |  |  | 1.00 |  |  | 1 |  |  | 0.00 |  |  | 0.00 |  |  | 0.00 |
| C1-11 | Uncertainty of the RF signal generator with power monitoring and controling by power sensor |  |  |  |  |  | 0.98 |  |  | Gaussian |  |  | 1.00 |  |  | 1 |  |  |  |  |  |  |  |  | 0.98 |  |  |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |
| C1-3 |  | Uncertainty of the network analyzer |  |  | 0.3 |  |  | 0.3 |  |  | 0.85 |  |  | Gaussian |  |  | 1.00 |  |  | 1 |  |  | 0.30 |  |  | 0.30 |  |  | 0.85 |
| B2-5 |  | Mismatch of transmit chain (i.e. between transmitting measurement antenna and BS) |  |  | 0.43 |  |  | 0.57 |  |  | 0.85 |  |  | U-shaped |  |  | 1.41 |  |  | 1 |  |  | 0.30 |  |  | 0.40 |  |  | 0.60 |
| B2-6 |  | Insertion loss of transmitter chain |  |  | 0.12 |  |  | 0.12 |  |  | 0.12 |  |  | Rectangular |  |  | 1.73 |  |  | 1 |  |  | 0.07 |  |  | 0.07 |  |  | 0.07 |
| B2-7 |  | RF leakage (SGH connector terminated & test range antenna connector cable terminated) |  |  | 0.01 |  |  | 0.01 |  |  | 0.01 |  |  | Gaussian |  |  | 1.00 |  |  | 1 |  |  | 0.01 |  |  | 0.01 |  |  | 0.01 |
| B2-8 |  | Influence of the calibration antenna feed cable |  |  | 0.21 |  |  | 0.29 |  |  | 0.29 |  |  | U-shaped |  |  | 1.41 |  |  | 1 |  |  | 0.15 |  |  | 0.21 |  |  | 0.21 |
| C1-4 |  | Uncertainty of the absolute gain of the reference antenna |  |  | 0.52 |  |  | 0.52 |  |  | 0.52 |  |  | Rectangular |  |  | 1.73 |  |  | 1 |  |  | 0.30 |  |  | 0.30 |  |  | 0.30 |
| B2-11 |  | Misalignment positioning system |  |  | 0 |  |  | 0 |  |  | 0 |  |  | Exp. normal |  |  | 2.00 |  |  | 1 |  |  | 0.00 |  |  | 0.00 |  |  | 0.00 |
| B2-4b |  | QZ ripple experienced by calibration antenna |  |  | 0.1 |  |  | 0.1 |  |  | 0.1 |  |  | Gaussian |  |  | 1.00 |  |  | 1 |  |  | 0.10 |  |  | 0.10 |  |  | 0.10 |
| B2-10 |  | Rotary joints |  |  | 0 |  |  | 0 |  |  | 0 |  |  | U-shaped |  |  | 1.41 |  |  | 1 |  |  | 0.00 |  |  | 0.00 |  |  | 0.00 |
| B2-1b |  | Misalignment and pointing error of calibration antenna |  |  | 0 |  |  | 0 |  |  | 0 |  |  | Exp. normal |  |  | 2.00 |  |  | 1 |  |  | 0.00 |  |  | 0.00 |  |  | 0.00 |
| B2-12 |  | Standing wave between SGH and test range antenna |  |  | 0.09 |  |  | 0.09 |  |  | 0.09 |  |  | U-shaped |  |  | 1.41 |  |  | 1 |  |  | 0.06 |  |  | 0.06 |  |  | 0.06 |
| B2-13 |  | Switching uncertainty |  |  | 0.1 |  |  | 0.1 |  |  | 0.1 |  |  | Rectangular |  |  | 1.73 |  |  | 1 |  |  | 0.06 |  |  | 0.06 |  |  | 0.06 |
| Combined standard uncertainty (1σ) (dB) |  |  |  |  |  |  |  | 1.15 | 1.19 | 1.55 |
| Expanded uncertainty (1.96σ – confidence interval of 95 %) (dB) |  |  |  |  |  |  |  | 2.25 | 2.33 | 3.03 |

------------------------------ Next modified section ------------------------------

Table 10.2.7-2: OTA test system specific measurement uncertainty values for the OTA sensitivity in Normal test conditions, FR2

|  | Expanded uncertainty (dB) |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | 24.25 GHz < f ≤ 29.5 GHz |  | 37 GHz < f ≤ 43.5 GHz | 43.5 GHz < f ≤ 48.2 GHz | 52.6 < f ≤ 71 GHz |
| Indoor Anechoic Chamber |  | 2.33 |  | 2.46 |  |  |
| Compact Antenna Test Range |  | 2.25 |  | 2.33 |  | 3.03 |
| Common maximum accepted test system uncertainty |  | 2.4 |  | 2.4 | 3.5 | 3.0 |

------------------------------ Next modified section ------------------------------

Table 10.2.8-2: Test Tolerance values for the OTA sensitivity, Normal test conditions

|  | 24.25 GHz < f ≤ 29.5 GHz | 37 GHz < f ≤ 43.5 GHz | 43.5 GHz < f ≤ 48.2 GHz | 52.6 < f ≤ 71 GHz |
| --- | --- | --- | --- | --- |
| Test Tolerance (dB) | 2.4 | 2.4 | 3.5 | 3.0 |

------------------------------ Next modified section ------------------------------

Table 11.2.3.4-1: CATR MU value derivation for OTA BS output power measurement, FR2

| UID | Uncertainty source | Uncertainty value (dB) |  |  | Distribution of the probability | Divisor based on distribution | ci | Standard uncertainty ui (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 24.25 < f ≤ 29.5 GHz | 37 < f ≤ 43.5 GHz | 52.6 < f ≤ 71 GHz |  | shape |  | 24.25 < f ≤ 29.5 GHz | 37 < f ≤ 43.5 GHz | 52.6 < f ≤ 71 GHz |
| Stage 2: BS measurement |  |  |  |  |  |  |  |  |  |  |
| A2-1a | Misalignment and pointing error of BS (for EIRP) |  | 0.20 |  |  | 0.20 |  |  |  | 0.20 |  |  |  | Exp. normal |  |  |  | 2.00 |  |  |  | 1 |  |  | 0.10 |  | 0.10 |  | 0.10 |  |
| C1-1 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) - high power |  | 0.50 |  |  | 0.70 |  |  |  |  |  |  |  | Gaussian |  |  |  | 1.00 |  |  |  | 1 |  |  | 0.50 |  | 0.70 |  |  |  |
| C1-10 | Uncertainty of the RF power measurement equipment (power meter, power sensor) - high power (EIRP) |  |  |  |  |  |  |  |  |  | 0.98 |  |  |  | Gaussian |  |  |  | 1.00 |  |  |  | 1 |  |  |  |  |  | 0.98 |  |
| A2-2a | Standing wave between BS and test range antenna |  | 0.03 |  |  | 0.03 |  |  |  | 0.21 |  |  |  | U-shaped |  |  |  | 1.41 |  |  |  | 1 |  |  | 0.02 |  | 0.02 |  | 0.15 |  |
| A2-3 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) |  | 0.01 |  |  | 0.01 |  |  |  | 0.00 |  |  |  | Gaussian |  |  |  | 1.00 |  |  |  | 1 |  |  | 0.01 |  | 0.01 |  | 0.00 |  |
| A2-4a | QZ ripple experienced by BS |  | 0.40 |  |  | 0.40 |  |  |  | 0.40 |  |  |  | Gaussian |  |  |  | 1.00 |  |  |  | 1 |  |  | 0.40 |  | 0.40 |  | 0.40 |  |
| A2-12 | Frequency flatness of test system |  | 0.25 |  |  | 0.25 |  |  |  | 0.25 |  |  |  | Gaussian |  |  |  | 1.00 |  |  |  | 1 |  |  | 0.25 |  | 0.25 |  | 0.25 |  |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |
| C1-3 | Uncertainty of the network analyzer |  | 0.30 |  |  | 0.30 |  |  | 0.85 |  |  |  | Gaussian |  |  |  | 1.00 |  |  |  | 1 |  |  |  | 0.30 |  | 0.30 |  | 0.85 |  |
| A2-5a | Mismatch of receiver chain between receiving antenna and measurement receiver |  | 0.43 |  |  | 0.57 |  |  | 0.57 |  |  |  | U-shaped |  |  |  | 1.41 |  |  |  | 1 |  |  |  | 0.30 |  | 0.40 |  | 0.40 |  |
| A2-6 | Insertion loss of receiver chain |  | 0.00 |  |  | 0.00 |  |  | 0.18 |  |  |  | Rectangular |  |  |  | 1.73 |  |  |  | 1 |  |  |  | 0.00 |  | 0.00 |  | 0.10 |  |
| A2-3 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) |  | 0.01 |  |  | 0.01 |  |  | 0.01 |  |  |  | Gaussian |  |  |  | 1.00 |  |  |  | 1 |  |  |  | 0.01 |  | 0.01 |  | 0.01 |  |
| A2-7 | Influence of the calibration antenna feed cable |  | 0.21 |  |  | 0.29 |  |  | 0.29 |  |  |  | U-shaped |  |  |  | 1.41 |  |  |  | 1 |  |  |  | 0.15 |  | 0.21 |  | 0.21 |  |
| C1-4 | Uncertainty of the absolute gain of the reference antenna |  | 0.52 |  |  | 0.52 |  |  | 0.52 |  |  |  | Rectangular |  |  |  | 1.73 |  |  |  | 1 |  |  |  | 0.30 |  | 0.30 |  | 0.30 |  |
| A2-8 | Misalignment positioning system |  | 0.00 |  |  | 0.00 |  |  | 0.00 |  |  |  | Exp. normal |  |  |  | 2.00 |  |  |  | 1 |  |  |  | 0.00 |  | 0.00 |  | 0.00 |  |
| A2-1b | Misalignment and pointing error of calibration antenna (for EIRP) |  | 0.00 |  |  | 0.00 |  |  | 0.00 |  |  |  | Exp. normal |  |  |  | 2.00 |  |  |  | 1 |  |  |  | 0.00 |  | 0.00 |  | 0.00 |  |
| A2-9 | Rotary joints |  | 0.00 |  |  | 0.00 |  |  | 0.00 |  |  |  | U-shaped |  |  |  | 1.41 |  |  |  | 1 |  |  |  | 0.00 |  | 0.00 |  | 0.00 |  |
| A2-2b | Standing wave between calibration antenna and test range antenna |  | 0.09 |  |  | 0.09 |  |  | 0.09 |  |  |  | U-shaped |  |  |  | 1.41 |  |  |  | 1 |  |  |  | 0.06 |  | 0.06 |  | 0.06 |  |
| A2-4b | QZ ripple experienced by calibration antenna |  | 0.01 |  |  | 0.01 |  |  | 0.01 |  |  |  | Gaussian |  |  |  | 1.00 |  |  |  | 1 |  |  |  | 0.01 |  | 0.01 |  | 0.01 |  |
| A2-11 | Switching uncertainty |  |  |  |  |  |  |  | 0.43 |  |  |  | Rectangular |  |  |  | 1.73 |  |  |  | 1 |  |  |  |  |  |  |  | 0.25 |  |
| Combined standard uncertainty (1σ) (dB) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.88 |  | 1.05 |  | 1.52 |  |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.73 |  | 2.07 |  | 2.98 |  |
| TRP summation error |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.20 |  | 1.20 |  | 1.20 |  |
| Total MU |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.11 |  | 2.39 |  | 3.21 |  |

------------------------------ Next modified section ------------------------------

Table 11.2.5.4-1: Reverberation chamber MU value derivation for OTA BS output power, FR2

| UID | Uncertainty source | Uncertainty value (dB) |  |  | Distribution of the probability | Divisor based on distribution | ci | Standard uncertainty ui (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 24.25 < f ≤ 29.5 GHz | 37 < f ≤ 43.5 GHz | 52.6 < f ≤ 71 GHz |  | shape |  | 24.25 < f ≤ 29.5 GHz | 37 < f ≤ 43.5 GHz | 52.6 < f ≤ 71 GHz |
| Stage 2: BS measurement |  |  |  |  |  |  |  |  |  |  |
| C1-1 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) - high power (EIRP, TRP) | 0.50 | 0.70 |  | Gaussian | 1.00 | 1 | 0.50 | 0.70 |  |
| C1-10 | Uncertainty of the RF power measurement equipment (power meter, power sensor) - high power (EIRP) |  |  |  |  | 0.98 |  |  | Gaussian |  |  | 1 |  |  | 1 |  |  |  |  |  | 0.98 |  |
| A6-1 | Impedance mismatch in the receiving chain | 0.20 | 0.20 | 0.20 | U-shaped | 1.41 | 1 | 0.14 | 0.14 |  |
| A6-2 | Random uncertainty | 0.10 | 0.10 | 0.10 | Rectangular | 1.73 | 1 | 0.06 | 0.06 |  |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |
| A6-3 | Reference antenna radiation efficiency | 0.30 | 0.30 |  | 0.30 |  |  | Gaussian |  |  | 1.00 |  |  | 1 |  |  | 0.30 |  | 0.30 |  | 0.30 |  |
| A6-4 | Mean value estimation of reference antenna mismatch efficiency | 0.27 | 0.27 |  | 0.27 |  |  | Gaussian |  |  | 1.00 |  |  | 1 |  |  | 0.27 |  | 0.27 |  | 0.27 |  |
| C1-3 | Uncertainty of the network analyzer | 0.30 | 0.30 |  | 0.85 |  |  | Gaussian |  |  | 1.00 |  |  | 1 |  |  | 0.30 |  | 0.30 |  | 0.85 |  |
| A6-5 | Influence of the reference antenna feed cable | 0.20 | 0.20 |  | 0.20 |  |  | Gaussian |  |  | 1.00 |  |  | 1 |  |  | 0.20 |  | 0.20 |  | 0.20 |  |
| A6-6 | Mean value estimation of transfer function | 0.27 | 0.27 |  | 0.27 |  |  | Gaussian |  |  | 1.00 |  |  | 1 |  |  | 0.27 |  | 0.27 |  | 0.27 |  |
| A6-7 | Uniformity of transfer function | 0.50 | 0.50 |  | 0.50 |  |  | Gaussian |  |  | 1.00 |  |  | 1 |  |  | 0.50 |  | 0.50 |  | 0.50 |  |
| Combined standard uncertainty (1σ) (dB) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.94 |  | 1.06 |  | 1.49 |  |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.85 |  | 2.08 |  | 2.93 |  |

------------------------------ Next modified section ------------------------------

Table 11.2.7-2: Test system specific MU values for the OTA BS output power test, Normal test conditions, FR2

|  | Expanded uncertainty ue (dB) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 24.25 < f < 29.5 GHz | 37 < f < 43.5 GHz | 43.5 < f < 48.2 GHz | 52.6 < f ≤ 71 GHz |
| Indoor Anechoic Chamber |  |  | - | - |
| Compact Antenna Test Range | 2.11 | 2.39 | - | 3.21 |
| Near Field Test Range |  |  | - | - |
| Reverberation chamber | 1.85 | 2.08 | - | 2.93 |
| Plane Wave Synthesizer |  |  | - | - |
| Common maximum accepted test system uncertainty | 2.1 | 2.4 | 2.6 | 3.2 |
| NOTE: MU estimation for 43.5 < f < 48.2 GHz was derived based on the linear approximation (based on MU values for lower frequency ranges). MU extrapolation approach was used instead of the typical derivation of the Expanded MU based on the MU budget calculations, as in case of lower frequency ranges. |  |  |  |  |

------------------------------ Next modified section ------------------------------

Table 11.2.8-2: Test Tolerance values for the OTA BS output power, Normal test conditions, FR2

|  | 24.25 < f < 29.5 GHz | 37 < f < 43.5 GHz | 43.5 < f < 48.2 GHz | 52.6 < f ≤ 71 GHz |
| --- | --- | --- | --- | --- |
| Test Tolerance (dB) | 2.1 | 2.4 | 2.6 | 3.2 |

------------------------------ Next modified section ------------------------------

Table 11.3.3.4-1: CATR MU value derivation for the EIRP measurement of the absolute OTA ACLR, FR2

| UID | Uncertainty source |  | Uncertainty value (dB) |  |  | Distribution of the probability | Divisor based on distribution | ci | Standard uncertainty ui (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 24.25 < f ≤ 29.5 GHz | 37 < f ≤ 43.5 GHz | 52.6 < f ≤ 71 GHz |  | shape |  | 24.25 < f ≤ 29.5 GHz | 37 < f ≤ 43.5 GHz | 52.6 < f ≤ 71 GHz |
| Stage 2: BS measurement |  |  |  |  |  |  |  |  |  |  |  |
| A2-1a | Misalignment and pointing error of BS (for EIRP) |  |  |  |  | 0.20 |  |  |  | 0.20 |  |  |  | 0.20 |  |  |  | Exp. normal |  |  |  | 2.00 |  |  |  | 1 |  | 0.10 |  | 0.10 |  | 0.10 |  |  |
| C1-7 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter, mixer) - low power (UEM, absolute ACLR) |  |  |  |  | 0.90 |  |  |  | 0.90 |  |  |  | 2.0 |  |  |  | Gaussian |  |  |  | 1.00 |  |  |  | 1 |  | 0.90 |  | 0.90 |  | 2.0 |  |  |
| A2-2a | Standing wave between BS and test range antenna |  |  |  |  | 0.03 |  |  |  | 0.03 |  |  |  | 0.21 |  |  |  | U-shaped |  |  |  | 1.41 |  |  |  | 1 |  | 0.02 |  | 0.02 |  | 0.15 |  |  |
| A2-3 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) |  |  |  |  | 0.01 |  |  |  | 0.01 |  |  |  | 0.00 |  |  |  | Gaussian |  |  |  | 1.00 |  |  |  | 1 |  | 0.01 |  | 0.01 |  | 0.00 |  |  |
| A2-4a | QZ ripple experienced by BS |  |  |  |  | 0.40 |  |  |  | 0.40 |  |  |  | 0.40 |  |  |  | Gaussian |  |  |  | 1.00 |  |  |  | 1 |  | 0.40 |  | 0.40 |  | 0.40 |  |  |
| A2-12 | Frequency flatness of test system |  |  |  |  | 0.25 |  |  |  | 0.25 |  |  |  | 0.25 |  |  |  | Gaussian |  |  |  | 1.00 |  |  |  | 1 |  | 0.25 |  | 0.25 |  | 0.25 |  |  |
| A2-19 | Uncertainty of the LNA (FR2 only) |  |  |  |  |  |  |  |  |  |  | 0.16 |  |  |  | Gaussian |  |  |  | 1.00 |  |  |  | 1 |  |  | 0.00 |  | 0.00 |  | 0.16 |  |  |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |  |
| C1-3 | Uncertainty of the network analyzer |  |  |  | 0.30 |  |  |  | 0.30 |  |  |  | 0.85 |  |  |  | Gaussian |  |  |  | 1.00 |  |  |  | 1 |  |  | 0.30 |  | 0.30 |  | 0.85 |  |  |
| A2-5b | Mismatch of receiver chain for low power |  |  |  | 0.72 |  |  |  | 0.72 |  |  |  | 0.72 |  |  |  | U-shaped |  |  |  | 1.41 |  |  |  | 1 |  |  | 0.51 |  | 0.51 |  | 0.51 |  |  |
| A2-6 | Insertion loss in receiver chain |  |  |  | 0.00 |  |  |  | 0.00 |  |  |  | 0.18 |  |  |  | Rectangular |  |  |  | 1.73 |  |  |  | 1 |  |  | 0.00 |  | 0.00 |  | 0.10 |  |  |
| C2-3 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) |  |  |  | 0.01 |  |  |  | 0.01 |  |  |  | 0.01 |  |  |  | Gaussian |  |  |  | 1.00 |  |  |  | 1 |  |  | 0.01 |  | 0.01 |  | 0.01 |  |  |
| A2-7 | Influence of the calibration antenna feed cable |  |  |  | 0.21 |  |  |  | 0.29 |  |  |  | 0.29 |  |  |  | U-shaped |  |  |  | 1.41 |  |  |  | 1 |  |  | 0.15 |  | 0.21 |  | 0.21 |  |  |
| C1-4 | Uncertainty of the absolute gain of the reference antenna |  |  |  | 0.52 |  |  |  | 0.52 |  |  |  | 0.52 |  |  |  | Rectangular |  |  |  | 1.73 |  |  |  | 1 |  |  | 0.30 |  | 0.30 |  | 0.30 |  |  |
| A2-8 | Misalignment positioning system |  |  |  | 0.00 |  |  |  | 0.00 |  |  |  | 0.00 |  |  |  | Exp. normal |  |  |  | 2.00 |  |  |  | 1 |  |  | 0.00 |  | 0.00 |  | 0.00 |  |  |
| A2-1b | Misalignment and pointing error of calibration antenna (for EIRP) |  |  |  | 0.00 |  |  |  | 0.00 |  |  |  | 0.00 |  |  |  | Exp. normal |  |  |  | 2.00 |  |  |  | 1 |  |  | 0.00 |  | 0.00 |  | 0.00 |  |  |
| A2-9 | Rotary joints |  |  |  | 0.00 |  |  |  | 0.00 |  |  |  | 0.00 |  |  |  | U-shaped |  |  |  | 1.41 |  |  |  | 1 |  |  | 0.00 |  | 0.00 |  | 0.00 |  |  |
| A2-2b | Standing wave between calibration antenna and test range antenna |  |  |  | 0.09 |  |  |  | 0.09 |  |  |  | 0.09 |  |  |  | U-shaped |  |  |  | 1.41 |  |  |  | 1 |  |  | 0.06 |  | 0.06 |  | 0.06 |  |  |
| A2-4b | QZ ripple experienced by calibration antenna |  |  |  | 0.01 |  |  |  | 0.01 |  |  |  | 0.01 |  |  |  | Gaussian |  |  |  | 1.00 |  |  |  | 1 |  |  | 0.01 |  | 0.01 |  | 0.01 |  |  |
| A2-11 | Switching uncertainty |  |  |  | 0.10 |  |  |  | 0.10 |  |  |  | 0.43 |  |  |  | Rectangular |  |  |  | 1.73 |  |  |  | 1 |  |  | 0.06 |  | 0.06 |  | 0.25 |  |  |
|  |  | Combined standard uncertainty (1σ) (dB) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.23 |  |  | 1.24 | 2.34 |
|  |  | Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.41 |  |  | 2.43 | 4.58 |
|  |  | TRP summation error |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.20 |  |  | 1.20 | 1.20 |
|  |  | Total MU |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.69 |  |  | 2.71 | 4.74 |

Table 11.3.3.4-2: CATR MU value derivation for the EIRP measurement of the relative OTA ACLR, FR2

| UID | Uncertainty source | Uncertainty value (dB) |  |  | Distribution of the probability | Divisor based on distribution | ci | Standard uncertainty ui (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 24.25 < f ≤ 29.5 GHz | 37 < f ≤ 43.5 GHz | 52.6 < f ≤ 71 GHz |  | shape |  | 24.25 < f ≤ 29.5 GHz | 37 < f ≤ 43.5 GHz | 52.6 < f ≤ 71 GHz |
| Stage 2: BS measurement |  |  |  |  |  |  |  |  |  |  |
| A2-1a | Misalignment and pointing error of BS (for EIRP) | 0.20 | 0.20 | 0.20 | Exp. normal | 2.00 | 1 | 0.10 | 0.10 | 0.10 |
| C1-8 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter, mixer) - relative (ACLR) | 0.75 | 0.90 | 2.0 | Gaussian | 1.00 | 1 | 0.75 | 0.90 | 2.0 |
| A2-4a | QZ ripple experienced by BS | 0.40 | 0.40 | 0.40 | Gaussian | 1.00 | 1 | 0.40 | 0.40 | 0.40 |
| A2-12 | Frequency flatness of test system | 0.25 | 0.25 | 0.25 | Gaussian | 1.00 | 1 | 0.25 | 0.25 | 0.25 |
| A2-19 | Uncertainty of the LNA (FR2 only) |  |  | 0.16 | Gaussian | 1.00 | 1 | 0.00 | 0.00 | 0.16 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |
| C1-3 | Uncertainty of the network analyzer | 0.30 | 0.30 | 0.85 | Gaussian | 1.00 | 1 | 0.30 | 0.30 | 0.85 |
| A2-5a | Mismatch of receiver chain between receiving antenna and measurement receiver | 0.43 | 0.57 | 0.57 | U-shaped | 1.41 | 1 | 0.30 | 0.40 | 0.40 |
| A2-6 | Insertion loss in receiver chain | 0.00 | 0.00 | 0.18 | Rectangular | 1.73 | 1 | 0.00 | 0.00 | 0.10 |
| A2-11 | Switching uncertainty | 0.10 | 0.10 | 0.43 | Rectangular | 1.73 | 0 | 0.06 | 0.06 | 0.25 |
| Combined standard uncertainty (1σ) (dB) |  |  |  |  |  |  |  | 0.99 | 1.14 | 2.28 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB) |  |  |  |  |  |  |  | 1.94 | 2.23 | 4.48 |
| TRP summation error |  |  |  |  |  |  |  | 1.20 | 1.20 | 1.20 |
| Total MU |  |  |  |  |  |  |  | 2.28 | 2.54 | 4.63 |

------------------------------ Next modified section ------------------------------

Table 11.3.5.4-1: Reverberation chamber MU value derivation for absolute ACLR measurement, FR2

| UID | Uncertainty source | Uncertainty value (dB) |  |  | Distribution of the probability | Divisor based on distribution | ci | Standard uncertainty ui (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 24.25 < f ≤ 29.5 GHz | 37 < f ≤ 43.5 GHz | 52.6 < f ≤ 71 GHz |  | shape |  | 24.25 < f ≤ 29.5 GHz | 37 < f ≤ 43.5 GHz | 52.6 < f ≤ 71 GHz |
| Stage 2: BS measurement |  |  |  |  |  |  |  |  |  |  |
| C1-7 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter, mixer) - low power (UEM, absolute ACLR) | 0.90 | 0.90 | 2.0 | Gaussian | 1.00 | 1 | 0.90 | 0.90 | 2.0 |
| A6-1 | Impedance mismatch in the receiving chain | 0.20 | 0.20 | 0.20 | U-shaped | 1.41 | 1 | 0.14 | 0.14 | 0.14 |
| A6-2 | Random uncertainty | 0.10 | 0.10 | 0.10 | Rectangular | 1.73 | 1 | 0.06 | 0.06 | 0.06 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |
| A6-3 | Reference antenna radiation efficiency | 0.30 | 0.30 | 0.30 | Gaussian | 1.00 | 1 | 0.30 | 0.30 | 0.30 |
| A6-4 | Mean value estimation of reference antenna mismatch efficiency | 0.27 | 0.27 | 0.27 | Gaussian | 1.00 | 1 | 0.27 | 0.27 | 0.27 |
| C1-3 | Uncertainty of the network analyzer | 0.30 | 0.30 | 0.85 | Gaussian | 1.00 | 1 | 0.30 | 0.30 | 0.85 |
| A6-5 | Influence of the reference antenna feed cable | 0.20 | 0.20 | 0.20 | Gaussian | 1.00 | 1 | 0.20 | 0.20 | 0.20 |
| A6-6 | Mean value estimation of transfer function | 0.27 | 0.27 | 0.27 | Gaussian | 1.00 | 1 | 0.27 | 0.27 | 0.27 |
| A6-7 | Uniformity of transfer function | 0.50 | 0.50 | 0.50 | Gaussian | 1.00 | 1 | 0.50 | 0.50 | 0.50 |
| Combined standard uncertainty (1σ) (dB) |  |  |  |  |  |  |  | 1.20 | 1.20 | 2.30 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB) |  |  |  |  |  |  |  | 2.36 | 2.36 | 4.50 |

Table 11.3.5.4-2: Reverberation chamber MU value derivation for relative ACLR measurement, FR2

| UID | Uncertainty source |  | Uncertainty value (dB) |  |  | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 24.25 < f ≤ 29.5 GHz | 37 < f ≤ 43.5 GHz | 52.6 < f ≤ 71 GHz |  |  |  | 24.25 < f ≤ 29.5 GHz | 37 < f ≤ 43.5 GHz | 52.6 < f ≤ 71 GHz |
| Stage 2: BS measurement |  |  |  |  |  |  |  |  |  |  |  |
| C1-8 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) - relative (ACLR) |  | 0.75 | 0.90 | 2.0 | Gaussian | 1.00 | 1 | 0.75 | 0.90 | 2.0 |
| A6-1 | Impedance mismatch in the receiving chain |  | 0.20 | 0.20 | 0.20 | U-shaped | 1.41 | 1 | 0.14 | 0.14 | 0.14 |
| A6-2 | Random uncertainty |  | 0.10 | 0.10 | 0.10 | Rectangular | 1.73 | 1 | 0.06 | 0.06 | 0.06 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |  |
| A6-3 | Reference antenna radiation efficiency |  |  | 0.30 |  | 0.30 |  | 0.30 |  | Gaussian |  | 1.00 |  | 1 |  | 0.30 |  | 0.30 |  | 0.30 |  |  |
| A6-4 | Mean value estimation of reference antenna mismatch efficiency |  |  | 0.27 |  | 0.27 |  | 0.27 |  | Gaussian |  | 1.00 |  | 1 |  | 0.27 |  | 0.27 |  | 0.27 |  |  |
| C1-3 | Uncertainty of the network analyzer |  |  | 0.30 |  | 0.30 |  | 0.85 |  | Gaussian |  | 1.00 |  | 1 |  | 0.30 |  | 0.30 |  | 0.85 |  |  |
| A6-5 | Influence of the reference antenna feed cable |  |  | 0.20 |  | 0.20 |  | 0.20 |  | Gaussian |  | 1.00 |  | 1 |  | 0.20 |  | 0.20 |  | 0.20 |  |  |
| A6-6 | Mean value estimation of transfer function |  |  | 0.27 |  | 0.27 |  | 0.27 |  | Gaussian |  | 1.00 |  | 1 |  | 0.27 |  | 0.27 |  | 0.27 |  |  |
| A6-7 | Uniformity of transfer function |  |  | 0.50 |  | 0.50 |  | 0.50 |  | Gaussian |  | 1.00 |  | 1 |  | 0.50 |  | 0.50 |  | 0.50 |  |  |
|  |  | Combined standard uncertainty (1σ) (dB) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.10 |  |  | 1.20 | 2.30 |
|  |  | Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.15 |  |  | 2.36 | 4.50 |

------------------------------ Next modified section ------------------------------

Table 11.3.7-3: Test system specific MU values for the absolute OTA ACLR, Normal test conditions, FR2

|  | Expanded uncertainty ue (dB) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 24.25 < f < 29.5 GHz |  | 37 < f < 43.5 GHz | 43.5 < f < 48.2 GHz |  | 52.6 < f ≤ 71 GHz |
| Indoor Anechoic Chamber |  |  |  | - |  |  |
| Compact Antenna Test Range | 2.69 |  | 2.71 | - |  | 4.74 |
| Near Field Test Range |  |  |  | - |  |  |
| Reverberation Chamber | 2.36 |  | 2.36 | - |  | 4.5 |
| Plane Wave Synthesizer |  |  |  | - |  |  |
| Common maximum accepted test system uncertainty | 2.7 |  | 2.7 | 2.9 |  | 4.7 |

Table 11.3.7-4: Test system specific MU values for the relative OTA ACLR, Normal test conditions, FR2

|  | Expanded uncertainty ue (dB) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 24.25 < f < 29.5 GHz | 37 < f < 43.5 GHz | 43.5 < f < 48.2 GHz |  |  | 52.6 < f ≤ 71 GHz |
| Indoor Anechoic Chamber |  | - | - | - |  |  |  |
| Compact Antenna Test Range |  | 2.28 | 2.54 | - |  |  | 4.63 |
| Near Field Test Range |  | - | - | - |  |  |  |
| Reverberation Chamber |  | 2.15 | 2.36 | - |  |  | 4.5 |
| Plane Wave Synthesizer |  | - | - | - |  |  |  |
| Common maximum accepted test system uncertainty |  | 2.3 | 2.6 | 2.8 (NOTE) |  |  | 4.6 |
| NOTE: MU estimation for 43.5 < f < 48.2 GHz was derived based on the linear approximation (based on MU values for lower frequency ranges). MU extrapolation approach was used instead of the typical derivation of the Expanded MU based on the MU budget calculations, as in case of lower frequency ranges. |  |  |  |  |

------------------------------ Next modified section ------------------------------

Table 11.3.8-3: Test Tolerance values for the absolute OTA ACLR, Normal test conditions, FR2

|  | 24.25 < f < 29.5 GHz | 37 < f < 43.5 GHz | 43.5 < f < 48.2 GHz | 52.6 < f ≤ 71 GHz |
| --- | --- | --- | --- | --- |
| Test Tolerance (dB) | 2.7 | 2.7 | 2.9 | 4.7 |

Table 11.3.8-4: Test Tolerance values for the relative OTA ACLR, Normal test conditions, FR2

|  | 24.25 < f < 29.5 GHz | 37 < f < 43.5 GHz | 43.5 < f < 48.2 GHz | 52.6 < f ≤ 71 GHz |
| --- | --- | --- | --- | --- |
| Test Tolerance (dB) | 2.3 | 2.6 | 2.8 | 4.6 |

------------------------------ Next modified section ------------------------------

Table 11.4.3.4-1: CATR MU value derivation for OTA OBUE measurement, FR2

| UID | Uncertainty source |  | Uncertainty value (dB) |  |  | Distribution of the probability | Divisor based on distribution | ci | Standard uncertainty ui (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 24.25 < f ≤ 29.5 GHz | 37 < f ≤ 43.5 GHz | 52.6 < f ≤ 71 GHz |  | shape |  | 24.25 < f ≤ 29.5 GHz | 37 < f ≤ 43.5 GHz | 52.6 < f ≤ 71 GHz |
| Stage 2: BS measurement |  |  |  |  |  |  |  |  |  |  |  |
| A2-18a | Misalignment and pointing error of BS (for TRP) |  |  |  |  | 0.30 |  |  |  | 0.30 |  |  |  | 0.30 |  |  |  | Exp. normal |  |  |  | 2.00 |  |  |  | 1 |  | 0.15 |  | 0.15 |  | 0.15 |  |  |
| C1-7 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter, mixer) - low power (UEM, absolute ACLR) |  |  |  |  | 0.90 |  |  |  | 0.90 |  |  |  | 2.0 |  |  |  | Gaussian |  |  |  | 1.00 |  |  |  | 1 |  | 0.90 |  | 0.90 |  | 2.0 |  |  |
| A2-2a | Standing wave between BS and test range antenna |  |  |  |  | 0.03 |  |  |  | 0.03 |  |  |  | 0.21 |  |  |  | U-shaped |  |  |  | 1.41 |  |  |  | 1 |  | 0.02 |  | 0.02 |  | 0.15 |  |  |
| A2-3 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) |  |  |  |  | 0.01 |  |  |  | 0.01 |  |  |  | 0.00 |  |  |  | Gaussian |  |  |  | 1.00 |  |  |  | 1 |  | 0.01 |  | 0.01 |  | 0.00 |  |  |
| A2-4a | QZ ripple experienced by BS |  |  |  |  | 0.40 |  |  |  | 0.40 |  |  |  | 0.40 |  |  |  | Gaussian |  |  |  | 1.00 |  |  |  | 1 |  | 0.40 |  | 0.40 |  | 0.40 |  |  |
| A2-12 | Frequency flatness of test system |  |  |  |  | 0.25 |  |  |  | 0.25 |  |  |  | 0.25 |  |  |  | Gaussian |  |  |  | 1.00 |  |  |  | 1 |  | 0.25 |  | 0.25 |  | 0.25 |  |  |
| A2-19 | Uncertainty of the LNA (FR2 only) |  |  |  |  |  |  |  |  |  |  | 0.16 |  |  |  | Gaussian |  |  |  | 1.00 |  |  |  | 1 |  |  | 0.00 |  | 0.00 |  | 0.16 |  |  |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |  |
| C1-3 | Uncertainty of the Network Analyzer |  |  |  | 0.30 |  |  |  | 0.30 |  |  |  | 0.85 |  |  |  | Gaussian |  |  |  | 1.00 |  |  |  | 1 |  |  | 0.30 |  | 0.30 |  | 0.85 |  |  |
| A2-5b | Mismatch of receiver chain for low power receiver |  |  |  | 0.72 |  |  |  | 0.72 |  |  |  | 0.72 |  |  |  | U-shaped |  |  |  | 1.41 |  |  |  | 1 |  |  | 0.51 |  | 0.51 |  | 0.51 |  |  |
| A2-6 | Insertion loss in receiver chain |  |  |  | 0.00 |  |  |  | 0.00 |  |  |  | 0.18 |  |  |  | Rectangular |  |  |  | 1.73 |  |  |  | 1 |  |  | 0.00 |  | 0.00 |  | 0.10 |  |  |
| A2-3 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) |  |  |  | 0.01 |  |  |  | 0.01 |  |  |  | 0.01 |  |  |  | Gaussian |  |  |  | 1.00 |  |  |  | 1 |  |  | 0.01 |  | 0.01 |  | 0.01 |  |  |
| A2-7 | Influence of the calibration antenna feed cable |  |  |  | 0.21 |  |  |  | 0.29 |  |  |  | 0.29 |  |  |  | U-shaped |  |  |  | 1.41 |  |  |  | 1 |  |  | 0.15 |  | 0.21 |  | 0.21 |  |  |
| C1-4 | Uncertainty of the absolute gain of the reference antenna |  |  |  | 0.52 |  |  |  | 0.52 |  |  |  | 0.52 |  |  |  | Rectangular |  |  |  | 1.73 |  |  |  | 1 |  |  | 0.30 |  | 0.30 |  | 0.30 |  |  |
| A2-8 | Misalignment positioning system |  |  |  | 0.00 |  |  |  | 0.00 |  |  |  | 0.00 |  |  |  | Exp. normal |  |  |  | 2.00 |  |  |  | 1 |  |  | 0.00 |  | 0.00 |  | 0.00 |  |  |
| A2-1b | Misalignment and pointing error of calibration antenna (for EIRP) |  |  |  | 0.00 |  |  |  | 0.00 |  |  |  | 0.00 |  |  |  | Exp. normal |  |  |  | 2.00 |  |  |  | 1 |  |  | 0.00 |  | 0.00 |  | 0.00 |  |  |
| A2-9 | Rotary joints |  |  |  | 0.00 |  |  |  | 0.00 |  |  |  | 0.00 |  |  |  | U-shaped |  |  |  | 1.41 |  |  |  | 1 |  |  | 0.00 |  | 0.00 |  | 0.00 |  |  |
| A2-2b | Standing wave between calibration antenna and test range antenna |  |  |  | 0.09 |  |  |  | 0.09 |  |  |  | 0.09 |  |  |  | U-shaped |  |  |  | 1.41 |  |  |  | 1 |  |  | 0.06 |  | 0.06 |  | 0.06 |  |  |
| A2-4b | QZ ripple experienced by calibration antenna |  |  |  | 0.01 |  |  |  | 0.01 |  |  |  | 0.01 |  |  |  | Gaussian |  |  |  | 1.00 |  |  |  | 1 |  |  | 0.01 |  | 0.01 |  | 0.01 |  |  |
| A2-11 | Switching uncertainty |  |  |  | 0.10 |  |  |  | 0.10 |  |  |  | 0.43 |  |  |  | Rectangular |  |  |  | 1.73 |  |  |  | 1 |  |  | 0.06 |  | 0.06 |  | 0.25 |  |  |
|  |  | Combined standard uncertainty (1σ) (dB) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.23 |  |  | 1.24 | 2.34 |
|  |  | Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.42 |  |  | 2.44 | 4.59 |
|  |  | TRP summation error |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.20 |  |  | 1.20 | 1.20 |
|  |  | Total MU |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.70 |  |  | 2.72 | 4.74 |

------------------------------ Next modified section ------------------------------

Table 11.4.5.4-1: Reverberation chamber MU value derivation for OTA OBUE measurement, FR2

| UID | Uncertainty source |  | Uncertainty value (dB) |  |  | Distribution of the probability | Divisor based on distribution | ci | Standard uncertainty ui (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 24.25 < f ≤ 29.5 GHz | 37 < f ≤ 43.5 GHz | 52.6 < f ≤ 71 GHz |  |  |  |  | shape |  |  | 24.25 < f ≤ 29.5 GHz |  | 37 < f ≤ 43.5 GHz | 52.6 < f ≤ 71 GHz |
| Stage 2: BS measurement |  |  |  |  |  |  |  |  |  |  |  |
| C1-7 |  | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter, mixer) - low power (UEM, absolute ACLR) |  |  | 0.90 | 0.90 | 2.0 |  |  | Gaussian |  | 1.00 |  | 1 | 0.90 |  | 0.90 | 2.0 |
| A6-1 |  | Impedance mismatch in the receiving chain |  |  | 0.20 | 0.20 | 0.20 |  |  | U-shaped |  | 1.41 |  | 1 | 0.14 |  | 0.14 | 0.14 |
| A6-2 |  | Random uncertainty |  |  | 0.10 | 0.10 | 0.10 |  |  | Rectangular |  | 1.73 |  | 1 | 0.06 |  | 0.06 | 0.06 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |  |
| A6-3 |  | Reference antenna radiation efficiency |  |  | 0.30 | 0.30 | 0.30 |  |  | Gaussian |  | 1.00 |  | 1 | 0.30 |  | 0.30 | 0.30 |
| A6-4 |  | Mean value estimation of reference antenna mismatch efficiency |  |  | 0.27 | 0.27 | 0.27 |  |  | Gaussian |  | 1.00 |  | 1 | 0.27 |  | 0.27 | 0.27 |
| C1-3 |  | Uncertainty of the network analyzer |  |  | 0.30 | 0.30 | 0.85 |  |  | Gaussian |  | 1.00 |  | 0.3 | 0.30 |  | 0.30 | 0.85 |
| A6-5 |  | Influence of the reference antenna feed cable |  |  | 0.20 | 0.20 | 0.20 |  |  | Gaussian |  | 1.00 |  | 1 | 0.20 |  | 0.20 | 0.20 |
| A6-6 |  | Mean value estimation of transfer function |  |  | 0.27 | 0.27 | 0.27 |  |  | Gaussian |  | 1.00 |  | 1 | 0.27 |  | 0.27 | 0.27 |
| A6-7 |  | Uniformity of transfer function |  |  | 0.50 | 0.50 | 0.50 |  |  | Gaussian |  | 1.00 |  | 1 | 0.50 |  | 0.50 | 0.50 |
|  |  |  | Combined standard uncertainty (1σ) (dB) |  |  |  |  |  |  |  |  |  |  |  | 1.20 |  | 1.20 | 2.30 |
|  |  |  | Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB) |  |  |  |  |  |  |  |  |  |  |  | 2.36 |  | 2.36 | 4.50 |

------------------------------ Next modified section ------------------------------

Table 11.4.7-2: Test system specific MU values for the OTA OBUE measurement, FR2

|  | Expanded uncertainty ue (dB) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 24.25 < f < 29.5 GHz | 37 < f < 43.5 GHz | 43.5 < f < 48.2 GHz | 52.6 < f ≤ 71 GHz |
| Indoor Anechoic Chamber | - | - | - |  |
| Compact Antenna Test Range | 2.70 | 2.72 | - | 4.74 |
| Near Field Test Range | - | - | - |  |
| Reverberation Chamber | 2.36 | 2.36 | - | 4.5 |
| Plane Wave Synthesizer | - | - | - |  |
| Common maximum accepted test system uncertainty | 2.7 | 2.7 | 2.9 | 4.7 |

------------------------------ Next modified section ------------------------------

Table 11.4.8-2: Test Tolerance values for the OTA OBUE, Normal test conditions, FR2

|  | 24.25 < f < 29.5 GHz | 37 < f < 43.5 GHz | 43.5 < f < 48.2 GHz | 52.6 < f ≤ 71 GHz |
| --- | --- | --- | --- | --- |
| Test Tolerance (dB) | 2.7 | 2.7 | 2.9 | 4.7 |

------------------------------ Next modified section ------------------------------

Table 12.2.4.3-2: Reverberation Chamber value derivation for TX spurious emissions, 43.5 GHz – 142 GHz

| UID | Uncertainty source |  |  | Uncertainty value (dB) |  |  | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 43.5 GHz< f ≤ 60 GHz | 71 < f ≤ 110 GHz | 110 < f ≤ 142 GHz |  |  |  | 43.5 GHz< f ≤ 60 GHz | 60< f ≤ 110 GHz | 110 < f ≤ 142 GHz |
| Stage 2: BS measurement |  |  |  |  |  |  |  |  |  |  |  |  |
| C1-7 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) - low power (UEM, absolute ACLR) |  |  | 0.60 | 2.00 | 2.30 | Gaussian | 1.00 | 1 | 0.60 | 2.00 | 2.30 |
| A6-1 | Impedance mismatch in the receiving chain |  |  | 0.45 | 0.45 | 0.45 | U-shaped | 1.41 | 1 | 0.32 | 0.32 | 0.32 |
| A6-2 | Random uncertainty |  |  | 0.10 | 0.10 | 0.10 | Rectangular | 1.73 | 1 | 0.06 | 0.06 | 0.06 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |  |  |
| A6-3 | Reference antenna radiation efficiency |  |  | 0.50 |  | 0.50 |  |  | 0.50 |  | Normal |  | 1.00 |  | 1 |  | 0.50 |  | 0.50 |  | 0.50 |  |
| A6-4 | Mean value estimation of reference antenna mismatch efficiency |  |  | 0.27 |  | 0.27 |  |  | 0.27 |  | Normal |  | 1.00 |  | 1 |  | 0.27 |  | 0.27 |  | 0.27 |  |
| C1-3 | Uncertainty of the network analyzer |  |  | 0.30 |  | 0.80 |  |  | 1.00 |  | Gaussian |  | 1.00 |  | 1 |  | 0.30 |  | 0.80 |  | 1.00 |  |
| A6-5 | Influence of the reference antenna feed cable |  |  | 0.20 |  | 0.20 |  |  | 0.20 |  | Normal |  | 1.00 |  | 1 |  | 0.20 |  | 0.20 |  | 0.20 |  |
| A6-6 | Mean value estimation of transfer function |  |  | 0.27 |  | 0.27 |  |  | 0.27 |  | Normal |  | 1.00 |  | 1 |  | 0.27 |  | 0.27 |  | 0.27 |  |
| A6-7 | Uniformity of transfer function |  |  | 1.50 |  | 1.50 |  |  | 1.50 |  | Normal |  | 1.00 |  | 1 |  | 1.50 |  | 1.50 |  | 1.50 |  |
|  |  |  |  |  |  |  | Combined standard uncertainty (1σ) (dB) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.80 | 2.73 | 3.01 |
|  |  |  |  |  |  |  | Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.53 | 5.34 | 5.91 |

------------------------------ Next modified section ------------------------------

Table 12.2.5-2: Spurious emissions MU values – FR2

|  | Expanded uncertainty (dB) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 30 MHz< f ≤ 6 GHz | 6 GHz< f 43.5 GHz | 43.5 GHz< f ≤ 60 GHz | 60 < f ≤ 110 GHz | 110 < f ≤ 142 GHz |
| General Directional Chamber | x | x | 4.94 |  |  |
| Compact Antenna Test Range | x | x | 4.96 |  |  |
| Reverberation Chamber | x | x | 3.53 | 5.34 | 5.91 |
| Common maximum accepted test system uncertainty | 2.5 | 2.7 | 5 | 5.30 | 5.90 |

------------------------------ Next modified section ------------------------------

Table C.2-2: Test equipment uncertainty values for FR2

| UID | Instrument | Standard uncertainty σ (dB) |  |  | Probability distribution |
| --- | --- | --- | --- | --- | --- |
|  |  | 24.25 < f ≤ 29.5 GHz | 37 < f ≤ 43.5 GHz | 52.6 < f ≤ 71 GHz |  |
| C1-1 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) - high power |  |  |  | 0.50 |  |  |  |  | 0.70 |  |  |  | Gaussian |  |  |
| C1-2 | RF signal generator |  |  |  | 0.90 |  |  |  |  | 0.90 |  |  |  | Gaussian |  |  |
| C1-3 | Network Analyzer |  |  |  | 0.30 |  |  |  |  | 0.30 | 0.85 |  |  | Gaussian |  |  |
| C1-7 | RF power measurement equipment (e.g. spectrum analyzer, power meter, mixer) - low power (UEM) |  |  |  | 0.90 |  |  |  |  | 0.90 | 2.0 |  |  | Gaussian |  |  |
| C1-8 | RF power measurement equipment (e.g. spectrum analyzer, power meter) - relative (ACLR) |  |  |  | 0.75 |  |  |  |  | 0.90 | 2.0 |  |  | Gaussian |  |  |
| C1-9 | RF power measurement equipment (e.g. spectrum analyzer, power meter) - relative (ACLR) |  |  |  | 1.25 |  |  |  |  | 1.45 | 2.61 |  |  | Gaussian |  |  |
| C1-10 |  | Uncertainty of the RF power measurement equipment (power meter, power sensor) - high power (EIRP) |  |  |  |  |  |  | 0.98 |  |  |  | Gaussian |  |  |

The following uncertainty distribution and standard uncertainty (σ) value for the reference antenna derived as the maximum of companies' proposals are adopted in all test methods to calculate the uncertainty budget.

Table C.2-3: Reference antenna uncertainty value

| UID | Instrument | Use case | Standard uncertainty σ (dB) |  |  | Probability |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | f ≤ 3 GHz | 3 GHz < f ≤ 6 GHz | 18 < f ≤142 GHz | distribution |
| C1-4 | Reference antenna | Calibration stage | 0.29 | 0.25 | 0.3 | Rectangular |

Table C.2-4: Void

Table C.2-5: Test equipment uncertainty values for out-of-band

| UID | Instrument |  |  | Standard uncertainty σ (dB) |  |  |  |  |  | Probability distribution |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 18<f≤26.5 GHz | 26.5<f ≤40 GHz | 40<f ≤60 GHz | 60<f ≤110 GHz | 110<f ≤142 GHz |  |
| C1-3 | Network Analyzer | 0.30 | 0.30 | 0.30 | 0.80 | 1.00 | Gaussian |
| C1-7 | RF power measurement equipment (e.g. spectrum analyzer, power meter) - low power (UEM) | 0.52 | 0.84 | 0.60 | 2.00 | 2.30 | Gaussian |

------------------------------ End of modified section ------------------------------