3GPP TSG-RAN4 Meeting #108	R4-2313996

# Toulouse, France, 21-25 August 2023

Title: 	WF on SAN RF

Agenda Item:	8.26.3

Source: 	Ericsson

Document for:	Approval

### EISREFSENS_50M definition

Agreement:

- BW for EISREFSENS_50M for Ka-band SAN is 66RB, i.e. 66*12*60*1000Hz.

- -1dB SNR can be reused for EIS for Ka-band SAN

### 2- FRC

Agreement:

- The G-FR2-A1-1, G-FR2-A1-2 and G-FR2-A1-3 for FR2-1 TN BS can be reused for Ka-band SAN

### 3- SAN Noise Figure

Agreement:

- GEO and LEO: 3.5 dB

5- TS 38.108 Work split

| Sub-clause | Leading company |
| --- | --- |
| 4.3 Requirement reference points | Ericsson |
| 4.6 Applicability of minimum requirements | Ericsson |
| 5 Operating bands and channel arrangement | THALES |
| 9.2 Radiated transmit power | THALES |
| 9.3 OTA Satellite Access Node output power | THALES |
| 9.4 OTA output power dynamics | Ericsson |
| 9.6 OTA transmitted signal quality | THALES |
| 9.7 OTA unwanted emissions | THALES |
| 10.1 General | CATT |
| 10.2 OTA sensitivity | CATT |
| 10.3 OTA reference sensitivity level | CATT |
| 10.4 OTA dynamic range | CATT |
| 10.5 OTA in-band selectivity and blocking | ZTE |
| 10.6 OTA out-of-band blocking | ZTE |
| 10.9 OTA in-channel selectivity | ZTE |
| New sub-clause(s), if any (tbc) | Ericsson |